3GPP TSG RAN WG1 #110b-e                              R1-2208460

E-Meeting, October 10th – 19th 2022

Agenda Item:	9.1.1.2

Source:		TCL Communication Ltd.

Title:			Discussion on two TAs for multi-DCI based on multi-TRP operation

Document for:	Discussion and Decision

1 Introduction

During RAN#94-e meeting, the Rel-18 WID of MIMO evolution for DL and UL was approved [1]. The following objective is related to this AI.

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation. - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contributions, our opinions on two TAs for multi-DCI based multi-TRP operation are presented.

2 Discussion

2.1 TAG association

The following agreement on the association between TAGs and target UL channels/signals was made in RAN1#110 meeting:

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, downselect one of the options in RAN1#110bis-e: - Option 1: Associate TAG to TCI-state/spatial relation - Option 2: Associate TAG to CORESETPoolIndex - Option 3: Associate TAG to DL RS group. For a UL transmission, UE adopts the TAG associated with the DL RS group to which the PL RS of the UL transmission belongs. - Option 4: Associate TAG to target UL channels/RSs directly for semi-static UL channels/RSs (e.g. P CSI PUCCH, P SRS, CG PUSCH), and further discuss how to associate TAG to dynamic UL channels/RSs(e.g. via associating TAG to CORESETPoolIndex additionally, etc.)

In current spec, a UE can know that which TA indicated by a TAG is used to transmit UL channels/signals according to the CC. After 2 TAs introduced, a CC in a serving cell may have different TAs in multi-DCI based multi-TRP scenario. In this case, the UE cannot know that which TA should be used. However, the UE knows which TRP the UL channel/signal should be transmitted to. In multi-DCI based multi-TRP scenario, CoresetPoolIndex is introduced to distinguish the TRP in a serving cell. Therefore, associating TAG to CoresetPoolIndex is a simple and distinct way.

- Support option 2: Associate TAG to CORESETPoolIndex.

2.2 UL overlapping

The following agreement on UL transmission overlapping was made in RAN1#110 meeting:

Agreement For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes: - whether to introduce scheduling restriction in overlapping part - whether to introduce dropping rules - whether specification impact is needed, or if the issue can be handled via implementation - whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18)

In current spec, a UE transmitting UL signals to different TRPs has the same time in advance due to 1 TA. In this case, UL signals from a UE to different TRPs in transmission time do not overlap. However, after introducing 2 TAs, a UE transmitting UL signals to different TRPs has the different time in advance. In this case, UL signals from a UE to different TRPs in transmission time may overlap. For a UE with single Tx, it cannot transmit UL signals simultaneously.

Although STxMP feature is agreed in Rel-18, the UL signals corresponding to different panels are limited. A UE can only simultaneously transmit PUCCH + PUCCH and PUSCH + PUSCH. A UE cannot simultaneously transmit PUCCH + PUSCH. If PUCCH corresponding to a panel overlaps with PUSCH corresponding to the other panel due to 2 TAs, how to handle this situation should be discussed.

- How to handle UL overlapping between PUCCH and PUSCH in STxMP scenario should also be discussed.

There are two operations to be introduced to handle this problem. One operation is at TRP side. At TRP side, a TRP can allocate UL resources properly to avoid overlapping. At UE side, a UE can avoid overlapping by whole/fractional dropping method or fallback method.

2.2.1 At TRP side

As shown in Figure 3, if the time difference ∆tTx between the start time of UE transmitting UL signal 1 and UE transmitting UL signal 2 is larger than the duration time T1 of UL signal 1 resource, the transmission time of UL signal 1 and UL signal 2 does not overlap. According to the analysis, the formula TA1 / 2 + ∆tTx = TA2 / 2 + ∆tRx is workable. Therefore, ∆tTx = (TA2 – TA1) / 2 + ∆tRx = ∆TA + ∆tRx. If ∆TA + ∆tRx > T1, the transmission time of UL signal 1 and UL signal 2 does not overlap. Sometimes, switching time between different beams for UE needs to be considered. Therefore, the condition of non-overlapping becomes ∆TA + ∆tRx + ∆ts > T1. The switching time of different beams is dependent on the UE capability. The duration time of UL signal and the reception time difference between different UL signals are dependent on the resource allocation by network. If network knowns these parameters in the above-mentioned condition, the overlapping can be avoided. It is proposed that the overlapping between signals that a UE transmits to difference TRPs can be avoided by network allocating UL signal resource for UE properly to satisfy the condition ∆TA + ∆tRx + ∆ts > T1.

Figure 3. A schematic diagram illustrating

- Recommend to introduce UL resource allocation restriction to handle UL overlapping at TRP side.

2.2.2 At UE side

(1) Whole dropping method

If UL signals transmitted to different TRPs by a UE have overlapping in time domain, the UE can choose one of signals to transmit to corresponding TRP according to some rules, for example, time priority rule and importance priority rule. For time priority rule, as shown in Figure 4(a), if both UL signal 1 and UL signal 2 are PUSCH with data and the transmission time of UL signal 1 is earlier than that of UL signal 2, a UE choose UL signal 1 to transmit to TRP1 with TA1 and does not transmit UL signal 2 to TRP2. For importance priority rule, as shown in Figure 4(b), if UL signal 1 is PUSCH with data and UL signal 2 is PUCCH with HARQ, a UE choose UL signal 2 to transmit to TRP2 with TA2 and does not transmit UL signal 1 to TRP1.

Figure 4

(2) Fractional dropping method

If UL signals transmitted to different TRPs by a UE have overlapping in time domain, the UE can choose one of signals to transmit to corresponding TRP and transmit a segment of another UL signal to another TRP. Choosing which signal to be truncated can also follow time priority rule or importance priority rule. For example, as shown in Figure 5, if both UL signal 1 and UL signal 2 are PUSCH with data, a UE can transmit segmental UL signal 1 to TRP1 with TA1 and transmit whole UL signal 2 to TRP2 with TA2. It should be explained that the truncated signal is UL signal 1 instead of UL signal 2 because DMRS is more important than data at the overlapping PUSCH in time domain.

Figure 5

(3) Fallback method

If UL signals transmitted to different TRPs by a UE have overlapping but received by different TRPs do not have overlapping in time domain, the UE can back to the current spec mode, that is, multi-TRP with 1 TA. For example, as shown in Figure 6, if there is overlapping in time domain, a UE can transmit UL signal 1 to TRP1 with TA1, then transmit UL signal 2 to TRP2 also with TA1. Although this mechanism lowers the receiving performance of UL signal 2, the resource of UL signal 2 is not waste. That means this method can save time-frequency resource.

Figure 6

- Recommend to introduce dropping method or fallback method to handle UL overlapping at UE side.

2.3 TA reporting

For multi-TRP with a non-idea backhaul, a TRP only knows its own TA but cannot know the TA corresponding to the other TRP. While a UE can know both two TAs corresponding to different TRPs. If a UE reports the TA corresponding to a TRP to the other TRP, TRPs can calculate the UL overlapping time and allocate UL resources properly to avoid.

Meanwhile, for multi-TRP with a non-idea synchronization, the additional error will be introduced for DL Tx, because the DL timing offset may be unfortunately compensated along with propagation delay. The DL timing offset can be calculated according to the difference between two TAs and the difference between two DL reception timings. The two DL reception timings corresponding to two TRPs can be measured by UE. For multi-TRP with a non-idea backhaul, one of TRPs only knows its own UL TA and DL timing but cannot know the UL TA and DL timing of the other TRP. If a UE reports the TA corresponding to a TRP and the difference between two DL reception timings to the other TRP, TRPs can calculate the DL timing offset and adjust DL transmission time to avoid additional error.

- Recommend to enable a UE to report the information about TAs and DL reception timings.

2.3.1 Report actively

To enable a UE to report the information about TAs and DL reception timings, the UE can be configured specific period PUCCH resources for reporting the information. Here, we take a TA report as an example. When the TA corresponding to a TRP is updated, the UE can report the TA to the other TRP through PUCCH corresponding to the other TRP. For example, as shown in Figure 7, if TA2 is updated by TAC2 at time tTAC2, a UE reports the updated TA2 to TRP1 at time toffset1+T1, which is the nearest time after tTAC1 and at which there is a specific PUCCH resource related to TRP1.

Figure 7

A UE can also report the information about TAs and DL reception timings through PUSCH actively. Similarly, we take a TA report as an example. When the TA corresponding to a TRP is updated, the UE can report the TA to the other TRP through PUSCH related to the other TRP if there is PUSCH resources. If there is not PUSCH resources, the UE needs to request PUSCH resources related to the other TRP. For example, as shown in Figure 8, if TA2 is updated by TAC2 at time tTAC2, a UE reports the updated TA2 to TRP1 at time toffset1, which is the nearest time after tTAC1 and at which there is a PUSCH resource related to TRP1.

Figure 8

- Recommend to consider that a UE reports report the information about TAs and DL reception timings through PUCCH or PUSCH actively.

2.3.2 Report passively

A TRP can maintain a timer related to TAs and DL reception timings which is corresponding to the other TRP and reported by a UE. When a TRP receives the TA and DL reception timing corresponding to the other TRP, the TRP flushes the timer immediately. When the timer is overtime, the TRP transmits a command to the UE to indicate the UE to report the TA and DL reception timing corresponding to the other TRP.

- Recommend to consider that a TRP indicates a UE to report the information about TAs and DL reception timings.

3 Conclusion

In this contributions, our opinions on two TAs for multi-DCI based multi-TRP operation are listed as follows.

- Support option 2: Associate TAG to CORESETPoolIndex.

- How to handle UL overlapping between PUCCH and PUSCH in STxMP scenario should also be discussed.

- Recommend to introduce UL resource allocation restriction to handle UL overlapping at TRP side.

- Recommend to introduce dropping method or fallback method to handle UL overlapping at UE side.

- Recommend to enable a UE to report the information about TAs and DL reception timings.

- Recommend to consider that a UE reports report the information about TAs and DL reception timings through PUCCH or PUSCH actively.

- Recommend to consider that a TRP indicates a UE to report the information about TAs and DL reception timings.