3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208466

# e-Meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on processing timeline for NACK-only mode2 to TS38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current specified PDSCH processing timeline is not sufficient for UE configured with NACK-only mode2 HARQ-ACK feedback to transmit the PUCCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add d3 to TProc,1 and the value of d3 depends on the HARQ-ACK feedback option and the PUCCH resources configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When UE is configured with NACK-only mode2 HARQ-ACK feedback option and all the PUCCH resources are not allocated within the same PRB, the UE may not provide a valid HARQ-ACK feedback corresponding to the scheduled PDSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213 (R1-2209832) |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3	UE PDSCH processing procedure time

If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and Koffset, if configured, and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after  TProc,1=N1+d1,1+d2+d32048+144∙ƙ2-μ∙TC+Text after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message.

-	N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is assumed to be transmitted regardless of whether or not the PDSCH reception provides a transport block for a HARQ process with disabled HARQ-ACK information as indicated by HARQ-feedbackEnabling-disablingperHARQprocess, if provided, and κ is defined in clause 4.1 of [4, TS 38.211].

-	For operation with shared spectrum channel access in FR1, is calculated according to [4, TS 38.211], otherwise =0.

-	If the PDSCH DM-RS position l1 for the additional DM-RS in Table 7.4.1.1.2-3 in clause 7.4.1.1.2 of [4, TS 38.211] is l1=12 then N1,0=14 in Table 5.3-1, otherwise N1,0=13.

-	If the UE is configured with multiple active component carriers, the first uplink symbol which carries the HARQ-ACK information further includes the effect of timing difference between the component carriers as given in [11, TS 38.133].

-	For the PDSCH mapping type A as given in clause 7.4.1.1 of [4, TS 38.211]: if the last symbol of PDSCH is on the i-th symbol of the slot where i < 7, then d1,1 = 7 - i, otherwise d1,1 = 0

-	If a PUCCH of a larger priority index would overlap with PUCCH/PUSCH of a smaller priority index, d2 for the PUCCH of a larger priority is set as reported by the UE; otherwise d2 = 0.

-	For UE processing capability 1: If the PDSCH is mapping type B as given in clause 7.4.1.1 of [4, TS 38.211], and

-	if the number of PDSCH symbols allocated is L ≥ 7, then d1,1 = 0,

-	if the number of PDSCH symbols allocated is L ≥ 4 and L ≤ 6, then d1,1 = 7- L.

-	if the number of PDSCH symbols allocated is L = 3 then d1,1 = 3 + min (d,1), where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	if the number of PDSCH symbols allocated is 2, then d1,1 = 3+d, where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	For UE processing capability 2: If the PDSCH is mapping type B as given in clause 7.4.1.1 of [4, TS 38.211],

-	if the number of PDSCH symbols allocated is L ≥ 7, then d1,1 = 0,

-	if the number of PDSCH symbols allocated is L ≥ 3 and L ≤ 6, then d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH,

-	if the number of PDSCH symbols allocated is 2,

-	if the scheduling PDCCH was in a 3-symbol CORESET and the CORESET and the PDSCH had the same starting symbol, then d1,1 = 3,

-	otherwise d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	For UE processing capability 2 with scheduling limitation when µPDSCH = 1, if the scheduled RB allocation exceeds 136 RBs, the UE defaults to capability 1 processing time. The UE may skip decoding a number of PDSCHs with last symbol within 10 symbols before the start of a PDSCH that is scheduled to follow Capability 2, if any of those PDSCHs are scheduled with more than 136 RBs with 30kHz SCS and following Capability 1 processing time.

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PDSCH-ServingCellConfig is configured for the cell and set to 'enable'.

-	PDSCH processing capability 2 is not applied to PDSCH scheduled by PDCCH with DCI format 4_0, 4_1, or  4_2.

-	If this PUCCH resource is overlapping with another PUCCH or PUSCH resource, then HARQ-ACK is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the HARQ-ACK message is transmitted on PUCCH.

-    if UE is indicated mode2 by moreThanOneNackOnlyMode for nack-only HARQ-ACK feedback configured by harq-FeedbackOptionMulticast, d3= N2 when PUCCH resources configured for nack-only mode2 are not within the same PRB, otherwise, d3=0.

Otherwise the UE may not provide a valid HARQ-ACK corresponding to the scheduled PDSCH. The value of Tproc,1 is used both in the case of normal and extended cyclic prefix.

For a PDSCH that consists of two PDSCH transmission occasions in time domain in one slot, d1,1 is calculated based on the first PDSCH transmission occasion in the slot, and as described above.

For PDSCH with mapping Type B, if PDSCH is scheduled by a PDCCH reception that includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], d1,1 for PDSCH processing time is determined by considering the PDCCH candidate that results in larger d1,1 value.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, or if none of the higher layer parameters is configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |
| 5 | 80 | 96 |
| 6 | 160 | 192 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |