3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208469

# e-Meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE behaviors of PDCCH monitoring for configured RM patterns to TS38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the RateMatchPattern is configured in ServingCellConfig or ServingCellConfigCommon, UE’s behavior is unclear in terms of whether monitoring the PDCCH if the DCI is not a unicast DCI format. When the RateMatchPattern is configured in pdsch-ConfigMulticast or pdsch-ConfigMCCH/pdsch-ConfigMTCH, UE’s behavior is unclear in terms of whether monitoring the PDCCH if the DCI is not a multicast DCI format considering the case of UE not supporting multipleCORESET but to receive MBS multicast in CFR within UE active BWP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding “ServingCellConfig, or ServingCellConfigCommon” and “pdsch-ConfigMCCH/pdsch-ConfigMTCH”. - Deleting “unicast” from “unicast DCI format” and “multicast” from “multicast DCI format”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior whether monitoring PDCCH is unclear for the concerned cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

< Unchanged parts are omitted >

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern of pdsch-Config or ServingCellConfig/ServingCellConfigCommon, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern of pdsch-Config or ServingCellConfig/ServingCellConfigCommon, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception scheduled by a DCI format as described in [6, TS 38.214]. If a PDCCH candidate that provides a DCI format is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern of pdsch-ConfigMulticast or pdsch-ConfigMCCH/pdsch-ConfigMTCH, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern of pdsch-ConfigMulticast or pdsch-ConfigMCCH/pdsch-ConfigMTCH, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception scheduled by a DCI format. If a PDCCH candidate that provides a DCI format is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.

A UE does not expect to be configured with dci-FormatsSL and dci-FormatsExt in a same USS.