3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2208472

e-Meeting, October 10 – 19, 2022

Agenda Item:	8.5

Source:	Huawei, HiSilicon

Title:	Maintenance of Rel-17 NR positioning

Document for:	Discussion and Decision

Introduction

In this paper, we discuss the maintenance aspects for Rel-17 NR positioning on the following issues.

DL-AoD Rx beam reporting

PRS collision detection timeline for UL

DL-AoD Rx beam reporting

There was debate over the condition to report the Rx beam, where Rel-16 condition as below in TS 38.214 and TS 37.355.

| TS 38.214 | When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. |
| --- | --- |
| TS 37.355 | nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements from the same DL-PRS Resource Set have been made with the same RX beam by the target device; otherwise it is not present.. |

It means that only when at least two RSRP measurements from the same PRS resource set are associated with the same Rx beam, the Rx beam index shall be reported, and otherwise the Rx beam shall not be reported.

In general, we think that it should be an unnecessary restriction to limit the measurements within a single PRS resource set, but there was no consensus to extend the condition to across multiple resource sets within the TRP. Nevertheless, we think that a more pressing need is to add the RSRPP to the condition of Rx beam index reporting.

- Proposal 1: Add another condition for reporting Rx beam index that corresponds to reporting at least 2 DL PRS-RSRPP measurements associated with the Rx beam within the DL PRS resource set.

Endorse the draftCR in R1-2209837 to TS 38.214.

Meanwhile it is worth clarifying that the condition of reporting Rx beam should not be interpreted as that the same Rx beam index does not apply to RSRP/RSRPP measurement from different PRS resource sets for the same TRP on a positioning frequency layer. It means that Rx beam index 0, if associated with RSRP1 for a PRS resource from PRS resource set 0 and RSRP2 for a PRS resource from PRS resource set 1 of the same TRP on the same positioning frequency layer actually refers to the same Rx beam.

Proposal 2: The same Rx beam index shall be interpreted as the same Rx beam over PRS resources from multiple PRS resource sets for a single TRP on a single positioning frequency layer.

the same Rx beam will NOT be assumed at network side if the same Rx beam index is indicated for different TRPs or for different positioning frequency layers.

PRS collision detection timeline with UL

In RAN1#110, the PRS collision detection time for higher priority DL signals/channels are agreed to reuse N2 after the determination of presence of the DL signals/channels [1].

Agreement With regards to the action time for the PRS prioritization over other DL channels Alt.1: support N = N2 (clause 6.4 of TS 38.214) for the subcarrier spacing of the PDCCH. Agreement Revise the previous agreement as follows: With regards to the action time for the PRS prioritization over other DL channels - Alt.1: support N = N2 (clause 6.4 of TS 38.214) for the subcarrier spacing of the PRS.

In general, the PRS measurement within the PPW should not be impacted by any real or potential UL, since the working assumption only deals with DL.

The above statement is correct when PRS and UL are not colliding with each other, but it is not explicit when it comes handling of collision of PRS and UL. The thing that is worth consideration is the dynamically indicated UL symbol on a semi-static flexible symbol, e.g. by DCI or by SFI according to the following description from TS 38.213.

For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - … - if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink or flexible. … If a UE is configured by higher layers to receive a DL PRS in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the DL PRS reception in the set of symbols of the slot.

Observation 1: The only remaining matter with regards to collision between PRS and UL within the PPW is the UL indicated dynamically by DCI or by SFI, which is semi-statically indicated as FL.

We think that the timeline to handle the collision between PRS and other DL signals/channels can be used for such indication. For the indication of UL that arrives too late when UE is not able to cancel the DL PRS reception in the slot, the indicated “UL symbols” may suffer from severe cross-link interference, and thus the measurement on the PRS symbols (assuming the neighbouring TRPs are still transmitting) cannot be guaranteed.

Observation 2: The PRS symbols indicated as UL, regardless of whether UE is able to cancel the reception, may suffer from severe cross-link interference.

Our preference in RAN1#110 is that such a late indication of UL is considered as an error case that UE is not expected to handle due to poor SINR or even problems on the AGC from cross-link interference exist. During the FL summary discussion, there was question asking whether spec still allows UE to receive the PRS, similar to the handling of DL. We think this can also be a wayforward understanding that the side condition to meet performance requirement is not satisfied.

Based on the discussion above, we have the following proposal with regards to the collision between PRS reception and dynamically indicated UL within the PPW.

- Proposal 3: Apply the same timeline for collision between PRS and UL indicated by DCI or SFI.

Endorse the draftCRs in R1-2209838 and R1-2209839 to TS 38.214 and TS 38.213, respectively.

Conclusion

In this contribution, we have discussed the remaining issue of Rel-17 positioning. Based on the discussion, we have the following observations and proposals.

Observation 1: The only remaining matter with regards to collision between PRS and UL within the PPW is the UL indicated dynamically by DCI or by SFI, which is semi-statically indicated as FL.

Observation 2: The PRS symbols indicated as UL, regardless of whether UE is able to cancel the reception, may suffer from severe cross-link interference.

- Proposal 1: Add another condition for reporting Rx beam index that corresponds to reporting at least 2 DL PRS-RSRPP measurements associated with the Rx beam within the DL PRS resource set.

Endorse the draftCR in R1-2209837 to TS 38.214.

Proposal 2: The same Rx beam index shall be interpreted as the same Rx beam over PRS resources from multiple PRS resource sets for a single TRP on a single positioning frequency layer.

the same Rx beam will NOT be assumed at network side if the same Rx beam index is indicated for different TRPs or for different positioning frequency layers.

- Proposal 3: Apply the same timeline for collision between PRS and UL indicated by DCI or SFI.

Endorse the draftCRs in R1-2209838 and R1-2209839 to TS 38.214 and TS 38.213, respectively.

- Reference

- RAN1, Chair’s Notes RAN1#110 v20, RAN1#110, Toulouse, France, 22nd – 26th Aug., 2022.