3GPP TSG RAN WG1 Meeting #110bis-e	R1-2208478

E-meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TRS occasions for IDLE mode UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some parameter names for Rel-17 UE power saving are not aligned with the parameter names used in TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates on the parameter names to align with TS 38.331： - Update the parameter name of “numberOfresources” to “nrofResources”; - Update “scramblingID” to “ScramblingID”; - Update “TRS-ResourceSetConfig” to “trs-ResourceSetConfig”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some parameter names for Rel-17 UE power saving are not aligned with the name used in TS 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.1.6.1.1	CSI-RS for tracking

< Unchanged parts are omitted >

A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a trs-ResourceSetConfig.

-	For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more NZP CSI-RS set(s), where a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.

-	For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE:

-	the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by nrofResources

-	the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

-	where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4

-	a single port CSI-RS resource with density  given by Table 7.4.1.5.3-1 from [4, TS 38.211].

-	the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP.

-	UE is not required to receive TRS occasions outside the initial DL BWP.

-	the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet.

-	the UE does not expect the TRS-ResourceSet to be configured with the periodicity of  slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks.

-	the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by TRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP.

-	powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet.

-	the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet.

-	One or more ScramblingIDs where if a single ScramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a ScramblingID.

-	the UE may assume the following quasi co-location type(s):

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block.

For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID.

< Unchanged parts are omitted >