3GPP TSG RAN WG1 #110bis-e	R1-2208521

# e-Meeting, October 10th – 19th, 2022

Source:	ZTE

Title:	Evaluation on AI for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

- Introduction

According to discussion in 3GPP RAN1#110 meeting [1], some progress has been made on evaluation methodology for CSI feedback enhancement. In this contribution, we provide our views on further details for evaluation methodology and share some initial evaluation results. In our companion contribution [2], representative sub-use case selection and potential specification impacts are discussed accordingly.

- Generic issues on evaluation methodology

  - System Level Simulation Evaluation

- Agreements in RAN1#109-e Agreement: For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) - Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. - FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference. - FFS: How to model the realistic channel estimation - FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation - Agreement: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows Base Baseline for performance evaluation; Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation.

After RAN1#109-e and RAN1#110, some remaining issues have been left.

- Issue#1: Whether full buffer traffic model is optionally taken for the purpose of calibration and eventual performance comparison with the benchmark release and drawing SI conclusions.

- Issue#2: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

- Issue#3: Whether Type I Codebook can be optionally considered at least for performance evaluation

For the Issue#1, we think that full buffer can be optionally taken into account for calibration purpose. At the same time, SI conclusions should be drawn according to evaluation results of FTP traffic models rather than full buffer.

Proposal 1: Full buffer traffic model can be optionally taken for calibration purpose and SI conclusions should be based on evaluation results of FTP traffic models.

- For the Issue#2, if ideal channel is used as a target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation, companies can easily identify the performance loss compared to the ideal channel.

Proposal 2: Intermediate results should be calculated based on ideal channel and AI/ML model output at least for calibration purpose.

Considering the Issue#3, it is not necessary to take Type I CB for performance evaluation since the AI CSI should target on improving performance of MU-MIMO. According to previous study, Type I CB can barely achieve performance gain over traditional Type II codebook . Therefore, we prefer Rel-16 Type II CB or Rel-17 Type II CB to be adopted as the baseline for performance comparison.

Proposal 3: Type I CB is not necessary to be taken as a baseline for performance evaluation.

  - Evaluation Metrics

- Agreements in RAN1#109-e Agreement: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including: - Method 1: Average over all layers - Note: is the eigenvector of the target CSI at resource unit i and K is the rank. is the output vector of the output CSI of resource unit i. is the total number of resource units. denotes the average operation over multiple samples. - Method 2: Weighted average over all layers - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues) - Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer) - Other methods are not precluded - FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s). - Agreements in RAN1#110 Agreement: For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted Agreement: For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

In order to evaluate and calibrate the performance of AI/ML models among companies, intermediate KPIs have been decided in RAN1#109-e and RAN1#110, and one remaining issue has been left.

- Issue#1: Whether to further down-select among options for SGCS calculation when rank>1

In the above agreement, several SGCS calculation methods are proposed for rank>1. Since Method 1 and Method 2 either puts on the same weight or different weight on different layers, it’s hard to identify the performance gain for each layer. Method 3 can be used to analyze intermediate performance gains from different layers, which actually influence the eventual system throughput.

Proposal 4: Method 3 can be adopted as a baseline for SCGS calculation when rank>1.

  - Methodology for Generalization

- Agreements in RAN1#110 Agreement The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - FFS other cases for generalization verification, e.g., - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. Agreement For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification Agreement For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point: - Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband) - Various sizes of CSI feedback payloads, FFS candidate payload number - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) - Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc. - Companies to report the selected configurations for generalization verification - Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

In order to evaluate and calibrate the performance of AI/ML models among companies, generalization evaluation methodologies have been agreed in RAN1#109-e and RAN1#110. There is one remaining issue

- Issue#1: Case 2A for generalization verification is FFS

- For case 2A, the AI/ML model is trained based on training data from the first Scenario/Configuration, and then updated based on training data from the second Scenario/Configuration different from the first Scenario/Configuration. Case 2A tends to evaluate the performance improvement on the second Scenario/Configuration compared with Case 2, since Case 2A targets on the performance of a fine-tuned model. While Case 2 simply cares about the generalization performance of the original model. In addition, Case 2A also differs from Case 3 that Case 2A only focuses on the performance in the second Scenario/Configuration. However,  Case 3 takes the performance of both Scenarios/Configurations into account. Therefore, it is more appropriate for Case 2A to be listed as an independent case for further study, which should not be merged into either Case 2 or Case 3.

Proposal 5: Case 2A can be listed as an independent case for verifying model generalization

- Evaluations on CSI compression in spatial and frequency domain

  - Model input and model output

- Agreements in RAN1#109-e Agreement: For the evaluation of the AI/ML based CSI compression sub-use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. - At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

In RAN1#109-e, AI/ML based CSI compression sub-use case was agreed. For ease of comparison, the data type & data size of the training data input to AI models are encouraged to be discussed and aligned if possible. In our contribution proposed for RAN1#109-e [4], we provided the model input and the model output details for rank=1. In this contribution, more details of rank>1 will be elaborated.

For rank>1, in our initial evaluation, we propose two cases for AI/ML model to compress and recover the sub-band eigenvectors for CSI compression sub-use case.

- Case 1: Single layer in model input and single layer in model output

For Case 1, we mix the sub-band eigenvectors of multiple layers from multiple UEs as training samples so the total number of training samples is , where  denotes the layer number and  denotes the number of UEs. For model testing/inference, the input of AI model is a group of sub-band eigenvectors obtained from the same layer, which can be a three-dimensional tensor with a size of , where 2 denotes real and imaginary parts of each complex channel coefficient,  denotes the number of transmit antenna ports, and  denotes the number of sub-bands. Then, the output of AI model is recovered eigenvectors. Essentially, the AI input is still one layer of data and the model structure is the same as rank=1. The detailed AI/ML framework is shown in Figure 1(a).

Figure 1(a) The input and output of AI/ML models for Case 1

- Case 2: Multiple layers in model input and multiple layers in model output

For Case 2, we generate the sub-band eigenvectors of multiple layers from multiple UEs as training samples so the total number of training sample is , where  denotes the number of UE samples. For model testing/inference, the input of AI model is a group of sub-band eigenvectors obtained from  layers, which can be a three-dimensional tensor with a size of , where 2 denotes real and imaginary parts of each complex channel coefficient,  denotes the layer number,  denotes the number of transmit antenna ports, and  denotes the number of sub-bands. Then, the output of AI model is recovered eigenvectors. Different from Case 1, the AI input is multiple layers of data and the model structure needs to be changed from rank=1. The detailed AI/ML framework is shown in Figure 1(b).

Figure 1(b) The input and output of AI/ML models for Case 2

Proposal 6: For rank>1, two cases on model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point.

- Case 1: Single layer in model input and single layer in model output

- Case 2: Multiple layers in model input and multiple layers in model output

  - CSI training dataset and inference dataset separation

In RAN1#110 meeting, the issue of CSI training dataset and inference dataset separation was raised for discussion, since inappropriate dataset separation for AI training/inference may cause an unconvincing conclusion. For example, if the testing data derives from the training dataset, the testing data and training data are so similar that the trained model may show better performance than the real model performance.. For dataset construction for training and inference,.there are at least two methods can be applied. With the first method, UE channel data may be generated for one TTI from multiple drops, which brings abundant channel diversity. With the second method, UE channel data may be generated for multiple TTIs from a few drops, which may introduce strong time correlation across different TTIs of the same drop. Two different dataset constructions may show different performance gains, where AI model will show real performance gains with the first method while inaccurate performance gains may be obtained using the second method. From our perspective, the first method can be adopted for dataset separation, i.e. training dataset and inference dataset follow multiple diverse drops.

- Proposal 7: For CSI dataset construction, training dataset and inference dataset should be separated in different drops at least for calibration purpose.

- Evaluations on CSI prediction involving temporal domain

According to discussion in 3GPP RAN1#110 meeting [1], some conclusions have been drawn on evaluation methodology for the sub use case of CSI prediction involving temporal domain as follows:

- Conclusions in RAN1#110 Conclusion: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point. Conclusion: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point. Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE. Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation, - 100% outdoor UE is assumed for UE distribution. - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles - UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h - Note: Companies to report the set/subset of speeds - 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including: - The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc. - The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc. - The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc. - Data pre-processing/post-processing - Loss function - Others are not precluded

  - Evaluation assumptions for temporal CSI prediction

In the AI-based CSI prediction design, the AI model is designed to derive the prediction of CSI in the future occasion as the output of model when using the historical CSIs as the input. The process of AI-based CSI prediction is illustrated in Figure 2.

Figure 2 The process of AI-based CSI prediction

For CSI prediction, the data generation generally follows the SLS simulation assumptions of AI-based CSI compression [3], where some specific assumptions for CSI prediction are listed:

- UE distribution: 100% outdoor

- UE speed: 30km/h

- CSI feedback periodicity: 5ms

- UE trajectory: Reflected by Doppler shift

- Non-AI baseline for CSI prediction: Nearest historical CSI (sample-and-hold)

Proposal 8: Evaluation assumptions for AI-based CSI prediction can reuse agreed assumptions in Rel-18 MIMO for Type-II codebook refinement under high/medium velocities as much as possible.

  - Model input and model output

For CSI prediction involving temporal domain, we design a single-sided AI model applied at UE in our initial evaluation. The AI/ML model aims at predicting the channel matrices in future occasions based on multiple historical channel measurements. In our evaluation, the input of AI model is a group of historical channel on PRBs from one receiving RF chain, which constructs a four-dimensional tensor with a size of , where 2 denotes real and imaginary parts of each complex channel coefficient,  denotes the number of historical CSI measurement,  denotes the number of transmission antenna ports, and  denotes the number of physical resource blocks. Then, the output of AI model is predicted channel on PRBs in a future occasion. The detailed AI/ML model is shown in Figure 3.

Figure 3 The input and output of AI/ML models for CSI prediction

Proposal 9: For AI-based CSI prediction, the raw channel matrix on PRBs as model input/output can be considered for intermediate evaluations at least for calibration purpose.

- Preliminary Evaluation Results for CSI compression

We perform a preliminary simulation on spatial-frequency domain CSI compression sub-use case to evaluate the performance of AI models and the Rel-16 eTypeII is also simulated for baseline comparison. The detailed simulation assumptions are shown in Table 9-1. In addition, detailed AI model training parameters are listed in Table 9-2. The system-level channel data is generated from 3000 simulation drops and 210 UEs per simulation drop, which results in 630000 samples in total. Then, the datasets are randomly divided into three parts which are training, validation, and testing datasets with 600000, 10000, and 20000 samples respectively. In our evaluation, ideal sub-band eigenvectors are used as the input of AI models for training, validation and testing. The number of parameters ranges from about 9M-11M, where the CSI generation part is about 4M-5M and the CSI reconstruction part is 5M-6M. In addition, the FLOPs of CSI generation part and CSI reconstruction part are approximately 25M-26M and 27M-29M, respectively.

  - Rank up to 1

In this section, we initially evaluate the intermediate KPIs, e.g. GCS/SGCS, between our AI approach and the Rel-16 eType II. The results are shown in Figure 4(a). According to the simulation results, the AI based CSI recovery shows about 3%-4% GCS gains and 7%-8% SGCS gains over the Rel-16 eType II. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 4 layers MU scheduling under FTP3 traffic model with 70% RU and the simulation results are shown in Figure 4(b). Only less than 1% UPT performance gain can be obtained since Rel-16 eTypeII can already achieve good performance.

Figure 4(a) Subband-level SGCS simulation results       Figure 4(b) Average UPT gain with RU=70%

Observation 1: AI based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS for rank=1. With the same feedback overhead, AI based CSI recovery can obtain 7%-8% SGCS gains.

Observation 2: AI based CSI reconstruction achieves very little UPT gain for the case of rank=1 and less than 1% average UPT gain over Rel-16 eTypeII with the same feedback overhead.

  - Rank up to 2

In this section, the intermediate and eventual simulation results for Rel-16 eTypeII and AI based CSI recovery are provided. Figure 5 illustrates the SGCS comparison between the AI model and Rel-16 Type II codebook for each layer under the rank=2 case. As can be seen, the 1st layer outperforms the 2nd layer in terms of SGCS, where the reason may be that the 1st layer contains less features compared to the 2nd layer. Moreover, AI outperforms Rel-16 Type II codebook in terms of SGCS for each layer, indicating higher accuracy of CSI recovery by AI. In addition, Case 1 with single-layer model input and single-layer model output generally shows better SGCS performance than Case 2 with multi-layer model input and multi-layer model output for each layer.

Figure 5 SGCS simulation results for each layer

Observation 3: AI based CSI reconstruction with Case 1 method (single-layer model input and single-layer model output) shows performance gains in SGCS per layer over the Rel-16 eType II for rank =2.

Observation 4: Case 1(single-layer model input and single-layer model output) can achieve better performance than Case 2 (multi-layer model input and multi-layer model output).

To evaluate the eventual performance of AI/ML approaches, we further evaluate average UPT versus feedback overhead under the up to 8 layers MU scheduling and FTP 3 traffic model with 50% and 70% resource utilization (RU). Regarding the SGCS performance between the two cases of AI models, Case1 AI model is adopted for the subsequent eventual throughput simulation. The simulation results are shown in Figure 6(a) and Figure 6(b).

Figure 6(a) Average UPT gain with RU=50%         Figure 6(b) Average UPT gain with RU=70%

Observation 5: With maximum rank up to 2 under the same feedback overhead, AI based CSI reconstruction has about 4%-6% average UPT gain over the Rel-16 eType II under the case of 50% RU and 5%-8.5% average UPT gain can be obtained by AI based CSI recovery under the case of 70% RU.

Observation 6: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for maximum rank up to 2.

Observation 7: AI/ML approaches show better throughput performance with heavy traffic load for up to rank=2.

  - Rank up to 4

In this section, the intermediate and eventual simulation results for Rel-16 eTypeII and AI based CSI recovery are provided. Figure 7 shows the comparison of SGCS between the AI model and Rel-16 Type II codebook for each layer under the rank=4 cases, in which Figure 7 provides the SGCS performance for rank=4. As can be seen, there are larger performance gains in the 3rd layer and the 4th layer than the 1st layer and the 2nd layer in terms of SGCS, where the reason may be that some feedback bits of layer 1/2 are sacrificed and distributed to layer 3/4 compared with eType II... However, AI still outperforms Rel-16 Type II codebook in terms of SGCS for each layer, indicating higher accuracy of CSI recovery by AI.

Figure 7 SGCS for each layer for rank=4

Observation 8: AI based CSI reconstruction shows performance gain in SGCS per layer over the Rel-16 eType II for rank =3/4.

Observation 9: AI based CSI reconstruction shows larger performance gains in layer 3/4 than layer 1/2 in terms of SGCS with the assumption of the same feedback overhead of each layer.

To evaluate the eventual performance of AI/ML approaches, we further evaluate average UPT versus feedback overhead under up to 12 layers MU scheduling and FTP 3 traffic model with 50% and 70% resource utilization (RU). The simulation results are shown in Figure 8(a) and Figure 8(b).

Figure 8(a) Average UPT gain with RU=50%         Figure 8(b) Average UPT gain with RU=70%

Observation 10: For up to rank=4, with the same feedback overhead, AI based CSI reconstruction has about 4%-13% average UPT gain over the Rel-16 eType II under the case of 50% RU and 6%-16% average UPT gain can be obtained by AI based CSI reconstruction under the case of 70% RU.

Observation 11: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to rank=4.

Observation 12: AI/ML approaches show better throughput performance with heavy traffic load for up to rank=4.

Proposal 10: The case of rank>1 should be prioritized in later discussion.

  - Model generalization

In this section, we provide some results for the evaluation of model generalization capability..

- Various deployment scenarios:

We simulate AI generalization for various deployment scenarios, where the following three cases are considered.

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Table 5-1 provides generalization performance in term of SGCS for scenarios of UMa, UMi, InH and mixed scenarios. The training dataset consists of 600000 samples and the testing dataset consists of 20000 samples for each generalization case. The mixed training dataset has two types, where the first type is Mixed 1 with UMa:UMi=1:1, and the second type is Mixed 2 with UMa:UMi:InH=1:1:1. For comparison, Table 5-1 also shows performance gains of baseline scheme(i.e., case 1). As shown in the Table 5-1, the generalization results of case 2 show large negative effect, where most of performance reduction is smaller than 0.02. However, the generalization results of case 3 show marginal performance loss over baseline scheme, indicating that the mixed dataset is beneficial to model generalization.

- Table 5-1 SGCS for various deployment scenarios

| Model training | Model testing Overhead: total 644bits with each layer for 322bits SGCS: Layer1/Layer2 SGCS gains: (Layer1/Layer2) |  |  |  |
| --- | --- | --- | --- | --- |
|  | UMa | UMi | InH |
| UMa | 0.9218/0.8661 (0/0) | 0.9137/0.8560 (-0.0177/-0.0252) | 0.9339/0.9087 (-0.0165/-0.0222) |
| UMi | 0.9226/0.8636 (0.0008/-0.0025) | 0.9314/0.8812 (0/0) | 0.9367/0.9084 (-0.0137/-0.0225) |
| InH | 0.8706/0.7972 (-0.0512/-0.0689) | 0.8510/0.7685 (-0.0804/-0.1127) | 0.9504/0.9309 (0/0) |
| Mixed 1: UMa:UMI=1:1 | 0.9226/0.8668 (0.0008/0.0007) | 0.9228/0.8696 (-0.0086/-0.0116) | 0.9403/0.9155 (-0.0101/-0.0154) |
| Mixed 2: UMa:UMI:InH=1:1:1 | 0.9211/0.8652 (0.0007/-0.0009) | 0.9217/0.8691 (-0.0097/-0.0121) | 0.9609/0.9420 (0.0105/0.0111) |

- Observation 13: Case 3(AI model trained with mixed deployment scenarios) shows good generalization performance for various deployment scenarios.

- Observation 14: The AI model trained with the scenario of UMa/UMi shows good generalization performance for InH dataset, while the AI model trained with the scenario of InH shows generalization degradation for UMa/UMi dataset.

- Various configurations on bandwidth:

We simulate AI generalization for different datasets with bandwidths of 10MHz (52RBs) and 20MHz (104RBs). According to agreements made in previous meetings, 4RBs per sub-band for 10MHz and 8RBs per sub-band for 20MHz are adopted, hence the AI input sizes keep the same for both datasets, which are . Besides, two generalization cases are designed as follows and the results are shown in Figure 9. Compared with the cases of training and testing following the same bandwidth configuration, there is no obvious performance loss for both Case A and Case B.

- Case A: 10M dataset for training/validation and 20M dataset for testing

- Case B: 20M dataset for training/validation and 10M dataset for testing

- Figure 9 AI generalization for different bandwidth configurations

Observation 15: AI/ML approaches can achieve good generalization performance for the case that the training/validation dataset and testing dataset are generated with different bandwidth configurations.

- Various configurations on antenna port number:

- We also simulate AI generalization for various antenna port numbers, where three cases of generalization are considered. Table 4-2 provides generalization performance in term of SGCS for port numbers of 16, 32 and their mixed version. The training dataset consists of 600000 samples and the testing dataset consists of 20000 samples for each generalization operation. For comparison, Table 5-2 also shows performance gains of baseline scheme(i.e., case 1). In addition, the generalizations of case 2 have large performance degradation, where generalization performance reduces more than 0.05. However, the case 3 generally shows better generalization performance than case 2, where the SGCS performance loss is less than 0.02..

- Table 5-2 SGCS for various antenna port number

| Model training | Model testing:Overhead:total 644bits with each layer for 322bits SGCS: Layer1/Layer2 SGCS gains: (Layer1/Layer2) |  |
| --- | --- | --- |
|  | 16 Ports(Zero padding to 32 ports) | 32 Ports |
| 16 Ports(Zero padding to 32 ports) | 0.9470/0.9047 (0/0) | TBD |
| 32 Ports | 0.9151/0.8525 (-0.0319/-0.0522) | 0.9218/0.8661 (0/0) |
| Mixed: 16 Ports:32 Ports = 1:1 | 0.9553/0.9168 (0.0083/0.0121) | 0.9136/0.8534 (-0.0082/-0.0127) |

- Observation 16: Case 3(AI model trained with mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

- Observation 17: AI model trained with the configuration of 32 antenna ports can maintain performance for 16 antenna ports.

- Proposal 11: Case 3(AI model trained with mixed dataset) can be utilized to study AI generalization as a starting point.

- Preliminary Evaluation Results for CSI Prediction

We perform a preliminary simulation on the sub-use case of CSI prediction to evaluate the performance of AI models and the nearest sampled CSI is selected for non-AI baseline comparison. For AI/ML model, detailed training parameters are listed in Table 9-3. The channel data is divided into three parts which are training, validation, and testing datasets with 80000, 2000, and 2000 samples respectively. In our evaluation, historical channel measurements are used as the input of AI models for training, validation and testing. The parameter number of our AI/ML models is about 14.3K. In addition, the FLOPs of prediction in a future occasion are 3.9M.

  - Performance comparison with non-AI approach

In this section, we initially evaluate the intermediate KPIs, e.g. NMSE, between our AI approach and the non-AI baseline. We simulate 4 historical CSIs and 10 historical CSIs as the input of AI model to predict 3 CSIs in future occasions, respectively. The results are shown in Table 6-1 and Table 6-2. According to the simulation results, the AI based CSI prediction completely outperforms non-AI baseline in both cases.

- Table 6-1 CSI prediction based on 4 historical CSIs

| # Predicted time | # P=1 (+5ms) |  | # P=2 (+10ms) |  | # P=3 (+15ms) |  |
| --- | --- | --- | --- | --- | --- | --- |
| # Type | # AI | # Non-AI | # AI | # Non-AI | # AI | # Non-AI |
| # NMSE(dB) | - -12.33 | - 0.6817 | - -6.482 | - 4.291 | - -5.322 | - 3.756 |

- Table 6-2 CSI prediction based on 10 historical CSIs

| # Predicted time | # P=1 (+5ms) |  | # P=2 (+10ms) |  | # P=3 (+15ms) |  |
| --- | --- | --- | --- | --- | --- | --- |
| # Type | # AI | # Non-AI | # AI | # Non-AI | # AI | # Non-AI |
| # NMSE(dB) | - -19.09 | - 1.174 | - -10.013 | - 4.284 | - -6.29 | - 3.367 |

Observation 18: AI-based CSI prediction completely outperforms the non-AI method.

Observation 19: The prediction accuracy of AI-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can still maintain its performance for a long time.

Observation 20: The prediction accuracy of AI-based approach improves with the increasing number of historical CSIs as model input.

  - Diverse historical CSI samples for AI CSI prediction

In order to evaluate the performance influenced by the length of observation window, we set diverse samples of historical CSIs as AI model inputs.The simulation results are shown in Table 6-3. According to the simulation results, the prediction accuracy of AI-based approach improves with the increasing number of historical CSIs as model input..

- Table 6-3 CSI prediction based on diverse samples of historical CSIs

| # Historical samples | - AI,T=1 | - AI,T=2 | - AI,T=3 | - AI,T=4 | - AI,T=6 | - AI,T=8 | - AI,T=10 | - Non-AI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # NMSE(dB) | - -0.8175 | - -8.205 | - -10.34 | - -12.12 | - -15.99 | - -17.42 | - -17.70 | - 0.0688 |

Observation 21: The prediction accuracy of AI-based approach presents positive correlation with the number of historical CSIs as model input.

- Proposal 12: The number of observation window and prediction window need to be discussed at least for calibration purpose.

- Conclusion

In this contribution, we discuss the evaluations on AI/ML for CSI feedback enhancement, and provide preliminary simulation results. We have the following observations and proposals.

Proposal 1: Full buffer traffic model can be optionally taken for calibration purpose and SI conclusions should be based on evaluation results of FTP traffic models.

Proposal 2: Intermediate results should be calculated based on ideal channel and AI/ML model output at least for calibration purpose.

Proposal 3: Type I CB is not necessary to be taken as a baseline for performance evaluation.

Proposal 4: Method 3 can be adopted as a baseline for SCGS calculation when rank>1.

Proposal 5: Case 2A can be listed as an independent case for verifying model generalization

Proposal 6: For rank>1, two cases on model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point.

- Case 1: Single layer in model input and single layer in model output

- Case 2: Multiple layers in model input and multiple layers in model output

- Proposal 7: For CSI dataset construction, training dataset and inference dataset should be separated in different drops at least for calibration purpose.

Proposal 8: Evaluation assumptions for AI-based CSI prediction can reuse agreed assumptions in Rel-18 MIMO for Type-II codebook refinement under high/medium velocities as much as possible.

Proposal 9: For AI-based CSI prediction, the raw channel matrix on PRBs as model input/output can be considered for intermediate evaluations at least for calibration purpose.

Proposal 10: The case of rank>1 should be prioritized in later discussion.

- Proposal 11: Case 3(AI model trained with mixed dataset) can be utilized to study AI generalization as a starting point.

- Proposal 12: The number of observation window and prediction window need to be discussed at least for calibration purpose.

Observation 1: AI based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS for rank=1. With the same feedback overhead, AI based CSI recovery can obtain 7%-8% SGCS gains.

Observation 2: AI based CSI reconstruction achieves very little UPT gain for the case of rank=1 and less than 1% average UPT gain over Rel-16 eTypeII with the same feedback overhead.

Observation 3: AI based CSI reconstruction with Case 1 method (single-layer model input and single-layer model output) shows performance gains in SGCS per layer over the Rel-16 eType II for rank =2.

Observation 4: Case 1(single-layer model input and single-layer model output) can achieve better performance than Case 2 (multi-layer model input and multi-layer model output).

Observation 5: With maximum rank up to 2 under the same feedback overhead, AI based CSI recovery has about 4%-6% average UPT gain over the Rel-16 eType II under the case of 50% RU and 5%-8.5% average UPT gain can be obtained by AI based CSI recovery under the case of 70% RU.

Observation 6: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for maximum rank up to 2.

Observation 7: AI/ML approaches show better throughput performance with heavy traffic load for up to rank=2.

Observation 8: AI based CSI reconstruction shows performance gain in SGCS per layer over the Rel-16 eType II for rank =3/4.

Observation 9: AI based CSI reconstruction shows larger performance gains in layer 3/4 than layer 1/2 in terms of SGCS with the assumption of the same feedback overhead of each layer.

Observation 10: For up to rank=4, with the same feedback overhead, AI based CSI reconstruction has about 4%-13% average UPT gain over the Rel-16 eType II under the case of 50% RU and 6%-16% average UPT gain can be obtained by AI based CSI reconstruction under the case of 70% RU.

Observation 11: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to rank = 4.

Observation 12: AI/ML approaches show better throughput performance with heavy traffic load for up to rank = 4.

- Observation 13: Case 3(AI model trained with mixed deployment scenarios) shows good generalization performance for various deployment scenarios.

Observation 14: The AI model trained with the scenario of UMa/UMi shows good generalization performance for InH dataset, while the AI model trained with the scenario of InH shows generalization degradation for UMa/UMi dataset.

Observation 15: AI/ML approaches can achieve good generalization performance for the case that the training/validation dataset and testing dataset are generated with different bandwidth configurations.

- Observation 16: Case 3(AI model trained with mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

- Observation 17: AI model trained with the configuration of 32 antenna ports can maintain performance for 16 antenna ports.

Observation 18: AI-based CSI prediction completely outperforms the non-AI method.

Observation 19: The prediction accuracy of AI-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can still maintain its performance for a long time.

Observation 20: The prediction accuracy of AI-based approach improves with the increasing number of historical CSIs as model input.

Observation 21: The prediction accuracy of AI-based approach presents positive correlation with the number of historical CSIs as model input.

- References

- Chairman’s notes, RAN1 #110.

- R1-2208522, Discussion on other aspects for AI CSI feedback enhancement, ZTE Corporation.

- Chairman’s notes, RAN1 #109-e

- R1-2203248, Evaluation assumptions on AIML for CSI feedback, ZTE Corporation

- Appendix

Table 9-1 SLS assumptions for AI/ML based CSI feedback

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. |
| MIMO layers |  | Maximum MU 4 layers for max rank=1 Maximum MU 8 layers for max rank=2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback) : 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | 2 OFDM symbols for PDCCH，type 2 for DMRS(24 REs/PRB/slot) CSI-RS overhead(32 REs/PRB/5 slot) |
| Traffic model |  | FTP 3 |
| Traffic load (Resource utilization) |  | RU 50% and 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 TypeII Codebook |

Table 9-2  Training parameters of AI/ML model for CSI compression

| Parameter | Value |
| --- | --- |
| Backbone | Transformer |
| Parameter type | Real value |
| Input CSI type | Eigenvectors of the ideal channel matrix estimated by UE |
| Output CSI type | Recovered eigenvectors by AI/ML model in gNB |
| Data-processing | Normalization |
| Quantization | Vector quantization |
| Training dataset | 600000 |
| Validation dataset | 10000 |
| Testing dataset | 20000 |
| Batch size | 400 |
| Optimizer | Adam |
| Loss function | MSE |

Table 9-3  Training parameters of AI/ML model for CSI prediction

| Parameter | Value |
| --- | --- |
| Backbone | ResNet |
| Parameter type | Real value |
| Input CSI type | Historical channel matrices measured by UE |
| Output CSI type | Predicted channel matrix by AI/ML model in UE |
| Training dataset | 80000 |
| Validation dataset | 2000 |
| Testing dataset | 2000 |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |