3GPP TSG RAN WG1 #110bis-e                                              R1-2208543

e-Meeting, October 10th – 19th, 2022

Agenda Item:     9.1.3.2

Source:	Spreadtrum Communications

Title:	Discussion on SRS enhancement targeting TDD CJT and 8 TX operation

Document for:	Discussion and decision

Introduction

In this contribution, we provide our views on SRS enhancement in aspects of interference randomization, capacity enhancement, 8Tx operation etc.

Scope of SRS enhancement in Rel-18

Agreement For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply): - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization; - RAN1 should strive to minimize the number of schemes supported in Rel-18 - SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation. - Target usage includes antenna switching, codebook/non-codebook based SRS

For the implementation of TDD CJT, enhancement on SRS interference management is necessary. The necessity of SRS capacity enhancement is for the requirement of more CSI measurement on cross-channels. Besides, TDD CJT requires high accurate channel estimation but SRS capacity enhancement may also cause severe interference. For solving these issues, interference randomization is also necessary. From our perspective, both SRS capacity enhancement and interference randomization are critical. Considering many alternatives proposed for both of the two categories, we may make a down-selection first in this meeting.

Proposal 1: SRS capacity enhancement and interference randomization are both critical for TDD CJT, they should be simultaneously studied in Rel-18.

TRP-specific SRS versus Common SRS

The CSI of TDD downlink mainly relies on channel reciprocity from estimation on SRS reception. For Rel-18 TDD CJT operation, UE may be simultaneously served by up to 4 TRPs doubling the supported TRP# in Rel-17. If we use the common SRS design meaning one SRS resource (set) can be shared among multiple TRPs, the power control of such SRS can be extremely difficult considering different location of TRPs and different channel features. Thus, it is natural to introduce TRP-specific SRS resource/set. Besides, considering the overhead issue, we more prefer to realize TRP-specific SRS in a resource level instead of set level as shown in Fig.1.

Figure.1 Structure of TRP-specific SRS

Proposal 2: Support to introduce TRP-specific SRS in Rel-18.

SRS interference randomization

CJT requires high accuracy of channel estimation, and even a slight nuance may trigger remarkable performance degradation. Thus, how to minimize the inter-SRS interference may become a bottleneck for CJT implementation.

Currently, SRS can be configured with hopping or not. For non-hopping SRS, the frequency location is relatively fixed and may suffer sever inter-SRS interference from other cells. Though hopping SRS can alleviate this issue in some degree, the interference could still be a critical problem for CJT implementation. For hopping SRSs with same high layer configurations, their patterns are exactly same and frequent mutual interference can be expected. Thus, we want to optimize SRS hopping procedure in Rel-18 by introducing some randomization factors. For example, C-RNTI and Cell-ID can be considered as a starting point for hopping pattern enhancement. As shown in Fig.2, even if the two users have same SRS configuration, but according to their unique randomization factor(s), the actual SRS patterns could be totally different.

Figure.2 Randomized SRS hopping pattern

Besides, as part of SRS hopping procedure the partial sounding may also suffer serious inter-SRS interference. We should also consider appropriate randomization technique for partial sounding procedure.

Proposal 3: Support to use C-RNTI or Cell-ID to randomize SRS hopping pattern.

In current specification, the cyclic shift used for a specific antenna port is fixed in a given configuration as shown below. If there is another UE in another cell has the same SRS configuration, they shall interfere heavily with each other.

αi=2πnSRScs,inSRScs,max nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise,

Similar issue also exists for the comb position, as shown below, the comb position of SRS is also fixed in a given configuration. Thus, inter-SRS interference may occur consistently.

The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise noffsetFH=b=0BSRSmSRS,bNscRBnb noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

To alleviate the interference, Comb/CS randomization could be considered. For example, the Comb/CS may change from time to time depend on the transmission occasion or some cell-specific/UE-specific factors.

Proposal 4: Comb/CS randomization could be considered.

SRS capacity enhancement

Increasing maximum CS# can directly enhance SRS capacity. In previous release, we have three kinds of 
KTC and each corresponds a maximum CS#. In Rel-18, we could at least increase nSRScs,max to 12 for KTC=2.

Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC

| KTC | nSRScs,max |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 | 6 |

Besides, TD-OCC is another way of increasing SRS capacity. We already have DFT-based OCC sequences for PUCCH and Hadmard OCC sequences for DMRS/CSI-RS in previous specification. As a starting point, both DFT-based OCC sequence and Hadmard OCC sequence could be considered as candidates for SRS TD-OCC.

In Rel-17, the supported PF factors are 2 and 4. Enlarging PF can also directly increase SRS capacity. Thus, we recommend to study larger PF values in Rel-18, e.g., PF=6, 8, 12, 16, 20 etc.

Proposal 5: Larger value of nSRScs,max for SRS should be studied in Rel-18.

Proposal 6: As a starting point, both DFT-based OCC sequence and Hadmard OCC sequence could be considered as candidates for SRS TD-OCC.

Proposal 7: Larger PF factors could be studied, e.g., PF=6, 8, 12, 16, 20 etc.

8 Tx UL operation

Agreement For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘antennaSwitching’; - FFS 8 ports in one or multiple SRS resources for ‘codebook’ Above does not imply support for 8 ports in one or multiple OFDM symbols Agreement For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR) Agreement For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. - FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols. Agreement For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

In last meeting, 8-port in one symbol is agreed. From our perspective, the maximum port per-symbol of legacy SRS should also be supported in Rel-18 which is 4 per-symbol. Besides, such constraint is also beneficial for co-existence with legacy SRS resources.

Proposal 8: Support 8-port SRS in multiple OFDM symbols, i.e., 4 ports per symbol.

Conclusion

In this contribution, we provide our view on SRS enhancement in Rel-18. The following proposals are achieved:

Proposal 1: SRS capacity enhancement and interference randomization are both critical for TDD CJT, they should be simultaneously studied in Rel-18.

Proposal 2: Support to introduce TRP-specific SRS in Rel-18.

Proposal 3: Support to use C-RNTI or Cell-ID to randomize SRS hopping pattern.

Proposal 4: Comb/CS randomization could be considered.

Proposal 5: Larger value of nSRScs,max for SRS should be studied in Rel-18.

Proposal 6: As a starting point, both DFT-based OCC sequence and Hadmard OCC sequence could be considered as candidates for SRS TD-OCC.

Proposal 7: Larger PF factors could be studied, e.g., PF=6, 8, 12, 16, 20 etc.

Proposal 8: Support 8-port SRS in multiple OFDM symbols, i.e., 4 ports per symbol.

References

[1] 3GPP TS 38.211, “Physical channels and modulation,” v17.2.0, 2022; ftp://ftp.3gpp.org.