3GPP TSG RAN WG1 #110bis-e		    R1- 2208547

e-Meeting, October 10th – 19th, 2022

Agenda Item:     9.2.2.1

Source:	Spreadtrum Communications, BUPT

Title:	             Discussion on evaluation on AIML for CSI feedback enhancement

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, one SID on AI/ML for Air interface is approved [1]. Three use cases are identified as initial set of use case shown below, and representative sub use cases should be recognized by RAN#98.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In RAN1#109e [2], spatial-frequency domain CSI compression using two-sided AI model has been agreed as one representative sub use for CSI enhancement shown below.

Agreement Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case. - Note: Study of other sub use cases is not precluded. - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

In this paper, we would focus on the sub use case: spatial-frequency domain CSI compression, and present initial evaluation results about the sub use case.

Discussion

AI model for CSI compression and recovery

In the previous meeting, it has been agreed that for the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part and an AI/ML-based CSI recovery part. So in the simulation, we use the Transformer model, where the encoder as the CSI generation part is located at the UE side and the decoder as the CSI recovery part is located at the gNB side, as shown in figure 1. The encoder mainly includes the embedding layer, multi-head attention layer and quantization layer. The decoder mainly includes the dequantization layer, embedding layer and multi-head attention layer. After the decoder, the CSI can be recovered through a fully connected layer. The AI model parameters are shown in table 1.

Table 1. AI model parameter for CSI compression and recovery

| Transformer Parameters | Values |
| --- | --- |
| Embedding dimension | 512 |
| Model Layer | 10 |
| Head of attention layer | 16 |
| Quantification method | Vector quantization |
| Quantization bit | 2 |

Figure 1. AI model for CSI compression and recovery

Evaluation results for CSI compression and recovery

In this section, we provide the data set generation, training parameter and evaluation results. For the data generation, we consider a Dense Urban (Macro only) scenario operating on 2GHz FDD spectrum, where 1140 UEs (57 cells, and 20 users per cell) are generated. The number of frequency domain sub-bands is 12. The detailed evaluation assumptions can refer to the Appendix. We collect 570000 samples, 90% of which are used as training sets and 10% as validation sets. The input of AI model is eigenvector of the channel matrix. The main training parameters are shown as table 2.

Table 2. Training parameter for CSI compression and recovery

| Training parameters | Value |
| --- | --- |
| Batch size | 512 |
| Epoch | 200 |
| Optimizer | Adam |
| Learning rate | 0.0001 |

In the last meeting [2], it has been agreed that SGCS (Squared Generalized Cosine Similarity) can be used as the intermediate KPIs. Therefore, based on the trained AI model, we provide the evaluation results on SGCS in figure 2, and the Rel-16 eType II codebook is used as the baseline for performance evaluation.

Figure 2. Comparison of SGCS for AI based CSI compression and recovery

From the evaluation results in figure 2, AI based CSI compression and recovery can achieve about 14% gain than Rel-16 eType II codebook for the same feedback bits. Accordingly, AI based CSI compression and recovery can save more than 50% of feedback bits than Rel-16 eType II codebook for the same SGCS. Therefore, it can be concluded that the AI based CSI compression and recovery can achieve better SGCS performance and lower feedback bits than Rel-16 eType II codebook.

Observation 1: The AI based CSI compression and recovery can achieve better SGCS performance and lower feedback bits cost than Rel-16 eType II codebook.

In practical deployment, if different models are separately trained for different scenarios, several AI models need to be deployed at UE side or delivered to UE. The cost is too large. Therefore, the generalization of AI model is very important, and the following agreement about generalization was achieved in the last meeting[2].

Agreement The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - FFS other cases for generalization verification, e.g., - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

For case 2, we use the model trained in the dense urban (Macro only) scenario to compare the inference performance in UMa (Urban Macro) and UMi (Urban Micro) with Rel-16 eType II codebook. The performance comparison results of SGCS are shown in Figure 3.

Figure 3. The verification of generalization for AI based CSI compression and recovery

According to figure 3, in both UMa and UMi scenarios, the performance of AI based CSI compression and recovery trained under the dense urban (Macro only) scenario is also better than that of Rel-16 eType II codebook. The performance of AI based CSI compression and recovery under UMa scenario is better than that under UMi scenario, since UMa scenario is similar to dense urban (Macro only) scenario.

Observation 2: The AI based CSI compression and recovery trained under different scenarios can also achieve better SGCS performance than Rel-16 eType II codebook.

Conclusion

In this contribution, we have the following proposal and observation:

Observation 1: The AI based CSI compression and recovery can achieve better GCS/SGCS performance and lower feedback bits cost than Rel-16 eType II codebook.

Observation 2: The AI based CSI compression and recovery trained under different scenarios can also achieve better GCS/SGCS performance than Rel-16 eType II codebook.

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Draft Report of 3GPP TSG RAN WG1 #109-e

- Appendix: Evaluation assumption for AI/ML based CSI feedback enhancement

Table 3. SLS assumptions for AI/ML based CSI feedback enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. |
| Frequency Range |  | 2GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Ideal channel estimation |
| Evaluation Metric |  | GCS and SGCS |
| Baseline for performance evaluation |  | Rel-16 eType II Codebook is the baseline. |