3GPP TSG-RAN WG1 Meeting #110bis-e		R1-2208580

e-Meeting, October 10th – 19th, 2022

Title:	[DRAFT] Reply LS on Terminology Alignment for Ranging/Sidelink Positioning

Response to:	S2-2207129

Release:	Release 18

Work Item:	FS_NR_pos_enh2/FS_Ranging_SL

Source:	vivo [RAN1]

To:	SA2

Cc:	RAN2, RAN3

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 thanks SA2 for the LS asking if the terminologies defined by SA2 are aligned and if any further updates are needed. RAN1 would provide the following answers to SA2’s question.

Q1:	SA2 asks that terminologies of Ranging, SL Reference UE, Target UE, Assistant UE, Located UE, Sidelink Positioning, Positioning, and Relative position, defined in SA2 are either aligned or mapped with RAN definitions.

Answer: In RAN1’s view, the Target UE is UE to be positioned which is aligned with the definition of Target UE defined in SA2. And, the Anchor UE in RAN1 is defined as the UE supporting positioning of the target UE which can include the SL Reference UE, Assistant UE, and Located UE defined in SA2. Besides, the Sidelink positioning in RAN1 is defined as Positioning UE using reference signals transmitted via PC5, which is including absolute position, relative position, or ranging.

Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface). - Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information. - Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE. - Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes. - SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity. - Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion. Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. - FFS: clarification of the knowledge of the location of the anchor UE

2	Actions

To SA2

RAN1 respectfully asks SA2 to take the above information into account in related work.

3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #111			November 14th – 18th, 2022   			Toulouse

TSG RAN WG1 Meeting #112			February 27th – March 03rd, 2023		Athens