3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2208609

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | x | rev | x | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on missing field descriptions of SCI 2-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022/9/24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SCI format 2-C, the ‘Resource combinations’ field contians two FRIVs, and the ‘Lowest subChannel indices’ field contains two lowest subchannel indices of the two FRIVs indicated in the field ‘Resource combinations’, respectively. Similarly, the ‘Resource selection window location’ field contains two resource selection windows. However, the description of these field only define the size of the fields, while the contents are unspecified. Consequently, it’s unclear on how the field indicates each unit, e.g., whether the first tuple is associated to the MSBs, or the LSBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the first unit in each field is indicated by the MSBs, and the second unit is indicated by the LSBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on SCI format 2-C fields. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.4.1.3	SCI format 2-C

SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast.

===omitted===

If the 'Providing/Requesting indicator' field is set to 0, all the remaining fields are set as follows:

-	Resource combinations – 2∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y bits as defined in Clause 8.1.5A of [6, TS 38.214], where

-	the first tuple is indicated by the log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y MSBs, and the second tuple is indicated by the left bits.

===omitted===

-	Lowest subChannel indices – 2∙log2N subChannel SL bits as defined in Clause 8.1.5A of [6, TS 38.214] , where the log2N subChannel SL MSBs indicates the starting sub-channel of the first resource in the first tuple, and the starting sub-channel of the first resource in the second tuple is indicated by the log2N subChannel SL LSBs.

===omitted===

-	Resource selection window location – 2∙10+log2(10∙2μ) bits as defined in Clause 8.1.4A of [6, TS 38.214], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211] , where the 10+log2(10∙2μ) MSBs indicates the starting time location of the resource selection window and the ending time location is indicated by the 10+log2(10∙2μ) LSBs.