3GPP TSG RAN WG1 #110bis-e		     R1-2208617

e-Meeting, October 10th – 19th, 2022

Source:	vivo

Title:	Discussion on HARQ-ACK feedback for PDSCH scheduled by DCI format 4-1

Agenda Item:	8.12

Document for:	Discussion and Decision

- Introduction

In this contribution, we share our view on HARQ-ACK feedback for PDSCH scheduled by DCI format 4-1.

- HARQ-ACK feedback for PDSCH scheduled by DCI format 4-1

For enabling/disabling HARQ-ACK feedback for multicast PDSCHs, the following was captured in the latest specification [1].

A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212].

However, for DCI 4_1, there is no “Enabling/disabling HARQ-ACK feedback indication” filed. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, for the PDSCHs scheduled by DCI format 4_2, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format associated with the G-RNTI or the G-CS-RNTI. For the PDSCHs scheduled by DCI format 4-1, UE behaviour on whether to transmit HARQ-ACK feedback is not clear based on the current specification. Since there is no RRC parameter to configure the UE whether to provide the HARQ-ACK information for PDSCH receptions scheduled by DCI format 4_1 in this case, default UE behaviour should be specified. For example, the UE always provides the HARQ-ACK information for PDSCH receptions scheduled by DCI 4_1, or the UE always does not provide the HARQ-ACK information for PDSCH receptions scheduled by DCI 4_1. Considering that the former option can get better performance, it is slightly preferred.

Proposal: If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI,

- For the PDSCHs scheduled by DCI format 4_1, the UE provides the HARQ-ACK information for PDSCH receptions associated with the G-RNTI or the G-CS-RNTI.

- For the PDSCHs scheduled by DCI format 4_2, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format 4_2 associated with the G-RNTI or the G-CS-RNTI.

- Conclusion

In this contribution, we discuss HARQ-ACK feedback for PDSCH scheduled by DCI format 4-1, and the following proposal is made.

Proposal: If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI,

- For the PDSCHs scheduled by DCI format 4_1, the UE provides the HARQ-ACK information for PDSCH receptions associated with the G-RNTI or the G-CS-RNTI.

- For the PDSCHs scheduled by DCI format 4_2, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format 4_2 associated with the G-RNTI or the G-CS-RNTI.

References

- TS 38.213 NR Physical layer procedures for control (Release 17) V17.3.0 2022-09