3GPP TSG RAN WG1 #110bis-e		R1-2208644

e-Meeting, October 10th – 19th, 2022

Source:	vivo

Title:	Physical channel design framework for sidelink on unlicensed spectrum

Agenda Item:	9.4.1.2

Document for:	Discussion and Decision

- Introduction

The WID for NR sidelink evolution has been approved for Rel-18 in RAN#94e [1]. The channel access mechanisms and the physical channel design framework for sidelink on unlicensed spectrum were agreed for further investigation:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we will discuss the physical channel design framework for sidelink (SL) on unlicensed spectrum.

- Physical Channel design Framework

SL sub-channel and interlace

In RAN#109e and RAN1#110 meeting, the contiguous and interlaced sub-channel were discussed, and the following agreements were achieved [3] and [4]:

Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc. Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions - FFS: whether/how to address IBE (In Band Emission) impact Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission (if supported), at least the following candidates can be discussed: - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - FFS: Other resource allocation granularity, e.g., RB-level - 1 sub-channel equals K interlaces if sub-channel is supported - FFS details - Other candidates are not precluded - FFS: mapping of PSCCH to frequency resources - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc. Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

It has been agreed to reuse sub-channel as the frequency domain resource allocation granularity in the interlace RB-based PSSCH transmission, thus, it’s necessary to define the mapping rule between sub-channel and interlace. According to the agreements that one sub-channel equals K interlace, then whether K is fixed as 1 or can be (pre-)configured should be discussed first. Considering the sub-channel size is configurable from Rel-16, and such flexibility is beneficial (e.g., to tradeoff between the resource utilization and the control signaling overhead, etc.) and desirable from system perspective, the option that K could be (pre-)configured is preferred.

Proposal 1:1 One sub-channel equals K interlace, where K is (pre-)configured.

Regarding the sub-channel definition and the resource indication in wideband case, the operating scenario should be discussed first. In NR-U, the communication is between one gNB and one UE, and the gNB can control the operation bandwidth (i.e., one or multiple RB sets) of the UE for communication. Thus, although different bandwidths are supported between different UEs, the BWP size, positions, DCI contents, are not needed to be aligned among UEs.

However, this is a different story in the SL case, because different UEs supporting different bandwidths should be able to communicate with each other in the same resource pool. Specifically, there are several different scenarios as shown in Figure 1.

Scenario 1 is the communication in fully overlapped RB set(s) between two SL UEs as shown in Figure 1(a). The second scenario (Scenario 2) is the communication in partially overlapped between two SL UEs as shown in Figure 1(b). And the third scenario (Scenario 3) is the communication between a BW-limited UE (i.e., supporting only one RB set) and another advanced UE (WB UE, i.e., supporting more than one RB set) as shown in Figure 1(c).

Figure 1：Scenarios in SL-U

Proposal 2: The SL-U design should support SL communication different UEs capable of different operating bandwidths (e.g., supporting different RB sets) in the resource pool.

If 1 sub-channel is confined within 1 RB set, the sub-channel can be indexed as shown in Figure 5, which is similar as Rel-16. In Scenario 1, the resource allocation and frequency resource indication mechanism in Rel-16 can be largely reused. The receiver could easily identify that whether part of TB is not transmitted because of LBT failure through decoding FRIV. However, in this case, for Scenario 2 and Scenario 3, the sub-channel index within RB set 1 from these UEs’ perspective would be different, because the resource pool of one UE might only contain subset of RB sets of the resource pool of another UE. This leads to different sub-channel indexing in two pools, as well as starting sub-channel, FRIV, etc. Consequently, the frequency resource indication would be misaligned between UEs.

It is worth noting that, this issue not only happens between a pair of UEs performing unicast transmissions, in which case the issue may be somehow mitigated by exchanging the UE capability. It also affects the case of groupcast and broadcast transmissions, and the other UEs performing sensing but not intending to receive the packets, in which cases no UE capability reporting is possible.

For example, a BW-limited UE reserved a yellow color resource in Figure 2. The BW-limited UE only supports one RB set bandwidth, so it indicates this resource in SCI with sub-channel index 0 ~ sub-channel index 5. However, sub-channel index 0 ~ sub-channel index 5 means the red color resource to another WB UE performing SL sensing operation. Consequently, the WB UE may exclude wrong resources, or may select improper resources, due to the incorrect sensing results.

Figure 2: FRIV misunderstanding

Observation 1: The sub-channel indexing, resource indication, etc., may not be aligned between UEs capable of different operating bandwidths, resulting in many issues such as packet reception failure, incorrect sensing and resource selection, etc.

To solve this problem, one alternative is that sub-channel can be indexed separately within each RB set as shown in Figure 4. As there are sub-channels with same index in different RB sets, to make the indication clearer, an RB set indication would be used to indicate which RB set the FRIV refers to. If the FRIV in each RB set just indicates the allocated frequency resources belonging to the corresponding RB set only, when the UE wants to indicate resources spanning over multiple RB sets, multiple FRIVs as well as an RB set indication associated with each of them are needed.

Another alternative is that all the UEs maintain a same sub-channel indexing, starting sub-channel and max number of sub-channels, i.e., a common ‘virtual’ resource pool, which may be larger than the BWPs or actual resource pools configured for the UEs. This common resource pool is used for indicating SL resources allocated in the actual resource pool. In this case the resource indications, even from different resource pools in different RB sets, can be aligned. This mechanism is somewhat like the common RB grid introduced in NR R15, or common interlace grid introduced in NR-U.

For example, the UEs can be configured with a resource pool spanning RB set 2 and 3, and a common ‘virtual’ resource pool spanning all the RB sets (0, 1, 2, and 3). Then the UE can be additionally indicated with a starting sub-channel index for the actual resource pool as Figure 3. Hence, the starting sub-channel of BW-limited UE in its supporting bandwidth is not index 0 but the indicated sub-channel index. Thus, the FRIV in Rel-16 can be reused, but based on the common ‘virtual’ resource pool instead of the actual resource pool. Note that, the max number of sub-channel used for deriving the FRIV in SCI, etc., is given by the common ‘virtual’ resource pool.

Nevertheless, still there is another issue in Scenario 3. When WB UE indicates a wideband transmission with the SCI transmitted in the lowest RB set of PSSCH as Rel-16, the BW-limited UE cannot receive the transmission if it locates in a RB set other than the RB set where the SCI is sent. It may cause the resource collision problem because the BW-limited UE cannot identify the resources reserved by the SCI in the other RB sets. To resolve this issue, the UE should transmit SCI in every RB set where the allocated PSSCH spans, and avoid to reserve resources in RB set other than the RB sets of the receiver.

Observation 2: When a UE transmits a TB spanning multiple RB sets, if the SCI is sent only in the sub-channel of one RB set (e.g., lowest RB set), the other UEs operating in different RB sets (e.g., due to bandwidth limited) cannot receive the transmission, neither for sensing nor for packet decoding.

Considering the cons and pros, the first alternative seems to be slightly simpler with less specification impacts.

Proposal 3: The sub-channel is indexed within an RB set.

Figure 3: the starting sub-channel index and sub-channel number indicated to UEs

| Figure 4 | Figure 5 |
| --- | --- |

If 1 sub-channel spans multiple RB sets, RB set indication should be transmitted along with FRIV to indicate that in which RB set the sub-channels are allocated. Otherwise receiver cannot identify the specific resource location when there are same sub-channel index in multiple RB sets.

Proposal 4: RB set indication should be transmitted along with FRIV if the allocated TB or PSSCH spans multiple RB sets.

Proposal 5：The PSCCH is transmitted in one sub-channel in each allocated RB set.

Meanwhile, SL-U should support intra-cell guard band as NR-U to mitigate the interference between adjacent RB sets. Hence, when discussing sub-channel indexing, the impact of guard band should be considered. The guard band may be excluded from resource pool or sub-channel mapping as shown in Figure 4, or included as shown in Figure 6. RAN1 should discuss which one should be used.

If the guard band is excluded from resource pool or sub-channel mapping, the resources of sub-channel will be discontinuous in resource pool and the starting interlace is different in each RB set. What’s more, the size of some sub-channels in one RB set might be different with others when the number PRBs in this RB set is not an integer multiple of the number of interlace. While if the guard band is included in the sub-channel mapping, how to use the sub-channel including guard band need to be discussed, as the guard band cannot be used when the resources on both sides of the guard band are not used. In addition, as shown in Figure 6, some PRBs will be left. If these PRBs are indexed in a sub-channel, the sub-channel size would be different from others.

Proposal 6: When discussing sub-channel indexing, the impact of guard band should be considered.

Figure 6

PSCCH on unlicensed spectrum

The additional starting symbols within a slot has been discussed with the following agreement.

Agreement For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission

In SL transmission, the PSCCH is transmitted on a sub-channel aligns with the lowest sub-channel of the corresponding PSSCH. This concept can be reused when interlaced RB-based transmission is configured, i.e., the PSCCH is transmitted on one sub-channel aligns with the lowest sub-channel of the corresponding PSSCH. The number of PRBs for PSCCH can be {10, 12, 15, 20, 25}. Considering the number of PRBs in each interlace will be 10 or 11, the number of PRBs for PSCCH in SLU can be 10 or 20 PRBs, which correspond to 1 or 2 interlaces.

Proposal 7: The PSCCH is transmitted on one sub-channel. The number of PRBs for PSCCH can be 10 or 20 for interlaced transmission in unlicensed spectrum.

One remaining issue is when the transmitted TB spans multiple RB sets, whether the PSCCH is transmitted in one of the RB set or in every RB set that the TB is spanning. As discussed in section 2.1, it is preferred to configure PSCCH in each RB set and transmit PSCCH in all the available RB sets.

Proposal 8: PSCCH is configured in each RB set and can be transmitted in all the available RB sets.

Similar as in NRU, due to the uncertainty of the channel availability, the start of the channel available time may not be aligned with the slot boundary. In order to perform SL transmission as quickly as possible when channel is available, it was proposed to transmit the SL based on the mini-slot. i.e., adding additional starting symbol within a slot.

However, there are some issues to be considered. For example, the mini-slot based SL transmission requires multiple PSCCH transmission occasions within a slot. When wideband operation is supported, PSCCH transmission occasion within each RB set may be necessary. Therefore, if mini-slot based SL transmission is performed over a wide bandwidth larger than 20MHz, UE may need to perform PSCCH detection at the beginning of each mini-slot and in each RB set, which may far beyond the UE capability. Therefore, the PSCCH candidate positions should be carefully studied for mini-slot based SL transmission.

Proposal 9: Further study the potential issues (such as PSCCH candidate positions) for mini-slot based SL transmission.

PSFCH on unlicensed spectrum

In RAN1#110 meeting, the regulatory requirements such as OCB and PSD, the location of the PSFCH resources, etc., have not been decided yet.

Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded - FFS details of above alternatives

The PSFCH resource is one PRB for SL transmissions on licensed spectrum, which cannot fulfill the OCB requirement on unlicensed spectrum. Instead, the PSFCH transmission needs to spread over the nominal channel bandwidth. One straightforward solution is to reuse the interlace for PSFCH transmission, e.g., one sub-channel containing one interlace to perform PSFCH transmission. Since one interlace contains at least 10 PRBs, each PSFCH channel can transmit over multiple PRBs, e.g., by repetition, to meet the OCB requirement.

Proposal 10: Interlaced structure is supported for PSFCH transmission, i.e., each PSFCH transmission occupies an interlace.

In unlicensed spectrum, the channel availability is uncertain. On the other hand, the PSFCH transmission occasion is pre-determined according to the (pre-)configuration. If channel access mechanism (either Type-1 or Type-2x) is required for PSFCH transmission [5], in each (pre-)configured PSFCH transmission occasion, the channel may be busy, and consequently the PSFCH transmission may fail. In order to enhance the transmission possibility of PSFCH, multiple PSFCH transmission occasions can be supported. With multiple transmission occasions, each PSSCH is mapped to multiple candidate PSFCHs. When the first PSFCH transmission fails, UE can try to transmit PSFCH in the next occasions. In case the PSFCH transmission succeeds in an early PSFCH occasion, UE simply skips the PSFCH transmissions in the later occasions.

Figure 7 Multiple PSFCH occasions

The system level simulations are performed to evaluated the performance, with the results shown in Figure 8. In the simulations, type1 LBT is required for each PSFCH transmission. For the ‘one-shot PSFCH’ case, the Rel-16 PSFCH occasion and mapping rule is reused for HARQ-ACK feedback, i.e., only a single PSFCH occasion associated with a PSSCH. If the LBT fails for the PSFCH transmission, it is handled in the same way as Rel-16, i.e., treated as NACK due to DTX.  While in the ‘multiple PSFCH’ case, up to 3 additional PSFCH occasions are associated to a PSSCH, as discussed above. The simulation results show that the multiple PSFCH occasion can improve the UE average throughput by 22% compared to the one-shot PSFCH transmissions. More details of the simulation assumptions can be found in Annex I.

Figure 8 System performance based on the LBE channel access mechanism

Moreover, as COT sharing is supported in SL, COT sharing can be used for PSFCH transmission. Tx UE can dynamically trigger the PFSCH transmission when channel is available, e.g., when Tx UE obtain a COT. Tx UE can indicate the shared PSFCH resources in the SCI, then the Rx UE can transmit the PSFCH in the COT initiated by Tx UE.

Proposal 11: If LBT based channel access mechanism is required for PSFCH, multiple candidate PSFCH occasions can be supported for PSFCH transmission.

Proposal 12: Sharing the COT of Tx UE for PSFCH transmission is supported in SL-U.

S-SSB on unlicensed spectrum

In RAN#109e and 110 meeting, the S-SSB were discussed, and the following agreements were achieved [3]:

Agreement For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

- Regarding the options to meet OCB and PSD:

Among the four options, the interlace structure will not only increase the complexity of PSBCH decoding but also increase difficulty in frequency offset compensation and time tracking based on S-PSS/S-SSS, thus option 1 is not preferred. Moreover, it is suggested to reuse the R16 design as far as possible to save overhead, thus new sequence as suggested in option 4 should be avoided. FDM solutions are used in NRU, in which the DL SSB and other signals are transmitted over a set of contiguous RBs in a FDM manner. However, FDM solution is not applicable to SL-U. Firstly, it was agreed that the SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused as the baseline. In R16, S-SSB is outside the resource pool and other SL channels such as PSSCH/PSCCH are inside the resource pool. If this principle is inherited by SL-U, there are no SL channels that can be FDMed with the S-SSB. Even if this restriction is removed, e.g., by allowing the S-SSB inside the resource pool, there are still several issues. Firstly, as PSSCH/PSCCH is transmitted in units of interlace, how can FDM manner between S-SSB and PSSCH/PSCCH be guaranteed? Secondly, if S-SSB is also transmitted in units of interlace, UEs sharing the same sync reference would not be able to receive the PSSCH/PSCCH multiplexed with S-SSB from each other due to half-duplex constraint. Thirdly, even if PSSCH/PSCCH FDMed with the S-SSB is used to transmit dummy data, the S-SSB coverage decreases. In contrast, transmitting multiple S-SSBs repetitions is a much more straightforward solution. Coverage and reliability of S-SSB can be guaranteed by the combination of S-SSBs in the frequency domain.

Proposal 13: Option3 is supported to meet the OCB and PSD requirement.

- S-SSB candidates in time domain

In an unlicensed spectrum, the channel may be busy in the intended S-SSB candidate if a LBT channel access mechanism (either Type-1 or Type-2x) is required for S-SSB transmission [5], resulting in S-SSB transmission failure. There are only a few S-SSB candidates in Rel-16, i.e., only one S-SSB candidate per 160ms at 15 kHz. Once the LBT failure happens, the UE needs to wait another 160ms for S-SSB transmission, which would prolong the synchronization procedure of the UE as well as slowing down the formation of synchronization clusters in the SL system, degrading the synchronization performance, or even worse, breaking the sidelink system due to loss of synchronization. Thus, in the case that LBT is required for S-SSB transmission, more S-SSB candidates should be supported in the unlicensed spectrum due to the uncertainty of access. For example, for 15 kHz, 4 or 8 S-SSB candidate locations can be supported in SL-U.

Figure 9 more candidate S-SSB candidates

Proposal 14: If a LBT channel access mechanism is required for S-SSB transmission, more candidate S-SSB occasions in the time domain should be supported to increase the opportunities for S-SSB transmission for each S-SSB SCS, the number of S-SSB L is as follows:

- For S-SSB SCS=15kHz, L=1,2,4,8

- For S-SSB SCS=30kHz, L=1,2,4,8,16

- For S-SSB SCS=60kHz in FR1, L=1,2,4,8,16,32

Further, adjacent S-SSB bursts in R16 are either next to each other or equally distributed, and there is only one S-SSB candidate in each S-SSB burst. Once the LBT launched for an S-SSB burst fails, it has to wait until the next S-SSB burst and to perform a new LBT. Even if LBT to an S-SSB candidate success, a new LBT should be launched for the next S-SSB candidate due to the gap between the bursts, which also increases the latency and complexity of the sync procedure.

Figure 10 S-SSB bursts

Proposal 15: S-SSB burst containing more than one S-SSB candidate located on a set of consecutive time resources is supported. There are one or more S-SSB bursts in a S-SSB period.

In R16 SL, the time locations of the S-SSB candidates are (pre-)configured semi-statically. In SL-U, the time locations of the S-SSB candidates should also be semi-statically defined. Otherwise, UE has to perform a full search all the time. UE may successfully access the channel in the middle of a S-SSB candidate. To occupy the channel, one way is to send the remaining S-SSB or to send a dummy signal from the start time of the accessed channel. Another way is to send a complete S-SSB from the start time, in which case, the time resource occupied by the S-SSB is shifted relative to the (pre-)configured S-SSB candidate resource.

Proposal 16: The time locations of the S-SSB candidates are (pre-)configured semi-statically.

Proposal 17: If the LBT succeeds in the middle of a S-SSB candidate, UE transmits either the remaining S-SSB signal or a dummy signal from the start time of the accessed channel.

- whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from the resource pool

If all the S-SSB candidates are excluded from the candidate pool, the number of candidates available for SL communication would decrease dramatically and be interrupted frequently, leading to low candidate efficiency as well as lower channel access success rate. Therefore, to improve candidate utilization, at least some of the S-SSB candidates should not be excluded from the candidate pool. Specifically, the S-SSB candidate sets can be classified into two types:

- Type-1 S-SSB candidate sets, e.g., consist of R16 S-SSB candidates

- Type-2 S-SSB candidate sets, e.g., consist of the additional R18 S-SSB candidates, within the candidate pool

Type-1 S-SSB candidate sets are used for S-SSB transmission/reception only. On the other hand, Type-2 S-SSB may be used for S-SSB transmission or data transmission, but not both at the same time. If a Type-1 S-SSB candidate set is used for S-SSB transmission or if UE detects S-SSB on a Type-1 S-SSB candidate set, the corresponding Type-2 S-SSB candidate sets can be used for data transmission as Rel-16 SL. For example, UE access the first type-1 S-SSB candidate set as shown in Figure 11, and the corresponding Type-2 S-SSB candidate set is used for PSSCH/PSCCH instead of S-SSB. When LBT fails on the second S-SSB candidate set but succeeds on the 2nd Type-2 S-SSB candidate set, UE should use the Type-2 S-SSB candidate set for S-SSB transmission. In this case, the other SL transmissions are prohibited to use these resources.

Figure 11 Relationship between pool and S-SSB

Proposal 18: Support Type-1 S-SSB candidate sets and Type-2 S-SSB candidate sets, where Type-2 S-SSB candidate sets are not excluded from the candidate resource pool.

Proposal 19: Type-2 S-SSB candidate sets can be used for other SL channels if the corresponding Type-1 S-SSB candidate sets are used for S-SSB.

Further, S-SSB should carry COT-related information, such as the COT starting point and duration, for COT sharing.

Proposal 20: If UE is allowed to share the COT initialized by S-SSB, the S-SSB should include COT information.

Power control in unlicensed band

Operation in unlicensed spectrum is subject to different regulation requirements in some regions and frequency bands. To constrain the inter-RAT (Wi-Fi, WiGig radar) and intra-RAT interference levels, any technology attempted accessing to the unlicensed spectrum (like SL-U) should comply with the transmit power requirement, in terms of EIRP and PSD. For instance, according to the ETSI regulation, the maximum mean EIRP and PSD in the 5150-5350 MHz band are limited to 23 dBm and 10 dBm/MHz, respectively, while in the 5470-5725 MHz band are limited to 30 dBm and 17 dBm/MHz, respectively.

Open-loop power control is adopted in NR Sidelink to determine the transmit power. The existing power control mechanism only restricts the maximum transmit power (e.g., 23 dBm), but does not consider the PSD. For example, considering the transmit power requirement for SL-U UEs to operate in 5150-5350 MHz band, the Peak PSD can be achieved if the SL-U UEs transmit with the maximum transmit power spreading over the 20MHz channels (23 dBm -10×log10 (20 MHz)=10 dBm/MHz). This implies that if the SL transmission with the maximum transmit power occupies with a smaller bandwidth (i.e., less than 20MHz), the PSD would exceed the PSD limit. This issue would be critical especially for PSFCH and S-SSB transmission, because this kind of transmission is very unlikely to occupy the full 20MHz channel bandwidth. Thus, SL-U power control should take the PSD limit into consideration.

Proposal 21: SL-U power control should take into account the PSD limit, especially for some SL channel such as PSFCH or S-SSB.

For a given PSD constraint of 10 dBm/MHz, the Peak PSD conducted from the SL-U transmission shall not be greater than 10 dBm in every 1MHz band. To make the full use of the maximum transmit power, SL-U power control under the PSD constraint should depend on the frequency domain distribution of the SL-U transmission. For example, according the Rel-16 power control mechanism, the transmit power is equally distributed over multiple PSFCHs to be transmitted in the same time, while the specific frequency domain location is irrelevant. However, on the unlicensed band, a (unevenly distributed) frequency location would exceed the PSD limit. As shown in Figure 12, when transmitting the same number of PSFCHs with transmit power of 5 dBm for each PSFCH, although the mapping of PSFCH occasion 1 (in the left) has no problem, the mapping of PSFCH occasion 2 (in the right) exceeds the peak PSD limit (10 dBm/MHz). Therefore, in order to comply with the PSD requirement, the power control mechanism should consider the frequency domain distribution of the PSFCH transmission.

Figure 12 Frequency domain distribution of PSFCH

Proposal 22: SL-U power control under the PSD constraint should consider the frequency domain distribution of the SL-U transmission.

- Conclusion

In this contribution, we focus on the physical channel design framework for SL on unlicensed spectrum, and have the following proposals:

Proposal 1:1 One sub-channel equals K interlace, where K is (pre-)configured.

Proposal 2: The SL-U design should support SL communication different UEs capable of different operating bandwidths (e.g., supporting different RB sets) in the resource pool.

Proposal 3: The sub-channel is indexed within an RB set.

Proposal 4: RB set indication should be transmitted along with FRIV if the allocated TB or PSSCH spans multiple RB sets.

Proposal 5：The PSCCH is transmitted in one sub-channel in each allocated RB set.

Proposal 6: When discussing sub-channel indexing, the impact of guard band should be considered.

Proposal 7: The PSCCH is transmitted on one sub-channel. The number of PRBs for PSCCH can be 10 or 20 for interlaced transmission in unlicensed spectrum.

Proposal 8: PSCCH is configured in each RB set and can be transmitted in all the available RB sets.

Proposal 9: Further study the potential issues (such as PSCCH candidate positions) for mini-slot based SL transmission.

Proposal 10: Interlaced structure is supported for PSFCH transmission, i.e., each PSFCH transmission occupies an interlace.

Proposal 11: If LBT based channel access mechanism is required for PSFCH, multiple candidate PSFCH occasions can be supported for PSFCH transmission.

Proposal 12: Sharing the COT of Tx UE for PSFCH transmission is supported in SL-U.

Proposal 13: Option3 is supported to meet the OCB and PSD requirement.

Proposal 14: If a LBT channel access mechanism is required for S-SSB transmission, more candidate S-SSB occasions in the time domain should be supported to increase the opportunities for S-SSB transmission for each S-SSB SCS, the number of S-SSB L is as follows:

- For S-SSB SCS=15kHz, L=1,2,4,8

- For S-SSB SCS=30kHz, L=1,2,4,8,16

- For S-SSB SCS=60kHz in FR1, L=1,2,4,8,16,32

Proposal 15: S-SSB burst containing more than one S-SSB candidate located on a set of consecutive time resources is supported. There are one or more S-SSB bursts in a S-SSB period.

Proposal 16: The time locations of the S-SSB candidates are (pre-)configured semi-statically.

Proposal 17: If the LBT succeeds in the middle of a S-SSB candidate, UE transmits either the remaining S-SSB signal or a dummy signal from the start time of the accessed channel.

Proposal 18: Support Type-1 S-SSB candidate sets and Type-2 S-SSB candidate sets, where Type-2 S-SSB candidate sets are not excluded from the candidate resource pool.

Proposal 19: Type-2 S-SSB candidate sets can be used for other SL channels if the corresponding Type-1 S-SSB candidate sets are used for S-SSB.

Proposal 20: If UE is allowed to share the COT initialized by S-SSB, the S-SSB should include COT information.

Proposal 21: SL-U power control should take into account the PSD limit, especially for some SL channel such as PSFCH or S-SSB.

Proposal 22: SL-U power control under the PSD constraint should consider the frequency domain distribution of the SL-U transmission.

- References

- RP-213678, WID on NR sidelink evolution, RAN #94e, Electronic Meeting, Dec. 6-17, 2021.

- ETSI EN 301 893, Harmonised Standard covering the essential requirements Harmonised Standard covering the essential requirements, v2.1.1, 2017-05.

- 3GPP RAN1#109-e, Chairman’s notes.

- 3GPP RAN1#110, Chairman’s notes.

- R1-2206041, “Channel access mechanism for sidelink on unlicensed spectrum”, vivo, RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- Annex I

Table 1 System level simulation assumption

| Parameter | Value |
| --- | --- |
| Deployment | Indoor |
| Communication type | Unicast |
| Carrier frequency | 6GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15KHz |
| Channel model | P2P link defined in TR37.885 |
| Link adaptive | Enable |
| Traffic model for SL | R17 sidelink commercial traffic model with periodic model 3 reduced by a factor of 5 |
| Channel access | LBE |
| Tx Power | 23 dBm |
| Antenna number | [M N P Mg Ng Mp Np]=[1 2 3 1 1 1 1] for SL-UE [M N P Mg Ng Mp Np]=[1 1 2 1 1 1 1] for AP |
| UE deploy | 10 pairs of UEs per 20MH, uniformly at random in the area |
| Max multiple PSFCH number | 3 |