3GPP TSG RAN WG1 #110bis-e                                                                                      R1-2208663

e-Meeting, October 10th - 19th, 2022

Source:	vivo

Title:	Discussions on network verified UE location for NR NTN

Agenda Item:	9.11.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the issues related to the UE location verification in NTN.

# Discussions

In RAN #96 meeting, network verified UE location related objectives have been discussed and updated as summarized in [1].

- Pending on the conclusion of the RAN SI FS_NR_NTN_netw_verif_UE_loc study item, study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3]. - RAN is expected to determine by RAN#98 whether the study has identified any need for Network verified UE location specification support in Rel-18.

Furthermore, a new study item has been created to study the accuracy of network verified UE location as summarized in [2].

- The objective of this RAN led SI is: - Study detailed regulatory requirements (e.g. accuracy, privacy, reliability, latency) for network-verified UE location for NTN and for potential use cases/services (i.e. emergency call, lawful intercept, public warning, charging/billing).

In RAN1 #110 meeting and the post meeting email discussions, most of the required evaluation assumptions have been agreed to evaluate the accuracy of RAT dependent positioning in NTN.

In this contribution, we provide our evaluation methodology, assumptions and initial evaluation results for RAT dependent positioning in NTN.

## 2.1	Methodology and evaluation assumptions

According to the assumptions agreed in RAN1, single satellite based positioning would be assumed, as illustrated in Figure 1.

Figure 1. RAT dependent positioning based on single satellite in NTN

In order to evaluate the RAT dependent positioning in NTN, DL-TDoA method is selected in our system level simulations where a set of UEs are dropped randomly in NTN cells. As is known, DL-TDoA positioning method makes use of the DL RSTD (and optionally DL-PRS-RSRP and DL-PRS-RSRPP) of downlink signals received from multiple TPs in terrestrial network, at the UE. The UE measures the DL RSTD (and optionally DL-PRS-RSRP and DL-PRS-RSRPP) of the received signals using assistance data received from the positioning server, and the resulting measurements are used along with other configuration information to locate the UE in relation to the neighboring TPs. For NTN, the method is almost the same as the one for TN except that the downlink signals are received at different time instances from single satellite.

Since most of the parameters agreed in RAN1 are expected to be reported by companies, we provide our evaluation assumptions provided in Table 1 below.

Table 1. Evaluation assumptions for RAT dependent positioning study in NTN

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural |
| Satellite Orbit | LEO-600km |
| Satellite parameters | Set-1satellite parameters provided in table 6.1.1.1-1 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1/2GHz, S-band |
| BW | 5MHz |
| Subcarrier spacing, kHz | 15kHz for FR1 |
| Number of satellites in view | 1 for single satellite case |
| Orbit inclination | 90 degree orbital plane relative to equator plane |
| UE type | Handheld terminal |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration as agreed under AI 9.12.1 |
| Positioning signals (Note 1) | PRS |
| Number of occasions per positioning estimate | 4 |
| Time window for measurement collection | 30s/60s/120s |
| Interference modelling (ideal muting, or other) | Idea muting |
| Reference Signal Transmission Bandwidth | 5MHz |
| Reference point for timing measurement | Satellite |
| Positioning algorithm | taylor series |
| UE speed | 3km/h |
| Maximum additional timing measurement error | {0, 20, 40, 60, 80, 100, 120} ns for 30s measurement gap; {0, 40, 80, 120, 160, 200, 240} ns for 30s measurement gap; {0, 60, 120, 180, 240, 300, 360} ns for 30s measurement gap. |
| Performance metrics | Horizontal accuracy (UE 2D position accuracy) |

## 2.2	Initial performance evaluation for DL-TDoA method

With the assumptions provided in previous section, positioning accuracy of DL-TDoA method in NTN are summarized in following figures and tables when different time gaps are assumed between 2 measurement time instances. Considering the satellite is moving which is different from TN, some additional timing errors within a range of {0 to 120ns} or {0 to 240ns} or {0 to 360ns} are included in order to analyze the maximum additional timing errors allowed to meet required positioning accuracy for verifying the UE location.

Figure 1. Positioning accuracy for a measurement gap of 30ns

Table 2. Positioning accuracy for a measurement gap of 30ns

| Max timing error(ns) | Accuracy(km) for 30s measurement gap |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| 0 | 0.18 | 0.35 | 0.88 | 1.77 |
| 20 | 0.5 | 0.91 | 0.146 | 2.56 |
| 40 | 0.97 | 1.53 | 2.61 | 4.53 |
| 60 | 1.41 | 2.09 | 3.14 | 6.29 |
| 80 | 1.75 | 2.61 | 3.92 | 7.48 |
| 100 | 2.36 | 3.33 | 5.89 | 9.35 |
| 120 | 2.69 | 3.65 | 6.99 | 12.24 |

Figure 2. Positioning accuracy for a measurement gap of 60ns

Table 3. Positioning accuracy for a measurement gap of 60 ns

| Max timing error(ns) | Accuracy(km) for 30s measurement gap |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| 0 | 0.19 | 0.34 | 0.6 | 1.12 |
| 40 | 0.42 | 0.72 | 1.11 | 2.31 |
| 80 | 0.91 | 1.55 | 2.58 | 4.84 |
| 120 | 1.19 | 1.96 | 3.52 | 6.26 |
| 160 | 1.53 | 2.23 | 3.8 | 7.83 |
| 200 | 2.13 | 2.87 | 4.96 | 9.2 |
| 240 | 2.74 | 4.44 | 6.96 | 10.67 |

Figure 3. Positioning accuracy for a measurement gap of 120ns

Table 4. Positioning accuracy for a measurement gap of 120ns

| Max timing error(ns) | Accuracy(km) for 30s measurement gap |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| 0 | 0.22 | 0.34 | 0.71 | 1.68 |
| 60 | 0.6 | 0.89 | 1.4 | 2.78 |
| 120 | 1.032 | 1.72 | 2.69 | 5.08 |
| 180 | 1.62 | 2.36 | 4.13 | 7.14 |
| 240 | 2.1 | 3.36 | 5.35 | 8.52 |
| 300 | 2.22 | 3.63 | 5.64 | 9.68 |
| 360 | 2.95 | 4.66 | 6.81 | 11.07 |

It can be observed that, with DL-TDoA method, the maximum timing measurement error that can be allowed to meet the accuracy requirement of 5-10km is about 100ns, 200ns and 300ns when the measurement gaps are 30s, 60s, 120s respectively. The larger the gap configured, the larger the additional timing measurement error is allowed to meet the target positioning accuracy.

Observation1:

- The maximum timing measurement error allowed to meet the positioning accuracy requirement of 5-10km is about 100ns when the measurement gap is 30s.

Observation2:

- The maximum timing measurement error allowed to meet the positioning accuracy requirement of 5-10km is about 200ns when the measurement gap is 60s.

Observation3:

- The maximum timing measurement error allowed to meet the positioning accuracy requirement of 5-10km is about 300ns when the measurement gap is 120s.

Observation4:

- The larger the measurement gap is configured, the larger the additional timing measurement error can be allowed to meet the target positioning accuracy.

Therefore, with proper measurement gap configuration, DL-TDoA method would be enough to meet the target positioning accuracy of 5 to 10km. Furthermore, DL-TDoA based method only relies on downlink signal measurement and does not depend on the UL TA report which has to be enhanced due to low granularity for other methods with UL measurement performed.

According to above, we have following proposal.

Proposal 1:

- Reuse existing DL-TDoA method already specified in TN which is enough for UE location verification in NTN.

# Conclusion

In this contribution, we provide the evaluation methodology, assumptions and initial evaluation results for RAT dependent positioning in NTN. According to the results, we have following observations and proposals:

Observation1:

- The maximum timing measurement error allowed to meet the positioning accuracy requirement of 5-10km is about 100ns when the measurement gap is 30s.

Observation2:

- The maximum timing measurement error allowed to meet the positioning accuracy requirement of 5-10km is about 200ns when the measurement gap is 60s.

Observation3:

- The maximum timing measurement error allowed to meet the positioning accuracy requirement of 5-10km is about 300ns when the measurement gap is 120s.

Observation4:

- The larger the measurement gap is configured, the larger the additional timing measurement error can be allowed to meet the target positioning accuracy.

Proposal 1:

- Reuse existing DL-TDoA method already specified in TN which is enough for UE location verification in NTN.

# References

- RP-222654, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, 3GPP TSG RAN #97-e meeting, September 12-16, 2022.

- RP-221820, New SID: Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR, 3GPP TSG RAN #96-e meeting, June 6-9, 2022.