3GPP TSG-RAN WG1#110-bis-e	R1-2208689

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | DRAFT | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on timing relationship for IoT-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2022-9-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The parameter names of CellSpecificKoffset and UESpecificKoffset are not aligned with that in higher layer specification TS 36.331 and TS 36.321. The higher layer parameter name of segment length is missing. Redundant definition of Kmac in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise CellSpecificKoffset to k-Offset to align with TS 36.331. Revise UESpecificKoffset to Differential Koffset to align with TS 36.321. Add the higher layer parameter names of segment length in the description of segmented pre-compensation. Remove redundant definition of Kmac. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The parameter names are not aligned between physical layer spec and higher layer spec. Redundant definition of Kmac will exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 5.1.1.1, 6.1.1, 7.2.3, 7.3, 8, 10, 16, 16.1.2, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.3	Transmission timing adjustments

<Unchanged parts are omitted>

For a BL/CE UE, for a timing advance command received on subframe n, the corresponding adjustment of the uplink transmission timing shall apply for the uplink PUCCH/PUSCH/SRS transmissions in subframe n+6+Koffset. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on the same narrowband and are overlapped due to the timing adjustment, the UE shall complete transmission of subframe n and is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on different narrowbands, and the timing adjustment occurs in the guard period for narrowband retuning, the UE is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n and which does not reduce the guard period. The value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

For a BL/CE UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the quantity Nsegmentprecompensation is provided by the higher layer parameter prach-TxDuration for PRACH, pucch-TxDuration for PUCCH, and pusch-TxDuration for PUSCH, as specified in 3GPP TS 36.331.

<Unchanged parts are omitted>

5.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	 and  if accumulation is enabled based on the parameter Accumulation-enabled or accumulationEnabledsTTI provided by higher layers and if the TPC command  is included in a PDCCH with DCI format 3/3A whose CRC parity bits are scrambled by TPC-PUSCH-RNTI and if the UE is configured with multiple UL SPS configurations.

-	where was signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or PDCCH/SPDCCH with DCI format 7-0A/7-0B or MPDCCH with DCI format 6-0A or PDCCH/MPDCCH with DCI format 3/3A on subframe/slot/subslot , and where  is the first value after reset of accumulation. For a BL/CE UE configured with CEModeA, subframe i-KPUSCH-Koffset is the last subframe in which the MPDCCH with DCI format 6-0A or MPDCCH with DCI format 3/3A is transmitted.

-	The value of Koffset for a BL/CE UE is

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

6.1.1	Timing

Throughout this clause, for a BL/CE UE, if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

otherwise,

-	Koffset=0, Kcell_offset=0.

<Unchanged parts are omitted>

7.2.3	Channel Quality Indicator (CQI) definition

<Unchanged parts are omitted>

-	In the time domain and for a BL/CE UE, the CSI reference resource is defined by a set of BL/CE downlink or special subframes where the last subframe is subframe n-nCQI_ref -Koffset,

-	where Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0;

<Unchanged parts are omitted>

7.3	UE procedure for reporting HARQ-ACK

<Unchanged parts are omitted>

Throughout this clause,

-	if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, and otherwise.

-	if the UE is configured with higher layer parameter shortTTI and for PDSCH transmissions in a subslot,  is determined based on higher layer parameter proc-Timeline-r15, where

-	= 4 if proc-Timeline-r15 is set to 'nplus4set1'

-	= 6 if proc-Timeline-r15 is set to 'nplus6set1' or 'nplus6set2'

-	= 8 if proc-Timeline-r15 is set to 'nplus8set2'

-	for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

8	Physical uplink shared channel related procedures

<Unchanged parts are omitted>

Throughout this clause, for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

10	Physical uplink control channel procedures

<Unchanged parts are omitted>

Throughout this clause,

-	if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,  and otherwise,

-	if the UE is configured with higher layer parameter shortTTI and the corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G is detected in a subslot,  is determined based on higher layer parameter min-proc-TimelineSubslot from .

-	for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

16	UE Procedures related to narrowband IoT

Throughout this clause,

-	for a NB-IoT UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

16.1.2	Timing synchronization

<Unchanged parts are omitted>

For a NB-IoT UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the quantity Nsegmentprecompensation is provided by the higher layer parameter nprach-TxDurationFmt01 for NPRACH format 0 and format 1, nprach-TxDurationFmt2 for NPRACH format 2, and npusch-TxDuration for NPUSCH, as specified in 3GPP TS 36.331.

<Unchanged parts are omitted>

16.6	Narrowband physical downlink control channel related procedures

Throughout this clause, if a NB-IoT UE is configured with higher layer parameter k-Mac, Kmac = k-Mac otherwise, Kmac = 0.

<Unchanged parts are omitted>

If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-durationKmacKmac=0. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration.

<Unchanged parts are omitted>