3GPP TSG RAN WG1 #110bis-e	R1-2208706

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment CR on the parameter names in TS 38.213 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  |  |  | Date: |  |  |  | 2022-10-10 |
| Category: | A |  |  |  |  |  |  |  | Release: |  |  |  | Rel-17/16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Misalignment of the parameter names of COT duration and CSI-RS validation between TS 38.213 and TS 38.331. 38.331-h20 CO-DurationsPerCell-r16 ::= SEQUENCE { servingCellId-r16 ServCellIndex, positionInDCI-r16 INTEGER(0..maxSFI-DCI-PayloadSize-1), subcarrierSpacing-r16 SubcarrierSpacing, co-DurationList-r16 SEQUENCE (SIZE(1..64)) OF CO-Duration-r16 } csi-RS-ValidationWithDCI-r16 ENUMERATED {enabled} |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alignment of the parameter names between TS 38.213 and TS 38.331, that is: - Change “csi-RS-ValidationWith-DCI” to “csi-RS-ValidationWithDCI” - Change “CO-DurationPerCell-r16” to “CO-DurationsPerCell-r16” |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation error may occur due to the non-existence of the parameter |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1, 11.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  |  |  |
| (show related CRs) |  |  | X |  |  | O&M Specifications |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |

11.1	Slot configuration

<Unchanged parts are omitted>

For a UE operation with shared spectrum channel access in FR1, or in FR2-2 when the UE is provided ChannelAccessMode2-r17 = 'enabled', if the UE is provided csi-RS-ValidationWithDCI, is not provided CO-DurationsPerCell, and is not provided SlotFormatCombinationsPerCell, and if the UE is configured by higher layers to receive a CSI-RS in a set of symbols of a slot, the UE cancels the CSI-RS reception in the set of symbols of the slot if the UE does not detect a DCI format indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot.

<Unchanged parts are omitted>

11.1.1	UE procedure for determining slot format

<Unchanged parts are omitted>

-	a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationsPerCell, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from co-DurationList. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by co-DurationList. If CO-DurationsPerCell is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

-	a reference SCS configuration for co-DurationList, by subcarrierSpacing

-	a location of a search space set group switching flag field in DCI format 2_0, by SearchSpaceSwitchTrigger, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling for the serving cell or the set of serving cells, provided by CellGroupsForSwitching, as described in clause 10.4.

If neither CO-DurationsPerCell-r16 nor SlotFormatCombinationsPerCell are provided and if channelAccessMode = "semiStatic" is provided, the procedures in this clause apply with assuming a channel occupancy time defined in clause 4.3 of [15, TS 37.213] is the remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time.