3GPP TSG RAN WG1 Meeting #110bis-e	R1-2208719

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | draft | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on misalignment for RRC parameters in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRC parameter name in TS38.214 is inconsistent with that in TS38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct RRC parameter name in TS38.214 to align with TS38.331； 1. Parameter ‘determineResourceSetTypeScheme1’ is modified to ‘sl-DetermineResourceType’; 2. Parameter ‘condition1A2Scheme1Disabled’ is modified to ‘sl-Condition1-A-2’; 3. Parameter ‘thresholdRSRPCondition1B1Option1Scheme1’ is modified to ‘sl-ThresholdRSRP-Condition1-B-1-Option1List’; 4. Parameter ‘thresholdRSRPCondition1B1Option2Scheme1’ is modified to ‘sl-ThresholdRSRP-Condition1-B-1-Option2List’; 5. Parameter ‘interUECoordinationScheme2’ is modified to ‘sl-IUC-Scheme2’; 6. Parameter ‘interUECoordinationScheme1’ is modified to ‘sl-InterUE-CoordinationScheme1’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment on RRC parameters between specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1	UE procedure for transmitting the physical sidelink shared channel

<Unchanged parts are omitted>

-	if [Providing/Requesting indicator] indicates SCI format 2-C is used to convey an explicit request for inter-UE coordination information:

-	the UE shall set value of the ['Priority'] field as indicated by higher layers.

-	the UE shall set value of the ['Number of subchannels'] field as indicated by higher layers.

-	the UE shall set value of the ['Resource reservation period'] field as indicated by higher layers.

-	the UE shall set value of the ['Resource selection window location'] field as indicated by higher layers.

-	the UE shall set value of the ['Resource set type'] field as indicated by higher layers if higher layer parameter sl-DetermineResourceType is configured to 'UE-B's request'; otherwise this field is omitted.

<Unchanged parts are omitted>

8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

<Unchanged parts are omitted>

When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications:

-	Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met:

-	the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined;

-	the higher layer parameter sl-Condition1-A-2 is not set to 'Disabled'.

When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s):

-	resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria:

-	the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option1List, where k=pi.

-	the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option2List, where k=pi.

-	resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined.

8.1.4B	UE procedure for determining a resource conflict

A UE configured with the higher layer parameter sl-IUC-Scheme2 enabling transmission of a resource conflict indication considers that a resource conflict occurs on a first reserved resource r1 indicated by a first received SCI format if at least one of the following conditions is satisfied:

-	the first reserved resource r1 overlaps with a second reserved resource r2 indicated by a second received SCI format, and

-	if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 1 enabled],

-	if the UE is a destination UE of a TB to be transmitted in r1, the RSRP measurement performed for the second received SCI format RSRP2 is higher than Thprio2,prio1 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively.

-	if the UE is a destination UE of a TB to be transmitted in r2, the RSRP measurement performed for the first received SCI format RSRP1 is higher than Thprio1,prio2 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively.

-	if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 4 enabled] and the UE supports [Option 4],

-	if the UE is a destination UE of a TB to be transmitted in r1, RSRP2-RSRP1 is higher than a (pre)configured RSRP threshold.

-	if the UE is a destination UE of a TB to be transmitted in r2, RSRP1-RSRP2 is higher than a (pre)configured RSRP threshold.

-	where the RSRP1, RSRP2 measurements are performed according to clause 8.4.2.1 and the parameter Th(pi,pj) is determined [as described in step 3 of clause 8.1.4].

-	the first reserved resource r1 occurs in a slot in which the UE does not expect to perform SL reception due to half-duplex operation and the UE is a destination UE of a TB to be transmitted in resource r1.

8.1.4C	UE procedure for using a received non-preferred resource set

A UE configured with the higher layer parameter sl-InterUE-CoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection:

-	the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set.

Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement.

Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0SL+ Tproc,1SL+ Tproc,2SL) after the resource of inter-UE coordination information transmission, where Tproc,2SL is equal to (Tproc,0SL+ Tproc,1SL) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2SL is equal to Tproc,0SL when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission.

Note: The case when Tproc,2SL is equal to Tproc,0SL is assuming that SCI format 2-C is received.

<Unchanged parts are omitted>