3GPP TSG RAN WG1 #110bis		R1-2208744

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.1.3.2

Source:	Lenovo

Title:	Discussion of SRS enhancement

Document for:	Discussion

# Introduction

In Rel-18, enhancements on downlink MIMO that facilitate the use of large antenna array, for both FR1 and FR2, are needed to fulfil the demand for evolution of NR deployments. As coherent joint transmission (CJT) improves coverage and average throughput in commercial deployments with high-performance backhaul and synchronization, enhancement on CSI acquisition for FDD and TDD, targeting FR1, can be beneficial in expanding the utility of multi-TRP deployments. SRS enhancement targeting TDD coherent JT is agreed as part of MIMO enhancement, where the corresponding object is as highlighted as following in the approved WID [1]:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In Rel-18, it is important to identify and specify necessary enhancements for uplink MIMO. As advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) become more relevant, introducing necessary enhancements to support 8 antenna ports as well as 4 and more layers for UL transmission can offer the needed improvement for UL coverage and average throughput. SRS enhancement for 8Tx is agreed as part of MIMO enhancement, where the corresponding object is as highlighted as following in the approved WID [1]:

In this contribution, we provide our views on SRS enhancement targeting TDD CJT and 8 TX operation.

# Discussion on SRS enhancement targeting TDD CJT

In RAN1 #110 meeting, the following agreement is made for SRS enhancement for CJT [2]:

Agreement For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply): - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization; RAN1 should strive to minimize the number of schemes supported in Rel-18 - SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation. Target usage includes antenna switching, codebook/non-codebook based SRS

## Scenario and SRS coordination assumption for TDD CJT

For TDD system, SRS can be used to obtain downlink CSI by exploiting channel reciprocity. SRS based DL CSI acquisition scheme has the benefit of lower CSI feedback overhead and higher CSI precision, compared with quantized PMI feedback. For cell-edge UEs, the uplink SINR and channel quality could be too low to perform SRS-based channel measurement with sufficient resolution, especially for power-limited UEs. Thus, it is important for gNB to make coordination on SRS resources and manage inter-TRP cross-SRS interference.

One typical cross-SRS interference scenario is shown in Fig.1. For a UE that is associated with CJT transmission, i.e., UE1, the received SRS at the candidate TRPs for CJT, e.g., TRP1, TRP2, will be used to obtain channel estimates by one coordinated TRP, e.g. TRP1, and another coordinated TRPs, e.g. TRP2. However, a concurrent SRS transmission from a neighboring UE2 to one of the candidate TRPs , e.g., TRP2, may use the same or overlapped time-frequency resources as the SRS resource for the CJT-based UE without any coordination. Under this setup, the estimated channel accuracy corresponding to the CJT-based UE1 will be degraded by TRP2, especially if the neighboring UE2 is closer to TRP2 than the CJT-based UE1 is. Therefore, SRS enhancement can be made for SRS resources from candidate TRPs to improve SRS channel estimation accuracy.

Fig.1 Example cross-SRS interference for CJT UE

As mentioned in the Rel.18 MIMO WID, enhancement on CSI acquisition for CJT may support up to 4 TRPs. In general, the larger the number of TRPs, the more coordination is needed for cross SRS resources, leading to increased scheduling realization complexity and more SRS capacity requirement. Therefore, we assume SRS enhancement are made based on coordination between cross-SRS transmitted to up to 4 coordinated TRPs for interference mitigation. The cross-SRS coordination can be made between SRS resources or SRS resource sets from coordinated TRPs, which depends on actual interference mitigation schemes. For example, coordination can be made between SRS resource sets if transmission power is considered for coordination, where SRS power control is made based on SRS resource set according to legacy schemes. The actual coordinated SRS resource number or SRS resource set number is related with TRP number for coordination.

Enhance SRS for CJT based on coordination between SRS resources or SRS resource sets to up to 4 TRPs for interference mitigation.

In our opinion, SRS enhancement for CJT should not be limited to TDD systems with perfect UL-DL channel reciprocity, and should consider scenarios with partial UL-DL channel reciprocity. Note that partial UL-DL channel reciprocity may apply to TDD systems with Tx-Rx imbalance at either node, and/or FR1 FDD systems with small duplexing distance, as illustrated in Rel. 17 FeType-II port-selection codebook discussions [3]. While SRS signaling may not suffice to obtain high-resolution CSI estimates for CJT under partial channel reciprocity, it can help significantly reduce the CSI feedback overhead compared with pure CSI-RS based CSI acquisition. In light of that, SRS enhancement for CJT with partial channel reciprocity should be evaluated, where it should be specified only if benefits of the achieved throughput gain, CSI feedback overhead reduction, or CSI-RS overhead reduction, exceeds the cost of the additional SRS overhead.

Study the potential gains/costs of SRS enhancement for CJT assuming partial UL-DL channel reciprocity.

## Consideration on SRS enhancement for TDD CJT

In RAN1#110 meeting, it is agree to support and specify SRS interference randomization and/or capacity enhancement are agreed enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization [2]. There was good discussion on the details of enhanced schemes. And some technical analysis is made for each enhanced scheme. But RAN1 should strive to minimize the number of schemes supported in Rel-18. Thus, we make further discussion on the details of enhanced schemes and show our preference for the priority of these enhanced schemes.

### Interference randomization for SRS enhancement

With randomized resources for SRS transmission, the inter-TRP cross-SRS interference can be randomized. In this way, the interference can be small when some resources with better orthogonality are used for cross-SRS. It can avoid persistent interference and provide good channel estimation performance for some transmission opportunity with smaller cross-SRS, without increasing the actual SRS overhead. This kind of schemes can be used to obtain accurate downlink CSI especially when UE load is not heavy. Many SRS enhancement schemes with interference randomization are agreed to be studied in RAN1#109-e meeting and further discussed in RAN1#110 meeting. Our view are shown as follows based on technical analysis.

Scheme1: Frequency-domain randomization. For the first kind of schemes, randomization is made on frequency-domain resource mapping for SRS transmission. In general, the frequency location, including RE level or RB level, can hop in different transmission opportunities. Comb hopping is one RE level hopping scheme. In detail, the Comb value(s) for the SRS port(s) can cycle in one set, such as 0,1,…,KTC-1. For 4 port SRS, Comb hopping scheme can be further discussed since different Comb values can be used for different antenna ports based on the scheme in specification. For RB level hopping, different schemes are proposed by different companies, such as frequency hopping on different bandwidths, additional frequency hopping on initial frequency position on top of legacy frequency hopping scheme, frequency hopping with more SRS bandwidth configuration, etc. These enhanced hopping schemes need additional network coordination to guarantee SRS multiplexing orthogonality between R18 UEs and legacy UEs. Moreover, channel estimation performance at gNB may degrade if non-uniform patterns in frequency/time domain are used. Thus, there are different views for supporting RB level frequency hopping. In our view, Comb hopping scheme has not much standard impact but can reduce inter-TRP cross-SRS interference to some extent. Thus, we support to put high priority for SRS enhancement with Comb hopping.

Prioritize Comb hopping for frequency domain randomizaiton.

Scheme 2: Code-domain randomization. For the second kind of schemes, randomization are made on code-domain resource mapping for SRS transmission. It may include cyclic shift hopping, sequence hopping and per-hop sequence from a long SRS sequence. It is agreed that PAPR performance should be considered when considering this type of SRS enhancement. And, new SRS sequence design needs more effort and may be undesirable since legacy SRS sequence is well designed and evaluated. So we do not prefer per-hop sequence from a long SRS sequence. For sequence hopping, it may give some challenge on interference coordination for inter-TRP SRS since sequence hopping can be disabled if groupOrSequenceHopping equals 'groupHopping' or 'neither' and sequence number v has to be fixed as 0 when sequence length smaller than 72 if groupOrSequenceHopping equals 'sequenceHopping' based on scheme in the specification. Also, only value {0, 1} can be used for sequence hopping. The actual interference mitigation effect may depend on evaluation. For cyclic shift hopping, the CS value(s) for the SRS port(s) can cycle in one set, such as 0,1,…,nSRScs,max-1, whereas the cyclic shift nSRScs∈0,1,…,nSRScs,max-1 and transmission comb number KTC∈2,4,8 are configured by the higher-layer parameter transmissionComb in legacy system. With CS hopping, the cross-SRS interference can be reduced when different CS values and the same base sequence are used for cross SRS transmission. For multiple port SRS, different Comb values may be used for different SRS ports and both cyclic shift and Comb can be determined together for one SRS port. Thus, the cyclic shift hopping and Comb hopping can be jointly designed.

Prioritize cyclic shift hopping for code domain randomization.

Scheme 3: Time-domain randomization. For the third kind of scheme, SRS are transmitted based on randomized transmission, where pseudo-random muting of SRS transmission is used. In our view, it will have impact on DL CSI acquisition when SRS is not transmitted. This is undesirable when gNB needs to schedule burst urgent data transmission but not having DL CSI. Moreover, a similar functionality can be carried out by semi-persistent or aperiodic SRS to some extent. So we prefer to put lower priority on this kind of enhanced scheme to allocate more time to study the other candidates for SRS enhancement.

Deprioritize randomized SRS transmission.

Scheme 4: Enhanced power control. For the fourth kind of scheme, per-TRP power control and/or power control of one SRS towards multiple TRPs. It has lower SRS overhead than multiple SRS towards multiple TRPs. In general, power control scheme is typical and direct scheme for reducing interference. Based on the agreement in RAN1#109-e meeting, the pathloss between UE and TRPs can be different by at least {3, 6, 10}dB when SRSs sent by a UE are utilized by multiple TRPs for channel estimation. Since possible large difference between UE and TRPs, enhanced power control mechanism for CJT UE is efficient to guarantee channel estimation quality for both TRPs when one SRS is received by multiple TRPs simultaneously. Especially, two pathloss values can be considered in the enhanced power control mechanism. We support more study on enhanced power control mechanism for one SRS towards to multiple TRPs.

Further study enhanced power control mechanism for one SRS towards to multiple TRPs.

Scheme 5: Flexible SRS transmission. For the fifth kind of schemes, enhanced signalling is required. For example, the enhanced signalling can be used to dynamically update the SRS configuration parameters. For SRS parameters, they may include frequency domain parameter, such as bandwidth, Comb, hopping location, power control parameter, RPFS parameters, dynamic activation for semi-persistent SRS, number of antennas in antenna switching, etc. In this way, SRS transmission can be randomized by dynamic change of these configuration parameters. However, it will increase dynamic signalling overhead. Currently, the candidates of SRS configuration parameters for dynamic updating are too many. More clarification and down-selection are needed since this is related with detail signalling design, such as bit number. Furthermore, it is not clear how much gain can be achieved compared with randomized hopping schemes, such as the Comb hopping scheme, CS hopping scheme. So we prefer to have further study and narrow down for this kind of schemes.

Further study and narrow down the flexible SRS transmission schemes.

Scheme 6: Enhanced SRS configuration. For the sixth kind of schemes, enhanced configuration of SRS transmission is made to enable more flexible SRS parameter assignment. For one scheme, configuration is made for v, (i.e., sequence index within a group) per SRS resource. We are not clear the details on how this scheme work together with sequence hopping. The actual benefit for this scheme simultaneously with enabling sequence hopping needs clarifying since the similar function is carried by sequence hopping. For another scheme, configuration is made for cyclic shift per SRS port per SRS resource. This scheme may violate the rule for multiple port SRS design, where large CS spacing is used between SRS ports, which can reduce interference between SRS ports and guarantee channel estimation performance for the channel with larger delay spread. So we prefer to reuse legacy SRS design as much as possible to reduce standard effort.

Reuse legacy SRS parameter configuration.

Scheme 7: Enhanced SRS resource mapping. For the seven kind of schemes, resource mapping for SRS transmission can be made based on network provided parameters or system parameters. The network provided parameters can be network-provided ID (different from cell ID). The network provided parameters can be used for generating SRS sequence to randomize the interference. However, it seems the benefit is not clear since some coordination can be realized for SRS sequence by existed parameter configuration. Thus, we prefer to reuse the legacy SRS resource mapping scheme for SRS transmission.

Reuse the legacy SRS resource mapping for SRS transmission.

### Capacity enhancements and/or overhead reduction for SRS enhancement

For SRS resource coordination, the same SRS resource for CJT should not be assigned to more than one UEs targeted by the same set of TRPs. The resource coordination can be made in time domain, frequency domain and CS domain. For time domain, specific OFDM symbols or slots can only be used for CJT-based UEs. For frequency domain, REs with specific Comb or PRBs can only be used for CJT-based UEs. For CS domain, different CS can be used for CJT-based UE and neighboring UE(s) for SRS transmission if the same base sequence is used. The resource coordination on time/frequency/CS domain would lead to significant increase in SRS overhead, especially when the number of candidate TRPs is large, e.g., 3 or 4. However, in Rel-17, several schemes for SRS capacity enhancement are already introduced, including RB-level partial frequency sounding (RPFS), more than 4 symbols SRS and comb value of 8. With SRS partial sounding, UE transmits on a part of the hopping bandwidth to save the number of RBs occupied by SRS. With comb 8, the frequency-domain resource occupied by SRS can be further reduced compared with comb 2 or comb 4. Therefore, the shortage issue for SRS capacity is alleviated to some extent. When SRS capacity is required to be enhanced for supporting more CJT UEs, several kinds of enhanced schemes are are agreed to be studied in RAN1#109-e meeting and further discussed in RAN1#110 meeting. Our view are shown as follows based on technical analysis.

Scheme I: TD OCC. For the first kind of schemes, TD OCC is used for multiplexing SRS for different UEs. In detail, different OCC codes can be configured for different UEs with multiple symbol repetition transmission. In this way, the capacity of SRS is improved without introducing additional resources, where SRS with multiple symbol repetition are multiplexed. From another view, TD-OCC can provide performance gain for the channel with large delay spread. If TD-OCC is used for SRS without repetition But, it will introduce additional symbol for SRS transmission and thus increase SRS overhead. Thus, TD-OCC for SRS without repetition can be only used for specific scenario, e.g. large delay spread if it is introduced. Moreover, TD-OCC is a simple and conventional enhancement scheme, which is already used for DMRS, CSI-RS. Therefore, we support to further study TD OCC scheme for SRS with repetition if enhanced SRS capacity is needed.

Scheme II: Increasing cyclic shifts. For the second kind of schemes, the maximum number of cyclic shifts is increased. Based on specification, nSRScs,max is 8, 12, 6 when KTC is 2, 4, 8, respectively. For nSRScs,max as 8 or 12, there may be no more room for increasing number of cyclic shift. For nSRScs,max as 6 in case of KTC as 8, it is decided in Rel.17 based on hot discussion.  To avoid repeated discussion, the motivation for further increasing maximum number of cyclic shifts needs to be clarified, especially for CJT UEs typically in cell edge, where delay spread in case of CJT is expected to be no smaller than the case of single-TRP. Thus, we support to keep legacy maximum cyclic shift value.

Scheme III: SRS precoding. For the third kind of schemes, precoded SRS is used for DL CSI acquisition. The precoder for SRS is determined based on SVD for downlink CJT channel according to multiple CSI-RS resources. For this scheme, precoded SRS can reduce interference and improve coverage. Also, SRS overhead is reduced. For example, SRS port number for 4R UE can be reduced from 4 to 1 or 2 based on assumption that the typical PDSCH layer is 1 or 2. However, some information for full MIMO channel is lost since the assumed precoding is used for SRS transmission. Also there is additional Tx/Rx calibration work. Based on the hot discussion, there are different views on this scheme. In our view, more study and discussion are needed to align views between companies on the performance gain and potential system realization impact.

Scheme IV: Extended partial frequency sounding. For the fourth kind of schemes, partial frequency sounding is proposed for extension. These schemes include larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b (bhop<b<BSRS) beside the last bandwidth BSRS. In Rel.17, 2 or 4 can be configured as frequency scaling factor. If larger value, e.g. 8, is used, CSI may be inaccurate in case of strong frequency selective channel since SRS is transmitted with larger frequency scaling factor. For starting RB location hopping, it is already supported in Rel.17 by configured RRC parameter startRBIndexAndFreqScalingFactor2-r17 or startRBIndexAndFreqScalingFactor4-r17. For partial frequency hopping on other bandwidth, similar mechanism can be introduced or discussed for enhanced frequency hopping. So the motivation should be further clarified for making further enhancement on partial frequency sounding, which is a new feature introduced in Rel.17. Thus, we prefer not to make extension for partial frequency sounding.

Based on the discussion on 4 kinds of schemes for SRS capacity enhancement, our proposal is:

Prioritize TD OCC for multiplexing UEs with multiple symbol repetition transmission for SRS capacity enhancement.

# Discussion on SRS enhancement for 8Tx UL operation

- In RAN1#110 meeting, the following agreements were reached on SRS to support 8Tx UL operation [2]:

Agreement: For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘antennaSwitching’; - FFS 8 ports in one or multiple SRS resources for ‘codebook’ Agreement: For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR). Agreement For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. - FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols. Agreement For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

It was clear that both codebook and non-codebook based PUSCH shall be supported in Rel-18 for 8Tx UE, then 8Tx SRS for codebook as well as SRS for non-codebook shall be supported in Rel-18. In this section, we provide our views on the enhancements on 8Tx SRS.

## 8Tx SRS for codebook

To support 8Tx SRS for codebook, the straightforward method is to configured 8 ports for a SRS resource. Then all the 8 antenna ports can be sounded by a same SRS resource in one OFDM symbol. The key issue is how to determine the CS and/or Comb offset parameter for different SRS ports. Some configuration examples are provided with to different comb sizes and coherent groups. In the following examples, we assume that antenna port 1000, 1001, 1004, 1005 are within a same coherent group, and port 1002, 1003, 1006, 1007 are within another coherent group; or port 1000, 1002 are within a 1st coherent group, port 1001, 1003 are within a 2nd coherent group, port 1004, 1005 are within a 3rd coherent group and port 1006, 1007 are within a 4th coherent group:

- For KTC=2 with nSRScs,max=8, 2 Comb offset + 4 CS for 8 Tx ports are required for a SRS resources, e.g.,

  - Ports 1000, 1001, 1004, 1005 are transmitted using {nSRScs, (nSRScs+2) mod nSRScs,max, (nSRScs+4) mod nSRScs,max, (nSRScs+6) mod nSRScs,max} with kTC

  - Ports 1002, 1003, 1006, 1007 are transmitted using {nSRScs, (nSRScs+2) mod nSRScs,max, (nSRScs+4) mod nSRScs,max, (nSRScs+6) mod nSRScs,max} with (kTC+1) mod KTC

- For KTC=4 with nSRScs,max=12, the following two schemes can be used

  - Scheme 1: 2 Comb offset + 4 CS for 8 Tx ports, e.g.,

    - Ports 1000, 1001, 1004, 1005 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with kTC

    - Ports 1002, 1003, 1006, 1007 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+2) mode KTC

  - Scheme 2: 4 Comb offset +4 CS for 8 Tx ports, e.g.,

    - If nSRScs < 6,

Ports 1000, 1001, 1004, 1005 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with kTC

Ports 1002, 1003, 1006, 1007 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+2) mode KTC

    - If nSRScs >= 6,

Ports 1000, 1002 are transmitted using {nSRScs, (nSRScs+6) mod nSRScs,max} with kTC

Ports 1001, 1003 are transmitted using {(nSRScs+3) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+1) mode KTC

Ports 1004, 1006 are transmitted using {nSRScs, (nSRScs+6) mod nSRScs,max} with (kTC+2) mode KTC

Ports 1005, 1007 are transmitted using {(nSRScs+3) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+3) mode KTC

- For KTC=8 with nSRScs,max=6, 4 Comb offset+ 2 CS for 8 Tx ports, e.g.,

  - Ports 1000, 1002 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with kTC

  - Ports 1001, 1003 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with (kTC+2) mode KTC

  - Ports 1004, 1006 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with (kTC+4) mode KTC

  - Ports 1005, 1007 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with (kTC+6) mode KTC

For a SRS resource configured with 8 SRS port, considering the configurations

      - For KTC=2 with nSRScs,max=8, 2 Comb offset + 4 CS

      - For KTC=4 with nSRScs,max=12

        - 2 Comb offset + 4 CS

        - 4 Comb offset +4 CS

      - For KTC=8 with nSRScs,max=6, 4 Comb offset + 2 CS

Considering the fact that 8 orthogonal resources will be used for a SRS resource with 8 SRS ports, the total number of UEs supporting 8 SRS ports on the same PRB set in a cell is limited. For example, a total number of 48 orthogonal resources are available for SRS, and up to six 8TX UE can be multiplexed on a same set of PRBs for UL sounding. To further increase the SRS capacity, SRS port multiplexed with TD-OCC is a method and has been discussed in section 2.2.2.

Another possible solution is to sounding 8 antenna ports in multiple SRS resources. For example, all 8 antenna ports can be sounded by two SRS resources transmitted on adjacent OFDM symbols at lease for partial-coherent and non-coherent UE. Since the channel condition corresponding to different coherent antenna groups are independent and the phase difference between different antennas within different coherent antenna groups are random, the channels corresponding to different coherent antenna groups can be sounded by different SRS resources in different symbols. The channel condition corresponding to different antennas within a same coherent antenna group are correlated and the phase difference between different antennas within a same coherent antenna group are fixed for a channel, therefore, the channels for antennas within a same coherent antenna group should be sounded together by a same SRS resource. One example is provided as follows.

Example of sounding 8 antenna port by two SRS resource sets:

SRS resource#0: SRS antenna port 1000, 1001, 1004, 1005

SRS resource#1: SRS antenna port 1002, 1003, 1006, 1007

SRS antenna port 1000, 1001, 1004, 1005 belong to the 1st coherent antenna group are sounded by SRS resource#0 and SRS antenna port 1002, 1003, 1006, 1007 belong to the 2nd coherent antenna group are sounded by SRS resource#1. SRS ports of SRS resource 0 and SRS ports of SRS resource 1 share the same Comb offset and CS. In other words, the SRS port 0, 1, 2, 3 of SRS resource 0 are mapped to SRS antenna port 1000, 1001, 1004, 1005, and the SRS port 0, 1, 2, 3 of SRS resource 1 are mapped to SRS antenna port 1002, 1003, 1006, 1007. It means that SRS resource 0 and SRS resource 1 are associated with different antenna ports. Different from SRS for antenna switching, a gap is not need between different SRS resources and the antenna ports within a same coherent antenna group should be associated with a same SRS resource.

To quickly obtain a more completed channel matrix, SRS resources for all 8 antennas sounding are expected to be transmitted on adjacent OFDM symbols. Further, all the SRS resources for all 8 antennas sounding should be configured with the same frequency hopping pattern and repetition parameters and should perform frequency hopping and/or repetition together. For the SRI indication for PUSCH scheduling, each SRI codepoint can be associated with multiple SRS resources for all 8 antenna sounding.

Support to sound 8 antenna port with one or more SRS resources.

## 8Tx SRS for non-codebook

To support 8 Tx non-codebook based PUSCH transmission, 8 single port SRS resources shall be supported as agreed in RAN1#109. If all the 8 single-port SRS resources can be transmitted in a slot, they shall be configured within a same SRS resource set as in Rel-17. However, if the UE can only transmit SRS in the last 6 OFDM symbols in a slot and only one SRS resource can be transmitted in one OFDM symbol, 8 OFDM symbols are required for all the 8 SRS resource transmission. Then all the SRS resources within an SRS resource set cannot be transmitted in a slot. For this case, all the 8 SRS resources can be configured in two or more SRS resource sets. While all of the 8 SRS resources should be configured with a same UL TCI state and all the SRS resource sets should be associated with a same NZP CSI-RS for precoding calculation. Besides, for aperiodic SRS resource sets, all the SRS resource sets shall be triggered by a same DCI.

All the 8 single port SRS resources can be configured within one or more SRS resource sets according to UE capability.

- All the SRS  resource sets are configured with a same UL TCI state, or

- All the SRS resoruce sets are associated with a same NZP CSI-RS resource.

The next issue is how to indicate the SRI(s) for the scheduled PUSCH transmission. One or more SRIs corresponding to one or more SRS resources can be indicated by the SRS resource indicator field similar in Rel-15. When all the 8 SRS resources are configured within a same SRS resource set, if the UE only support up to rank 4 UL transmission, 162 SRI combinations shall be indicated with Rel-15 SRI indication mechanism by define multiple SRI indication tables. To minimize the s standardization effort, bitmap based SRI indication can be introduced. For example, a 8-bit bitmap can be used for SRI indication, where each bit of the bitmap corresponds to a SRS resource.

Support bitmap based SRI indication in DCI  for non-codebook based PUSCH scheduling.

# Conclusion

In this contribution document, we discussed our views on SRS enhancement targeting CJT scenarios, as well as 8Tx UL operation. Based on the discussion points in the document, we have the following proposals:

- Enhance SRS for CJT based on coordination between SRS resources or SRS resource sets to up to 4 TRPs for interference mitigation.

- Study the potential gains/costs of SRS enhancement for CJT assuming partial UL-DL channel reciprocity.

- Prioritize Comb hopping for frequency domain randomizaiton.

- Prioritize cyclic shift hopping for code domain randomization.

- Deprioritize randomized SRS transmission.

- Further study enhanced power control mechanism for one SRS towards to multiple TRPs.

- Further study and narrow down the flexible SRS transmission schemes.

- Reuse legacy SRS parameter configuration.

- Reuse the legacy SRS resource mapping for SRS transmission.

- Prioritize TD OCC for multiplexing UEs with multiple symbol repetition transmission for SRS capacity enhancement.

For a SRS resource configured with 8 SRS port, considering the configurations

      - For KTC=2 with nSRScs,max=8, 2 Comb offset + 4 CS

      - For KTC=4 with nSRScs,max=12

        - 2 Comb offset + 4 CS

        - 4 Comb offset +4 CS

      - For KTC=8 with nSRScs,max=6, 4 Comb offset + 2 CS

Support to sound 8 antenna port with one or more SRS resources.

All the 8 single port SRS resources can be configured within one or more SRS resource sets according to UE capability.

- All the SRS  resource sets are configured with a same UL TCI state, or

- All the SRS resoruce sets are associated with a same NZP CSI-RS resource.

- Support bitmap based SRI indication in DCI  for non-codebook based PUSCH scheduling.

# References

| [1] | RP-213598, Samsung, "New WID: MIMO Evlution for Downlink and Uplink," Online, Dec. 6-17, 2021. |
| --- | --- |
| [2] | 3GPP TSG RAN WG1 #110, "Draft Meeting Report," Toulouse, France, August 22nd – 26th, 2022. |
| [3] | R1-2208742, Lenovo, "CSI enhancements for high mobility and coherent JT," Online, October 10th – 19th, 2022. |