3GPP TSG RAN WG1 #110bis-e R1-2208759 e-Meeting, October 10th – 19th, 2022 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on symbols overlapping between UL signal and non-serving cell SSB in 38.213. Source to WG:; ZTE. Source to TSG:; RAN1. Work item code:; NR_feMIMO-Core; Date:; 2022-9-30. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17). Reason for change:; Regarding invalid symbols of PUCCH repetition, it includes the symbol of SSBs configured for L1 beam measurement/reporting. Herein the SSBs configured for L1 beam measurement/reporting may be different from the SSBs associated with active TCI states. However, according to the current specification, it may lead to the ambiguity that the SSBs configured for L1 beam measurement/reporting should be with active TCI states, which is incorrect and should be modified. Besides, the description of “...SS/PBCH blocks configured for L1 beam measurement/reporting” is too general and quite unclear. In the current specification, SSBs for L1 beam measurement/reporting corresponding to the SSBs configured in CSI-SSB-ResourceSet of a reporting setting configured by CSI-ReportConfig, that should be explicitly captured in the specification to avoid any ambiguity.. -. Summary of change:; Clarifying the symbol of SSBs configured for L1 beam measurement/reporting is not related to the SSBs with active TCI state and the description of SSBs for L1 beam measurement/reporting.. Consequences if not approved:; SSBs configured for L1 beam measurement/reporting may be incorrectly interpreted as the SSBs associated with active TCI states, and the definition of SSBs for L1 beam measurement/reporting will be agnostic.. Clauses affected:; 9.2.6, 11.1, 11.1.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR..

## 9.2.6	PUCCH repetition procedure

<Unchanged parts are omitted>

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB if provided or, if the UE is not provided dl-OrJoint-TCIStateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or is a symbol of an SS/PBCH block configured in a SS/PBCH block set configured by CSI-SSB-ResourceSet of a reporting setting configured by CSI-ReportConfig.

<Unchanged parts are omitted>

## 11.1	Slot configuration

<Unchanged parts are omitted>

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided dl-OrJoint-TCIStateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH block(s) configured in a SS/PBCH block set configured by CSI-SSB-ResourceSet of a reporting setting configured by CSI-ReportConfig, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

If a UE

-	is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided dl-OrJoint-TCIStateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH block(s) configured in a SS/PBCH block set configured by CSI-SSB-ResourceSet of a reporting setting configured by CSI-ReportConfig, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

<Unchanged parts are omitted>

### 11.1.1	UE procedure for determining slot format

<Unchanged parts are omitted>

For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in clause 4.1 or, if the UE is not provided dl-OrJoint-TCIStateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH block(s) configured in a SS/PBCH block set configured by CSI-SSB-ResourceSet of a reporting setting configured by CSI-ReportConfig, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

<Unchanged parts are omitted>