3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208791

e-Meeting, October 10th– 19th, 2022

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.214 | CR | xxxx | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on TCI indication of SRS in Unified TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-17 Unified TCI framework, whther the SRS resource for codebook-based or non-codebook based PUSCH transmission follows the indicated Rel-17 TCI state is configrued through RRC. If the SRS for codebook or non-codebook transmission does not follow the indicated Rel-17 TCI state, the TCI state for each SRS resource is separately configured. Thus it is possible that the TCI state on SRS resoucre for codebook-based or non-codebook based PUSCH is different from the TCI state applied on PUSCH. On the other hand, per the specification, the PUSCH shall use the same antenna ports as the SRS resource indicated in the scheduling DCI. If different TCI states are applied on PUSCH and the indicated SRS resource, it is technically impossible to maintain same antenna ports on PUSCH and SRS. Thus the current specification would break. To fix this issue, it is needed to make sure same TCI state is applied on PUSCH and the corresponding SRS resource. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sections 6.1.1.1 and 6.1.1.3 of TS 38.214, specify that same TCI state is expected on PUSCH and SRS resource for codebook-based or non-codebook based PUSCH transmission in unified TCI framework. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of PUSCH transmission with same antenna ports of indicated SRS resource will break. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1, 6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.1.1	Codebook based UL transmission

<Unchanged parts are omitted>

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3.

If an SRS resource set with parameter usage set to 'codebook' is not configured with followUnifiedTCIstate, the UE shall expect that the configured DLorJoint-TCIState or UL-TCIState of the SRS resource(s) is same to the indicated indicated DLorJointTCIState or UL-TCIstate.

<Unchanged parts are omitted>

#### 6.1.1.2	Non-Codebook based UL transmission

<Unchanged parts are omitted>

The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as .

If an SRS resource set with parameter usage set to 'NonCodebook' is not configured with followUnifiedTCIstate, the UE shall expect that the configured DLorJoint-TCIState or UL-TCIState of the SRS resource(s) is same to the indicated indicated DLorJointTCIState or UL-TCIstate.

<Unchanged parts are omitted>