3GPP TSG RAN WG1 #110bis-e		R1-2208801

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Discussions on Carrier Phase Measurement for NR Positioning

Agenda Item:	9.5.2.2

Document for:	Discussion and Decision

- Introduction

The Rel-18 SID on expanded and improved NR positioning [1] includes the RAN1-centric objective of studying solutions for accuracy improvement based on carrier phase measurements. The following pertinent agreements on carrier phase measurement study were made in previous meeting:

Agreement The impact of integer ambiguity on NR carrier phase positioning and potential solutions to resolve the integer ambiguity will be studied in the SI. Agreement The study of the accuracy improvement based on NR carrier phase measurements in Rel-18 SI may include: - UE-based and UE-assisted carrier phase positioning, - UL carrier phase positioning and DL carrier phase positioning. - NR carrier phase positioning with the carrier phase measurements of one carrier frequency or multiple frequencies - Combination of NR carrier phase positioning with another standardized Rel. 17 positioning method, e.g., DL-TDOA, UL-TDOA, Multi-RTT, etc. - Note: The use of “carrier phase positioning” does not necessarily mean it is a standalone positioning method - FFS: whether SL carrier phase positioning is to be discussed in Rel-18 SI Agreement - The impact of multipath for the carrier phase positioning will be evaluated during the SI - The methods of mitigating the impact of multipath for the carrier phase positioning will be studied during the SI, if it is considered to be necessary after the evaluation. Agreement - For the purposes of discussion, for NR downlink and/or uplink carrier phase positioning, the carrier phase (CP) at a RF frequency at a receiver is a phase that is a function of the signal propagation time from an Tx antenna reference point of a transmitter (e.g., a TRP or a UE) to a Rx antenna reference point of the receiver (e.g., a UE or a TRP). - The propagation time can be expressed in a fractional part of a cycle of the RF frequency and a number of integer cycles, but the CP may be independent of the number of integer cycles. Agreement The use of PRUs to facilitate NR carrier phase positioning can be evaluated in the SI by RAN1. Further evaluate the following multipath mitigation methods for the carrier phase positioning, which include, but are not limited to, the following: - The methods of estimating the carrier phase of the first path - Note: Both time-domain and frequency-domain methods can be considered - LOS/NLOS/ Multi-path indication for the carrier phase measurements for improving the accuracy of the position calculation - Rel-17 LOS/NLOS indicator can be used as the starting point - measurements of the first path and additional paths - E.g. carrier phase measurements, timing measurements - other channel information, such as RSRP/RSRPP, CIR/CFR, etc.

In this contribution, we will discuss the issues related with potential phase measurement solutions for NR positioning and evaluation results.

- DL Carrier Phase Measurement

In NR, we have specified the timing measurement based and angle-based positioning techniques. In DL-TDOA, UL-TDOA and multi-RTT method, time-based measurement is used to estimate location of UE. In DL-AoD and UL-AoA method, the departure angle or arrival angle of UE with respect to the TRP is used to estimate the location of UE. In theory, the accuracy of time-base measurement is limited by the bandwidth of positioning reference signal. And the performance of angel-based method is limited by the resolution of angles and beams. Rel-18 SID of NR positioning includes the objective of studying phase measurement-based positioning technique. In phase measurement-based method, the receiver can measure the phase of received positioning reference signal and difference between the transmit signal and receive signal is a function of the distance between the TRP and the UE. Given that, the location of UE can be estimated based on the phase measurement of positioning reference signal of multiple TRPs.

Figure 1 Example of phase measurement.

An example of phase measurement-based positioning is shown in Figure 1. Each TRP transmit PRS signal to the UE and the PRS signal can be written as:

skt=Akt∙ej2πfct with k=1,2,3,4

And the UE side, the received PRS signal of each TRP that passes through a given transmission delay is:

ykt=Akt-τk∙ej2πfc(t-τk)

where τk= dkc is the transmission time corresponding to the distance between TRP k and the UE. For each TRP k, the UE can measure the phase of received positioning reference signal:

ωk=2πdkλ-N×2π+δ

where  N is ambiguity integer wavelength and  δ is the measurement/estimation error.  With that, the distance between TRP and UE can be estimated as:

dk=ωk2π+N×λ

To study the phase measurement-based NR positioning technique, we shall focus on the current DL PRS and UL SRS for positioning, which are supported in NR system. It is not preferred to introduce new reference signal for phase measurement-based positioning. The existing DL PRS resource can be used by the UE to measure phase of DL transmission. For UE-assisted positioning method, the UE can report the phase measurement result to the LMF and the LMF can then estimate the UE location. For UE-based method, the UE can measure the phase of DL PRS resource and then estimate the UE location based on the phase measurement of DL PRS of multiple TRPs and the TRP coordinates.

Proposal 1: The NR phase measurement-based positioning shall be based on the existing DL PRS and SRS for positioning.

System-level simulations were conducted to evaluate the performance of carrier phase measurement for NR positioning for the scenario of InF-SH with the following parameters:

| Parameter |  |
| --- | --- |
| Scenario TS 38.857 | InF-SH |
| Single carrier frequency, or multiple carrier frequencies, GHz | 3.5GHz |
| Bandwidth, MHz | 100MHz |
| Subcarrier spacing, kHz | 30KHz |
| RS signal descriptions (PRS or posSRS, Number of OFDM simbles, Comb size) | PRS |
| NR Carrier phase positioning method (DL, UL, or DL+UL(RTT)) | DL |
| R16/R17 positioning method (if it is used together with CPP) | DL-DToA |
| Carrier phase estimation techniques | Time-domain estimation |
| Differential positioning techniques if used (e.g., single differential, double differential, etc.) | None |
| Integer ambiguity resolution techniques | Cost function + estimated ambiguity range |
| Multipath mitigation techniques | First path detection |
| Single-measurement instance CPP, or multiple measurement instances CPP | Single-measurement |
| UE position calculation algorithm | Chan |
| Network synchronization assumption (e.g., 0ns, 10ns, ..) | 0ns |
| UE/TRP Initial phase offset | 0 |
| CFO/Doppler | 0 |
| Oscillator-drifts | 0 |
| ARP errors | 0 |
| Phase Center Offsets | 0 |
| Phase noise (FR2) | 0 |
| Additional notes, if any |  |

Various methods were evaluated:

- DL-TDoA in InF-SH was evaluated to provide baseline performance.

- Carrier phase measurement-based method with assuming known integer ambiguity: the UE measures the carrier phase of received DL PRS from multiple TRPs. To resolve the issue of integer ambiguity, it is assumed that the integer ambiguity in each carrier phase measurement is known by the LMF. That can be considered as the upper bound of carrier phase measurement for NR positioning. In this method, assume there is no integer ambiguity,

- Carrier phase measurement + using DL-TDoA to estimate the integer ambiguity: in this method, the LMF can first use the DL-TDoA method and DL RSTD measurement to estimate the integer wavelength of distance between UE and each TRP.

The positioning performance of those methods in InF-SH is illustrated in Figure 2 and summarized in Table 1.

Figure 2: Performance of carrier phase measurement

The positioning accuracy evaluation results for various carrier phase measurement are summarized in the following Table:

Table 1: NR carrier phase positioning – horizontal accuracy

| Positioning method | @50% | @90% |
| --- | --- | --- |
| DL-TDoA | 0.14m | 0.24m |
| Carrier phase measurement with assuming zero integer ambiguity | 0.007m | 0.03m |
| Carrier phase measurement + DL-TDoA to estimate integer ambiguity | 0.14m | 0.26m |

We can make the following observations:

- The performance of carrier phase measurement-based positioning depends on the estimation of integer ambiguity critically.

- Using existing NR positioning (e.g., DL-DToA) to suppress the impact of integer ambiguity does not improve the performance.

- Carrier phase measurement + using DL-DToA to estimate integer ambiguity provide very similar performance as the DL-DToA method.

Observation 1: Integer ambiguity in carrier phase measurement would impair the performance of NR positioning significantly.

Observation 2: Combining carrier phase measurement with DL-TDOA method does not provide better performance than the DL-TDOA method.

To further study the impact of ambiguity and study how/whether to use other positioning method to mitigate the impact of ambiguity, the positioning performance of DL-TDOA was further evaluated and the equivalent integer wavelength of positioning error resulted in DL-DTOA method was studied. The DL-DTOA method has positioning errors and the positioning errors could be more than one wavelength and thus the error can contain integer number of wavelength. The distribution of integer wavelength contained in the positioning error of DL-DTOA method in the scenario of InF-SH is illustrated in Fig. 3. We can observe that with 85% of the positioning error contain one or more wavelength and 57% of the positioning errors contain two or more wavelengths. Thus, if the positioning results of DL-DTOA is used to resolve the integer ambiguity for carrier phase measurement, the integer wavelengths error in DL-DTOA method would be naturally the part of the positioning error in carrier phase measurement.

Figure 3: distribution of integer wavelength in positioning error of DL-DTOA method

Observation 3: Using other positioning method to resolve the integer ambiguity could cause error of integer wavelengths.

To evaluate the negative impact of residual integer wavelength in positioning measurement, numerical analysis was conducted. Figure 4 illustrates the negative impact on positioning performance of various residual integer wavelength. We can observe that residual integer wavelength can impair performance drastically.

Figure 4: Impact of integer ambiguity

One method to resolve the issue of integer ambiguity is to measure carrier phase of multiple carrier frequencies. The UE can measure the carrier phase of multiple carrier frequency points: {f1,f2,⋯,fM}  of one TRP and obtain the carrier phase measurement accordingly: {ω1,ω2,⋯,ωM}. Given that, we have the following M equations:

d=ω12π+N1×cf1

d=ω22π+N2×cf2

…

d=ωM2π+NM×cfM

where d is the distance between the UE and the TRP, c is the light speed and Nm is the integer ambiguity corresponding to the carrier phase measurement of carrier frequency fm. Then the carrier phase measurement of multiple frequency points can be used to resolve the issue of integer ambiguity. One method can be to minimize the cost function:

{d,N1,N2,…,NM}=argmin{d,N1,N2,…,NM}m=1Md-cfmωm2π+Nm2

Proposal 2: Study using carrier phase measurement of multiple carrier frequencies to resolve the issue of integer ambiguity.

One technical difficulty of solving the above minimization problem is that all the Nms are integer and thus there is no closed form solution to the problem.  One solution is to exhaustively search all the possible values of Nms to find the optimal values of them that can minimize the above cost function. However, the significant computational complexity might make this approach impractical.

# The method to resolve integer ambiguity

As discussed in previous section, using carrier phase measurement of multiple carrier frequencies could resolve the issue of integer ambiguity. To reduce the computational complexity of finding the optimal integer values that minimize the cost function, we can combine a legacy positioning method with carrier phase measurement. The legacy positioning method can be first use to estimate the location of UEs, which can be used to determine the range of possible integer values of  Nm, and then an exhaustive search on the determined range of possible values of  Nm to find the optimal values that minimize the cost function.

The method can be summarized as the following steps:

Step 1: the UE measure and reports positioning measurement for legacy timing-based or angle-based method, e.g., DL RSTD, or PRS RSRP or UE Rx-Tx time difference.

Step 2: the UE measures the carrier phase on multiple REs of DL PRS resource. On the REs {k1,k2,⋯,kM}, the UE measures the carrier phase measurement: {ω1,ω2,⋯,ωM}. Assume the carrier frequency on RE #0 is fc and the subcarrier spacing is Δf. Then for one TRP, the carrier phase measurements have the following equations:

d=ω12π+N1×cfc+k1Δf

d=ω22π+N2×cfc+k2Δf

…

d=ωM2π+NM×cfc+kMΔf

Step 3: the LMF can first estimate the UE location based on the legacy time-based or angle-based positioning method.

Step 4: According to the UE location estimation in Step 3, the LMF can estimate the integer wavelength NM corresponding to each frequency fc+kmΔf.

Step 5: The LMF can estimate the NM by minimizing the following cost functions over the value range of [NM-∆,NM+∆]:

{L,N1,N2,…,NM}=argmin{L,N1,N2,…,NM}m=1ML-cfc+kmΔfφm2π+Nm2

Simulation was conducted to evaluate the performance of this method. Integer ambiguity = 3 is assumed in the simulation and the proposed method is used to find the optimal value of NM. Carrier phase measurements of 5 REs with frequency spacing = 25MHz are measured by the UE. The error in estimated distance between TRP and UE is used to evaluate the performance, which is illustrated in Figure 5. We can observe that the proposed method can resolve the issue of integer ambiguity.

Figure 5: Performance of estimating integer ambiguity

Therefore, we propose that the UE measures the carrier phase of multiple frequencies and report the carrier phase measurement of multiple frequencies along with the positioning measurement results of other NR positioning method.

Proposal 3: The UE can measure and report carrier phase of multiple REs in DL PRS and the UE reports the carrier phase measurement and positioning measurement results of legacy positioning method.

# TRP Tx Antenna Phase Measurement

Another potential phase measurement for positioning is the UE can measure the relative phase between the PRS signal transmitted from different TRP Tx antennas. The relative phase can be used to calculate the angle of departure of the UE and then the system can estimate the UE location based on that.   Consider an example shown in Figure 3. The distance between TRP antennas is d. Assume the relative phase between TRP antennas is ω. Then the angle of departure of the UE with respect to the TRP can be calculated as:

θ=cos-1ωλ2πd

Figure 6: phase measurement of Antenna Ports

Therefore, the relative phase measurement between TRP antenna ports can be used to enhance the performance of DL-AoD positioning technique.

Proposal 4: Study measuring and reporting the relative phase of different Tx antenna ports of TRP for positioning.

To enable measuring relative phase of Tx antenna ports, the UE needs to receive DL PRS signal transmitted from different TRP antenna port. But, the current design of DL PRS is single-port transmission. Multiple Tx antenna ports are not visible to the UE. Therefore, to support measuring relative phase of Tx antenna ports, enhancement on DL PRS transmission is needed. One potential enhancement is to multi-antenna port DL PRS transmission, through which the UE can measure the signal phase of different Tx antenna ports.

Proposal 5: Study the PRS enhancement for supporting measuring relative phase of Tx antenna ports of TRP

The following PRS enhancements can be considered for supporting measuring relative phase of Tx antenna ports:

- We can consider to support multi-port DL PRS. In Rel-16/17, the DL PRS is single-port transmission. With that, the UE is not able to measure signal of different TRP antenna ports. A multi-port DL PRS transmission can support the UE to measure the signal phase of different TRP Tx antenna ports.

- Another alternative method is we still use the single-port DL PRS resource and map DL PRS resource to each TRP Tx antenna port. For example, a TRP has 4 Tx antenna ports and the system can associate one DL PRS resource to each TRP Tx antenna port. The UE can measure the received phase pf each DL PRS resource and then calculate the relative phase between TRP Tx antenna ports according to the mapping between DL PRS resource and TRP Tx antenna port.

# UL Phase Measurement

The carrier phase measurement of uplink positioning signal can also be used to enhance NR positioning. Similarly, the TRP can measure the carrier phase from SRS for positioning:

ω=2πdλ-N×2π+δ

where  N is ambiguity integer wavelength and  δ is the measurement error.  With that, the distance between TRP and UE can be estimated as:

d=ω2π+N×λ

Similar to the carrier phase measurement of DL PRS, the carrier phase measurement of UL also has the issue of ambiguity integer. To resolve the issue, the TRP can measure and report the carrier phase measurement of multiple frequency points of SRS for positioning and the TRP can also measure and report the positioning measurement of legacy UL positioning methods, e.g., UL RTOA or UL-AoA measurement. The LMF can the use the legacy timing-based or angle-based method to obtain the coarse estimation of value N and then find the optimal value of N through exhaustive searching on the estimated range of N.

Similarly, the UL method can be summarized as the following steps:

Step 1: the TRP measure and reports positioning measurement of legacy positioning method, UL RTOA, or UL AoA or gNB Rx-Tx time difference.

Step 2: the TRP measures the carrier phase on multiple REs of SRS resource for positioning. On the REs {k1,k2,⋯,kM}, the TRP measures the carrier phase measurement: {ω1,ω2,⋯,ωM}. Assume the carrier frequency on RE #0 is fc and the subcarrier spacing is Δf. Then for one UE, the carrier phase measurements have the following equations:

d=ω12π+N1×cfc+k1Δf

d=ω22π+N2×cfc+k2Δf

…

d=ωM2π+NM×cfc+kMΔf

Step 3: the LMF can first estimate the UE location based on the legacy positioning method.

Step 4: According to the UE location estimation in Step 3, the LMF can estimate the integer wavelength NM corresponding to each frequency fc+kmΔf.

Step 5: The LMF can estimate the NM by minimizing the following cost functions over the value range of [NM-∆,NM+∆]:

{L,N1,N2,…,NM}=argmin{L,N1,N2,…,NM}m=1ML-cfc+kmΔfφm2π+Nm2

Proposal 6: The TRP measures and reports carrier phase of multiple REs in SRS for positioning and the TRP also reports the carrier phase measurement and positioning measurement results of legacy positioning method.

- Conclusions

In this contribution, we present our views on phase measurement for NR positioning and the following proposals are made:

Proposal 1: The NR phase measurement-based positioning shall be based on the existing DL PRS and SRS for positioning.

Observation 1: Integer ambiguity in carrier phase measurement would impair the performance of NR positioning significantly.

Observation 2: Combining carrier phase measurement with DL-TDOA method does not provide better performance than the DL-TDOA method.

Observation 3: Using other positioning method to resolve the integer ambiguity could cause error of integer wavelengths.

Proposal 2: Study using carrier phase measurement of multiple carrier frequencies to resolve the issue of integer ambiguity.

Proposal 3: The UE can measure and report carrier phase of multiple REs in DL PRS and the UE reports the carrier phase measurement and positioning measurement results of legacy positioning method.

Proposal 4: Study measuring and reporting the relative phase of different Tx antenna ports of TRP for positioning.

Proposal 5: Study the PRS enhancement for supporting measuring relative phase of Tx antenna ports of TRP

Proposal 6: The TRP measures and reports carrier phase of multiple REs in SRS for positioning and the TRP also reports the carrier phase measurement and positioning measurement results of legacy positioning method.

- Reference

RP-222616 Study on expanded and improved NR positioning