3GPP TSG RAN WG1 #110bis-e		R1-2208806

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Discussions on Timing Advance Management

Agenda Item:	9.12.2

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID on further NR mobility enhancement [1] includes the following RAN1-centric objectives:

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2]

In this contribution, we will discuss the potential issues of current timing advance mechanism for supporting L1/L2 based inter-cell mobility for mobility latency reduction and then identify the potential enhancements needed for timing advance management.

- Discussion on Timing Advance

Timing advance in NR is used to control the uplink signal transmission timing. The gNB can keep measuring the time difference between PUSCH/PUCCH/SRS reception and the uplink subframe timing and then send a Timing advance command to the UE that indicates the UE to adjust the PUSCH/PUCCH/SRS transmission so that they are better aligned with the uplink subframe timing at the gNB side.

NR supports two mechanisms for delivering the timing advance information to the UE: the first mechanism is through RAR message during RACH and the second mechanism is a MAC CE message carrying a Timing advance command. The method of RAR is appliable during the initial attaching to a cell. When the UE switches to a new cell, the UE can start a random RACH procedure and the system can provide an initial timing advance command to the UE in the RAR message. Once the initial access is completed, the UE can adjust the UL transmission based on MAC CE Timing advance command.

If the current mechanisms are applied to L1/L2 based inter-cell mobility, large latency could be caused and thus the performance of L1/L2 based inter-cell mobility would be impaired significantly.

Figure 1

Take the example shown in Figure 1. A UE moves from cell 1 to cell 2. If the current mechanism of TA is applied here, the procedure would be as shown in Figure 2.

Figure 2

As shown in Figure 2, to obtain the UL synchronization to the cell 2, the UE has to conduct RACH procedure to the cell 2 and receives the initial TA command in the RAR message. Then after the connection with cell 2 is completed, the cell 2 can send MAC CE TA command to adjust the uplink timing.

Observation 1: The current TA mechanism could cause large latency in L1/L2-based inter-cell mobility.

- Enhancements of Timing Advance

As discussed in previous section, the current mechanism of TA could cause large latency to inter-cell mobility due to that the UE has to conduct RACH to obtain the initial TA towards the target cell. Therefore, to improve the latency of L1/L2 based inter-cell mobility, one enhancement is to enable the system to obtain the TA towards the target cell before the HO procedure is started. In other word, we need to support mechanism to enable the system to obtain TA of non-serving cell.

Proposal 1: Support to specify mechanism to enable system to obtain TA of non-serving cell.

One way to obtain TA of a non-serving cell is PDCCH order RACH. In current specification, the system can trigger RACH through PDCCI order. But, the current PDCCH order RACH can only be associated with SSB of serving cell. To support measuring the TA of a non-serving cell, the PDCCH order RACH shall be associated with SSB of a non-serving cell. With that, the serving cell gNB can trigger the UE to send a RACH towards an non-serving cell so that the system can measure and obtain the TA for that UE towards the non-serving cell.

Proposal 2: Support to enhance PDCCH order RACH to associate with SSB of a non-serving cell.

Another method to obtain the TA of one non-serving cell is to send SRS towards a non-serving cell. The gNB of the non-serving cell can receive and then measure the arrival timing of that SRS, which can then be used to calculate the TA of that non-serving cell. One thing to be noted is that NR positioning supports SRS resource for positioning, where the spatial relation info of one SRS resource for positioning can be SSB or PRS of a non-serving cell. The same mechanism can be utilized here. The gNB can configure a joint TCI state or UL TCI state with a SSB or CSI-RS resource of a non-serving cell to a SRS resource. That SRS resource can be measured by the non-serving cell to obtain the TA.

Proposal 3: Support transmitting SRS to a non-serving cell for measuring TA of non-serving cell.

Another necessary enhancement is to indicate the TA value of the target cell before the UE starts the HO procedure. In current specification, the earliest TA is delivered in RAR message and other TA values are delivered in MAC CE after the RRC connection is completed. To improve the latency of L1/L2-based inter-cell mobility, it is necessary to indicate the TA of a non-serving cell before the UE handovers to the target cell.

Proposal 4: Support to indicate the TA of non-serving cell, for example, indicating the TA value of the target cell before the UE handovers to the target cell.

- Conclusions

In this contribution, we investigate the issues of TA management for L1/L2-based inter-cell mobility. Accordingly, the following proposals were made:

Observation 1: The current TA mechanism could cause large latency in L1/L2-based inter-cell mobility.

Proposal 1: Support to specify mechanism to enable system to obtain TA of non-serving cell.

Proposal 2: Support to enhance PDCCH order RACH to associate with SSB of a non-serving cell

Proposal 3: Support transmitting SRS to a non-serving cell for measuring TA of non-serving cell.

Proposal 4: Support to indicate the TA of non-serving cell, for example, indicating the TA value of the target cell before the UE handovers to the target cell.

- Reference

RP-222332 Further NR Mobility Enhancements