3GPP TSG-RAN WG1 Meeting #110bis-e			R1-2208813

Electronic Meeting, October 10 – 19, 2022

Title:	[Draft] Reply LS on Pemax,c of S-SSB transmission when multiple resource pool is configured in a carrier

Response to:	R1-2208348(R4-2214421)

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	OPPO [to be RAN1]

To:	RAN4

Cc:	RAN2

Contact Person:

Name:

Tel. Number:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 thanks RAN4’s LS in R1-2208348(R4-2214421) and would like to provide our responses to the following questions asked by RAN4.

Question 1: RAN4 agreed not to define PEMAX,c for S-SSB configured Tx Power in TS38.101-1 as highlighted yellow below and would like to know if this can have any impact to RAN1/2, for example, Pcompensation defined in idle mode procedure in TS 38.304. 6.2E.4 Configured transmitted power for V2X 6.2E.4.1 General The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f, c = MIN {PEMAX,c, PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c } PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X, PRegulatory,c } where - PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively; - For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,c is the value given by IE sl-maxTransPower, defined by TS 38.331. - For the total transmitted power PCMAX,S-SSB, the PCMAX_L,f,c and PCMAX_H,f,c are defined as follows: PCMAX_L,f,c = MIN {PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c} PCMAX_H,f,c = MIN {PPowerClass, V2X, PRegulatory,c} RAN4 would like to know if there is an impact on RAN1 and RAN2 about the agreement RAN4 made on S-SSB.

RAN1 response:

If not to define PEMAX,c for S-SSB, gNB cannot control the maximum transmission power of S-SSB, which may cause interference to UL in case of SL and UL share the same carrier and FDMed in the same slot. It is preferred to introduce cell specific transmission power parameter which is used to configure maximum transmission power of S-SSB at least for Rel-17.

2. Actions:

To RAN4 and RAN2 group

RAN1 respectfully asks RAN4 and RAN2 to take the above responses into account in the future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#111                       14-18 November 2022	Toulouse

RAN1#112                       27 Februrary-03 March 2023	Athens