3GPP TSG RAN WG1 #110bis-e			R1-2208817

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CPS sensing window |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the description of CPS sensing window, the UE behaivor for periodic transmissions and aperiodic transmissions is not clearly distinguished. The case of “When the minimum M slots for CPS cannot be guaranteed” is only possible for aperiodic transsmissions, but it is used for periodic transmissions in current specification. In addition, the definition of t'y0SL is missing for aperiodic transmissions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Differentiate the case of periodic and aperiodic transmissions in the description of CPS window. -Delete “When the minimum M slots for CPS cannot be guaranteed…” in the case of periodic transmissions. -Add the deinfition of t'y0SL for aperiodic transmissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior of performing contiguous parital sensing will be unclear for both periodic and aperiodic transmissions if this CR is not approved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<omitted text>

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and if Prsvp_TX≠0, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. Prsvp_TX≠0  The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. Prsvp_TX=0

When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, or when the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

<omitted text>