3GPP TSG RAN WG1 #110bis-e		R1-2208820

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Evaluation methodology and results of SL positioning

Agenda Item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

In RAN1#110 the evaluation methodology of SL positioning was discussed with agreements achieved as below: [1]:

Agreement

For SL positioning evaluation in IIOT use case, companies should report how to drop anchor UEs and how to select anchor UEs

Agreement

Adopt the tables in section 3 of R1-2207606 as templates to collect SL positioning simulation results from each company.

Agreement

- In the evaluation, relative positioning or ranging is performed between two UEs within X m, where X value(s) are reported by companies, and companies should also report the minimum distance used in the evaluations for each use case. The assumption used for X will be included in the TR for each set of results.

Agreement

For SL positioning evaluation purpose, the following assumptions are further adopted

- Companies should report whether SL-PRS and other SL signals are FDMed or not FDMed, and whether other SL signals are present

- Adopting system level simulations (rather than the link level simulations) as the baseline tool

- For SL positioning evaluation in highway scenario or urban grid scenario, the performance metrics can include absolute horizontal accuracy, relative horizontal accuracy, ranging with distance accuracy, and ranging with direction accuracy (optionally).

- In highway and urban grid scenarios, companies can further consider other UE types, e.g. pedestrian UE or VRU devices.

In this paper we provide our evaluation results.

Simulation results

In line with the evaluation methodology agreed in previous meetings the performance of TDOA based absolute positioning are evaluated in highway, urban and IIoT scenarios. In the TDOA method evaluation, only RSUs are used as anchor nodes assuming all of them are perfectly synchronized. The evaluation results are shown in Figure 1~3 and summarized in corresponding tables respectively.

Table 1 Common assumption for all scenarios if they are different from or not specified in Agreements

| Parameter | Value |
| --- | --- |
| Carrier frequency | 6GHz |
| Subcarrier spacing | 30kHz |
| Reference Signal Transmission Bandwidth | 20/40/100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-2, fully staggered |
| Reference signal including PRS, SRS and SL-PRS (type of sequence, number of ports, …) | Gold Sequence, 1 port |
| Number of symbols used per occasion | 2 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | No boosting |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Not modelled |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Max likelihood |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | CHAN |
| Synchronization assumptions | Perfect |
| UE/gNB RX and TX timing error assumption | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | N/A |
| Additional notes, if any | N/A |

2.1 Highway

In this section we provide evaluation assumptions and evaluation results for Highway scenario.

Table 2.1-1 Assumptions for highway if they are different from or not specified in Agreements

| Parameters | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| UE Antenna model | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) |
| TRP antenna model | (1, 2, 2, 1, 1) for RSU | (1, 2, 2, 1, 1) for RSU | (1, 2, 2, 1, 1) for RSU |
| BS/RSU deployment for absolute positioning | uniformly located with 200m spacing on both sides of highway symmetrically | uniformly located with 200m spacing on both sides of highway symmetrically | uniformly located with 200m spacing on both sides of highway symmetrically |
| Bandwidth | 20MHz | 40MHz | 100MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | NA | NA | NA |
| Positioning method | TDOA | TDOA | TDOA |

Figure 1 CDF of TDOA based absolute positioning errors (Highway)

Table 2.1-2 Simulation results for highway for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, 20MHz, TDOA, absolute positioning | 1.76m | 2.12m | 3.12m | 4.99m | No | No |
| Case #2, 40MHz, TDOA, absolute positioning | 0.74m | 1.24m | 1.85m | 2.29m | NO | NO |
| Case #3, 100MHz, TDOA, absolute positioning | 0.31m | 0.52m | 0.75m | 1.1m | Yes | NO |

2.2 Urban

In this section we provide evaluation assumptions and evaluation results for Urban scenario.

Table 2.2-1 Assumptions for urban if they are different from or not specified in Agreements

| Parameters | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| UE Antenna model | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) |
| TRP antenna model | (1, 2, 2, 1, 1) for RSU | (1, 2, 2, 1, 1) for RSU | (1, 2, 2, 1, 1) for RSU |
| BS/RSU deployment for absolute positioning | Located at intersection | Located at intersection | Located at intersection |
| Bandwidth | 20MHz | 40MHz | 100MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | NA | NA | NA |
| Positioning method | TDOA | TDOA | TDOA |

Figure 2 CDF of TDOA based absolute positioning errors (Urban)

Table 2.2-2 Simulation results for urban for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, 20MHz, TDOA, absolute positioning | 4.56m | 5.61m | 8.99m | 13.59m | NO | NO |
| Case #2, 40MHz, TDOA, absolute positioning | 3.55m | 5.05m | 6.05m | 9.05m | NO | NO |
| Case #3, 100MHz, TDOA, absolute positioning | 2.56m | 3.99m | 4.93m | 5.66m | NO | NO |

2.2 IIoT

In this section we provide evaluation assumptions and evaluation results for IIoT scenario.

Table 2.3-1 Assumptions for IIoT if they are different from or not specified in Agreements

| Parameters | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| UE Antenna model | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) |
| TRP antenna model | (1, 2, 2, 1, 1) for RSU | (1, 2, 2, 1, 1) for RSU | (1, 2, 2, 1, 1) for RSU |
| BS/RSU deployment for absolute positioning | TRPs in InF-SH defined in TR 38.857 is replaced with anchor UEs | TRPs in InF-SH defined in TR 38.857 is replaced with anchor UEs | TRPs in InF-SH defined in TR 38.857 is replaced with anchor UEs |
| Bandwidth | 20MHz | 40MHz | 100MHz |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | NA | NA | NA |
| Positioning method | TDOA | TDOA | TDOA |

Figure 3 CDF of TDOA based absolute positioning errors (IIoT)

Table 2.3-2 Simulation results for IIoT for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, 20MHz, TDOA, absolute positioning | 0.98m | 1.33m | 2.05m | 3.49m | NO | NO |
| Case #2, 40MHz, TDOA, absolute positioning | 0.48m | 0.66m | 0.99m | 1.56m | NO | NO |
| Case #3, 100MHz, TDOA, absolute positioning | 0.23m | 0.33m | 0.50m | 0.87m | Yes | NO |

From the evaluation results it can be seen that, in highway scenario, the Set-A horizontal accuracy requirement for absolute positioning (1.5 m for 90% of UEs) can be satisfied with 100MHz SL PRS bandwidth, but Set-B requirements (0.5 m for 90% of UEs) cannot be satisfied.; In urban scenario, due to the lack of LOS signal, both Set-A and Set-B accuracy requirements for absolute positioning cannot be satisfied.  In IIoT scenario, the Set-A requirement for absolute positioning (1 m for 90% of UEs) can be satisfied with 100MHz SL PRS bandwidth, but Set-B requirements (0.2 m for 90% of UEs) cannot be satisfied.

Observation 1:

- In Highway scenario, Set-A horizontal accuracy requirement for absolute positioning can be satisfied with 100 MHz SL PRS bandwidth, but cannot be satisfied with smaller bandwidth, Set-B requirement cannot be satisfied with 100 MHz or smaller SL PRS bandwidth;

- In urban scenario, both Set-A and Set-B horizontal accuracy requirement for absolute positioning cannot be satisfied with 100 MHz or smaller SL PRS bandwidth;

- In IIoT scenario, Set-A horizontal accuracy requirement for absolute positioning can be satisfied with 100 MHz SL PRS bandwidth, but cannot be satisfied with smaller bandwidth, Set-B requirement cannot be satisfied with 100 MHz or smaller SL PRS bandwidth;

Conclusion

In this contribution, we provided our evaluation results for Highway, Urban and IIoT scenarios, we have following observation:

Observation 1:

- In Highway scenario, Set-A horizontal accuracy requirement for absolute positioning can be satisfied with 100 MHz SL PRS bandwidth, but cannot be satisfied with smaller bandwidth, Set-B requirement cannot be satisfied with 100 MHz or smaller SL PRS bandwidth;

- In urban scenario, both Set-A and Set-B horizontal accuracy requirement for absolute positioning cannot be satisfied with 100 MHz or smaller SL PRS bandwidth;

- In IIoT scenario, Set-A horizontal accuracy requirement for absolute positioning can be satisfied with 100 MHz SL PRS bandwidth, but cannot be satisfied with smaller bandwidth, Set-B requirement cannot be satisfied with 100 MHz or smaller SL PRS bandwidth;

References

- RAN1 Chairman’s Notes, RAN1#110, August, 2022.