3GPP TSG RAN WG1 #110bis-e R1-2208830 E-meeting, October 10th-19th, 2022 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 17.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on interpretation SFN indicating epoch time. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_NTN_solutions-Core; Date:; 2022-10-10. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; 1. In RAN1 meeting #107e, the following agreement on epoch time determination was made and was not reflected in the specification. Agreement - When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. - Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted. - When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number. 2. How to interpret the SFN indicating epoch time is unclear.. -. Summary of change:; 1. Clarify the epoch time can be explicitly indicated by a SFN and a sub-frame number or be implicitly determined by the end of SI window based on the agreement. 2. Clarify the interpretation of SFN indicating epoch time, i.e., the SFN indicates a frame which is nearest to the frame where the ephemeris message is received.. Consequences if not approved:; 1. Missing agreement on epoch time determination. 2. Potential ambiguity in interpretation SFN indicating epoch time.. Clauses affected:; 4.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

### 4.2	Transmission timing adjustments

<Unchanged parts are omitted>

Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as:

Delaycommont= TACommon2+ TACommonDrift2×t−tepoch+TACommonDriftVariant2×t−tepoch2

where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepocℎ is the epoch time of TACommon, TACommonDrift, and TACommonDriftVariant [12, TS 38.331]. The epoch time is indicated by a SFN and a sub-frame number, if provided, or is the end of SI window where the higher-layer ephemeris parameters are received otherwise, for which the SFN indicates a frame nearest to the frame where the higher-layer ephemeris parameters are received. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

<Unchanged parts are omitted>