3GPP TSG RAN WG1 #110bis-e		R1-2208832

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Discussion on NW energy savings performance evaluation

Agenda Item:	9.7.1

Document for:	Discussion and Decision

Introduction

A study item (SI) on network energy savings for NR was approved in RAN#94-e meeting[1]. As a part of the objectives of the SI, one sub-agenda is to define a base station energy consumption model, an evaluation methodology and KPIs.

During RAN1#110 meeting, extensive discussion was conducted on the BS energy consumption model, including power state categorization, relative power values and transition time/energy for various power state, power scaling method and the remaining issues of reference configuration. The detailed agreements are listed in the appendix.

In this contribution, we provide our view and analysis on the BS energy consumption model firstly, including reference configuration, transition time/energy and power scaling method. Then, the average BS energy consumption per slot can be calculated based on the model. Finally, some preliminary simulation results and analyses on the potential NW energy saving schemes are also provided.

BS energy consumption model

A reliable BS energy consumption model is crucial for NW energy saving performance evaluation. The power consumption computed based on the model should show relatively consistent trend with the actual BS power consumption. According to the Objective 1 of the SID, RAN1 should adapt the framework of the UE power consumption model to the BS side. With regards to this matter, the following aspects have been considered for BS power consumption model in the last two meeting:

- Reference configuration for Set1-FR1/Set2-FR1/Set3-FR2, including system BW, SCS, number of TxRUs and total DL power level.

- Multiple power states including sleep and non-sleep modes with relative power value and additional transition energy/transition time.

- Power scaling method in spatial/frequency/power/time domain, etc.

Power state and transition time/energy

In RAN1#110 meeting, the BS power consumption model was extensively discussed and the table showing a whole picture of the BS power consumption has been agreed as follows:

Agreement

For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set.

- FFS: One or multiple values for relative power and transition time.

| Power state | Characteristic | Relative Power | Additional transition energy3 | Total transition time |
| --- | --- | --- | --- | --- |
| Deep sleep1 | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. | P1=1 | E1 | T1 |
| Light sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1) | P2 | E2 | T2 |
| Micro sleep | There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state. | P3 | 0 | 0 |
| Active DL | There is only DL transmission. | P4 | NA | NA |
| Active UL | There is only UL reception. FFS: Whether multiple P5 values are needed to address low power UL mode | P5 | NA | NA |
| Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined. |  |  |  |  |

- For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study.

- FFS: Optionally, a state machine where BS may transit between sleep modes without entering non-sleep mode can be considered. Companies are to report the involved sleep modes and the assumptions for inter-sleep mode transition time used in their evaluations.

- FFS: Details on how to use the above table for low power uplink reception (e.g. for WUS).

According to the agreed framework, two active states without differentiation of various channels/signals and three sleep modes are defined in the BS consumption model. Besides, it has stipulated that there is neither DL transmission nor UL reception during the sleep state. In this case, the gNB should determine the target sleep mode i from the beginning based on the current traffic load, and perform one-shot transition between the active state and i-th sleep mode, as shown in Fig 1.

Fig 1. One-shot transition between the active state and i-th sleep mode

For evaluatiing the BS power consumption, the relative power value and total transition time have been carefully discussed in the last meeting, and two cat of values provided in the following table are taken as a working assumption for reference configuration set 1. In this matter, companies can choose one or both cat of values to calculate the relative energy consumption for the interest of study.

Working Assumption

For reference configuration set 1, the values are provided as below. FFS set2 and set 3.

| Power state | Relative Power P |  | Total transition time T |  |
| --- | --- | --- | --- | --- |
| Deep sleep | 1 | 1 | Cat 1: 50ms | Cat 2: 10s |
| Light sleep | Cat 1: 25 | Cat 2: 2.1 | Cat 1: 6 ms | Cat 2: 640 ms |
| Micro sleep | Cat1: 55 | Cat 2: 5.5 | 0 | 0 |
| Active DL | Cat 1: 280 | Cat 2: 32 | N.A. | N.A. |
| Active UL | Cat 1: 110 | Cat 2: 6.5 | N.A. | N.A. |

Agreement

- The total transition time for set 2 and set 3 is the same as that for set 1.

- Companies are encouraged to check the input and values provided in section 2.1.4.2 of R1-2208312 for further determination.

The design logic of values for reference configuration set 1 can be reused for set 2 and set 3 for the consistence of the model. It has been agreed that the same transition time as set 1 is adopted for set 2 and set 3, but the relative power value will be further discussed. Considering the total DL power level of 49dBm for set 2 FR1 and 33dBm for set 3 FR2 is smaller than that of set 1, the lower relative power values than set1 are expected.

Proposal 1: For reference configuration set 2 and set 3, lower relative power values than set 1 are expected.

The additional transition energy is not stabilized yet. Similar to UE power saving study, the additional transition energy and total transition time also include energy and time for both ramp up and ramp down transition. Thus, the additional transition energy Ei can be calculated based on Ei≈12×∆i×Ti, where ∆i=P3-Pi is the power difference between micro sleep and sleep mode i, Ti is the corresponding total transition time of sleep mode i. Then, the concrete transition energy for set 1 can be computed as shown in proposal 2. The computing formula also applies to set 2 and set 3.

Proposal 2: For set 1, the following additional transition energy for the three sleep modes is taken as working assumption:

| Power state | Additional transition energy Ei (relative power x ms) |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 |
| Light sleep | 90 | 1088 |
| Micro sleep | 0 | 0 |

Proposal 3: The computing formula of transition energy Ei for set 1 also applies to set 2 and set 3.

Scaling method

In RAN1#110 meeting, it was agreed that the relative power value of the active DL/UL state in the power consumption model is provided based on reference configuration:

Agreement

For non-sleep mode, the relative power value in power model table for UL reception and/or DL transmission is provided based on reference configuration.

Considering the actually used resource may be less than that in the reference configuration, power scaling should be applied to reflect the more realistic BS power consumption. The detailed scaling method has been deeply discussed in the last meeting but not stabilized. In our understanding, the BS power scaling should be considered at least in time, spatial, frequency and power domain.

Providing a single formula is helpful to derive the ultimate BS power consumption after scaling. Firstly, there should a static part in the formula accounting for the power which is irrelevant to any transmission/reception. In our understanding, the relative power of micro sleep P3 can be considered as the static part, and the dynamic part P4-P3 will be scaled with the percentage of the active resource. Secondly, in spatial domain, the power is linearly scaled with the percentage of active TRxRUs, i.e., sa. Thirdly, to address the non-linear PA efficiency effects, jointly scaling in frequency and power domain sf*spηsf, sp can be further introduced, where sf, sp and ηsf, sp is the resource usage ratio in frequency domain, the PSD scaling factor relative to reference configuration and the PA efficiency respectively.

Based on the analysis above, the power consumption of DL transmission can be provided by

P=P3+ sa*sf*spηsf, sp*P4-P3

Proposal 4: The power consumption of DL transmission is provided by

P=P3+ sa*sf*spηsf, sp*P4-P3

- sa, sf, sp and ηsf, sp is the percentage of active TRxRUs, the resource usage ratio in frequency domain, the PSD scaling factor relative to reference configuration and the PA efficiency respectively.

Power scaling in time domain is direct because immediate transition is assumed in the BS power consumption model. When a slot-level model is used, the BS power consumption P can apply to the active DL symbols within a slot and the unoccupied symbols can be treated as micro sleep. In this case, the BS power consumption over a slot can be scaled by 1-α*P3+α*P, where α is the ratio of the number of active DL symbols within a slot.

Proposal 5: When a slot-level model is used, the BS power consumption over a slot can be scaled by 1-α*P3+α*P, where α is the ratio of the number of active DL symbols within a slot.

BS energy consumption

In RAN1#110 meeting, the evaluation methodology of NES gain was discussed, and the agreement is shown as follows:

Agreement

For evaluation purpose, network energy saving gain is computed based on the energy consumptions for a technique and the baseline over the same duration.

To evaluate the NES gain, the BS energy consumption over a time duration should be computed firstly. When the power state at different time instant is determined, the total energy consumption Etotal including the energy consumption Estate of each power state and additional transition energy Etrans is provided by

Etotal=Estate+Etrans=i=15Pi*Di+i=13Ei*Li

where Pi and Di are relative power value and time duration in unit of slot for power state i, Ei and Li are transition energy and the number of transitions for sleep mode i.

Observation 1: The total energy consumption Etotal including the energy consumption Estate of each power state and additional transition energy Etrans is provided by

Etotal=Estate+Etrans=i=15Pi*Di+i=13Ei*Li

where Pi and Di are relative power value and time duration in unit of slot for power state i, Ei and Li are transition energy and the number of transitions for sleep mode i.

Evaluation methodology and simulation results

Evaluation assumptions

In RAN1#110 meeting, extensive discussions on the simulation assumption were conducted and the related agreements were made as follows:

Agreement

- For FR1, adopt the Reference SLS configurations in Annex-A in R1-2208312 as baseline SLS assumptions.

  - Other carrier frequencies can be optionally considered.

- FFS For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption.

  - BS antenna configurations

    - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

    - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ)

  - Traffic model & UE density

    - Follow previous agreements with adjusted UE density

  - Total transmit power per TRxP

    - Value scaled from that in set 3 reference configuration considering BW

- Further adjustment/clarification can be discussed in the next meeting.

The reference SLS configurations provide good alignment on the simulation assumptions so that the evaluation results from different contribution comparable. However, we note that some parameters need to be updated, e.g., the channel model is 3D-Uma based on TR 38.901, the percentage of high loss and low loss building type is 100% low loss.

Proposal 6: Update the following parameters in the reference SLS configurations:

- The channel model is 3D-Uma based on TR 38.901.

- The percentage of high loss and low loss building type is 100% low loss.

Evaluation results

Some preliminary simulation results for the NW energy saving schemes in time, frequency and spatial domains are provided in this section. The detailed simulation parameters can be found in the appendix.

Comparison between legacy and NES BS:

To effectively evaluate the energy saving gains of the candidate NES techniques, it is necessary to align the evaluation baseline for BS energy saving study. The parameters provided in the reference configuration can be taken as baseline simulation assumption, e.g.., system BW is 100MHz and the number of Tx/Rx RUs is 64. However, whether power scaling and sleep mode is supported by legacy BS needs further discussion. The energy consumption comparison for the BS without power scaling and sleep mode, with power scaling and only micro sleep, and with power scaling and ideal sleep is provided in Fig 2, where the difference is obvious.

Fig 2. Energy consumption comparison between legacy and NES BS

Proposal 7: RAN1 should discuss whether power scaling and sleep mode is supported by legacy BS to align the evaluation baseline for BS energy saving study.

Frequency and spatial domain energy saving:

The potential energy schemes in frequency and spatial domain are evaluated firstly. The parameters provided in the reference configuration are taken as baseline simulation assumption, i.e., system BW is 100MHz and the number of Tx/Rx RUs is 64. For comparison, the BW and the number of Tx/Rx RUs is reduced to 20MHz and 8 Tx/Rx RUs for energy saving performance evaluation in frequency and spatial domain respectively.

The RU, BO and ρ comparison are provided in our simulation results, where RU is resource utilization, BO is buffer occupancy (i.e., sum of the period of time during which the BS has data to transmit / total simulation time), and ρ is packet completion rate.

Fig 3. RU, BO and ρ comparison for FTP3 model with 0.5MB packet size and 200ms inter-arrival time

As shown in Fig 3, energy saving in frequency domain will significantly increase the RU and BO due to available BW reduction, resulting in the reduced packet completion rate. Similarly, for energy saving in spatial domain, the RU and BO increase about 20%, further leading a slight reduction of the packet completion rate.

As proposed in the agreement, latency, UPT and energy consumption are also considered in the energy saving performance evaluation. As shown in Fig 4, the latency and UPT is severely affected by BW reduction, which results that more slots are used for transmission and the energy consumption is increased instead. On the other hand, Tx/Rx RUs reduction can provide 15% energy saving gain but with 60% performance loss.

Fig 4. Latency, UPT and energy consumption comparison for FTP3 model with 0.5MB packet size and 200ms inter-arrival time

For energy saving evaluation in Fig 4, the energy consumption is computed based on the total energy consumption formula Etotal and further averaged to one slot. In addition, two statistical methods on energy consumption are used in our evaluation results. For ideal sleep, the BS can enter into the ideal sleep mode as long as the time interval is larger than the corresponding transition time. For only micro sleep, the BS can only enter into the micro sleep mode due to unawareness of the traffic arrival time.

Observation 2: For FTP3 model, BW reduction will result in remarkable performance loss on latency and UPT and increased energy consumption.

Observation 3: For FTP3 model, Tx/Rx RUs reduction can provide 10% energy saving gain but with 60% performance loss on latency and UPT.

Time domain energy saving:

Similar to UE power saving study, the potential energy saving evaluation in time domain are associated with DRX configuration. Furthermore, UE-specific DRX and UE-aligned DRX pattern are considered in our simulation. As shown in Fig 5, introducing DRX will reduce the RU, further leading to increased BO, resulting in reduced packet completion rate.

Fig 5. RU, BO and ρ comparison for FTP3 model with 0.5MB packet size and 200ms inter-arrival time

Fig 6. Latency, UPT and energy consumption comparison for FTP3 model with 0.5MB packet size and 200ms inter-arrival time

On the other hand, as shown in Fig 6, introducing DRX will increase UE delay and decrease UE throughput, and bring a little energy saving gain compared with no DRX. In addition, comparing with UE specific DRX, UE aligned DRX pattern has slight impact on UE throughput.

Observation 4: For FTP3 model, introducing DRX will increase UE delay and decrease UE throughput, and bring a little energy saving gain.

Observation 5: Comparing with UE specific DRX, UE aligned DRX pattern has slight impact on UE throughput.

To have a comprehensive evaluation on the NW energy saving schemes, the simulation results with FTP3 IM model are also provided. As shown in Fig 7, the observation of FTP3 IM model is similar to that of FT3 model, BW reduction and Tx/Rx RUs reduction leads to 70% and 40% performance loss on UE throughput. However, the energy saving gain is marginal due to the little energy saving space.

Observation 6: For FTP3 IM model, BW reduction and Tx/Rx RUs reduction bring little energy saving gain with 70% and 40% performance loss on UE throughput.

Fig 7. Latency, UPT and energy consumption comparison for FTP3 IM model with 0.1MB packet size and 2s inter-arrival time

On the other hand, when DRX (320ms DRX period) is used for the energy saving evaluation in time domain, significant performance degradation happens as shown in Fig 8. If the BS can only enter into micro sleep when DRX is not configured, the energy saving gain is about 30%. In addition, comparing with UE specific DRX, UE aligned DRX pattern bring little energy saving gain with slight impact on UE throughput.

Observation 7: For FTP3 IM model, introducing DRX will increase UE delay and decrease UE throughput, and brings energy saving gain of 30%.

Observation 8: Comparing with UE specific DRX, UE aligned DRX pattern bring little energy saving with slight impact on UE throughput.

Fig 8. Latency, UPT and energy consumption comparison for FTP3 IM model with 0.1MB packet size and 2s inter-arrival time

Conclusion

In this contribution, the BS energy consumption model is discussed and the average BS energy consumption per slot can be calculated based on the model. Then, we present some preliminary simulation results and analyses on the potential NW energy saving schemes. The following proposals and observations are made:

Proposal 1: For reference configuration set 2 and set 3, lower relative power values than set 1 are expected.

Proposal 2: For set 1, the following additional transition energy for the three sleep modes is taken as working assumption:

| Power state | Additional transition energy Ei (relative power x ms) |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 |
| Light sleep | 90 | 1088 |
| Micro sleep | 0 | 0 |

Proposal 3: The computing formula of transition energy Ei for set 1 also applies to set 2 and set 3.

Proposal 4: The power consumption of DL transmission is provided by

P=P3+ sa*sf*spηsf, sp*P4-P3

- sa, sf, sp and ηsf, sp is the percentage of active TRxRUs, the resource usage ratio in frequency domain, the PSD scaling factor relative to reference configuration and the PA efficiency respectively.

Proposal 5: When a slot-level model is used, the BS power consumption over a slot can be scaled by 1-α*P3+α*P, where α is the ratio of the number of active DL symbols within a slot.

Proposal 6: Update the following parameters in the reference SLS configurations:

- The channel model is 3D-Uma based on TR 38.901.

- The percentage of high loss and low loss building type is 100% low loss.

Proposal 7: RAN1 should discuss whether power scaling and sleep mode is supported by legacy BS to align the evaluation baseline for BS energy saving study.

Observation 1: The total energy consumption Etotal including the energy consumption Estate of each power state and additional transition energy Etrans is provided by

Etotal=Estate+Etrans=i=15Pi*Di+i=13Ei*Li

where Pi and Di are relative power value and time duration in unit of slot for power state i, Ei and Li are transition energy and the number of transitions for sleep mode i.

Observation 2: For FTP3 model, BW reduction will result in remarkable performance loss on latency and UPT and increased energy consumption.

Observation 3: For FTP3 model, Tx/Rx RUs reduction can provide 10% energy saving gain but with 60% performance loss on latency and UPT.

Observation 4: For FTP3 model, introducing DRX will increase UE delay and decrease UE throughput, and bring a little energy saving gain.

Observation 5: Comparing with UE specific DRX, UE aligned DRX pattern has slight impact on UE throughput.

Observation 6: For FTP3 IM model, BW reduction and Tx/Rx RUs reduction bring little energy saving gain with 70% and 40% performance loss on UE throughput.

Observation 7: For FTP3 IM model, introducing DRX will increase UE delay and decrease UE throughput, and brings energy saving gain of 30%.

Observation 8: Comparing with UE specific DRX, UE aligned DRX pattern bring little energy saving with slight impact on UE throughput.

Reference

[1]       RP-220297, “Revised SI: Study on network energy savings for NR”, Huawei, RAN#95-e, March 2022

[2]       RAN1#110 meeting chairman notes

[3]       3GPP TR 38.840: Study on User Equipment (UE) power saving in NR

Appendix

simulation assumptions

Table 1 The evaluation assumption for BS power consumption model

| Channel model | 3D-Uma based on TR 38.901 |
| --- | --- |
| Device deployment | 5 UEs per TRP. 80% indoor, 20% outdoor |
| Percentage of high loss and low loss building type | 100% low loss |
| Inter-site distance | 500m |
| Network Topology | 7*3 Sector |
| Carrier Frequency | 2.6GHz |
| Multiple access | OFDMA |
| Duplexing | TDD (for set 1 ref. config.) |
| Numerology | 30kHz, 14 OFDM symbol slot |
| Guard band ratio on simulation bandwidth | TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth) |
| Simulation bandwidth | 100MHz |
| Frame structure | DDDSU |
| UT attachment | Based on RSRP |
| Wrapping around method | Geographical distance based wrapping |
| Traffic model | FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time), FTP3 IM (0.1MB as packet size, 2s as mean inter-arrival time) |
| BS antenna height | 25 m |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| Antenna configuration at TRxP | For 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8). based on 38.802 |
| UE power class | 23dBm |
| UE noise figure | 9 dB |
| UE antenna element gain | 0 dBi |
| UE antenna height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height |
| Antenna configuration at UE | For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, 0.8)λ |
| Modulation | Up to 256 QAM |
| Transmission scheme | SU-MIMO |
| SU dimension | For 4Rx: Up to 4 layers |
| DL CSI measurement | Precoded CSI-RS based |
| DL codebook | non-PMI transmission |
| SRS transmission | For UE 4 Tx ports: Non-precoded SRS |
| CSI feedback | CQI, RI: every 5 slot; Subband based |
| Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Non-ideal |
| SSB period | 20ms |

Agreements at RAN1#110 meeting

Agreement

For non-sleep mode, the relative power value in power model table for UL reception and/or DL transmission is provided based on reference configuration.

Agreement

For set 2 FR1 FDD TxRx reference configuration, confirm the WA as 32 in reference configuration.

Agreement

The total DL power level is 49 dBm for set 2 FR1 FDD reference configuration.

Agreement

For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set.

- FFS: One or multiple values for relative power and transition time.

| Power state | Characteristic | Relative Power | Additional transition energy3 | Total transition time |
| --- | --- | --- | --- | --- |
| Deep sleep1 | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. | P1=1 | E1 | T1 |
| Light sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1) | P2 | E2 | T2 |
| Micro sleep | There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state. | P3 | 0 | 0 |
| Active DL | There is only DL transmission. | P4 | NA | NA |
| Active UL | There is only UL reception. FFS: Whether multiple P5 values are needed to address low power UL mode | P5 | NA | NA |
| Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined. |  |  |  |  |

- For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study.

- FFS: Optionally, a state machine where BS may transit between sleep modes without entering non-sleep mode can be considered. Companies are to report the involved sleep modes and the assumptions for inter-sleep mode transition time used in their evaluations.

- FFS: Details on how to use the above table for low power uplink reception (e.g. for WUS).

Working Assumption

For reference configuration set 1, the values are provided as below. FFS set2 and set 3.

| Power state | Relative Power P |  | Total transition time T |  |
| --- | --- | --- | --- | --- |
| Deep sleep | 1 | 1 | Cat 1: 50ms | Cat 2: 10s |
| Light sleep | Cat 1: 25 | Cat 2: 2.1 | Cat 1: 6 ms | Cat 2: 640 ms |
| Micro sleep | Cat1: 55 | Cat 2: 5.5 | 0 | 0 |
| Active DL | Cat 1: 280 | Cat 2: 32 | N.A. | N.A. |
| Active UL | Cat 1: 110 | Cat 2: 6.5 | N.A. | N.A. |

Agreement

For evaluation purpose,

- a load (L) of a cell is a percentage of resources used for UE specific PDSCH / PUSCH

- The following load scenarios are considered

| Load scenario | Characteristics |
| --- | --- |
| Idle/empty load | - Include cell-specific signals and channels, and - L = 0 |
| low load | - Include cell-specific signals and channels, and - 0 < L≤15 |
| Light load | - Include cell-specific signals and channels, and - 0 < L≤ [30] |
| Medium load | - Include cell-specific signals and channels, and - [30] < L≤ [50] |
| For CA, the companies report whether the load is defined per CC or across all CCs. |  |

Agreement

- For FR1, urban micro can be optionally considered.

- For FR2, urban micro is prioritized, with ISD=200 m is assumed.

Agreement

It is up to company report which traffic model is used among the agreed three traffic models in their evaluations.

- Other models may be used as well. Parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

Agreement

For set 3 FR2 reference configuration, the total DL power level and EIRP limit is set as 33 dBm and 63 dBm respectively. Note EIRP limit is also scaled with the number of TxRU.

Agreement

For evaluation purpose, network energy saving gain is computed based on the energy consumptions for a technique and the baseline over the same duration.

Agreement

For initial evaluations, there is always a non-sleep mode assumed between adjacent sleep modes.

Agreement

Companies to report the assumption details for the reception of a low-power UL channel/signal, if used, including power states, additional transition energy, and transition times, receiver details (e.g. architecture and receiver sensitivity), and other impact/change on the power consumption model.

Agreement

Update the RAN1 agreements with the following changes

In the evaluation,

- A load (L)% of a cell is a percentage of resources used for UE specific PDSCH/PUSCH.

- The following load scenarios are considered.

| Load scenario | Characteristics |
| --- | --- |
| Idle/empty load | - Include cell-specific signals and channels, and - L = 0 |
| low load | - Include cell-specific signals and channels, and - 0 < L≤15 |
| Light load | - Include cell-specific signals and channels, and - 150 < L≤30 |
| Medium load | - Include cell-specific signals and channels, and - 30 < L≤50 |
| For CA, the companies report whether the load is defined per CC or across all CCs. |  |

Agreement

During the transition time period, relative power of sleep mode i iis assumed to be consumed. Additional transition energy and total transition time also include energy and time for both ramping down and ramping up

Agreement

- The total transition time for set 2 and set 3 is the same as that for set 1.

- Companies are encouraged to check the input and values provided in section 2.1.4.2 of R1-2208312 for further determination.

Conclusion

Companies are encouraged to check discussion in section 2.2.2 of R1-2208312 for scaling discussion in the next meeting.

Agreement

- FFS whether to set exact requirements/QoS target for UPT and/or latency impact

- Other KPIs can be optionally reported, conditioned with clear definition/descriptions provided.

- Note for potential new channel/signals, e.g. WUS from UE, the assumption for detection reliability at BS side is reported (performance and complexity impact would subject to results and further discussion).

Agreement

It is up to company report the use of UE C-DRX.

- the baseline configuration (for alignment/calibration) for C-DRX, if reported, can be as below;

- Other inactivity timer values can be optionally reported

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |
| DRX Period | 160 ms | 320 ms | 40 ms |
| DRX Inactivity timer | 100 ms | 80 ms | 10 ms |
| On duration | FR1: 8 ms FR2: 4 ms | FR1: 10 ms FR2: 5 ms | FR1: 4 ms FR2: 2 ms |

Agreement

- For FR1, adopt the Reference SLS configurations in Annex-A in R1-2208312 as baseline SLS assumptions.

  - Other carrier frequencies can be optionally considered.

- FFS For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption.

  - BS antenna configurations

    - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

    - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ)

  - Traffic model & UE density

    - Follow previous agreements with adjusted UE density

  - Total transmit power per TRxP

    - Value scaled from that in set 3 reference configuration considering BW

- Further adjustment/clarification can be discussed in the next meeting.