3GPP TSG RAN WG1 #110bis-e                                                    	 R1-2208849

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	On general aspects of AI/ML framework

Agenda Item:	9.2.1

Document for:	Discussion

Introduction

The general aspects of AI/ML framework have been discussed in RAN1#109-e and RAN1#110-e meeting. The agreements/conclusions from previous meetings are listed below. In this paper, we further introduce our understandings and suggestions regarding the collaboration levels, AI/ML model transfer, deployment, inference and training procedures, general evaluation methodology, complexity issues, and specification impacts.

Agreement

- Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

- Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model by learning the input/output relationship in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing do not assume subsequent tuning of the model. |
| Online training | TBD - need more discussion |
| Offline training | TBD - need more discussion |
| On-UE training | Online/offline training at the UE |
| On-network training | Online/offline training at the network |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| Model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Model deployment | Delivery of a fully developed and tested model runtime image to a target UE/gNB where inference is to be performed. |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple model exchanges, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online (field) data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Model update | Retraining or fine tuning of an AI/ML model, via online/offline training, to improve the model inference performance. |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data e.g., clustering is a common example of this. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

1.	Level x: No collaboration

2.	Level y: Signaling-based collaboration without model transfer

3.	Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIs

Note: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note: Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

Discussions

General principle for identifying representative sub use cases

In RAN1#109-e and RAN1#110-e meetings, some sub use cases have been identified for the first-round study. Meanwhile, companies are proposing some other sub use cases. In our view, the primary target of the study item is to focus on the study on the identified sub use cases. Other sub use cases can still be considered but with carefully selection, considering the following aspects.

- Potential performance gain (e.g., shown in preliminary evaluation results).

- Feasible evaluation methodology and valid training data set (incl. training set generation methodology).

- Reasonable non-AI/ML-based baseline for performance gain analysis.

- Potential specification impacts.

Considering the study capacity of each AI/ML use case, focusing on 1 – 2 representative use cases is preferred. But we are not suggesting to stop/deprioritize discussions for other sub use cases at this moment. They can be further studied and discussed in AI 9.2.x.2. And if some sub use case has no clear non-AI/ML-based baseline for performance or potential specification impacts, the sub use case can still be studied in AI 9.2.x.2. But the sub use cases with all the four aspects should be studied/evaluated with a higher priority, especially in AI 9.2.x.1.

Proposal 1: Focus on the identified representative sub use cases in AI 9.2.x.1 for the corresponding studies on their evaluation methodology, KPI, and performance evaluation results. Investigate the following aspects for other potential sub use case in AI 9.2.x.2.

- Potential performance gain (e.g., shown in preliminary evaluation results).

- Feasible evaluation methodology and valid training data set (incl. training set generation methodology).

- Reasonable non-AI/ML-based baseline for performance gain analysis.

- Potential specification impacts.

Note: Sub use cases without 3) and/or 4) will not be precluded. But target to limit the number of the representative sub use cases to 1 or 2 for each use case.

Evaluations and calibration

In RAN1#109-e and RAN1#110-e, it has been agreed to use 3gpp channel models (TR 38.901) as the baseline for evaluations.

But we still think more works need to be done to align different understandings on data for this study, e.g. how to align the data sets used for evaluations. One way to make it would be aligning the method to generate date sets for each sub use cases. We can make some detailed parameter setting lists to describe how to generate the data. Then different companies can follow the aligned setting list to generate their local data and to train/test their results. This way would be a basic approach in Rel-18 study from our understanding.

Another way is to directly use some common data set(s). Although we can define parameter lists to align how to generate the data, it is still risky that companies cannot align for all the parameters and random factors that used to generate a channel or a data. Then the data set used for companies may still be different. We can tolerate these differences when comparing the simulation results. But if we want to calibrate the performance gain, it would better to use the completely-aligned data set. For each sub use case, common data set(s) provided by companies or third parties could be used as references, at least for calibration. In [4], some data sets and reference models relate to CSI, BM and Position are provided for free downloading.

In RAN1#109-e, the following options were discussed in the email thread for different levels of calibrations:

- Option 1: Common dataset and reference model for calibration.

- Option 2: Common dataset without reference model for calibration.

- Option 3: Reference model without common dataset for calibration.

- Option 4: No calibration.

Based on the above analysis, we think at least one among Option 1-3 is needed. Option 4 will stop us from finding the problems in the evaluation, and not benefit for the study.

In principle, the data set captured from field test would be helpful to improve the quality of AI/ML training set. But the use of field data in AI/ML study needs to be carefully evaluated and cautiously considered. Only the field data with fundamental generalization characteristics and reflecting typical 5G channel conditions (e.g. captured from a number of cells covering various types of deployment scenarios) are qualified for the AI/ML training and testing. Otherwise, a unique field data set lack of generalization is not conducive to a meaningful and reliable evaluation conclusion. And the field data should also be aligned for evaluation in different companies. If different companies use different field data to evaluate their AI/ML models, the comparison between companies’ results does not make much sense because the channel characteristics of the field data can be completely different.

Proposal 2: Consider at least one among Option 1-3 for calibration of AI/ML evaluation.

- Option 1: Common dataset and reference model for calibration.

- Option 2: Common dataset without reference model for calibration.

- Option 3: Reference model without common dataset for calibration.

- Option 4: No calibration.

Proposal 3: The use of field data set can be considered in Rel-18 AI/ML study, but needs to be carefully evaluated. The field data set should provide fundamental generalization characteristics and reflecting typical 5G channel condition (e.g. captured from a number of cells covering various types of deployment scenarios) are qualified for the AI/ML training and testing.

Complexity analysis for the AI/ML algorithms

In RAN1#109-e meeting, the complexity issues were addressed by some companies. We think the complexity and latency issue related to AI/ML inference should be addressed with higher priority.

Training complexity is also an important issue for research and deploy AI/ML-based algorithms. But it is not clear yet what role the 5G air interface would play in the AI/ML training. In RAN1#110-e, the training complexity was listed in the common KPIs (if applicable) for evaluating performance benefits of AI/ML. As introduced in Section 2.5, we assume the offline training in servers is the default format for AI/ML training. The further study is needed for the necessity of online training or training on UE and gNB in Rel-18 study. Hence the training complexity should not be a part of complexity evaluation at this moment. Component companies can provide information about the complexity for their model training. But the training complexity is not applicable for as a compulsory metric for Rel-18 AI/ML evaluation.

Power consumption is also an important issue for AI/ML inference/training. But again, the evaluation metric/methodology is still unclear at this moment. We suggest not to consider the power consumption as a metric for evaluation at this moment. Companies can provide information about AI/ML power consumption if they have.

FLOPs (floating-point operations) can be used in this study item to evaluate the complexity of AI/ML-based solutions. It a widely-used complexity metric in computer science and AI/ML research, which means how many operations are needed for the inference of an AI/ML model or an algorithm.

Here, it needs to be noted that if we change the small “s” into a big “S”, then we get a new index FLOPS. FLOPS (floating-point operations per second) is a metric to evaluate the computation capabilities of an AI platform, e.g. 1 FLOPS means a platform could finish 1 FLOPs within 1 second. TOPS (Tera Operations Per Second) is another metric to evaluate the computation power. Although there are some differences between the definition and the actual implementation of calculation, we can approximately regard the two metrics as closely equivalent.

For example, EVCsiNet introduced in [3] for AI/ML-based CSI feedback is with a computation complexity of about 10MFLOPs. If we consider a reference computation capability (e.g. 10 TFLOPS/10TOPS), the ideal CSI inference latency using EVCsiNet would be about 1μs. Considering non-ideal effects like computation resource scheduling, storage and I/O, the actual latency may be larger than the ideal value, e.g. 100 times larger. But the inference latency is still under 1ms which cand meet the latency requirement for 5G CSI feedback. For BM and positioning, the similar evaluations show acceptable computation complexities, as listed in Table.1.

For each use case, a threshold on maximum computation complexity in terms of FLOPs can be defined. If the complexity of a proposed model is lower than the threshold, the evaluation results of the model can be treated as the valid results.

Table 1: Inference latency for AI models with different computing complexity

| Use case | Computing complexity | Reference computation capability | Ideal inference latency |
| --- | --- | --- | --- |
| CSI (AI encoder&decoder for CSI feedback, Resnet based AI solution) [4] | 10 MFLOPs | 10 TFLOPS | 1μs |
| CSI (AI encoder&decoder for CSI feedback, Transformer based AI solution) [4] | 40 MFLOPs | 10 TFLOPS | 4μs |
| BM (spatial domain prediction) [4] | 0.22 MFLOPs | 10 TFLOPS | 0.22μs |
| Position (CIR as input) [4] | 3 MFLOPs | 10 TFLOPS | 0.3μs |
| Position (DL-RSTD as input) [4] | 0.15 MFLOPs | 10 TFLOPS | 0.15μs |

Note: Considering the actual implementation such as computation resource scheduling, storage and I/O, we can assume that the actual latency would be larger than the ideal value, e.g. 100 times larger.

Proposal 4: Focus on complexity evaluation for AI/ML inference. The AI/ML training complexity and AI/ML inference/training power consumption are not applicable as a compulsory metric for evaluation.

Proposal 5: The computation complexity for an AI/ML algorithm can be evaluated in terms of FLOPs.

Proposal 6: For sub each use case, the maximum acceptable computation complexity can be defined based on the tolerable of inference latency for this sub use case.

Collaboration levels

Collaboration levels:

In RAN1#109-e meeting, the following network-UE collaboration levels were agreed.

1.	Level x: No collaboration

2.	Level y: Signaling-based collaboration without model transfer

3.	Level z: Signaling-based collaboration with model transfer

In our understanding, the above collaboration levels are for AI/ML inference, not necessarily for AI/ML training. For an AI/ML model, training, deployment, inference and management are different procedures. Therefore, we propose to divide the whole picture into four aspects, i.e. training / deployment / inference / management. Although these aspects may influence each other, we believe that it is beneficial to discuss them separately. The collaboration levels need to be identified first. AI/ML training collaboration framework can be further studied.

In RAN1#110-e meeting, most of companies are fine with keeping the above agreement in the general framework AI. We also think this is a good general definition for AI/ML inference collaboration framework because it is applicable for all AI/ML use cases. Then the further refinement of the collaboration levels can be considered in each use case. For example, for AI/ML-based CSI compression, Level y can be further categorized into:

Level y: Signaling-based collaboration without model transfer

  - y(a): Signaling exchange before AI/ML inference (i.e. one-side AI/ML model inference)

  - y(b): Signaling exchange before and during AI/ML inference (i.e. two-side AI/ML model inference)

Level y/z boundary:

In RAN1#110-e meeting, the moderator asked the following question:

Should non-3gpp-based model delivery (e.g., via application layer from OAM or OTT sever) be viewed as model transfer (hence level z) or not (hence level y)?

(Clarification: The non-3pp-based model delivery still involves 3gpp signaling, such as the network configuring a model ID to the UE and the UE subsequently downloading the model corresponding to the model ID from an OTT server via the non-3gpp-based method.)

From the definition perspective, Level z can include 3GPP-based and non-3GPP-based model delivery. Then Level y is only related to model inference. But considering the non-3GPP-based model delivery will bring substantially different specification impacts than 3GPP-based model delivery, it is also OK not to regard non-3GPP-based model delivery as a “model transfer” and categorize it into Level y. Non-3GPP-based model delivery procedure itself will not be defined in 3GPP specification. Only some signaling will be defined to support the non-3GPP-based model delivery and model registration. In this sense, treating non-3GPP-based model delivery as a “signaling-based collaboration without model transfer” is also reasonable, and more suitable for the future work.

Non-3gpp-based model delivery is naturally the default model transfer mechanism as we can do nowadays. Hence the Rel-18 study can focus on Level y. 3GPP-based model delivery can be studied also under Level z, whose advantage over non-3gpp-based model delivery needs to be verified. But defining the model transfer format will be complicated. And the evaluation methodology for 3GPP-based model delivery is unclear at all. So we suggest to prioritize Level y including non-3gpp-based model delivery and in Rel-18 study.

Proposal 7: Study the collaboration levels for AI/ML inference and training separately. Confirm that the agreement in RAN1#109-e defines the collaboration levels for AI/ML inference in general framework.

- Non-3gpp-based model delivery (e.g., via application layer from OAM or OTT sever) is not viewed as model transfer, hence is categorized into level y.

- Further refinement of the collaboration levels can be considered in each use case.

- FFS collaboration levels for training.

Life Cycle Management

An agreement on AI/ML Life Cycle Management was reached in RAN1#110-e meeting. But several points are still open. Here we introduce our understandings

Model transfer and model registration:

For “Signaling-based collaboration with model transfer”, we understand the model transfer is to enable the model switching or updating. When the current model’s performance degrades (observed by performance monitoring), UE or gNB needs to switching to another model. If the required model is not available, the UE or gNB needs to obtain the model via 5G network. One possibility is to transfer the full model to replace the old model (i.e. model transfer for model switching). Another possibility is to transfer the partial model. For example, the model structure is reused. Only some parameters of the model are updated (model transfer for model updating).

Model transfer for AI/ML training is more complicated. A large amount of training data, updated model and training results need to be exchanged between UE and gNB frequently/iteratively. It can be further studied in long-term, but may not be suitable for Rel-18 study.

We understand the model transfer for AI/ML inference can be implemented with two types:

- Type 1: Non-3GPP-based model transfer + model registration for 3GPP-based model scheduling

- Type 2: 3GPP-based model transfer.

With Non-3GPP-based model transfer, the model can be transferred between vendors’ AI/ML servers. The model transfer procedure is not visible in 5G air interface standards. But after the model is transferred and deployed by the UE and/or gNB, there should be a signaling can be used to activate and schedule the model. Hence a “model registration” procedure needs to be defined in 5G standards, with which 5G air interface signaling can indicate a model transferred via non-3GPP channels.

With 3GPP-based model transfer, the model can be transferred between gNB and UE via a 5G channel. So the “model registration” procedure may not needed because it has been identified when it is transferred in the 5G channel. After the model is deployed by the UE and/or gNB, it can be directly be activated and scheduled by 5G signaling. But transfer the file like AI/ML model in 5G control plain is a new procedure. Some requirement study is needed, e.g., the required KPI (e.g. packet size, data rate, latency, reliability), so to select the design (e.g. transfer the model in which layer/channel). Some file format may be defined to support 3GPP-based model transfer.

But it should be noticed that even the 3GPP-based model transfer is supported, it does not mean the model deployment is naturally 3GPP-based. The model format during transfer procedure (as a 5G control plain package) cannot be directly used for model deployment.

Model deployment:

We understand the model deployment is the different procedure from model transfer. The deployment phase refers to that after a node receives a model, some engineering operations are required to make the model available to use at that node, e.g. specific optimization, compiling and testing.

Nowadays the non-AI 5G algorithms are deployed in non-3GPP manner, i.e., the algorithm is developed, tested by each vendor proprietarily. The AI 5G algorithms will evolve/update more frequently/actively than non-AI 5G algorithms. And some AI/ML algorithm (e.g. two-side model) needs close alignment/cooperation between UE and gNB. Hence the 3GPP-based model deployment was proposed by some companies in RAN1#109-e, i.e., some model deployment format may be defined in 3GPP standards to enable a more open and interoperable model deployment. The target is to enable Vendor B to compile, test and deploy Vendor A’s AI/ML model in a “plug-and-play” manner. We are open to study the possibility. But so far, we have not seen much details to justify its feasibility.

Model configuration:

We support the study on model configuration. Some parameters need to be configured before a model is used for inference for some use case. If the configuration is semi-static, it needs to be performed based on 3GPP signaling. However, there may be different understandings about the contents of the model configuration, which can be clarified during the study.

Proposal 8: For Rel-18 study on AI/ML Life Cycle Management,

- Model registration for non-3GPP-based delivered model should be studied.

  - This is the default type.

- Study on 3GPP-based model transfer with lower priority. First focus on following aspects:

  - Required KPI (e.g. packet size, data rate, latency, reliability), so to select the design (e.g. in which layer/channel).

  - Model transfer format (if needed).

- Study AI/ML model delivery for training with lower priority.

Proposal 9: Study the model deployment procedure separately from model delivery.

Proposal 10: Model configuration procedure should be studied.

Training operation

In RAN1#110-e, a working assumption was agreed for definitions of offline training and online training.

As mentioned above, we think the training operation should not be treated in high priority in the early stage of the Rel-18 study. The training is very important step in enabling AI/ML-based algorithm. But the primary target of the study item is to analyze and evaluation the performance of the AI/ML-based algorithm for each use case. Hence the study on inference operation should be treated with high priority.

Now companies perform offline training on their servers and submit the evaluation results. It shows the non-3GPP-based offline training is workable for AI/ML for 5G air interface. Training is a more complicated procedure than inference. It will consume huge study capacity if we treat it as another focus.

Another problem is that we have not seen a clear evaluation methodology for checking the performance of different training approaches. For example, how can we compare the performance of 3GPP-based training and non-3GPP training, offline training and online training? If the study on training is prioritized, we need first clarify the corresponding evaluation methods.

During the Rel-18 study on AI/ML training, the offline training should be treated with high priority and as the default training type. Real-time online training and the training performed on UE and gNB are much more challenging than that on cloud servers, e.g. requiring high-level computation, storage, communications resource, and requiring UE staying in the environment collecting the training data which may unreasonably restrict the end user’s behavior.

And in RAN1#110-e, it was agreed that the 3gpp discussion should be proceeded without waiting for online/offline training terminologies. We think the guidance is reasonable. We can first agree on the most necessary notation and terminology list. The target of this work should be for categorizing/prioritizing AI/ML approaches. And the notation and terminology definitions for AI/ML inference should be prioritized over that for AI/ML training.

Proposal 11: In the early stage of Rel-18 study, prioritize study of the AI/ML inference over the study of AI/ML training.

- Study offline training with high priority and as the default training type.

Performance monitoring

Performance monitoring needs to be studied to support AI/ML model changing and switching between AI/ML-based and non-AI/ML based operations. But some details need to be clarified on this aspect:

- What is the performance monitoring for?

- What metrics are used for the performance monitoring?

- Which node controls the monitoring?

- How to predict the performance of an unused model?

- How to evaluate the performance of the performance monitoring?

The performance monitoring can be used to detect the performance degradation. When the performance degrades, three actions can be taken: First, gNB/UE can try to switch to a better model if the model has been pre-loaded on board. Second, if the wanted model is not available yet, the gNB/UE can download the model. Finally, if the model is not available in the model library, a model re-tuning/re-training can be performed. From our preliminary perspective, the performance monitoring procedure can be designed despite while action will be taken. The task of performance monitoring is to detect the performance degradation. Model switching, model transfer and re-tuning/re-training can be based on the same monitoring result.

For the above three actions, we think the communication performance-based metrics (e.g. MSE, BLER, throughput) are enough for the performance monitoring. Some AI/ML-specific performance metrics may be needed for training performance monitoring, e.g., training convergence speed. But we think the performance monitoring for training should be studied with low priority.

AI/ML performance degradation can be detected by gNB or UE in different cases. Especially for the two-side model inference, gNB and UE may need to cooperate to detect the reason of inference performance degradation. Hence it is suggested to study both gNB-initiated and UE-initiated performance monitoring.

If it is determined to switch to a better model, the question needs to be answered is that what the performance the new model will provide. Can the new model perform better that the current model? The performance prediction mechanism should be studied.

Finally, the performance of performance monitoring mechanism may be evaluated too, e.g., the monitoring/predition accuracy.

Proposal 12: Target to design a unified AI/ML inference monitoring mechanism supporting AI/ML model switching, model transfer and model re-training.

- Model re-training is considered with low priority.

- Consider communication performance-based metrics (e.g. MSE, BLER, throughput) as starting point.

- Study gNB-initiated and UE-initiated performance monitoring

- Study performance prediction mechanism for an unused model.

- Study evaluation methodology for performance monitoring approaches.

Proposal 13: Study on AI/ML training performance monitoring is low priority.

Conclusion

Based on the above discussions, our proposals for general aspect of AI/ML framework are listed below:

Proposal 1: Focus on the identified representative sub use cases in AI 9.2.x.1 for the corresponding studies on their evaluation methodology, KPI, and performance evaluation results. Investigate the following aspects for other potential sub use case in AI 9.2.x.2.

- Potential performance gain (e.g., shown in preliminary evaluation results).

- Feasible evaluation methodology and valid training data set (incl. training set generation methodology).

- Reasonable non-AI/ML-based baseline for performance gain analysis.

- Potential specification impacts.

Note: Sub use cases without 3) and/or 4) will not be precluded. But target to limit the number of the representative sub use cases to 1 or 2 for each use case.

Proposal 2: Consider at least one among Option 1-3 for calibration of AI/ML evaluation.

- Option 1: Common dataset and reference model for calibration.

- Option 2: Common dataset without reference model for calibration.

- Option 3: Reference model without common dataset for calibration.

- Option 4: No calibration.

Proposal 3: The use of field data set can be considered in Rel-18 AI/ML study, but needs to be carefully evaluated. The field data set should provide fundamental generalization characteristics and reflecting typical 5G channel condition (e.g. captured from a number of cells covering various types of deployment scenarios) are qualified for the AI/ML training and testing.

Proposal 4: Focus on complexity evaluation for AI/ML inference. The AI/ML training complexity and AI/ML inference/training power consumption are not applicable as a compulsory metric for evaluation.

Proposal 5: The computation complexity for an AI/ML algorithm can be evaluated in terms of FLOPs.

Proposal 6: For sub each use case, the maximum acceptable computation complexity can be defined based on the tolerable of inference latency for this sub use case.

Proposal 7: Study the collaboration levels for AI/ML inference and training separately. Confirm that the agreement in RAN1#109-e defines the collaboration levels for AI/ML inference in general framework.

- Non-3gpp-based model delivery (e.g., via application layer from OAM or OTT sever) is not viewed as model transfer, hence is categorized into level y.

- Further refinement of the collaboration levels can be considered in each use case.

- FFS collaboration levels for training.

Proposal 8: For Rel-18 study on AI/ML Life Cycle Management,

- Model registration for non-3GPP-based delivered model should be studied.

  - This is the default type.

- Study on 3GPP-based model transfer with lower priority. First focus on following aspects:

  - Required KPI (e.g. packet size, data rate, latency, reliability), so to select the design (e.g. in which layer/channel).

  - Model transfer format (if needed).

- Study AI/ML model delivery for training with lower priority.

Proposal 9: Study the model deployment procedure separately from model delivery.

Proposal 10: Model configuration procedure should be studied.

Proposal 11: In the early stage of Rel-18 study, prioritize study of the AI/ML inference over the study of AI/ML training.

- Study offline training with high priority and as the default training type.

Proposal 12: Target to design a unified AI/ML inference monitoring mechanism supporting AI/ML model switching, model transfer and model re-training.

- Model re-training is considered with low priority.

- Consider communication performance-based metrics (e.g. MSE, BLER, throughput) as starting point.

- Study gNB-initiated and UE-initiated performance monitoring

- Study performance prediction mechanism for an unused model.

- Study evaluation methodology for performance monitoring approaches.

Proposal 13: Study on AI/ML training performance monitoring is low priority.

References

- R1-2208178, Summary#4 of General Aspects of AI/ML Framework, Moderator (Qualcomm), 3GPP TSG RAN WG1 #110, Toulouse, France, 22nd – 26th August 2022