3GPP TSG RAN WG1 #110bis-e		R1-2208854

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Evaluation methodology and preliminary results on AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.1

Document for:	Discussion and Decision

- Introduction

As a promising technology, AI/ML based solutions have been widely used and has shown impressive performance in various areas, e.g., computer vision, speech recognition, natural language processing (NLP), language translation and so on.

Motivated by the great success of AI/ML in the afore-mentioned areas, 3GPP also started study on AL/ML for wireless communications in SA2 and RAN3. On the basis of the existing study in 3GPP, RAN#94e meeting decided to start a new study item (SI) on AI/ML for NR air interface in Rel-18 [1]. According to the new SID, this study will focus on three typical use cases of physical layer in the first step:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In RAN1#109e and RAN1#110 meetings, lots of agreements for the typical scenarios and evaluation assumptions were achieved for the AI/ML based positioning, some of which are as below [3][4]:

Agreement The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning. Agreement For InF-DH channel, the prioritized clutter parameters {density, height, size} are: - {60%, 6m, 2m}; - {40%, 2m, 2m}. - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. Agreement For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857. Agreement For AI/ML-based positioning evaluation, the baseline performance to compare against is that of existing Rel-16/Rel-17 positioning methods. - As a starting point, each participating company report the specific existing positioning method (e.g., DL-TDOA, Multi-RTT) used as comparison. Agreement Synthetic dataset generated according to the statistical channel models in TR38.901 is used for model training, validation, and testing. Agreement The entry “UE horizontal drop procedure” in the simulation parameter table for InF is updated to the following. UE horizontal drop procedure; Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull.. Agreement The entries “UE antenna height” and “gNB antenna height” in the simulation parameter table for InF is updated to the following. UE antenna height; Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH). …; …. gNB antenna height; Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}.. Agreement If spatial consistency is enabled for the evaluation, companies model at least one of: large scale parameters, small scale parameters and absolute time of arrival, where - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = for InF (Section 7.6.3.1 of TR 38.901) - the small scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 Agreement If spatial consistency is enabled for the evaluation of AI/ML based positioning, the baseline evaluation does not incorporate spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). - It is optional to implement spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). Agreement For evaluation of AI/ML based positioning, the computational complexity can be reported via the metric of floating point operations (FLOPs). - Note: For AI/ML assisted methods, computational complexity for the AI/ML model is only one component of the overall complexity for estimating the UE’s location. - Note: Other metrics to measure the computational complexity are not precluded. Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area. Agreement For AI/ML-based positioning, both approaches below are studied and evaluated by RAN1: - Direct AI/ML positioning - AI/ML assisted positioning Agreement For AI/ML-based positioning, study impact from implementation imperfections. Agreement For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”. Agreement To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time. Agreement When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including: - Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA) - Percentage of training data without label, if incomplete labeling is considered in the evaluation - Imperfection of the ground truth labels, if any Agreement For evaluation of AI/ML based positioning, study the performance impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies. Agreement For AI/ML-based positioning, for evaluation of the potential performance benefits of model finetuning, report at least the following: - training dataset setting (e.g., training dataset size necessary for performing model finetuning) - horizontal positioning accuracy (in meters) before and after model finetuning. Agreement To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: - UE/gNB RX and TX timing error. - The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG).

In this contribution, we will continue to discuss the evaluation of AI-based positioning accuracy enhancement, including the setting for generalization and assumption and so on. Moreover, we will present our evaluation results for some AI/ML-based schemes.

Discussion

AI-based schemes

During the evolution of R16 NR, 3GPP developed multiple RAT-based positioning methods to provide high accuracy positioning for NR cellular system:

- DL-TDOA positioning

- UL-TDOA positioning

- DL-AoD positioning

- UL-AoA positioning

- Multi-RTT positioning

Theoretically, AI/ML can be introduced to work along with each of the above positioning schemes. In our initial evaluation, we focus on the combination / integration of DL-TDOA positioning and AI/ML. The same approach and evaluation methodologies can be directly extended to other NR positioning methods.

For the existing NR mechanism of DL-TDOA positioning method, UE will measure DL RSTD or estimate the location based on the reception of DL PRS.  Then, UE will report the DL RSTD measurements, optionally along with DL PRS RSRP, or the estimated location to LMF. For UE-assisted DL-TDOA positioning method, LMF will further calculate the location of the target UE based on the reported measurement results, by using traditional algorithm (e.g., CHAN algorithm). We refer to the traditional NR DL-TDOA as “DL-TDOA”.

Based on categories agreed in the previous RAN1 meeting, there will be two different categories of AI-based positioning schemes as below:

- Direct AI/ML positioning: The AI model will estimate the location of a given UE based on the measurement results of UE/TRP

- AI/ML assisted positioning: The AI model will estimate some intermediate results based on the measurement results of UE/TRP and the LMF can use these intermediate results to calculate the location of the given UE by using non-AI method or a separate AI method.

Fig.1: Illustration of different categories of AI-based schemes.

In order to evaluate the potential advantages of AI/ML based positioning over traditional positioning algorithm(s), we firstly consider some specific schemes for direct AI/ML positioning, which are based on the different assumptions on the type and acquisition of measurement results and will have different impacts on the system design and specifications.

The first AI-based scheme is based on LMF’s implementation:

- UE measures and reports the DL RSTD results via the existing mechanism.

- Then, LMF uses a Neural Network (NN) to estimate the location.

For this scheme, the whole air interface is the same as R16/17 positioning mechanism and it is totally up to LMF implementation.  In other words, the AI/ML operations are transparent to UE. Accordingly, the advantages of this scheme are no impact on specification and easier for the deployment of AI/ML. We refer to this scheme as “Direct: + DL-TDOA” in subsequent discussions.

Regarding TDOA, only partial information can be attracted from the reception of DL PRS. That is to say, some information, more or less, cannot be exploited at LMF side. Thus, another way is to measure and report the original channel information to LMF. In our evaluation, UE estimates the channel impulse response (CIR) and reports its normalized versions without quantization to LMF. LMF uses a NN to estimate the location. We refer to this method as “Direct: Normalized CIR”. For this scheme, a new type of measurements and reporting format will be needed.

Since the reported CIR is normalized, information regarding the received power of DL PRS is missing. UE can also report this information via DL PRS RSRP. We refer to it as “Direct: Normalized CIR + RSRP”.  Similarly, RSRP measurements can also be used for other AI-based schemes.

In summary, the following schemes are used in our initial evaluation:

- DL-TDOA: CHAN algorithm is used to estimate the location based on measurement results of DL RSTD

- Direct: DL-TDOA: A trained NN is used to estimate the location based on measurement results of DL RSTD

- Direct: DL-TDOA + RSRP: A trained NN is used to estimate the location based on measurement results of DL RSTD and associated RSRP

- Direct: Normalized CIR: A trained NN is used to estimate the location based on measurement results of normalized CIR

- Direct: Normalized CIR + RSRP: A trained NN is used to estimate the location based on measurement results of normalized CIR and associated RSRP

For AI/ML assisted positioning, two different schemes are investigated in our evaluations. The first one of AI/ML assisted positioning is as below (Indirect: Normalized CIR of all TRPs)

- Normalized CIR information corresponding to all TRPs are used for the input of AI/ML mode

- The outputs of AI/ML model are the estimated TOAs corresponding to all TRPs.

- For the AI model training, the TOAs calculated based on the distance between UE and TRPs are used as the labels.

The second scheme for AI/ML assisted positioning is as below (Indirect: Normalized CIR of single TRP)

- Normalized CIR information corresponding to single TRP are used for the input of AI/ML mode

- The AI model is performed for each TRP and get the estimated TOA corresponding to each TRP.

- For the AI model training, the TOAs calculated based on the distance between UE and TRPs are used as the labels.

Then, a traditional algorithm (i.e., CHAN algorithm) is used to estimate UE the location.

Since additional information of RSRP have marginal impact on the performance, we only evaluate normalized CIR information as the input for the AL/ML assisted positioning.

Scenarios setting

In our evaluation, different levels of spatial consistency were enabled to investigate their impacts on the performance of different AI schemes. To be specifically, two different settings of spatial consistency as below were used in our evaluations

- Setting 1 “w/ spatial consistency”: In this case, the following three models are enabled

  - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance =  for InF (Section 7.6.3.1 of TR 38.901)

  - the small scale parameters are according to Section 7.6.3.1 of TR 38.901

  - the absolute time of arrival is according to Section 7.6.9 of TR 38.901

- Setting 2 “w/o spatial consistency”: In this case, only the following two models are enabled

  - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance =  for InF (Section 7.6.3.1 of TR 38.901)

  - the absolute time of arrival is according to Section 7.6.9 of TR 38.901

The difference between these two settings is whether the spatial consistency modeling of small scale parameters is enabled or not.

Generation of data sets

In our evaluation, the data sets are generated by system-level simulator. The measurement samples are based on the reception of DL PRS within one slot. For each reception of DL PRS, UE will do the measurement corresponding to each scheme, e.g., DL-RSTD, CIR and RSRP. The label associated with each sample is the known location information of the target UE, or the TOA calculated based on the distance between UE and TRPs.

For the evaluation of each AI/ML-based scheme, 80,000 or 800,000 samples are generated with associated labels. There are three different alternatives in our evaluation to general samples for each scheme:

- Alt.1: We simulate 80,000 drops, and only 1 UE is randomly dropped per drop. Compared to the second alternative, the data of UEs have less correlation among each other.

  - 2% of the samples are randomly selected for testing and the remaining 98% samples are used for training.

- Alt.2: We simulate 1 drop and 80,000 UEs are dropped in this drop. As the number of UEs in a drop increases, the UEs are closed to each other. Thus, the data for close UEs are with higher correlations.

  - 2% of the samples are randomly selected for testing and the remaining 98% samples are used for training.

- Alt.3: We simulate 10 drops and 80,000 UEs are dropped in this drop. Alt.3 is a middle ground between Alt.1 and Alt.2. For Alt.3, the total number of samples are 800,000.

  - For each drop, 2% of the samples are randomly selected for testing and the remaining 98% samples are used for training.

Alt.2 can be regarded as a snapshot of a fixed scenario, while Alt.1 is reflecting a set of many similar scenarios with the same scenario and the same TRP deployment. Thus, in some sense, Alt.2 can be used to show some upper bound of AI-based solutions, but the corresponding trained AI modes may suffer from the generalization issue. Some other alternative(s) to generate the data sets are in the middle ground between Alt.1 and Alt.2, e.g., Alt.3.

NN models

For each sample of CIR, we use 256 time-domain complex values in our evaluation.  We use the three-dimensional matrix with the size of [M, 256, 2], which are generated from the CIRs corresponding to M TRPs, as the input of neural network.  Since the input size are relatively large, we can consider the input as an “image”. With this intuitive, we choose ResNet for our evaluation as ResNet is widely used for image classification tasks.

Fig.2 illustrates the basic structure of our AI model for direct AI/ML positioning. To be specific, based on ResNet backbone, the input is directly added to the output of a residual block (consisting of 9 Conv2D layers with 64 filters and 1 Conv2D layer with 2 filters, 3×3 kernels are used for these filters), then followed by 1 dense layer with 2 nodes to provide the NN output. The NN outputs are the estimated values of the location (x, y).

Fig.2: Illustration of the ResNet model for direct AI/ML positioning

For the AI/ML assisted positioning, a similar ResNet model is used for the scheme “Indirect: Normalized CIR of all TRP” except the following two aspects:

- The module “Dense with 2 nodes” is replaced by a dense layer with 18 nodes, where the output of AI model are the estimated TOAs corresponding to the 18 TRPs.

- The input is only normalized CIR information whereas CIR and RSRP information are used for the input of direct AI/ML positioning.

The AI model for the scheme “Indirect: Normalized CIR of single TRP” is also similar to the ResNet model in Fig.2:

- The module “Dense with 2 nodes” is replaced by a dense layer with 1 node, where the output of AI model are the estimated TOAs corresponding to a single TRP.

- The input is only normalized CIR information whereas CIR and RSRP information are used for the input of direct AI/ML positioning.

Evaluation results

Based on the above-mentioned data sets and AI models, we present some evaluation results in this section to show the positioning accuracy of each AI-based or non-AI-based scheme and the generalization performance of AI models as well. We use InF-DH scenario in the simulations, where clutter setting {clutter density, height, size} is set to be {0.6,6m,2m} or {0.4, 2m, 2m}. In order to reduce the workload for the evaluation, we assume no timing error except for the evaluation of generalization performance.

Impact of spatial consistency and data set construction

To investigate the impact of spatial consistency and different data set constructions on AI model performance, we evaluate the performance of direct AI/ML positioning for InF-DH with the clutter setting {0.6, 6m, 2m} and with the following different configurations:

- Whether spatial consistency of small-scale parameters is enabled or not

- Different alternatives for data set construction

The evaluation results for the scheme “Direct: Normalized CIR + RSRP” are shown in Fig.3, Fig.4 and Table 1. Fig. 3 illustrates the evaluation results of the positioning accuracy for the data set generated by {1 drop, 80,000 UEs per drop}, whereas Fig. 4 illustrates the performance for the data set generated by {10 drops, 80,000 UEs per drop}.  Table 1 also shows the positioning accuracy achieved for 90% UE with different configurations.

For the AI/ML assisted positioning, the evaluation results of the scheme “Indirect: Normalized CIR of all TRP” are shown in Fig.5 and Table.2. Fig. 5 and Table 2 illustrate the evaluation results of the positioning accuracy for InF-DH with clutter setting {60%, 6m, 2m}.

Based on the evaluation results shown in Fig. 3-5 and Table 1-2, we can get the following observation:

Observation 1: For InF-DH scenario with clutter setting {0.6, 6m, 2m}, if the data set is generated by modeling the spatial consistency of small-scale parameters, some performance gain can be observed compared to the configuration that the spatial consistency of small-scale parameters is not modeled.

- The performance gain is quite limited

In the following sections, the spatial consistency of small-scale parameters is always enabled in our evaluations unless explicit stated “without spatial consistency”.

Fig. 3: CDF of positioning error for “Direct: Normalized CIR + RSRP” and

data set based on {1 drop, 80,000 UEs per drop}

Fig. 4: CDF of positioning error for “Direct: Normalized CIR + RSRP” and

data set based on {10 drop, 80,000 UEs per drop}

Table 1: Positioning accuracy achieved for direct AI/ML positioning (“Direct: Normalized CIR + RSRP”)

AI model deployed at UE or LMF, without model generalization, ResNet model

| Accuracy achieved @90% (m) | w/ spatial consistency | w/o spatial consistency |
| --- | --- | --- |
| 1 drop, 80,000 UEs per drop | 0.33 | 0.40 |
| 10 drop, 80,000 UEs per drop | 0.52 | 0.86 |
| 80,000 drop, 1 UE per drop | 4.35 | 4.35 |

Fig. 5: CDF of positioning error for AI/ML assisted positioning (“Indirect: Normalized CIR of all TRPs”)

Table 2: Positioning accuracy for AI/ML assisted positioning (“Indirect: Normalized CIR of all TRPs”)

AI model deployed at UE, without model generalization, ResNet model

|  | Accuracy achieved @90% (m) |
| --- | --- |
| 1 drop, 80000 UEs per drop, w/ spatial consistency | 0.52 |
| 1 drop, 80000 UEs per drop, w/o spatial consistency | 0.72 |
| 10 drops, 80000 UEs per drop, w/ spatial consistency | 1.03 |
| 80000 drops, 1 UEs per drop, w/ spatial consistency | 5.78 |

By comparing the simulation results based on different data sets, we can also notice that performance gap is quite large for different alternatives of data set construction.

Observation 2: For InF-DH scenario with clutter setting {0.6, 6m, 2m}, no matter the data set is generated by modeling the spatial consistency of small scale parameters or not, the performance gaps are quite large for different alternatives of data set construction.

- The number of drops has larger impact than the enabling/disabling of spatial consistency for small scale parameters on the AI/ML performance

- When the number of drops increased, the achieved accuracy become worse

Therefore, in order to keep the consistency among companies and ensure the results from different sources are comparable, we suggest the following proposal:

Proposal 1: For the evaluation of AI/ML-based positioning accuracy improvement, companies are encouraged to report the following parameters regarding the data set construction to keep the results from different sources be comparable:

- X drops

- Y UEs per drop

- FFS: values of X, Y

Comparison of two schemes for AI/ML assisted positioning

As we discussed, there are two different schemes for AI/ML assisted positioning:

- Indirect: Normalized CIR of all TRPs

- Indirect: Normalized CIR of single TRP

In this section, we will do evaluations to illustrate and compare the performance of these two schemes. In these evaluations, the spatial consistency of small-scale parameters is enabled and the clutter setting is {60%, 6, 2}.  Fig.6, Fig.7 and Fig.8 show the CDF of positioning accuracy for different AI/ML assisted positioning schemes for different data set constructions.  Table 3 summarizes the results of these two schemes and the traditional non-AI method (i.e., “DL-TDOA”) as well.

Fig. 6: Performance comparison of different schemes for AI/ML assisted positioning

Fig. 7: Performance comparison of different schemes for AI/ML assisted positioning

Fig. 8: Performance comparison of different schemes for AI/ML assisted positioning

Table 3: Performance comparison of different schemes for AI/ML assisted positioning

| Accuracy achieved @90% (m) | Scheme 1: Indirect: Normalized CIR of all TRPs | Scheme 2: Indirect: Normalized CIR of single TRP | Non-AI method on the drop(s) |
| --- | --- | --- | --- |
| 1 drop, 80000 UEs per drop | 0.52 | 2.42 | 8.2 |
| 10 drops, 80000 UEs per drop | 1.03 | 7.17 | 10.16 |
| 80000 drops, 1 UEs per drop | 5.78 | 14.47 | 10.55 |

From Fig.6-8 and Table 3, we can see that the scheme “Indirect: Normalized CIR of single TRP” has much worse performance than the scheme “Indirect: Normalized CIR of all TRPs”. Meanwhile, the traditional non-AI scheme achieves comparable performance with the scheme “Indirect: Normalized CIR of single TRP”.

Observation 3: For AI/ML assisted positioning

- The performance of “Indirect: Normalized CIR of single TRP” is much worse than “Indirect: Normalized CIR of all TRPs”

- The performance of “Indirect: Normalized CIR of single TRP” is comparable to the traditional non-AI scheme

As the scheme “Indirect: Normalized CIR of single TRP” for AI/ML assisted positioning cannot get satisfying performance, we will only consider “Indirect: Normalized CIR of all TRPs” in later sections.

Proposal 2: For AI/ML assisted positioning, if the output of AI model is timing-based result (e.g., TOA), prioritize the scheme where the measurement results corresponding to all TRPs are used as the input of AI model

- the scheme where the measurement results corresponding to single TRP are used as the input of AI model is deprioritized.

Generalization: training and testing data sets are generated from different drops

In this section, we will investigate the generalization performance from the perspective of different data setting are used for training and testing, i.e., the training and testing data set are generated from different drops. To be specifically, two different settings are used in our evaluations

- Data setting D-1: 2 drops, 80000 UEs per drop

  - All samples from 1 drop are used for training

  - All samples from another drop are used for testing

- Data setting D-2: 10 drops, 80000 UEs per drop

  - All samples from 5 drops are used for training

  - All samples from another 5 drops are used for testing

In these evaluations, the spatial consistency of small-scale parameters is enabled and the clutter setting is {60%, 6, 2}.  Fig. 9 and Fig.10 show the performance for data setting D-1 and D-2, respectively. The performance of the traditional non-AI method (i.e., “DL-TDOA”) for the same drop(s) is also provided for comparison. Table 4 summarizes their performance and the performance with the same data setting as well.

Fig. 9: Generalization performance: 1 drop for training and another drop for testing

Fig. 10: Generalization performance: 5 drops for training and another 5 drops for testing

Table 4: Generalization performance: training and testing data sets are generated from different drops

|  | Accuracy achieved @90% (m) | DL-TDOA | Direct: DL-TDOA+RSRP | Direct: Normalized CIR + RSRP | Indirect: Normalized CIR for all TRPs |
| --- | --- | --- | --- | --- | --- |
| w/o generalization | 1 drop for both training and testing | 8.20 | 0.48 | 0.33 | 0.52 |
| w/ generalization | 1 drop for training Another drop for testing | 9.92 | 10.53 | 10.11 | 11.29 |
| w/o generalization | 10 drops for both training and testing | 10.16 | 0.46 | 0.52 | 1.03 |
| w/ generalization | 5 drops for training Another 5 drops for testing | 10.2 | 9.3 | 6.55 | 7.4 |

Based on the results showed in Fig.8-10 and Table 4, we can get the following observation:

Observation 4: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated from different drops, there will be large performance degradation for AI-based positioning.

Generalization: training and testing data sets are generated with different clutter settings

In this section, we will investigate the generalization performance by using different clutter settings for the data set generalizations of training and testing. That’s is to say, the clutter setting for training and testing data sets are mismatched.  To be specifically, four different settings are used in our evaluations

- Data setting C-1: 2 drops, 80000 UEs per drop, {60%,6,2} for training, {40%,2,2} for testing

  - All samples from 1 drop with the clutter setting {60%,6,2} are used for training

  - All samples from another drop with the clutter setting {40%,2,2}  are used for testing

- Data setting C-2: 2 drops, 80000 UEs per drop, {40%,2,2} for training, {60%,6,2}for testing

  - All samples from 1 drop with the clutter setting {40%,2,2}  are used for training

  - All samples from another drop with the clutter setting {60%,6,2}  are used for testing

- Data setting C-3: 10 drops, 80000 UEs per drop, {60%,6,2} for training, {40%,2,2} for testing

  - All samples from 5 drops with the clutter setting {60%,6,2} are used for training

  - All samples from another 5 drops with the clutter setting {40%,2,2} are used for testing

- Data setting C-4: 80000 drops, 1 UE per drop, {60%,6,2} for training, {40%,2,2} for testing

  - All samples from 78400 drops with the clutter setting {60%,6,2} are used for training

  - All samples from another 1600 drops with the clutter setting {40%,2,2} are used for testing

Fig.11, Fig.12, Fig.13 and Fig.14 show the performance for data setting C-1, C-2, C-3 and C-4, respectively. The performance of the traditional non-AI method (i.e., “DL-TDOA”) for the same drop(s) is also provided for reference. Table 5 summarizes the results corresponding to the positioning accuracy achieved at 90%.

Fig. 11: Generalization performance with different clutter settings (Data setting C-1)

Fig. 12: Generalization performance with different clutter settings (Data setting C-2)

Fig. 13: Generalization performance with different clutter settings (Data setting C-3)

Fig. 14: Generalization performance with different clutter settings (Data setting C-4)

Table 5: Generalization performance: training and testing data sets are with different clutter settings

| Accuracy achieved @90% (m) | DL-TDOA | Direct: DL-TDOA+RSRP | Direct: Normalized CIR + RSRP | Indirect: Normalized CIR for all TRPs |
| --- | --- | --- | --- | --- |
| Data setting C-1 | 3.86 | 8.05 | 15.75 | 16.09 |
| Data setting C-2 | 10.45 | 10.8 | 8.61 | 8.88 |
| Data setting C-3 | 3.74 | 7.78 | 8.67 | 8.49 |
| Data setting C-4 | 3.84 | 4.86 | 6.82 | 6.66 |

Based on the results showed in Fig.11-14 and Table 5, we can get the following observation:

Observation 5: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated with different clutter settings, there will be large performance degradation for AI-based positioning.

Generalization: mixed data set for training

In this section, we continue to investigate the generalization performance for AI/ML model trained by mixed data set. To be specifically, the following data settings are used in our evaluations

- Data setting M-1: 1 drop with {60%,6,2} + 1 drop with {40%,2,2}  for training, {40%,2,2} for testing

- Data setting M-2: 1 drop with {60%,6,2} + 1 drop with {40%,2,2}  for training, {60%,6,2} for testing

Fig.15 and Fig.16 show the performance for data setting M-1 and M-2, respectively. Table 6 summarizes their performance and the performance with the same data setting as well.

Fig. 15: Generalization performance with mixed data set for training (Data setting M-1)

Fig. 16: Generalization performance with mixed data set for training (Data setting M-2)

Table 6: Generalization performance: AI model training based on mixed data sets

|  | Accuracy achieved @90% (m) | Direct: DL-TDOA+RSRP | Direct: Normalized CIR + RSRP | Indirect: Normalized CIR for all TRPs |
| --- | --- | --- | --- | --- |
| w/ generalization | Data setting M-1 Mixed for training {40%,2,2} for testing | 0.44 | 0.38 | 0.55 |
| w/ generalization | Data setting M-2 Mixed for training {60%,6,2} for testing | 0.78 | 0.46 | 0.66 |
| w/o generalization | {60%,6,2} for training {60%,6,2} for testing | 0.48 | 0.33 | 0.52 |

Based on the results showed in Fig.15-16 and Table 6, we can have the following observation:

Observation 6: For the InF-DH scenario, if the training of AI model is based on the mixed data set with different clutter settings, the AI model inference for the data set with one of these clutter settings can achieve good performance.

Generalization: training data set w/o NW synchronization error, testing data set w/ NW synchronization error

In this section, we continue to investigate the generalization performance by considering the impact of NW synchronization error. To be specifically, the following settings are used in our evaluations

- Data setting S-1: 1 drop w/o NW synchronization error for training, another 1 drop w/ NW synchronization error for testing

- Data setting S-2: 10 drops w/o NW synchronization error for training, another 10 drops w/ NW synchronization error for testing

In these evaluations, the spatial consistency of small-scale parameters is enabled and the clutter setting is {60%, 6, 2}. The model of a truncated Gaussian distribution as captured in TR 38.857 [2] is adopt to generate NW synchronization error where the value of RMS T1 is set to 50ns.

- The synchronization error generation for each TRP is independent of other TRPs

- The signals from the same TRP to different UEs will use the same synchronization error

In order to avoid the impact of different UE locations on the generalization performance, the UE locations are also kept the same for the corresponding drops with/without NW synchronization errors in the evaluations. For example, if drop 1-10 and drop 11-20 are without and with NW synchronization errors, respectively, then UE locations are the same for drop k and drop k+10 (k=1,…,10)

Fig.17 and Fig.18 show the performance for data setting S-1 and S-2, respectively. Table 7 summarizes their performance and the performance with the same data setting as well.

Fig. 17: Generalization performance with NW synchronization error (Data setting S-1)

Fig. 18: Generalization performance with NW synchronization error (Data setting S-2)

Table 7: Generalization performance: Training w/o NW syn error, Test w/ NW syn error

|  | Accuracy achieved @90% (m) | DL-TDOA | Direct: DL-TDOA+RSRP | Direct: Normalized CIR + RSRP | Indirect: Normalized CIR for all TRPs |
| --- | --- | --- | --- | --- | --- |
| w/ generalization | Data setting S-1 1 drop w/o NW sync error for training, 1 drop w/ NW sync error for testing | 29.05 | 5.29 | 6.54 | 27.85 |
| w/o generalization | 1 drop w/o NW sync error for training and testing | 8.2 | 0.48 | 0.33 | 0.52 |
| w/ generalization | Data setting S-2 10 drops w/o NW sync error for training, 10 drops w/ NW sync error for testing | 36.91 | 7.11 | 11.77 | 33.62 |
| w/o generalization | 10 drops w/o NW sync error for training and testing | 10.16 | 0.46 | 0.52 | 1.03 |

Based on the results showed in Fig.17-18 and Table 7, we can have the following observation:

Observation 7: For the InF-DH scenario, if the training data set of AI model is generated without NW synchronization error, the AI model performance for the case with NW synchronization error will suffer larger performance loss

- In this case, the AI/ML assisted positioning scheme “Indirect: Normalized CIR for all TRPs” achieves similar performance as the traditional non-AI based scheme.

Generalization: training data set w/o UE timing error, testing data set w/ UE timing error

In this section, we continue to investigate the generalization performance by considering timing error at UE side. To be specifically, the following settings are used in our evaluations

- Data setting T-1: 1 drop w/o UE timing error for training, another 1 drop w/ UE timing error for testing

- Data setting T-2: 10 drops w/o UE timing error for training, another 10 drops w/ UE timing error for testing

In these evaluations, the spatial consistency of small-scale parameters is enabled and the clutter setting is {60%, 6, 2}. The model of a truncated Gaussian distribution captured in TR 38.857 [2] is adopt to generate UE timing error where the RMS value T1 is set to 10ns.

In order to avoid the impact of different UE locations on the generalization performance, the UE locations are also kept the same for the corresponding drops with/without UE timing errors in the evaluations.  Fig.19 and Fig.20 show the performance for data setting T-1 and T-2, respectively. Table 8 summarizes their performance and the performance with the same data setting as well.

Fig. 19: Generalization performance with UE timing error (Data setting T-1)

Fig. 20: Generalization performance with UE timing error (Data setting T-2)

Table 8: Generalization performance: Training w/o UE timing error, Test w/ UE timing error

|  | Accuracy achieved @90% (m) | DL-TDOA | Direct: DL-TDOA+RSRP | Direct: Normalized CIR + RSRP | Indirect: Normalized CIR for all TRPs |
| --- | --- | --- | --- | --- | --- |
| w/ generalization | Data setting T-1 1 drop w/o UE timing error for training, 1 drop w/ UE timing error for testing | 8.38 | 0.48 | 6.18 | 26.79 |
| w/o generalization | 1 drop w/o UE timing error for training and testing | 8.2 | 0.48 | 0.33 | 0.52 |
| w/ generalization | Data setting T-2 10 drops w/o UE timing error for training, 10 drops w/ UE timing error for testing | 10.28 | 0.456 | 12.393 | 39.73 |
| w/o generalization | 10 drops w/o UE timing for training and testing | 10.16 | 0.46 | 0.52 | 1.03 |

Based on the results showed in Fig.19-20 and Table 8, we can have the following observation:

Observation 8: For the InF-DH scenario, if the training data set of AI model is generated without UE timing error, the AI model inference performance for the case with UE timing error will suffer larger performance loss

- In this case, the AI/ML assisted positioning scheme “Indirect: Normalized CIR for all TRPs” is more sensitive to the UE timing error, e.g., its performance is even worse than the traditional non-AI based scheme

Based on the above results showed in Section 2.5, we can get the following observation for the performance of direct AI/ML positioning and AI/ML assisted positioning:

Observation 9: For performance of direct AI/ML positioning and AI/ML assisted positioning in the InF-DH scenario,

- If the training and testing data sets are with the same configuration/setting, the direct AI/ML positioning have better performance than AI/ML assisted positioning.

- Regarding the generalization performance, the direct AI/ML positioning achieves similar performance with AI/ML assisted positioning in some cases, and achieves much better performance in some other cases

  - E.g., for the case where the training data set of AI model is generated without timing error (e.g., UE timing error, NW synchronization error) and the AI model inference is done for the data with timing error, the direct AI/ML positioning have better performance than AI/ML assisted positioning.

Complexity of AI models

In our initial assessment on the complexity of different AI models, FLOP and the number of trainable parameters are used as the basic metrics. The corresponding complexity of the AI modes used in our evaluations are summarized in Table 9.

Table 9: Complexity of AI/ML modes used in our evalautions

|  | Direct (CIR-based scheme) | Assisted (All TRPs for Input) | Assisted (Single TRP for Input) |
| --- | --- | --- | --- |
| MFLOPs | ~ 5.32 | ~ 2.96 | ~ 0.66 |
| No. of trainable parameters | ~ 2.66M | ~ 1.48M | ~ 0.33M |

Both CIR and RSRP information are used as the input for direct AI/ML positioning, whereas only CSI information is used as the input for the AI/ML positioning. Thus, the AI model for direct AI/ML positioning has a larger number of trainable parameters and needs more FLOPS.

- Conclusions

In this contribution, we discussed AI based positioning accuracy enhancement from the different aspects including AI-based schemes, generation of data sets, AI modes used in evaluation. We also present our initial evaluations on the positioning accuracy of AI-based methods and complexity of different AI approaches as well. Based on the discussion and evaluation results, we have the following observations and proposals.

Observation 1: For InF-DH scenario with clutter setting {0.6, 6m, 2m}, if the data set is generated by modeling the spatial consistency of small-scale parameters, some performance gain can be observed compared to the configuration that the spatial consistency of small-scale parameters is not modeled.

- The performance gain is quite limited

Observation 2: For InF-DH scenario with clutter setting {0.6, 6m, 2m}, no matter the data set is generated by modeling the spatial consistency of small scale parameters or not, the performance gaps are quite large for different alternatives of data set construction.

- The number of drops has larger impact than the enabling/disabling of spatial consistency for small scale parameters on the AI/ML performance

- When the number of drops increased, the achieved accuracy become worse

Observation 3: For AI/ML assisted positioning

- The performance of “Indirect: Normalized CIR of single TRP” is much worse than “Indirect: Normalized CIR of all TRPs”

- The performance of “Indirect: Normalized CIR of single TRP” is comparable to the traditional non-AI scheme

Observation 4: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated from different drops, there will be large performance degradation for AI-based positioning.

Observation 5: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated with different clutter settings, there will be large performance degradation for AI-based positioning.

Observation 6: For the InF-DH scenario, if the training of AI model is based on the mixed data set with different clutter settings, the AI model inference for the data set with one of these clutter settings can achieve good performance.

Observation 7: For the InF-DH scenario, if the training data set of AI model is generated without NW synchronization error, the AI model performance for the case with NW synchronization error will suffer larger performance loss

- In this case, the AI/ML assisted positioning scheme “Indirect: Normalized CIR for all TRPs” achieves similar performance as the traditional non-AI based scheme.

Observation 8: For the InF-DH scenario, if the training data set of AI model is generated without UE timing error, the AI model inference performance for the case with UE timing error will suffer larger performance loss

- In this case, the AI/ML assisted positioning scheme “Indirect: Normalized CIR for all TRPs” is more sensitive to the UE timing error, e.g., its performance is even worse than the traditional non-AI based scheme

Observation 9: For performance of direct AI/ML positioning and AI/ML assisted positioning in the InF-DH scenario,

- If the training and testing data sets are with the same configuration/setting, the direct AI/ML positioning have better performance than AI/ML assisted positioning.

- Regarding the generalization performance, the direct AI/ML positioning achieves similar performance with AI/ML assisted positioning in some cases, and achieves much better performance in some other cases

  - E.g., for the case where the training data set of AI model is generated without timing error (e.g., UE timing error, NW synchronization error) and the AI model inference is done for the data with timing error, the direct AI/ML positioning have better performance than AI/ML assisted positioning.

Proposal 1: For the evaluation of AI/ML-based positioning accuracy improvement, companies are encouraged to report the following parameters regarding the data set construction to keep the results from different sources be comparable:

- X drops

- Y UEs per drop

- FFS: values of X, Y

Proposal 2: For AI/ML assisted positioning, if the output of AI model is timing-based result (e.g., TOA), prioritize the scheme where the measurement results corresponding to all TRPs are used as the input of AI model

- the scheme where the measurement results corresponding to single TRP are used as the input of AI model is deprioritized.

Table 9: Complexity of AI modes

|  | Direct (CIR-based scheme) | Assisted (All TRPs for Input) | Assisted (Single TRP for Input) |
| --- | --- | --- | --- |
| MFLOPs | ~ 5.32 | ~ 2.96 | ~ 0.66 |
| No. of trainable parameters | ~ 2.66M | ~ 1.48M | ~ 0.33M |

- Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- TR 38.857, Study on NR Positioning Enhancements

- Chair’s notes of RAN1#109e meeting

- Chair’s notes of RAN1#110 meeting