3GPP TSG RAN WG1 #110bis-e		R1-2208855

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	On sub use cases and other aspects of AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.2

Document for:	Discussion and Decision

- Introduction

As a promising technology, AI/ML based solutions have been widely used and has shown impressive performance in various areas, e.g., computer vision, speech recognition, natural language processing (NLP), language translation and so on.

Motivated by the great success of AI/ML in the afore-mentioned areas, 3GPP also started study on AL/ML for wireless communications in SA2 and RAN3. On the basis of the existing study in 3GPP, RAN#94e meeting decided to start a new study item (SI) on AI/ML for NR air interface in Rel-18 [1]. According to the SID, this study will focus on three typical use cases of physical layer in the first step:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In the recent RAN1 meetings, there were intensive discussions on the typical use cases and the potential spec impacts. Some agreements were made in RAN1#110 as below [3]:

Agreement For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected. - Direct AI/ML positioning - AI/ML assisted positioning - Note 1: the selection does not intend to provide any indication of the prospects of any future normative project. - Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results Conclusion Defer the discussion of prioritization of AI/ML positioning based on collaboration level until more progress on collaboration level discussion in agenda 9.2.1. Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded Agreement Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity Conclusion To use the following terminology defined in TS 38.305 when describe their proposed positioning methods - UE-based - UE-assisted/LMF-based - NG-RAN node assisted

In this contribution, we will continue to discuss the use case of AI-based positioning accuracy enhancement, with the focus on sub use cases, the collaboration levels and the corresponding spec impacts on NR.

# Discussion

## Typical sub use cases/schemes

During the evolution of R16 NR, 3GPP developed multiple RAT-based positioning methods to provide high accuracy positioning for NR cellular system:

- DL-TDOA positioning

- UL-TDOA positioning

- DL-AoD positioning

- UL-AoA positioning

- Multi-RTT positioning

Meanwhile, during the discussion of RAN1#109e and RAN1#110 meeting, two different categories of AI/ML-based positioning schemes were agreed as below:

- Cat.1: Direct AI/ML positioning

- Cat.2: AI/ML assisted positioning

Theoretically, AI/ML can be introduced to work along with each of the above positioning methods (e.g., DL-TDOA, DL-AOD, etc.). To simplify the discussion, in the following discussions, we will take DL-TDOA as an example of the positioning methods to discuss the two categories of AI/ML-based positioning schemes. The discussions and proposals can be easily extended to other existing positioning methods.

There are different schemes for AL/ML to improve positioning accuracy, which may have different levels of collaborations between NW and UEs and thereby have different spec impacts. Whether to introduce some specification enhancement or not should be justified based on the comparison with the ones based on implementation and without any need of specification enhancement, from the perspective of complexity, performance and so on. Thus, we should select at least one AI/ML-based solution replying on UE/NW implementation and having no specification impact as a baseline for the justification of any enhancement(s) for NR specification. Regarding the DL-TDOA positioning method, LMF can deploy an AI model (e.g., neural network) to calculate the location of a given UE based on the existing measurement results reported by UE (i.e., RSTD) and no any change is needed for the existing air interface. We refer to this scheme as “Direct: DL RSTD” and it includes the following steps:

- UE measures and reports the DL RSTD results by following the existing mechanism.

  - Optionally, RSRP measurements can also be measured and reported. In this case, we refer to it as “Direct: DL RSTD + RSRP”.

- LMF uses an AI model (e.g., neural network) to estimate the location of a given UE based on the reported Dl RSTD measurement results.

For this scheme, the whole air interface is the same as R16/17 positioning mechanism. Therefore, AI operations can be totally up to LMF implementation and transparent to UE. The advantages of this scheme are no impact on specification and easier for the deployment of AI/ML.

In order to further improve the performance, other types of measurement results can be considered for the direct AI/ML positioning. In theory, for RSTD measurement results, only partial information can be attracted from the reception of DL PRS. That is to say, some information, more or less, is missing and cannot be exploited by AI model(s) at LMF side. Thus, one way is to let UE measure and report the channel information to LMF. A possible scheme consists the following steps (We refer to it as “Direct: Normalized CIR”):

- UE estimates the channel impulse response (CIR) and reports its normalized versions to LMF.

  - Optionally, RSRP measurements can also be measured and reported. In this case, we refer to it as “Direct: Normalized CIR + RSRP”.

- LMF uses an AI model to estimate the location of a given UE.

The scheme “Direct: Normalized CIR + RSRP” can also deployed in UE sided and used for UE based positioning method. For this scheme, new type of measurements at UE side and new reporting format are needed to facilitate the AI operations.

In summary, we have the following proposal:

Proposal 1: For direct AI/ML positioning, study the following input options for the AI model inference at LMF side:

- Option1: Existing UE measurement/reporting (e.g., DL RSTD and the corresponding RSRP)

- Option2: New type of UE measurement/reporting (e.g., Normalized CIR and/or the corresponding RSRP)

For the AI/ML assisted positioning, there are two stages for the whole positioning procedure:

- Stage 1: Some intermediate results were generated by the AI/ML model

- Stage 2: A non-AI method or a separate AI/ML model is used to calculate the location based on these intermediated results

For Stage 2, whether a non-AI method or an AI/ML method is used may be transparent to the other side from 3GPP perspective. Thus, we will only focus on the AI/ML functionality of Stage 1 for the AI/ML assisted positioning.

For NR positioning, there have existed various types of reporting / measurement results, e.g., NLOS/LOS information, DL RSTD and so on. If the output of AI/ML model in Stage 1 is any of the existing types, we refer to this scheme as “Assisted: an existing type of measurement”. As this is study item, we also need to investigate whether there is some new type of intermediate result(s) better than any existing measurement results. We use “Assisted: a new type of measurement” to refer to the scheme where AI model produces a new type of UE measurement different from any existing one, e.g., TOA.

Proposal 2: For AI/ML assisted positioning, study the following output options for the AI model inference at UE side

- Option1: Existing types of measurement (e.g., NLOS/LOS identification, RSTD)

- Option2: New types of measurement (e.g., TOA)

One aspect of AI/ML assisted positioning need to be clarified. During the discussion, different companies have different understanding on the AI/ML assisted positioning regarding the information of how many TRPs are used as the input of the AI/ML model inference. Speaking of this, there are two different alternatives:

- Alt.1: The information of single TRP is used as the input for AI/ML model inference. The same model is used for each TRP.

- Alt.2: The information of all measurement TRPs are used as the input for AI/ML model inference.

According to the evaluation results in our companion tdoc, the performance of Alt.1 if TOA is the output of AI/ML model is much worse than Alt.2, and Alt.1 cannot meet the requirement of positioning accuracy. Thus, Alt.2 should be focused on for AI/ML assisted positioning.

Proposal 3: For AI/ML assisted positioning, the following alternative should be prioritized if the TOA-like output is used for AI/ML model

- The measurement results corresponding to all TRPs are used as the input for AI/ML model inference.

In our understanding, AI/ML assisted positioning is not promising for UE-based positioning method. Thus, in the following discussion, we will only focus on UE-assisted or NG-RAN node assisted positioning method  when we are talking about AI/ML assisted positioning.

## Data collection

### Data collection for AI model training

To train an AI/ML model, the positioning-related measurement results and associate ground-truth labels are used. For direct AI/ML positioning, the ground-truth labels are the location of the UEs that do the measurement. For AI/ML assisted positioning, the ground-truth labels are the ideal information of UE measurement/reporting (e.g., LOS/NLOS identification, RSTD, TOA). In general, no matter what the contents of the labels are, the ground-truth labels are difficult to be obtained for UE and NW.

- LMF cannot get the ground-truth labels for the measurement results reported by normal UEs. Otherwise, the positioning procedure is not needed and LMF can directly provide the location information of the given UE.

- In theory, a normal UE can get some information of its own location via some other technologies, e.g., GPS,  UWB and so on. However, there some potential challenges for normal UE to ground-truth labels, e.g.,

  - Other technologies may not be available for some scenarios

  - The accuracy of location information is uncertainty

Thus, NW and normal UE seem unable to collect the data with ground-truth labels for AI/ML model training. In Rel-17, a new type of UE, also known as PRU (Positioning Reference Unit), was introduced to improve the positioning performance. The key characteristic of PRU is its known location.  Thus, by utilizing PRU, the samples with associate ground-truth labels can be collected.

Proposal 4: If UE-based or UE-assisted positioning method is used, regarding the data collection for AI model training, study the feasibility/mechanism that the measurement results with associated ground-truth labels are obtained via PRU

- Applicable to both model training at UE side and model training at NW side

For the NG-RAN node assisted positioning method, the ground-truth labels can also be collected by PRU. Meanwhile, the measurement results (i.e., data samples) are reported by TRP.

Proposal 5: If NG-RAN node assisted positioning method is used, regarding the data collection for AI model training, study the feasibility/mechanism that the measurement results are reported from TRP and the associated ground-truth labels are obtained via PRU

- Applicable to model training at NW side

Table 1: Data collection for AI model training

|  | Positioning method | AI model inference at which side | Data samples (measurement results) | Ground-truth label |
| --- | --- | --- | --- | --- |
| Direct AI/ML positioning | UE-based | UE | PRU | PRU |
|  | UE-Assisted | LMF | PRU | PRU |
|  | NG-RAN node assisted | LMF | TRP | PRU |
| AI/ML assisted positioning | UE-Assisted | UE | PRU | PRU |
|  | NG-RAN node assisted | TRP | TRP | PRU |

### Data collection for AI model inference

When direct AI/ML positioning is used for UE-assisted positioning method, the input of AI model inference will be based on UE reporting of the measurement results. If the existing type of measurement results are reused, the current spec/signaling/procedure can be reused to facilitate the AI model inference. If new type of measurement result is justified and introduced, some new or enhanced signaling/mechanism for the reporting of new measurement type will be needed.

When direct AI/ML positioning is used for UE-base positioning method or AI/ML assisted positioning is used, the input will be based on the measurement results at UE side. If the model is trained by UE/chipset vendor, the inputs are up to UE implementation and are likely transparent from the perspective of air interface. On the other side, if the model is trained by NW vendor, the size/content of inputs will have to be pre-defined/pre-configured. Similar discussions are also applicable to the NG-RAN node assisted positioning method. Thus, we have the following proposal.

Proposal 6: For the data collection used for AI model inference

- When direct AI/ML positioning is used for UE-assisted positioning method, the target UE will report the measurement results to LMF

  - FFS: type of measurement, RS configuration for measurement

- When direct AI/ML positioning is used for UE-base positioning method or AI/ML assisted positioning is used for UE-assisted positioning method, the UE will collect measurement for the input of AI model

  - If the model is trained at the same side, the inputs/data collection are up to UE implementation and transparent from the perspective of air interface

  - If the model is trained at NW side and AI model inference is carried out at UE side, the size/contents of inputs will need to be pre-defined or pre-configured.

- When direct AI/ML positioning is used for NG-RAN node assisted positioning method, the TRP will report the measurement results to LMF

- When direct AI/ML positioning or AI/ML assisted positioning is used for NG-RAN node assisted positioning method, the TRP will collect measurement for the input of AI model

## AI model training and deployment

The main phases of AI/ML based positioning consist of model training and inference. The collaboration levels and potential spec impacts will be different for model training phase and inference phase. Thus, we prefer to discuss model training and inference separately.

In generally, there are two types of AI model training: online training and offline training. Online training can update the AI model upon the reception of timely samples, which may be more flexible for the dynamic operation environment. On the other hand, online training will suffer more issues:

- How to evaluate/justify the performance/benefit of online training

  - The well-established methodology for online training of AI model is still missing in 3GPP

- More spec impact on NR system

  - E.g., new procedure(s)/signal(s) to support online training

- Potential more overhead due to data collection/information sharing

- Feasibility for UE/gNB to deploy an updated AI/ML models in (nearly) real-time manner

- …

Considering the limited timeline of Rel-18 AI study, we prefer to focus on offline training in R18 and continue to study online training in later release(s).

Proposal 7: For both direct AI/ML positioning and AI/ML assisted positioning, offline training of AI model(s) is prioritized in Rel-18

- Study online training in the future release(s)

From the perspective of AI model deployment, two types of AI models were defined in RAN1#109e meeting

- One-side model

- Two-side model.

In our understanding, one-side model is enough for positioning from the perspective of performance. Even for the AI/ML assisted positioning with AI scheme for Stage 2, we can still regard the AI model in Stage 1 as one-side model since the AI operations in Stage 1 and Stage 2 are independent of each other. RAN1#110 meeting agreed to support at least one-side model. The controversial point is whether to support two-side model or not. Two-side model will lead to more challenging issues, more spec impact, larger evaluation efforts and higher implementation complexity, but without significant benefits. Meanwhile, two-side AI model is studied for CSI compression and we can have better understanding on the feasibility and spec impact on this type of AI model after the completion of AI-based CSI compression study. Thus, it is not urgent to study two-side AI model for positioning and thereby, we propose not to study two-side model for AI/ML based positioning in Rel-18 study item.

Proposal 8: For AI/ML based positioning, not support to study two-side AI/ML model within Rel-18 SI timeline.

If the AI model is trained by one side and the inference is performed in the other side, it will have to support model transfer between this two sides, which is expected to lead to more challenges for spec design, product implementation and practical deployment. For example, more standardization efforts will be needed for the mechanism supporting AI/ML model transfer between different nodes (e.g., AI model is transferred from network to UE). Moreover, the optimization/deployment of transferred AI/ML model may require advanced UE capability. Thus, we suggest to focus on the schemes with the model training and inference are done in the same side and without model transfer in Rel-18.

Proposal 9: For both direct AI/ML positioning and AI/ML assisted positioning, the AI model training and inference are assumed to be done in the same side in Rel-18, i.e., no model transfer

- AI model training and inference at UE side, or

- AI model training and inference at NW side

- Note: model transfer can be studied in future release(s).

## AI model inference

For UE-based positioning method, if AI model is used to estimate the location, then estimated location information can be reported by existing LPP signaling. One potential enhancement is to report the associated confidence information of estimated location additionally.

Regarding the direct AI/ML scheme for UE-assisted positioning method, there are some potential enhancements from the following aspects:

- Whether can some new type of measurement/reporting improve the performance of AI model? If so, what’s the new measurement/reporting type and the associated LPP signaling from UE to LMF?

- The current measurement/reporting can be used as the input of AI model in a transparent way. Whether there is some enhancement needed to improve the AI model performance based on existing measurement/reproting, e.g., more bits to report the existing measurement results. The parts will involve the LPP signaling from UE to LMF.

- If any of the above enhancement is introduced, then some new LPP signaling from LMF to UE will be needed for the configuration/triggering of measurement/reporting.

The above-mentioned aspects are also applicable to other cases/combinations in similar way. Table 2 provides a summary on the spec impacts of AI model inference.

Table 2: Spec impacts of AI model inference

|  | Positioning method | AI model inference at which side | Spec impacts |
| --- | --- | --- | --- |
| Direct AI/ML positioning | UE-based | UE | 1. whether additional information (e.g., confidence of the AI estimated location) is needed or not on top of location information (LPP signaling from UE to LMF) |
|  | UE-Assisted | LMF | 1. whether/what new type of measurement /reporting (LPP signaling from UE to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (LPP signaling from UE to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration including RS (LPP signaling from LMF to UE) |
|  | NG-RAN node assisted | LMF | 1. whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (NRPPa signaling from gNB to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration (NRPPa signaling from LMF to gNB) |
| AI/ML assisted positioning | UE-Assisted | UE | 1. whether/what new type of measurement /reporting (LPP signaling from UE to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (LPP signaling from UE to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration (LPP signaling from LMF to UE) |
|  | NG-RAN node assisted | TRP | 1. whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (NRPPa signaling from gNB to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration (NRPPa signaling from LMF to gNB) |

Proposal 10: For AI/ML based positioning,

- if UE-based positioning method is used, study the following aspect on spec impact

  - whether additional information (e.g., confidence of the AI estimated location) is needed or not on top of location information (LPP signaling from UE to LMF)

- if UE-based positioning method is used, study the following aspects on spec impact

  - whether/what new type of measurement /reporting (LPP signaling from UE to LMF)

  - whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (LPP signaling from UE to LMF)

  - whether/what enhancement for measurement/reporting triggering/configuration (LPP signaling from LMF to UE)

- if NG-RAN node assisted positioning method is used, study the following aspects on spec impact

  - whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF)

  - whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (NRPPa signaling from gNB to LMF)

  - whether/what enhancement for measurement/reporting triggering/configuration (NRPPa signaling from LMF to gNB)

## Model monitoring and management (selection, activation, deactivation, switching, and fallback operation)

For AI-based CSI compression or beam prediction, there are some existing metric(s), in an explicit or implicit way, to reflect the performance of AI model inference. For example, some potential metrics can be used for AI-based CSI compression:

- Some metrics of communication performance can be used to reflect the AI model performance implicitly. If an AI model achieves good performance, then the system will have good throughput and high transmission reliability (e.g., lower BLER). Thus, the throughput or BLER can be used a metric to monitor the AI model performance.

- The estimated channel by UE can be also used to compare the output of AI model. Thus, the accuracy of re-constructed channel/precoder can be monitored.

For the positioning, the output of AI model usually is not used for communication. Thus, the metrics reflecting communication performance cannot reflect the performance of AI model inference. Meanwhile, a normal UE cannot get the ground-truth information for location information or some intermediate results (e.g., TOA, TDOA, NLOS/LOS identification, etc.). Thus, the PRU seems the only choice to collect information for the AI model monitoring. Meanwhile, there are some open issues regarding the usefulness/efficiency for PRU use for AI model monitoring, for example

- The PRU may not be deployed in each cell or each scenario. In such a cell or scenario, no PRU can used for AI model monitoring.

- The PRU and the target UE are in different locations. Thus, can the performance of AI model for a UE in one location reflect the performance of the same AI model for another UE in a different location? There maybe need further study and justification.

Proposal 11: For AI/ML based positioning, study from which side/component the data are collected for AI model monitoring

- If PRU is utilized to collect data for AI model performance monitoring, further study is needed to evaluate/justify whether the performance of the same AI model for PRU and a given UE in different locations are the same or similar.

## Collaboration level and potential impact on specification

In this section, we will discuss the above-mentioned mechanisms from the perspective of collaboration levels between NW and UE and the corresponding specification impacts.

RAN1#109e meeting made the following agreement for the general framework of collaboration level [2]:

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels Level x: No collaboration Level y: Signaling-based collaboration without model transfer Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

### Direct AI/ML positioning

In general, direct AI/ML positioning can be used for both UE-based positioning method and UE assisted positioning method.

For UE-based positioning method, the inference of AI/ML model is performed only at UE side. In this case, this model can be regarded as UE-side model. UE will use this AI/ML model to generate the estimated location, which can be reported by existing NR signaling. If this model is trained by UE/chipset vendor, there seems no need to specify the input as it is up to implementation.

Observation 1: For UE-based positioning method, if the model for direct AI/ML positioning is trained by UE/chipset vendor, the reporting can reuse existing NR signaling and there is no strong motivation to specify the input

Thus, we have the following proposal:

Proposal 12: For UE-based positioning method, collaboration level x is prioritized for direct AI/ML positioning in Rel-18

- FFS: level y, e.g., signaling for model monitoring

For UE-assisted positioning method, the inference of AI/ML model is performed only at network side. In this case, this model can be regarded as network-side model. Network will use this AI/ML model to generate the estimated location based on the measurement results reported by UE.  If this approach is based on the existing UE reporting (e.g., the scheme “Direct: DL RSTD + RSRP”), the corresponding AI operations at network side can be transparent to UE. Thus, no collaboration between UE and network is needed in this case. Consequently, these will no spec impact from the perspective of RAN side.

Observation 2: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on existing UE measurement and reporting, the AI operations at network side can be transparent to UE.

Thus, we have following proposal:

Proposal 13: For UE-assisted positioning method, collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing measurement and reporting (e.g., the scheme “Direct: DL RSTD +RSRP”)

If the input of direct AI/ML positioning is some new type(s) of UE measurement (e.g., the scheme “Direct: Normalized CIR + RSRP”), some specification enhancement(s) is likely to be needed, e.g.,

- Signaling for the configuration of new type of measurement and reporting

- New reporting format

- New type of measurement at UE side and corresponding requirement(s)

Although the new type of measurement result and reporting are introduced with the aim to facilitate the AI/ML operation at LMF, whether the AI/ML operation or non-AI operation is applied at LMF is transparent to UE. In other word, LMF can still use traditional algorithm(s) to calculate the location information of the given UE based on the new measurement results if it wants. Thus, the corresponding spec enhancement(s) is only regarding the new type of measurement and reporting, without any explicit/direct information on AI/ML operations.

Observation 3: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on new type(s) of UE measurement/reporting, specification enhancement will be needed.

- e.g., new reporting format, new type of measurement and corresponding requirement

Thus, we have following proposal:

Proposal 14: For UE-assisted positioning method, collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of UE measurement/reporting (e.g., the scheme “Direct: Normalized CIR + RSRP”)

### AI/ML assisted positioning

For AI/ML assisted positioning, the AI/ML model of Stage 1 will produce some intermediate results, based on which the location will be calculated. For the perspective of air interface, there seems no much difference between direct AI/ML positioning and AI/ML assisted positioning for UE-based positioning method. Thus, the following discussions will focus on UE-assisted positioning method.

For UE-assisted positioning method, the inference of AI/ML model of Stage 1 is performed only at UE side. This model can be regarded as UE-side model. UE will use this AI/ML model to generate UE measurement results for some existing type(s) and the generated measurement results can be reported by existing NR signaling. If this model is trained by UE/chipset vendor, there seems no need to specify the input as it is up to UE implementation.

Observation 4: For UE-assisted positioning method, if the model for AI/ML assisted positioning is trained by UE/chipset vendor and generates UE measurement results for some existing type(s), the reporting can reuse existing NR signaling and there is no strong motivation to specify the input of AI/ML model

Thus, we have following proposal:

Proposal 15: For UE-assisted positioning method, collaboration level x is prioritized for AI/ML assisted positioning in Rel-18 if the outputs of AI model are some existing type(s) of UE measurement (e.g., the scheme “Assisted: an existing type of measurement”)

- FFS: level y, e.g., signaling for model monitoring

If the output of AI model for AI/ML positioning assisted is some new type of UE measurement (e.g., the scheme “Assisted: a new type of measurement”), some specification enhancement(s) may be needed, e.g.,

- Signaling for the configuration of new type of measurement and reporting

- New reporting format

- New type of measurement at UE side and corresponding requirement(s)

Observation 5: For UE-assisted positioning method, if the outputs of the model for AI/ML assisted positioning are some new type(s) of UE measurement, specification enhancement will be needed.

- e.g., new reporting format, new type of measurement and corresponding requirement

Thus, we have following proposal:

Proposal 16: For UE-assisted positioning method, collaboration level y is prioritized for AI/ML assisted positioning if the outputs of AI model are some new type(s) of UE measurement (e.g., the scheme “Assisted: a new type of measurement”)

In the above discussion, we haven’t included life cycle management. We summarize the above discussions in Table 3 as below:

Table 3: Potential spec impact(s) and collaboration level(s)

| AI schemes/ sub use cases | Positioning method | Model | Potential impact on NR specifications | Collaboration level between NW and UE |
| --- | --- | --- | --- | --- |
| direct AI/ML positioning | UE-based | UE-side model | whether additional information (e.g., confidence of the AI estimated location) is needed or not on top of location information (LPP signaling from UE to LMF) | Level x FFS: level y |
| direct AI/ML positioning based on existing measurement/reporting (e.g., “Direct: DL RSTD + RSRP”) | UE-assisted | Network-side model | whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) | Level x FFS: level y |
| direct AI/ML positioning based on new measurement/reporting (e.g., “Direct: Normalized CIR + RSRP”) | UE-assisted | Network-side model | - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at UE side and corresponding requirement(s) | Level y |
| AI/ML assisted positioning based on existing type(s) of UE measurement/reporting (e.g., “Assisted: an existing type of measurement”) | UE-assisted | UE-side model (Stage 1) | / | Level x FFS: level y |
| AI/ML assisted positioning based on new type(s) of UE measurement/reporting (e.g., “Assisted: a new type of measurement”) | UE-assisted | UE-side model (Stage 1) | - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at UE side and corresponding requirement(s) | Level y |
| Note: Any potential spec impact of life cycle management is not included here. |  |  |  |  |

- Conclusions

In this contribution, we discussed the sub use cases for AI based positioning accuracy improvement. Starting with some general principles for the use case selection, we discussed some typical sub use cases for AI-based positioning accuracy enhancement, followed by the discussion on potential impact on NR specification and collaboration between NW and UE. Based on the above discussion, we have the following proposals and the summary of spec impact and collaboration level for typical sub use cases.

Proposal 1: For direct AI/ML positioning, study the following input options for the AI model inference at LMF side:

- Option1: Existing UE measurement/reporting (e.g., DL RSTD and the corresponding RSRP)

- Option2: New type of UE measurement/reporting (e.g., Normalized CIR and/or the corresponding RSRP)

Proposal 2: For AI/ML assisted positioning, study the following output options for the AI model inference at UE side

- Option1: Existing types of measurement (e.g., NLOS/LOS identification, RSTD)

- Option2: New types of measurement (e.g., TOA)

Proposal 3: For AI/ML assisted positioning, the following alternative should be prioritized if the TOA-like output is used for AI/ML model

- The measurement results corresponding to all TRPs are used as the input for AI/ML model inference.

Proposal 4: If UE-based or UE-assisted positioning method is used, regarding the data collection for AI model training, study the feasibility/mechanism that the measurement results with associated ground-truth labels are obtained via PRU

- Applicable to both model training at UE side and model training at NW side

Proposal 5: If NG-RAN node assisted positioning method is used, regarding the data collection for AI model training, study the feasibility/mechanism that the measurement results are reported from TRP and the associated ground-truth labels are obtained via PRU

- Applicable to model training at NW side

Proposal 6: For the data collection used for AI model inference

- When direct AI/ML positioning is used for UE-assisted positioning method, the target UE will report the measurement results to LMF

  - FFS: type of measurement, RS configuration for measurement

- When direct AI/ML positioning is used for UE-base positioning method or AI/ML assisted positioning is used for UE-assisted positioning method, the UE will collect measurement for the input of AI model

  - If the model is trained at the same side, the inputs/data collection are up to UE implementation and transparent from the perspective of air interface

  - If the model is trained at NW side and AI model inference is carried out at UE side, the size/contents of inputs will need to be pre-defined or pre-configured.

- When direct AI/ML positioning is used for NG-RAN node assisted positioning method, the TRP will report the measurement results to LMF

- When direct AI/ML positioning or AI/ML assisted positioning is used for NG-RAN node assisted positioning method, the TRP will collect measurement for the input of AI model

Proposal 7: For both direct AI/ML positioning and AI/ML assisted positioning, offline training of AI model(s) is prioritized in Rel-18

- Study online training in the future release(s)

Proposal 8: For AI/ML based positioning, not support to study two-side AI/ML model within Rel-18 SI timeline.

Proposal 9: For both direct AI/ML positioning and AI/ML assisted positioning, the AI model training and inference are assumed to be done in the same side in Rel-18, i.e., no model transfer

- AI model training and inference at UE side, or

- AI model training and inference at NW side

- Note: model transfer can be studied in future release(s).

Proposal 10: For AI/ML based positioning,

- if UE-based positioning method is used, study the following aspect on spec impact

  - whether additional information (e.g., confidence of the AI estimated location) is needed or not on top of location information (LPP signaling from UE to LMF)

- if UE-based positioning method is used, study the following aspects on spec impact

  - whether/what new type of measurement /reporting (LPP signaling from UE to LMF)

  - whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (LPP signaling from UE to LMF)

  - whether/what enhancement for measurement/reporting triggering/configuration (LPP signaling from LMF to UE)

- if NG-RAN node assisted positioning method is used, study the following aspects on spec impact

  - whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF)

  - whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (NRPPa signaling from gNB to LMF)

  - whether/what enhancement for measurement/reporting triggering/configuration (NRPPa signaling from LMF to gNB)

Proposal 11: For AI/ML based positioning, study from which side/component the data are collected for AI model monitoring

- If PRU is utilized to collect data for AI model performance monitoring, further study is needed to evaluate/justify whether the performance of the same AI model for PRU and a given UE in different locations are the same or similar.

Proposal 12: For UE-based positioning method, collaboration level x is prioritized for direct AI/ML positioning in Rel-18

- FFS: level y, e.g., signaling for model monitoring

Proposal 13: For UE-assisted positioning method, collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing measurement and reporting (e.g., the scheme “Direct: DL RSTD +RSRP”)

Proposal 14: For UE-assisted positioning method, collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of UE measurement/reporting (e.g., the scheme “Direct: Normalized CIR + RSRP”)

Proposal 15: For UE-assisted positioning method, collaboration level x is prioritized for AI/ML assisted positioning in Rel-18 if the outputs of AI model are some existing type(s) of UE measurement (e.g., the scheme “Assisted: an existing type of measurement”)

- FFS: level y, e.g., signaling for model monitoring

Proposal 16: For UE-assisted positioning method, collaboration level y is prioritized for AI/ML assisted positioning if the outputs of AI model are some new type(s) of UE measurement (e.g., the scheme “Assisted: a new type of measurement”)

Table 3: Potential spec impact(s) and collaboration level(s)

| AI schemes/ sub use cases | Positioning method | Model | Potential impact on NR specifications | Collaboration level between NW and UE |
| --- | --- | --- | --- | --- |
| direct AI/ML positioning | UE-based | UE-side model | whether additional information (e.g., confidence of the AI estimated location) is needed or not on top of location information (LPP signaling from UE to LMF) | Level x FFS: level y |
| direct AI/ML positioning based on existing measurement/reporting (e.g., “Direct: DL RSTD + RSRP”) | UE-assisted | Network-side model | whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) | Level x FFS: level y |
| direct AI/ML positioning based on new measurement/reporting (e.g., “Direct: Normalized CIR + RSRP”) | UE-assisted | Network-side model | - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at UE side and corresponding requirement(s) | Level y |
| AI/ML assisted positioning based on existing type(s) of UE measurement/reporting (e.g., “Assisted: an existing type of measurement”) | UE-assisted | UE-side model (Stage 1) | / | Level x FFS: level y |
| AI/ML assisted positioning based on new type(s) of UE measurement/reporting (e.g., “Assisted: a new type of measurement”) | UE-assisted | UE-side model (Stage 1) | - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at UE side and corresponding requirement(s) | Level y |
| Note: Any potential spec impact of life cycle management is not included here. |  |  |  |  |

- Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Chair’s notes of RAN1#109e meeting

- Chair’s notes of RAN1#110 meeting