3GPP TSG RAN WG1 #110bis-e 	                                                            	 R1-2208863

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Discussion on XR specific capacity enhancements techniques

Agenda Item:	9.10.2

Document for:	Discussion and Decision

# Introduction

In RAN1#110 meeting, XR-specific capacity enhancements techniques were discussed with the following agreements:

Agreement

RAN1 to make decision on the following in RAN1#110bis-e

- Support single DCI scheduling multi-PDSCHs/PUSCHs which is currently supported for FR2-2 to other SCS in FR1/FR2

Conclusion

There is no consensus in RAN1 on the benefits of enhancing SPS for the purpose of XR capacity enhancement.

Agreement

When DG is used as the baseline scheme, for the performance evaluation scheduling, after SR is triggered, both BSR and UL data can be transmitted using the UL grant after SR.

- Companies are encouraged to provide the size of resources by the first UL grant after SR

Agreement

Whether/how to enhance BSR to improve capacity performance of XR traffic is within RAN2 scope and is not handled by RAN1.

- Note that companies should indicate if and what BSR enhancement is assumed in their RAN1 proposals on CG and DG enhancements.

- RAN1 can evaluate BSR enhancement to improve capacity performance

In this contribution, we discuss the capacity enhancement for supporting XR services.

# Discussion

XR service is a quasi-periodic traffic with jitter and various packet sizes, if SPS and configured grant transmission is used to support XR services, the following enhancement should be studied:

- Non-integer period

In earlier RAN1 meetings, it was agreed that 60 frames-per-second (fps) is baseline for both DL and UL video stream and 30 fps, 90 fps as well as 120 fps can be also optionally evaluated. Based on the formula of arrival time of packet, the corresponding periodicities are {33.33ms, 16.67ms, 11.11ms, 8.33ms}. It is easy to observe that the periodicity of XR/CG and periodicity of CG are not matched. One straightforward way is to introduce non-integer number(s) in the periodicity of CG. The UE needs to calculate/derivate the PUSCH occasion(s) based on the non-integer value by using the enhanced CG occasion calculation formula, i.e. the current CG occasion formula with ceiling operation:

- CG Type 1:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	ceiling{(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)}
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

- CG Type 2:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	ceiling{[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]}
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

The following periodicities are supported for CG depending on the configured subcarrier spacing [symbols]

- {100/3/14, 50/3/14, 100/9/14, 25/3/14} for 15kHz

- 0.5 x {100/3/14, 50/3/14, 100/9/14, 25/3/14} for 30kHz

- 0.25 x {100/3/14, 50/3/14, 100/9/14, 25/3/14} for 60kHz with normal CP

- 0.25 x {100/3/12, 50/3/12, 100/9/12, 25/3/12} for 60kHz with ECP

- 0.125 x {100/3/14, 50/3/14, 100/9/14, 25/3/14} for 120kHz

- 0.0625 x {100/3/14, 50/3/14, 100/9/14, 25/3/14} for 480kHz

- 0.03125 x {100/3/14, 50/3/14, 100/9/14, 25/3/14} for 960kHz

Figure 1 shows an example of the calculated CG PUSCH occasions.

Figure 1: Example of CG PUSCH occasions calculated for a 60fps period

Another option to resolve the periodicity mismatch issue is to configure a periodic CG pattern, and each CG pattern has multiple CG-PUSCH occasions. Let us take 60fps for UL video as an example, as shown in Figure 2, the periodicity of the CG pattern is 50ms, 9 CG PUSCH occasions are configured in each period, which can be used to resolve the periodicity mismatch issue, jitter issue and large packet size issue at the same time.

Figure 2: Example of configured CG PUSCH pattern for a 60fps period

Proposal 1: The following solutions to resolve the periodicity mismatch issue should be further studied:

- Option 1: Introduce non-integer values for CG periodicity and add ceiling operation in CG occasion calculation formula.

- Option 2: Introduce the periodic CG pattern in which multiple non-consecutive sets of PUSCH occasions can be configured in one period.

- Large and various packet size

As mentioned above, multi-PUSCH transmission per packet should be used to support XR and CG services. For configured grant transmission, how to transmit multiple PUSCHs in each period should be studied. One example is that multiple PUSCH transmission occasions per period are configured for one CG configuration. Another example is that multiple CG configurations with different resource allocations are configured to UE, the most appropriate CG configuration(s) is used to transmit each packet. Then, fast CG activation may be needed.

Proposal 2: For CG transmission, the following solutions for XR-specific packet size adaptation should be further studied:

- Alt.1: The CG configuration supports multiple PUSCH transmission occasions in each period.

- Alt.2: A UE can have multiple CG configurations, one or more of which can be fast activated.

For CG PUSCH, UE can transmit CG-UCI to early terminate the CG transmission. Specifically, when multiple PUSCH transmission occasions are configured for one CG configuration, CG-UCI, transmitted in the early PUSCH transmission occasion(s) within a given period, can indicate the last configured PUSCH transmission occasion(s) within the given period is dropped. When gNB receives CG-UCI indicating early termination, the last configured PUSCH occasion(s) can be dynamically scheduled for other transmission to improve the system efficiency. Dynamic determination of MCS, TDRA or FDRA used to transmit CG PUSCH by UE should be further studied.

During the simulation for XR capacity with simulation parameters in Table 2, when CG PUSCH is configured, the maximum packet size is used to allocate CG PUSCH resource. The RBs allocated for a UE with CG PUSCH cannot be re-allocate to other UE even the RBs are redundant for the UE when the real-time packet size is less than the maximum packet size. When early terminate of CG transmission is applicable:

- Case 1: Among the occasions configured for CG PUSCH in a period, the last N≥1 occasion(s) could be released and each released occasion has to be released in full.

- Case 2: Among the occasions configured for CG PUSCH in a period, the first occasion where any RB is released can have partial release, i.e., some of RBs in the occasion are released but some others are retained as needed; the later occasions after this first occasion, if any, are released in full. The first occasion where any RB is released cannot be the first occasion configured for CG PUSCH in a period.

- Case 3: Same as Case 2, except that the first occasion where any RB is released can be the first occasion configured for CG PUSCH in a period.

Figure 3: 3 cases for early termination of CG transmission

The Table 1 shows the comparison between legacy CG transmission and new CG transmission scheme with early termination. About 19%, 33.2% and 33.3% RB releases are respectively observed for cases 1~3. For case 1, CG-UCI overhead is the lowest, since slot-level indication is enough. The gain of case 2 is almost same as case 3, and the advantages of case 2 include:  simple reception of CG-UCI which is multiplexed on fixed REs and no additional time delay for decoding the first CG PUSCH.

Table 1 Comparison of legacy CG vs. CG with early termination

|  |  | Avg. number of consumed RBs (per UE per packet) |
| --- | --- | --- |
| Legacy CG transmission |  | 840.8 |
| Early terminate the CG transmission | Case 1 | 679.1 |
|  | Case 2 | 562.2 |
|  | Case 3 | 560.5 |

Proposal 3: Early terminate the CG transmission should be further studied.

# Conclusions

In this contribution, we discuss the capacity enhancement for supporting XR services with following proposals:

Proposal 1: The following solutions to resolve the periodicity mismatch issue should be further studied:

- Option 1: Introduce non-integer values for CG periodicity and add ceiling operation in CG occasion calculation formula.

- Option 2: Introduce the periodic CG pattern in which multiple non-consecutive set of PUSCH occasions can be configured in one period.

Proposal 2: For CG transmission, the following solutions for XR-specific packet size adaptation should be further studied:

- Alt.1: The CG configuration supports multiple PUSCH transmission occasions in each period.

- Alt.2: A UE can have multiple CG configurations, one or more of which can be fast activated.

Proposal 3: Early terminate the CG transmission should be further studied.

# References

- RP-213587, New SID “Study on XR Enhancements for NR”, Nokia

- R1-2204028, “Discussion on XR specific power saving techniques”, OPPO

# Appendix

The simulation parameters are introduced by the Table 2.

Table 2: Simulation parameters for Dense Urban

| Parameters | Value |
| --- | --- |
| Scenario | Dense Urban hexagonal layout with (19, 3) Sectors |
| Inter-BS distance | 200m |
| Carrier frequency | 4GHz |
| Duplex Mode / Simulation bandwidth | 100MHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) (dH, dV) = (0.5λ, 0.5λ) |
| UE Antenna Configuration | 4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Transmit Power | 51dBm |
| Antenna Height | 3 m for BS and 1.5 m for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE speed | 3km/h |
| Scheduling Algorithm | SU-MIMO+PF |
| Channel estimation | Realist |
| BS receiver | MMSE-IRC |