3GPP TSG RAN WG1 #110bis-e R1-2208864 e-Meeting, October 10th – 19th, 2022 CR-Form-v12.2. DRAFT CHANGE REQUEST. 38.213; CR; -; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on e-type 3 HARQ codebook. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_IIOT_URLLC_enh-Core; Date:; 2022-9-30. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19). Reason for change:; In RAN1 #107, the following agreements were made for eType-3 HARQ-ACK codebook: Agreement If more than one (M>1) enhanced Type 3 HARQ-ACK codebook is configured and the triggering DCI with the ‘one-shot HARQ-ACK request’ set to ‘1’, - If the FDRA field is not valid, i.e. all “1s” or all “0s” as per Rel-16, then PDSCH is not scheduled: - If a new field with N= log2 (M) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE uses the MCS field to indicate one of M configured e-Type 3 HARQ-ACK CBs …... When maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, there are two MCS fields in DCI format 1_1. If the FDRA field is not valid and enhanced Type 3 codebook indicator field is not configured in DCI format 1_1, which MCS field used to indicate one of the configured e-Type 3 HARQ-ACK CBs should be defined. We propose the MCS field for TB 1 is used, which is same as the indication of HARQ_retx_offset for HARQ-ACK retransmission.. Summary of change:; When no PDSCH is scheduled and enhanced Type 3 codebook indicator field is not configured in DCI format 1_1, the MCS field for TB 1 is used to indicate a pdsch-HARQ-ACK-EnhType3Index value.. Consequences if not approved:; Undetermined UE implementation when the UE observes maxNrofCodeWordsScheduledByDCI = 2 and receives a DCI format 1_1 which intends to trigger eType-3 CB (with no PDSCH scheduling) but meanwhile is not configured with enhanced Type 3 codebook indicator field.. Clauses affected:; 9.1.4. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR..

### 9.1.4	Type-3 HARQ-ACK codebook determination

*** Unchanged text is omitted ***

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. If the UE is provided pdsch-HARQ-ACK-EnhType3List and the DCI format includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-EnhType3List corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is provided by the value of MCS field in the DCI format. If there are two MCS fields for two transport blocks in the DCI format, the pdsch-HARQ-ACK-EnhType3Index value is provided by the value of MCS field for transport block 1 in the DCI format. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format".

*** Unchanged text is omitted ***