3GPP TSG RAN WG1 #110bis-e   	                           R1-2208899

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.2.2.1

Source:	LG Electronics

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	   Discussion and Decision

- Introduction

In the RAN1#110 meeting, several agreements related to evaluation methodology and assumptions were made. Also, some conclusions related to time-domain CSI prediction were made. This contribution discusses on evaluation methodology for AI/ML based CSI feedback enhancement.

- Discussions on evaluation methodology

Agreement @ RAN1#109e

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

Baseline scheme for evaluation

As captured above, Rel-16/17 Type II CSI are selected as baseline scheme if SLS is adopted. The FFS point is whether to optionally adopt Type I CSI or not. Type I CSI consists of SD domain basis and selection matrix, and thereby it provides limited performance especially for MU-MIMO. However, required payload is relatively small (<80bits for SB reporting). On the other hand, Rel-16/17 Type II CSI consists of SD / FD basis and combining matrix. The flexibility of linear combining of multiple SD / FD domain basis provides good MU-MIMO performance at the expense of high required payload. The spatial-frequency domain CSI compression using two-sided AI/ML model is targeting Rel-16/Rel-17 Type II CSI, and the state-of-art CSI feedback mechanism is normally selected as a baseline in order to fairly compare performance. If Type I CSI is optionally selected as a baseline, the performance benefit may be exaggerated. In that sense, Type I CSI is not preferred as a baseline scheme for performance comparison purpose and Rel-16 and Rel-17 Type II CSI seem sufficient.

Proposal 1. Type I CSI is not preferred as a baseline for the AI/ML based CSI reporting.

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

    - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

AI/ML model generalization

As captured above, agreement related to AI/ML model generalization were made in RAN1#110. In Case 1, training dataset and inference/test dataset are the same, so it can be considered as a baseline. In Case 2, training dataset and inference/test dataset are different. Case 3 employs mixed datasets for training and conducts inference/test on one dataset from a single scenario/configuration. The FFS point is whether to include another case (i.e., Case 2A) which considers fine-tuning. In our view, the exact definition of fine-tuning is firstly needed before we discussing this issue. Moreover, in the generalization perspective, Case 2A can be included in Case 3. Therefore, Case 2A and Case 3 can be merged.

Proposal 2. For AI/ML model generalization, Case 2A and Case 3 can be merged.

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

    - FFS: whether to add O2I carpenetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

    - Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Evaluation methodology for temporal domain CSI prediction

As captured above, some evaluation mythologies for temporal-domain CSI prediction were captured as a conclusion in the Chairman’s note, since this sub-use case was not chosen as another representative sub-use case. First of all, as expressed in our companion contribution [1], we are not supportive for temporal-domain CSI prediction. Instead, it is preferred to focus on the spatial-frequency domain compression considering limited time line.

If the AI/ML based CSI prediction sub-use case is to be selected as a sub use case, there are still remaining issues on the evaluation methodology. First issue is whether to add car-penetration loss. Normally, CSI prediction is applied for high mobility scenario. In this sense, 10, 20, 30, 60 and 120km/h are assumed for UE speed. The assumption of such high speed is reasonable when UE is on the vehicle. Therefore, O2I car-penetration loss in TR 38.901 should be taken into account for evaluation of CSI prediction. The exact model is 	 where  is the basic outdoor path loss,  μ = 9, and σP = 5.

Proposal 3. If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, O2I car-penetration loss in TR 38.901 should be taken into account for evaluation.

In Rel-18 MIMO session, spatial-frequency-time domain compression is under discussion, and agreed evaluation methodology is captured in [2]. Thus, it is preferred to re-use EVM in [2] as many as possible. First, for mobility modeling, spatial consistency procedure A and B can be considered for CSI prediction. Second, for the baseline scheme, it should be carefully determined by state-of-art scheme in order to avoid over-estimated performance gain. So, it needs to be discussed what is the reasonable baseline for the CSI predication. Lastly, assumptions on measurement / prediction window needs to be aligned in order for fair performance comparison among companies. Based on the above discussion, following is proposed.

Proposal 4. If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, at least followings need to be further discussed

- Spatial consistency

- Reasonable baseline scheme

- Measurement / prediction window

- Conclusion

This contribution discussed on evaluation methodology for AI/ML based CSI feedback enhancement. Based on the above discussion, following three proposals are proposed.

Proposal 1. Type I CSI is not preferred as a baseline for the AI/ML based CSI reporting.

Proposal 2. For AI/ML model generalization, Case 2A and Case 3 can be merged.

Proposal 3. If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, O2I car-penetration loss in TR 38.901 should be taken into account for evaluation.

Proposal 4. If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, at least followings need to be further discussed

- Spatial consistency

- Reasonable baseline scheme

- Measurement / prediction window

Reference

[1] R1-2208900, “Other aspects on AI/ML for CSI feedback enhancement”, LG Electronics

[2] R1-2205289, “EVM for Rel-18 MIMO CSI enhancements”, Samsung