3GPP TSG RAN WG1 #110bis-e	                                         R1-2208909

e-Meeting, October 10th – 19th, 2022

Source:	CATT

Title:	Discussion on support of positioning in FR2-2

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on the issue on support of positioning in FR2-2 in RAN2 LS R1-2208325(R2-2208810) [1].

Discussion on support of positioning in FR2-2

In the RAN2 LS R1-2208325(R2-2208810) [1], RAN2 informs that NR operation is extended up to 71 GHz and new numerologies (i.e., 480 kHz SCS, 960 kHz SCS) are introduced in FR2-2 (52.6 GHz–71 GHz) for data and control channels and reference signals in Rel-17. Moreover, RAN2 thinks FR2-2 is assumed to be applicable to other Rel-17 features unless otherwise specified based on the below agreement from 71GHz WI in RAN2:

From chairman’s note in RAN2#117-e meeting, Applicability of FR2-2 to other Rel-17 features 4.3-1: From RAN2 point of view, FR2-2 are assumed to be also applicable to other Rel-17 features, unless otherwise specified (e.g. if the feature is only for FR1). No impact to the specification expected for cases where we don't specify otherwise. - Companies can bring up cases (e.g. for some WIs where FR2-2 has not been discussed at all) where differentiation is needed by contributions to May meeting.

In this LS, RAN2 also mentions in POS WI, there has been no discussion on the support of positioning (especially related to RAT-dependent positioning) in FR2-2 and thus RAN2 thinks it is unclear whether SRS for positioning/DL-PRS can be used with the new 480/960 kHz SCS in FR2-2 or not.

Then, RAN2 asks RAN1 and RAN4 the following question in the LS [1]:

Question: Can SRS for positioning/DL-PRS with 480/960 kHz SCS be supported in FR2-2 in R17?

In our point of view, first of all, since only SCS 15, 30, 60 and 120kHz are applicable to PRS, no PRS for 480/960 kHz SCS has been defined in Rel-17. Secondly, the positioning in FR2-2 has not been discussed in RAN1. Positioning on FR2-2 should be a new feature, which may have some additional specification impact, including the impact on the definition of PRS resource and positioning physical layer procedures. It should not simply conclude that PRS/SRS with 480/960 kHz SCS can be directly supported in FR2-2 in R17 without first studying how to support it. The related research on support of positioning in FR2-2 can be carried out in Rel-19. For Rel-17, Therefore, we prefer that SRS for positioning/DL-PRS with 480/960 kHz SCS cannot be supported in FR2-2 in R17.

Proposal 1: Provide the following response to RAN2: SRS for positioning/DL-PRS with 480/960 kHz SCS cannot be supported in FR2-2 in R17.

Conclusion

In this contribution, we provide our views on the issue on support of positioning in FR2-2 in RAN2 LS R1-2208325(R2-2208810). Our proposal is given as follows:

Proposal 1: Provide the following response to RAN2: SRS for positioning/DL-PRS with 480/960 kHz SCS cannot be supported in FR2-2 in R17.

References

  - R2-2208810, “LS on support of positioning in FR2-2”, RAN2, 3GPP TSG RAN WG2 Meeting #119-e.