3GPP TSG RAN WG1 #110bis-e	   R1-2208928

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XXXX | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on multicast DCI format to enable/disable HARQ-ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-0X-XX |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RAN1 agreed that the “enabling/disabling HARQ-ACK feedback indication” is included in DCI format 4_2 scrambled by G-RNTI and G-CS-RNTI based on RRC signalling configuration. However, in TS 38.213(v 17.3.0), the UE determines HARQ-ACK enabling/disabling based on an indication by the ‘multicast DCI format’, rather than ‘DCI format 4_2’. - The current specification doesn’t define the behavior of UE for HARQ-ACK information of PDSCH scheduled by DCI format 4_1 when ‘dci-enabler’ is configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modifying ‘multicast DCI’ to ‘DCI format 4_2’. - Adding description for UE behavior for PDSCH scheduled by DCI format 4_1 when ‘dci-enabler’ is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of multicast DCI format to enable/disable HARQ-ACK can not align with the RAN1 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

18	Multicast Broadcast Services

<Unchanged part is omitted>

A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the DCI format 4_2 associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212], and provides the HARQ-ACK information for PDSCH receptions scheduled by the DCI format 4_1 associated with G-RNTI or the G-CS-RNTI.

<Unchanged part is omitted>