3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208945

# e-Meeting, October 10th – 19th, 2022

Source:	CATT

Title:	On unified TCI framework extension for multi-TRP operation

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

Introduction

In RAN #94 e-meeting, the enhancement for extending Rel-17 unified TCI framework for multi-TRP use cases was agreed [1]. In this contribution, our views on extending Rel-17 unified TCI to multi-TRP operation as well as multi-panel UL transmission are analyzed, including power control related issues.

Extension for indication of multiple DL/UL TCI states

In RAN1 #110 meeting, the following agreements on unified TCI for multi-TRP has been achieved [2]:

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings:

- Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s)

  - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group

  - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions

- Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

Switching between multi-TRP and single TRP operation is not precluded

Agreement

On unified TCI framework extension, at least for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation

- FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP

- Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1

- Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17

Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI

Agreement

On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings:

- Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

- Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2

- Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

  - FFS: Details of CORESET group(s)

FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings:

- Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group

- Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group

- Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

- Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

In this section, CJT-based PDSCH transmission, TCI state association, beam indication signaling and TRP-specific BFR enhancement will be discussed.

Applicable scenarios

In the last meeting, it was agreed to support applying unified TCI framework to CJT-based PDSCH reception(s). According to the agreement of CJT, up to 4 TRP/TRP-groups would be supported in Rel-18. Generally, 4 joint/DL TCI states should be indicated to support 4 TRP/TRP-groups. This means 4 TRS could be used, where each TRS estimates the delay spread of one TRP/TRP-group. On the other hand, coherent TRS transmission has been discussed for CJT, where the number of TCI state is less than that of TRP/TRP-groups. From our perspective, whether coherent TRS transmission could be applied depends on the distribution between TRP/TRP-groups and UE. For example, if TRP/TRP-groups are located at the two sides of UE (like Rel-17 HST-SFN), 2 joint/DL TCI states may be indicated, where two TRS are transmitted to estimate the delay spread of the two directions. Considering different deployment of TRP/TRP-groups, coherent TRS transmission should be allowed. Therefore, for CJT in Rel-18, up to 4 joint/DL TCI states could be supported.

Proposal 1: For CJT-based PDSCH transmission, up to 4 joint/DL TCI states could be indicated.

In Rel-17, either joint TCI mode or separate DL/UL TCI mode is configured to UE by RRC. Considering mTRP as shown in Figure 1, there are two TRP-to-UE links. As channel characteristics are independent for each link, it is possible that one link (e.g. link-0) may suffer from MPE and the other one does not. Then, hybrid configuration of joint TCI and separated DL/UL TCI seems necessary to be supported in Rel-18. As TCI state mode could only be configured statically by RRC, TCI state mode is preferred to be configured per link instead of per UE. For example, link-0 would be configured with separate DL/UL TCI and link-1 would be configured with joint TCI by RRC. The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP may include both joint TCI state and separate DL/UL TCI state(s).

Figure 1: TCI state mode configuration for multi-TRP

Proposal 2: In Rel-18, TCI state mode (i.e. joint TCI or separate DL/UL TCI) could be configured per link for multi-TRP.

TCI state association

Within the unified TCI framework, when single or multiple TCI states are indicated, UE has to correctly associate each TCI state to corresponding channel resource(s) or transmission repetition(s). Therefore, Rel-18 has to specify the TCI state association with resource and/or time domain repetition. Such association could be defined for each channel independently.

TCI state update for S-DCI based multi-TRP

In Rel-17 multi-TRP, TCI state ID(s) is directly configured for PDCCH/PUCCH. In addition, in Rel-15/16, TCI state ID and SpatialRelationInfo are configured for CSI-RS and SRS, respectively. However, such configuration is not applicable to Rel-18 unified TCI, where association with the indicated TCI states would be configured instead for these channels or reference signals. To define this association, TCI state indicators (the order index of the indicated TCI states, e.g. TCI_ind_0 denotes the first TCI state, TCI_ind_1 denotes the second TCI state,…), which refer to the indicated TCI states, could be introduced in Rel-18.

Proposal 3: To define the association, TCI state indicators referring to the indicated TCI states could be introduced in Rel-18.

For Rel-17 multi-TRP PDCCH, PDCCH could be transmitted by two associated search spaces with each search space or its associated CORESET transmitting from one individual TRP. For Rel-17 HST-SFN, each CORESET is associated with two TCI states for SFN transmission. For PDCCH in Rel-18, in the case of S-DCI, each CORESET could be associated with one or two joint/DL TCI states. According to the agreement of the last meeting, four alternatives have been proposed for the association, where RRC, MAC-CE or a fixed rule could be used. In our opinion, using RRC to change association for CORESET is sufficient considering flexibility. Different to Rel-16, RRC configuration aims to indicate association, not to update TCI state of CORESET. Namely, in Rel-16, the applied TCI state on CORESET could only be updated by MAC-CE. However for unified TCI, the actual applied TCI state on CORESET depends on beam indication MAC-CE/DCI, not RRC. It is even flexible than that of Rel-16, because the TCI state of CORESET could be dynamically changed by DCI. Since TCI state is configured per CORESET in Rel-15/16, we prefer to use RRC configuration to inform association between the indicated TCI state(s) and a CORESET (Alt1-1). For PDCCH repetition, only one of the indicated TCI state is associated with the CORESET. If two TCI states are both associated, SFN transmission would be used.

Proposal 4: To support multi-TRP PDCCH transmission, use RRC configuration to inform association between the indicated TCI state(s) and a CORESET (Alt1-1).

For Rel-16 S-DCI based multi-TRP, PDSCH could be transmitted from two TRPs simultaneously (SDM-1a/FDM-2a/FDM-2b) or be transmitted from two TRPs according to a time domain repetition pattern (TDM3/TDM4). For PDSCH in Rel-18, in the case of SDM-1a/FDM-2a/FDM-2b, there is no need to define the association since both TCI states have to be used. For TDM-3/TDM-4, the association between TCI state and each time domain repetition has been specified in Rel-16, which could be reused in Rel-18.

Proposal 5: For SDM/FDM-based multi-TRP PDSCH transmission, there is no need to define TCI state association. For TDM-based multi-TRP PDSCH transmssion, the association between TCI state and each time domain repetition defined in Rel-16 could be reused.

For Rel-17 PUCCH, each resource is configured with one or two SpatialRelationInfo. In the case of two, time domain repetition transmission is used and the PUCCH resource would be transmitted to two TRPs. Similarly, for PUCCH in Rel-18, each PUCCH resource could be associated with one or two indicated TCI states. The association is implemented by TCI state indicators. Similar as PDCCH, under unified TCI framework, TCI state of each PUCCH resource could be dynamically changed by DCI, which is more flexible than that of Rel-17. Therefore, RRC is sufficient to be used to configure the association for each PUCCH resource.

Proposal 6: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource (Alt1-1).

For PUSCH multi-TRP transmission in Rel-17, the TX beam of each time domain repetition is determined by the SpatialRelationInfo of a SRS indicated by an SRI. Following the same mechanism, one extension scheme is to associate a joint/UL TCI state with a SRS resource set. But according to Rel-17 unified TCI, SRS resources within an SRS resource set could share the indicated TCI state. If SRS does not share the indicated TCI state, the association scheme would be problematic. To avoid such issue, another candidate scheme is to associate a joint/UL TCI state directly with each repetition. The association could be determined per DCI according to the codepoint of the SRS resource set(s) field of the DCI. The SRS resource set(s) field in DCI format 0_1/0_2 would be re-defined as the TCI state association. For example, “codepoint = 01” means single TRP mode and PUSCH is associated with TCI_ind_1. “codepoint = 10” means multi-TRP mode and PUSCH repetition with TCI_ind_0 and TCI_ind_1 order. In addition, for Type 1 CG PUSCH, similar as PDCCH, TCI state association is defined for each Type 1 CG PUSCH configuration. Considering Type 2 CG PUSCH, as it has to be scheduled by DCI, the same scheme as that of PUSCH (per DCI association) is used.

Proposal 7: To support Rel-17 multi-TRP PUSCH transmission, use an indicator field in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 (Alt-1).

TCI state update for M-DCI based multi-TRP

For M-DCI based multi-TRP, CORESETPoolIndex is configured for each CORESET. For each TRP (represented by a CORESETPoolIndex), an independent DCI (corresponding to the CORESETPoolIndex) would be used for beam indication. In this way, the TCI state and CORESET association has already be supported through CORESETPoolIndex in the current spec. In addition, DL and UL transmission is also implemented independently per TRP. Consequently, with unified TCI in Rel-18, each indicated TCI state (or TCI state pair) would be used for UE-dedicated reception on all CORESETs with a same CORESETPoolIndex, and their scheduled UE-dedicated PDSCH and PUSCH. For PUCCH, each PUCCH resource could be associated with one CORESETPoolIndex. Therefore, DCI associated with one of CORESETPoolIndex values would indicate the TCI state(s) corresponding to the same CORESETPoolIndex value.

In the last meeting, cross-TRP beam indication has also been proposed for M-DCI based multi-TRP. Namely, DCI associated with one of CORESETPoolIndex values could indicate the TCI state(s) corresponding to different or both CORESETPoolIndex values. Although M-DCI based multi-TRP is mainly applied to non-ideal backhaul, it is also applicable for ideal backhaul scenario. For the latter case, cross-TRP beam indication is beneficial to increase the opportunity for TCI state update. In our opinion, cross-TRP beam indication could be further studied for M-DCI based multi-TRP in Rel-18. CORESETPoolIndex may be employed in DCI format 1_1/1_2 to indicate the TCI state(s) of the corresponding TRP.

Proposal 8: For M-DCI based multi-TRP, DCI associated with one of CORESETPoolIndex values would indicate the TCI state(s) corresponding to the same CORESETPoolIndex value.

Proposal 9: For M-DCI based multi-TRP, cross-TRP beam indication could be further studied in Rel-18.

TCI state update for reference signals

In Rel-17 unified TCI framework, reference signals (e.g. CSI-RS, SRS) could follow the indicated TCI state(s) or not depending on the configuration. If not, Rel-15/16 TCI state update signaling/configuration design(s) are reused. The same rule should be extended to Rel-18. If RS is configured to share the indicated TCI state(s), the association of the TCI state(s) to the RS has to be designed.

Same as Rel-17, P/SP CSI-RS can never share the indicated TCI states in Rel-18, and the legacy RRC/MAC-CE signalling mechanism is always used for TCI state update. For aperiodic CSI-RS, the association could be achieved by its triggering DCI. Namely, aperiodic CSI-RS could follow the TCI state of the CORESET carrying the triggering DCI. This mechanism is also applied to aperiodic SRS. There may be the case that the triggering DCI is transmitted by SFN, where two TCI states are associated with the CORESET. Then one of the TCI states should be specified for aperiodic CSI-RS/SRS. If P/SP SRS is configured to follow the indicated TCI state(s), each SRS resource could be associated with one indicated TCI state. The association is implemented by TCI state indicators.

Proposal 10: For aperiodic CSI-RS/SRS, if configured to follow the indicated TCI state, they could follow the TCI state of the CORESET carrying the triggering DCI. If the triggering DCI is transmitted by SFN, one of the TCI states should be specified.

Single TRP and multi-TRP switching

In Rel-16, single TRP and multi-TRP PDSCH transmission are dynamically switched according to the number of TCI states indicated in the scheduling DCI format 1_1/1_2. Similarly, for Rel-17 PUSCH, dynamic switching is also supported according to the “SRS resource set” field in DCI format 0_1/0_2. Although common TCI states are indicated for all the channels in Rel-18, the same transmission flexibility as Rel-16/17 should be maintained for different channels. Namely, each channel (PDCCH/PDSCH/PUCCH/PUSCH) could determine its single TRP or multi-TRP transmission separately. For PDSCH/PUSCH, similar as current spec, dynamic switching is achieved by its scheduling DCI. For PDCCH, switching between single TRP and multi-TRP depends on gNB implementation. If PDCCH is transmitted on associated search spaces, multi-TRP is used. If a CORESET is associated with two TCI states, PDCCH would be transmitted in SFN. Otherwise, single TRP is used. For PUCCH, the switching could either be done using DCI or similar as that of PDCCH.

Proposal 11: For flexibility, each channel could determine its single TRP or multi-TRP transmission separately.

In Rel-15/16 S-DCI based multi-TRP, switching between single TRP and multi-TRP PDSCH transmission is determined by the number of TCI states in the scheduling DCI. Namely, when the TCI codepoint indicated in DCI format 1_1/1_2 with DL assignment is associated with a single TCI state, the scheduled PDSCH is assumed to be transmitted by single TRP. When two TCI states are indicated, the scheduled PDSCHs are to be received based on both TCI states. These indicated TCI states would be applied to the scheduled PDSCH transmission. In Rel-17, beam application time (BAT) has been defined for unified TCI. In case of DCI indication, to guarantee the robustness of beam indication, BAT is defined as the first slot that is at least Y symbols after the last symbol of the acknowledgement of the joint or separate DL/UL beam indication. Such BAT definition would impact PDSCH dynamic switching in Rel-18. Taking Figure 2 as an example, the TCI codepoint indicated in the former DCI is associated with two TCI states. These two TCI states are applied after Y symbols after the ACK/NACK. For the latter DCI, the TCI codepoint is associated with only one TCI state. It intends for single TRP transmission. The number of activated TCI states during PDSCH transmission is not be aligned with the intended single TRP/multi-TRP transmission scheduled by DCI format 1_1/1_2. In this way, switching between single TRP and multi-TRP could not be done immediately after DCI indication.

Observation 1: Switching between single TRP and multi-TRP could not be done immediately after DCI indication due to BAT.

Figure 2：Issue on single TRP and multi-TRP dynamic switching for PDSCH

To deal with the problem, we should try to decouple beam indication and DL assignment. In our opinion, a new DCI field could be introduced in DCI format 1_1/1_2 with DL assignment, which is used to indicate single TRP or multi-TRP transmission for the scheduled PDSCH. In this way, the indicated TCI state(s) only aims for beam indication, and the specified BAT mechanism in Rel-17 is reused. On the other hand, the indicated single TRP or multi-TRP transmission in this new DCI field would be applied to the schedule PDSCH immediately, which is independent of the BAT. It is possible that the applied number of TCI states and the indicated single TRP/multi-TRP are not aligned. For example, in the dot circle of Figure 2, two TCI states are applied, but DCI indicates single TRP transmission. Therefore, only one of the two TCI states is used for PDSCH transmission. The selection of TCI state could also been indicated by the new DCI field, where 2bits are used for both single TRP/multi-TRP indication and TCI state selection. Another example is only one TCI state is applied, but DCI indicates multi-TRP transmission. This could be regarded as an error case, which is not allowed to be configured by gNB. Therefore, with this scheme, gNB would flexibly switch between single TRP and multi-TRP for the scheduled PDSCH.

Proposal 12: In Rel-18, introduce new DCI field(s) in DCI format 1_1/1_2 with DL assignment to indicate switching between single TRP and multi-TRP.

Beam indication signaling

In Rel-16/17 multi-TRP, one TCI state pool is used for both S-DCI and M-DCI based multi-TRP. Considering TCI state update for S-DCI, TCI states are activated by one MAC-CE and one DCI with a single TCI field indicates the corresponding TCI state(s). For M-DCI, for each CORESETPoolIndex, one MAC-CE and one DCI with a single TCI field are used. In the last meeting, TRP-specific TCI state pool has been proposed by some companies, which is equivalent to introduce a TRP-ID. In our opinion, such TRP-ID is not necessary for S-DCI, and has already been defined by CORESETPoolIndex for M-DCI. There is no obvious benefit to support TRP-specific TCI state pool in Rel-18. In addition, to allow more degrees of freedom for multi-TRP beam indication, for S-DCI, more than one TCI fields in the DCI has also been discussed, where each TCI field corresponds to one TRP. This scheme would increase the DCI overhead and impact the legacy MAC-CE command, since one codepoint of MAC-CE could only be related to the TCI state(s) of a single TRP. For beam indication flexibility, we prefer a more straightway to increase the number of bits in the TCI field. Consequently, Rel-16/17 beam indication mechanism for multi-TRP should be reused in Rel-18.

Proposal 13: For S-DCI based multi-TRP, a single TCI state pool, one activation MAC-CE and one DCI with one TCI field are used for beam indicaiton.

Proposal 14: For M-DCI based multi-TRP, a single TCI state pool is used and for each CORESETPoolIndex, one activation MAC-CE and one DCI with one TCI field are used for beam indicaiton.

Enhancement to TRP-specific BFR

TRP-specific BFR has been supported in Rel-17, which should be extended to unified TCI. In Rel-17, both explicit BFD RS and implicit BFD RS have been specified. For explicit BFD RS, two BFD RS sets (each corresponding to one TRP) could be configured for either S-DCI or M-DCI. However, for implicit BFD RS, two BFD RS sets are only applicable to M-DCI. Since for M-DCI, each CORESETPoolIndex configured by RRC corresponds to one TRP. Then, the RS within the TCI states for those CORESETs associated with the same CORESETPoolIndex could be used as BFD RS for one TRP. But there is no such TRP-ID like parameter for S-DCI. Therefore it is difficult for UE to discriminate two BFD RS sets. Considering unified TCI framework, as mentioned above, TCI state indicators may be introduced for S-DCI. In this way, implicit TRP-specific BFD RS is also feasible for S-DCI in Rel-18.

Proposal 15: Implicit TRP-specific BFD RS is also supported for S-DCI in Rel-18.

Simultaneous multi-panel UL transmission

Association of panel and TCI state

In Rel-17, UE-initiated UL panel activation and selection is supported for fast uplink panel selection, via UE reporting a list of UE capability values along with the pairs of SSBRI/CRI and L1 RSRP/L1 SINR in a beam reporting instant. Each UE capability value comprises the maximum supported number of SRS antenna ports, and the maximum supported numbers of SRS antenna ports of any two UE capability values are different.

For STxMP UL transmission in Rel-18, up to 2 TRPs and up to 2 panels is considered. When two TCI states are selected, the two TCI states may correspond to same or different panels. In terms of RF functionality, in most cases, a UE panel is not able to generate two analog beams simultaneously. Therefore indicating two beams corresponding to the same panel for simultaneous UL transmission should be avoided. That means that for the configuration of unified TCI for simultaneous UL transmission, gNB has to know the correspondence between panels and TCI states.

Proposal 16: For STxMP UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

There are various approaches to identify the correspondence between panels and TCI states.

One approach is to use UE capability value to indicate the association between SSB resources/CSI-RS resources and panels as that for Rel-17 fast panel selection. Note that in Rel-17, two different UE capability values must correspond to two different maximum supported numbers of SRS antenna ports. That is, if a UE receives two CSI-RS resources with two panels, respectively, and the two panels correspond to same maximum supported number of SRS antenna ports, same UE capability value would be reported. Then gNB does not know whether the two CSI-RS resources correspond to same panel or different panels. The two UL panels deployed by UE may correspond to same or different maximum number of SRS antenna ports, and same or different maximum number of MIMO layers. Therefore allowing UE capability values for two panels correspond to same or different maximum number of SRS antenna ports, and same or different maximum supported number of MIMO layers should be supported.

Proposal 17: For STxMP UL transmission, consider enhancing UE capability values to represent panels. The following two enhancements can be considered:

- Two different UE capability values correspond to same or different maximum supported number of SRS antenna ports are supported;

- Using UE capability value to indicate maximum supported number of MIMO layers.

Another approach is using group-based beam reporting to faciliate gNB to know which beams can be used for UL transmission simultaneously. UE can report two groups of beams, with each group corresponds to a panel. Then any two SSB resources/CSI-RS resources in different beam groups can be used for UL transmission simultaneously.

Proposal 18: For STxMP UL transmission, group-based beam reporting is considered, with each group corresponds to a panel.

Association between TCI states and DMRS ports for PUSCH

In RAN1 #110 meeting, there was a working assumption on supporting SDM scheme for S-DCI based STxMP PUSCH transmission. Since the SDM scheme provides diversity gain and multiplexing gain compared to sing panel transmission, it is our view that the working assumption should be confirmed. Besides, SFN scheme also can be supported for S-DCI based STxMP PUSCH transmission. For SFN scheme, all of the layers/DMRS ports of one PUSCH are transmitted from two different UE panels simultaneously, which can improve reliability and coverage for uplink, especially when blockage happens for one TRP.

Proposal 19： The following two schemes are supported for S-DCI based STxMP PUSCH transmission:

- SDM scheme;

- SFN scheme.

In Rel-16 and Rel-17, dynamic switching of single TRP and multi-TRP is supported for all DL multi-TRP transmission schemes and UL multi-TRP transmission schemes scheduled by S-DCI. It is natural to also support dynamic switching of single-panel and multi-panel transmission for Rel-18 simultaneous multi-panel UL transmission scheduled by S-DCI.

Proposal 20: For S-DCI based STxMP PUSCH transmission, dynamic switching of single-panel and multi-panel transmission is supported.

Both Rel-17 multi-TRP PUSCH transmission schemes and Rel-18 S-DCI based STxMP PUSCH transmission schemes map the PUSCH transmission to two beams to two TRPs, respectively.  Same indication scheme for dynamic switching of single TRP and multi-TRP under unified TCI framwork can be used for Rel-17 multi-TRP PUSCH transmission schemes and Rel-18 S-DCI based STxMP PUSCH transmission schemes.

Proposal 21: On dynamic switching of single-panel and multi-panel transmission for Rel-18 S-DCI based STxMP PUSCH transmission under unified TCI framework, same scheme as that for dynamic switching of single-TRP and multi-TRP for Rel-17 multi-TRP PUSCH transmission shall be used.

M-DCI based STxMP PUSCH transmission also can be considered in Rel-18, with two PUSCHs scheduled by two PDCCHs associated with different CORESETPoolIndex values. The two PUSCHs can be fully/partially/non-overlapped in time and frequency domain. For such transmission scheme, one important issue is that which PUSCHs can be transmitted simultaneously. One possible solution is that PUSCHs scheduled by PDCCH resources associated with different CORESETPoolIndex can be transmitted simultaneously.

Proposal 22: For M-DCI based STxMP PUSCH transmission, transmitting PUSCHs scheduled by PDCCH resources associated with different CORESETPoolIndex simultaneously is considered.

For PUSCH transmission with configured grant, same TCI indicating schemes as that for Rel-17 multi-TRP PUSCH transmission with configured grant can be considered.

Proposal 23: On TCI state determination for STxMP PUSCH transmission with Type 1/Type 2 configured grant under unified TCI framework, same scheme as that for Rel-17 multi-TRP UL repetition with Type 1/Type 2 configured grant can be used.

Unified TCI for PUCCH

Different to PUSCH, only one layer is supported for a PUCCH resource. Therefore the enhancement on simultaneous multi-panel PUCCH transmission is mainly for reliability/coverage improvement. For S-DCI based STxMP PUCCH transmission, SFN scheme, FDM-A scheme and FDM-B scheme can be supported, with details of schemes are provided in our companion contribution [3].

Proposal 24: For STxMP PUCCH transmission, SFN scheme, FDM-A scheme and FDM-B scheme are supported.

In Rel-17, a PUCCH resource can be associated with one or two spatialRelationInfos, and the spatialRelationInfo(s) are activated by MAC-CE. When one spatialRelationInfo is activated, the PUCCH resource corresponds to single-TRP transmission, and when two spatialRelationInfos are activated, the PUCCH resource corresponds to multiple- TRP transmission. For Rel-18 M-TRP scenarios under unified TCI framework, we prefer to use similar TCI state indication scheme for control channels, i.e., similar as that for PUCCH transmissions with Rel-17 TDM repetition schemes, RRC configuration is used to configure the TCI state(s) associated with PUCCH resources.

Proposal 25: For S-DCI based STxMP PUCCH transmisison, RRC configuration is used to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource.

For a UE capable of transmitting with multi-panel simultaneously, simultaneous PUCCH transmission with two PUCCH resources can also be considered. The two PUCCHs can be fully/partially/non-overlapped in time and frequency domain. And the two PUCCH resources may be scheduled by S-DCI or M-DCI. If the two PUCCH resources are scheduled by M-DCI, the two PDCCHs carrying the scheduling DCIs can be associated with different CORESETPoolIndex values. Similar as PUSCH transmission scheduled by M-DCI, transmitting PUCCHs triggered by PDCCH resources associated with different CORESETPoolIndex can be considered.

Proposal 26: For M-DCI based STxMP PUCCH transmission, transmitting PUCCHs triggered by PDCCH resources associated with different CORESETPoolIndex simultaneously is considered.

Power control enhancement for Multi-TRP transmission

In RAN1 #110 e-meeting, one issue discussed is how to determine the UL PC parameter setting(s) if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting for PUCCH/PUSCH M-TRP transmission, there are three alternatives:

- Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP

  - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting

- Alt2: No change from Rel-17 unified TCI framework, i.e., the UE should apply the one single default UL PC parameter setting configured in the corresponding UL BWP regardless the UL PC parameter setting is absent from one or both of indicated joint/UL TCI states

- Alt3: A joint/UL TCI state for PUCCH/PUSCH transmission is always associated with a UL PC parameter setting for PUCCH/PUSCH

In RAN1 #105 e-meeting, it was agreed that for Rel-17 unified TCI framework, if UL PC parameter is not associated with the indicated TCI state, the setting of UL PC parameters(P0, alpha, closed loop index) per channel/signal per BWP is independent of the UL or joint TCI states for each of the PUSCH and PUCCH. To be specific, the default UL PC parameter settings are configured in BWP-UplinkDedicated. This conclusion can be straightly extended to Rel-18 unified TCI multi-TRP transmission.

BWP-UplinkDedicated ::=             SEQUENCE {

pucch-Config                        SetupRelease { PUCCH-Config }

pusch-Config                        SetupRelease { PUSCH-Config }

......

ul-powerControl-r17                Uplink-powerControlId-r17                                         OPTIONAL,  -- Cond NoTCI-PC

}

Per TRP default power control parameter has been supported in Rel-17 M-TRP PUSCH/PUCCH repetition due to the different pathloss between two UE-to-TRP links. It should also be supported for M-TRP with Rel-18 unified TCI. Applying the one single UL PC parameter configured in the corresponding UL BWP, i.e. Alt2 will degrade the performance of another UE-to-TRP link.

Default power control parameter is supported in Rel-17 unified TCI, it should also be supported for Rel-18 unified TCI, so Alt.3. is not preferred.

Proposal 27: Support Alt1, i.e. two default UL PC parameter settings configured in BWP-UplinkDedicated if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting for PUCCH/PUSCH M-TRP transmission.

In RAN1 #109 e-meeting, the following agreement on power control enhancement for Multi-TRP transmission were achieved [4]:

Agreement

On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion.

- FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported

- FFS: UL PC enhancement for CB and non-CB SRS in above case

FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

In RAN1 #109 e-meeting, it was agreed that the power control parameter settings for NR Rel-17 TDM PUSCH/PUCCH repetition can be determined by the indicated joint or UL TCI state associated to the corresponding PUSCH/PUCCH transmission occasion. One issue to be further discussed is how to extend it to NR Rel-18 MTRP scheme(s) with STxMP, if supported. For STxMP transmission, since the transmission power of each panel might be different and the maximum transmission power for each panel is limited, panel-specific power control should be considered. Suppose UE has two panels, for once transmission, when two TCI states are indicated to UE to indicate two PUSCH transmission to the same/different TRPs, UE can determine two individual Tx PC parameter settings accordingly and use it to perform open-loop power control of each transmission.

Proposal 28: Support enhancement of power control for UL single DCI in NR Rel-18 uplink STxMP, if supported.

Proposal 29: For Rel-18 STxMP, two sets of individual power control parameters are determined according to the DLorjoint TCI state/UL TCI state indicated by gNB for multi-TRP transmission, use the determined power control parameter setting to perform open loop power control.

Proposal 30: Reuse two TPC fields in Rel-17 S-DCI based MTRP PUCCH/PUSCH repetition to achieve TRP/panel-specific closed-loop power control.

For NR Rel-18 STxMP transmission, considering each panel has its own maximum transmission power, and the total transmission power of a UE is also limited. Power control mechanisms need to be considered for these limitations. Suppose UE has two panels, the maximum transmission power for each panel is Pcmax1, Pcmax2. The maximum total transmission power of the UE is Pcmax which is configured per CC [5]. Basically, power control mechanism of the following cases should be studied:

Case1: P1 + P2 <= Pcmax

Case2: P1 + P2  > Pcmax

where P1 and P2 represents the transmission power of panel 1 and panel 2 determined from equation (1) provided by [5]  respectively, with Pcmax replaced by Pcmax1 and Pcmax2 in the calculation.

(1)

For Case 2, how to determine actual transmission power of panel 1 and panel 2 on the basis of P1 and P2 to meet the requirement of total transmission power limitation need to be further studied.

Figure 3: Power restriction cases in NR Rel-18 uplink simultaneous transmission

For Case1, only power control of per-panel power limitation need to be considered, whether the current framework(i.e. equation (1) to determine transmission power of PUSCH transmission) can be extended to the case of per-panel power limitation needs to be further studied. In detail, the definition of Pcmax in equation (1) is a cell- specific parameter, it is determined from SIB parameter and UE power class limitation. If per-panel power limitation is considered, Pcmax,i for panel i shall be used. Pcmax,i  can be obtained by UE capability reporting and RRC configuration.

For Case 2, power control schemes considering both per-panel power limitation and total transmission power limitation need to be considered. Suppose UE has two panels, the maximum transmission power for panel 1 and panel is Pcmax1 = 23dBm, Pcmax2 = 23dBm. The maximum total transmission power of the UE Pcmax = 20dBm. For once transmission, UE determines its real PUSCH transmission power P1 = 21dBm and P2 = 22dBm individually according to equation (1) with Pcmax replaced by Pcmax1 and Pcmax2 respectively. In addition, considering Pcmax is limited by 20dBm and P1 + P2  > Pcmax, power control mechanism, e.g. power back-off mechanism can be used to reduce the transmission power of both TRPs or one of the TRPs, detailed power back-off mechanisms can be further studied.

Proposal 31: In order to support TRP-specific UL power control in sDCI-mTRP operation. The case of Per-panel power limitation and/or total power limitation should be considered. For the case of total power limitation, some power back-off mechanisms can be further studied. For the case of per-panel power limitation, the formula in TS38.213 to determine PUSCH transmission power needs to be further refined.

Conclusions

In this contribution, we discuss extending Rel-17 unified TCI to multi-TRP use cases in Rel-16/Rel-17 as well as CJT and multi-panel UL transmission. The power control related issues are also analyzed. We have the following observation and proposals:

Observation 1: Switching between single TRP and multi-TRP could not be done immediately after DCI indication due to BAT.

Proposal 1: For CJT-based PDSCH transmission, up to 4 joint/DL TCI states could be indicated.

Proposal 2: In Rel-18, TCI state mode (i.e. joint TCI or separate DL/UL TCI) could be configured per link for multi-TRP.

Proposal 3: To define the association, TCI state indicators referring to the indicated TCI states could be introduced in Rel-18.

Proposal 4: To support multi-TRP PDCCH transmission, use RRC configuration to inform association between the indicated TCI state(s) and a CORESET (Alt1-1).

Proposal 5: For SDM/FDM-based multi-TRP PDSCH transmission, there is no need to define TCI state association. For TDM-based multi-TRP PDSCH transmssion, the association between TCI state and each time domain repetition defined in Rel-16 could be reused.

Proposal 6: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource (Alt1-1).

Proposal 7: To support Rel-17 multi-TRP PUSCH transmission, use an indicator field in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 (Alt-1).

Proposal 8: For M-DCI based multi-TRP, DCI associated with one of CORESETPoolIndex values would indicate the TCI state(s) corresponding to the same CORESETPoolIndex value.

Proposal 9: For M-DCI based multi-TRP, cross-TRP beam indication could be further studied in Rel-18.

Proposal 10: For aperiodic CSI-RS/SRS, if configured to follow the indicated TCI state, they could follow the TCI state of the CORESET carrying the triggering DCI. If the triggering DCI is transmitted by SFN, one of the TCI states should be specified.

Proposal 11: For flexibility, each channel could determine its single TRP or multi-TRP transmission separately.

Proposal 12: In Rel-18, introduce new DCI field(s) in DCI format 1_1/1_2 with DL assignment to indicate switching between single TRP and multi-TRP.

Proposal 13: For S-DCI based multi-TRP, a single TCI state pool, one activation MAC-CE and one DCI with one TCI field are used for beam indicaiton.

Proposal 14: For M-DCI based multi-TRP, a single TCI state pool is used and for each CORESETPoolIndex, one activation MAC-CE and one DCI with one TCI field are used for beam indicaiton.

Proposal 15: Implicit TRP-specific BFD RS is also supported for S-DCI in Rel-18.

Proposal 16: For STxMP UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

Proposal 17: For STxMP UL transmission, consider enhancing UE capability values to represent panels. The following two enhancements can be considered:

- Two different UE capability values correspond to same or different maximum supported number of SRS antenna ports are supported;

- Using UE capability value to indicate maximum supported number of MIMO layers.

Proposal 18: For STxMP UL transmission, group-based beam reporting is considered, with each group corresponds to a panel.

Proposal 19： The following two schemes are supported for S-DCI based STxMP PUSCH transmission:

- SDM scheme;

- SFN scheme.

Proposal 20: For S-DCI based STxMP PUSCH transmission, dynamic switching of single-panel and multi-panel transmission is supported.

Proposal 21: On dynamic switching of single-panel and multi-panel transmission for Rel-18 S-DCI based STxMP PUSCH transmission under unified TCI framework, same scheme as that for dynamic switching of single-TRP and multi-TRP for Rel-17 multi-TRP PUSCH transmission shall be used.

Proposal 22: For M-DCI based STxMP PUSCH transmission, transmitting PUSCHs scheduled by PDCCH resources associated with different CORESETPoolIndex simultaneously is considered.

Proposal 23: On TCI state determination for STxMP PUSCH transmission with Type 1/Type 2 configured grant under unified TCI framework, same scheme as that for Rel-17 multi-TRP UL repetition with Type 1/Type 2 configured grant can be used.

Proposal 24: For STxMP PUCCH transmission, SFN scheme, FDM-A scheme and FDM-B scheme are supported.

Proposal 25: For S-DCI based STxMP PUCCH transmisison, RRC configuration is used to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource.

Proposal 26: For M-DCI based STxMP PUCCH transmission, transmitting PUCCHs triggered by PDCCH resources associated with different CORESETPoolIndex simultaneously is considered.

Proposal 27: Support Alt1, i.e. two default UL PC parameter settings configured in BWP-UplinkDedicated if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting for PUCCH/PUSCH M-TRP transmission.

Proposal 28: Support enhancement of power control for UL single DCI in NR Rel-18 uplink STxMP, if supported.

Proposal 29: For Rel-18 STxMP, two sets of individual power control parameters are determined according to the DLorjoint TCI state/UL TCI state indicated by gNB for multi-TRP transmission, use the determined power control parameter setting to perform open loop power control.

Proposal 30: Reuse two TPC fields in Rel-17 S-DCI based MTRP PUCCH/PUSCH repetition to achieve TRP/panel-specific closed-loop power control.

Proposal 31: In order to support TRP-specific UL power control in sDCI-mTRP operation. The case of Per-panel power limitation and/or total power limitation should be considered. For the case of total power limitation, some power back-off mechanisms can be further studied. For the case of per-panel power limitation, the formula in TS38.213 to determine PUSCH transmission power needs to be further refined.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94-e, December 6th – 17th, 2021.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110.

- R1-2206380, Discussion on UL precoding indication for multi-panel transmission, CATT, RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109.

- 3GPP RAN1, TS 38.213 V17.0.