3GPP TSG RAN WG1 #110bis-e			R1-2208953

e-Meeting, October 10th – 19th, 2022

Source:	CATT

Title:	NR enhancement for XR capacity improvement

Agenda Item:	9.10.2

Document for:	Discussion and Decision

Introduction

In the RAN1 Rel-17 “Study on XR Evaluation for NR” [3], the eXtended Reality (XR) and Cloud Gaming (CG) have summarized the study results of the use cases and services, which are considered as one of the important features for NR in Rel-18 and beyond. Many of the XR and CG use cases are characterized by quasi-periodic traffic (with possible jitter from network transport) with high data rate in DL (i.e., video steam) combined with the frequent UL (i.e., pose/control update) and/or UL video stream. Both DL and UL traffic are also characterized by relatively strict packet delay budget (PDB). According to the capacity evaluation results in TR38.838 [2], there are performance gaps and limitation of current NR systems in supporting XR applications. Potential and necessary enhancements of NR system would be needed to better support XR applications.

According to the scope of Study Item “Study on XR Enhancements for NR” [3], the XR-specific capacity improvements mainly focus on XR-specific capacity improvements, which include the enhancements of SPS and CG as well as dynamic scheduling enhancements. As the agreements in RAN1#110, the potential schemes for the capacity enhancement of XR were listed. Both the capacity performance and power saving gain of the proposed enhancement schemes need to be coupled to be presented in the evaluation results for their performance gain.

Agreement RAN1 to make decision on the following in RAN1#110bis-e - Support single DCI scheduling multi-PDSCHs/PUSCHs which is currently supported for FR2-2 to other SCS in FR1/FR2 Conclusion There is no consensus in RAN1 on the benefits of enhancing SPS for the purpose of XR capacity enhancement Agreement When DG is used as the baseline scheme, for the performance evaluation scheduling, after SR is triggered, both BSR and UL data can be transmitted using the UL grant after SR. - Companies are encouraged to provide the size of resources by the first UL grant after SR Agreement Whether/how to enhance BSR to improve capacity performance of XR traffic is within RAN2 scope and is not handled by RAN1. - Note that companies should indicate if and what BSR enhancement is assumed in their RAN1 proposals on CG and DG enhancements. - RAN1 can evaluate BSR enhancement to improve capacity performance Agreement Deprioritize the study of CQI report for different BLER and/or different XR traffic to improve XR capacity performance. Agreement Deprioritize the study of intra/inter UE prioritization/multiplexing enhancements to improve XR capacity performance. For future meetings: Companies are requested to follow the following agreement and conclusion from RAN1#109-e. Check final FL summary for details. Agreement - Rel-17 evaluation methodology for XR capacity enhancement captured in TR 38.838 is used as the baseline evaluation methodology for XR capacity enhancement of Rel-18 SI on XR enhancements. Conclusion - Companies are encouraged to use the capacity Excel sheet attached with TR 38.838 in RP-213652 for recording the simulation results that are provided in their contributions.

In this contribution, the DL capacity enhancements of dynamic scheduling scheme with the results of their capacity performance and power saving gain are shown in Section 2. The SPS enhancement for improving the DL capacity of XR service and the potential CG enhancements for UL XR-specific traffic are provided in Section 3.1 and Section 3.2, respectively, where the capacity performance and the power saving gain are both presented. The other DG scheduling enhancement schemes and SPS/CG enhancement schemes are analyzed in Section 2.3 and Section 3.3, respectively. The evaluation assumptions of the schemes in our contribution including the traffic model and simulation parameters as a whole section are listed in the Annex.

Dynamic scheduling enhancement

According to the study of Rel-17 “Study on XR Evaluation for NR” [3] in RAN1, the video stream in XR application, such as VR/AR and CG, are periodically generated based on the source codec frame generation rate, e.g. 60 FPS. For DL traffic in VR/CG, the packet inter-arrival time at the gNB would be varied caused by the delay jitter of network transport. The packet arrival of the UL XR traffic would be periodic at the UE MAC/physical layer for scheduled UL transmission without delay jitter. The pose/control information as the classical UL XR traffic is usually generated with the 4ms periodicity with a small packet size, e.g. 100 bytes.

The dynamic scheduling can dynamically allocate resource for the transmission at the instance of XR packets to meet the stringent QoS requirements of low latency and large packet size. However, the price of dynamic scheduling for XR is the UE power consumption, where in UE always on the standby for the XR packet arrival without any power consumption reduction in reducing PDCCH monitoring. The C-DRX is usually configured in NR for the power saving purpose. When the C-DRX is configured for the XR-specific transmission, the miss-alignment between the XR-packet arrival time and the On-duration of C-DRX would result in excessive delay of XR packet scheduling and leads to the capacity degradation, which had been evaluated and captured in TS 38.838[2]. When the XR packet arrives at the DRX-OFF, it would wait for the next on-duration, which results in the additional transmission delay. If the longer on-duration is configured, it would lead to the larger power consumption for UE. Thus, the C-DRX mechanism should be enhanced for the XR-specific traffic.

In this section, the DG enhancement schemes including the XR-dedicated PDCCH monitoring window and the XR-specific playoutDelayForMediaStartup for the gNB scheduling awareness are proposed for capacity enhancement based on the C-DRX mechanism.

The XR-dedicated PDCCH monitoring window for the C-DRX enhancement

The C-DRX had been introduced as the UE power saving feature since the LTE system and continuing in NR system. C-DRX cycle is usually configured as an integrate value for all different services in all serving cells by MAC protocol. The periodicity of XR application generated from source codec, such as VR/AR and CG, is not an integer value, which is 16.67ms for 60 FPS frame generation rate. The packet inter-arrival time at the gNB would vary due to the delay jitter from the network transport XR packets. If the packet arrives at the period during the DRX-OFF, the packet would be buffered and transmitted at next DRX-ON

According to the NR MAC specification [4] as follows, the MAC would control the PDCCH monitoring for all traffic when the C-DRX is configured for the UE.

TS 38.321, Section 5.7 [4]: “The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213.”

If DRX is configured in RRC_CONNECTED state for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation. If the drx-Cycle or drx-startOffset is adapted only for XR-specific traffic, it would affect the other traffic and have impacted to the UE power saving for other services. Thus, the enhancement of the dynamic scheduling based on the DRX configuration should not impact the PDCCH monitoring and resource allocation of other traffic, such as robust traffic arrival of eMBB, used by the UE in the same time in achieving UE power saving.

Proposal 1：The dynamic scheduling enhancement for XR traffic should not impact the PDCCH monitoring and resource allocation of other traffic, such as robust traffic arrival of eMBB used by the UE in the same time in achieving UE power saving.

The XR-dedicated PDCCH monitoring window for DL

The XR-dedicated PDCCH Monitoring Window (XR-PMW) is proposed for the XR-specific traffic transmission without impacting the PDCCH monitoring and resource allocation of other traffic with robust traffic arrival. As shown in Figure 1, UE is preconfigured with a fixed PDCCH monitoring cycle based on the periodicity of XR data arrival and expected delay jitter the monitoring window disassociated with the PDCCH monitoring controlled by C-DRX. The C-DRX functionality would not affect the PDCCH monitoring and data scheduling of the XR-specific traffic. The periodicity of XR-specific monitoring can be configured based on the XR frame generation rate, e.g. 16.67ms for 60FPS. The length of the PDCCH monitoring window can be configured to dynamically adapt to the network delay jitter. The UE is required to monitor PDCCH at the preconfigured monitoring window in each monitoring cycle regardless it is DRX ON or OFF.  The PDCCH monitor window could be configured in coordination among XR UEs to allow the system capacity gain through MU-MIMO scheduling. Furthermore, the power saving techniques of PDCCH skipping or go-to-sleep indication could be used along with the XR-specific monitoring window to further reduce the PDCCH monitoring with details in [5]. In order to get the link adaptation gain for the XR transmission during the monitoring window, UE could feedback the CSI or beam measurement at the beginning of PDCCH monitoring cycle. Since XR-specific PDCCH monitoring window would be customized for PDCCH monitoring outside and within Active Time of C-DRX, UE needs to be configured with a XR specific RNTI, e.g., XR-RNTI.

Figure 1: The XR-specific PDCCH monitoring window (XR-PMW) scheme

We evaluated the capacity and power saving performance of XR-PMW compared with the different baseline schemes. The evaluated performance of the capacity gain and power saving gain compared with the Baseline scheme 1 and Baseline scheme 2 are provided in Table 1 and Table 2, in which the baseline schemes are evaluated based on link adaptation with SU-MIMO scheduling, while the XR-PMW scheme and its power saving schemes are evaluated based on MU-MIMO scheduling since the gNB could configure UEs in the same XR-PMW monitoring window to achieve MU-MIMO gain comparing to no coordination of UE groups in the baseline scheme with dynamic scheduling. The two baseline schemes based on the agreed baseline of the SU-MIMO in RAN1#110 are listed as following:

- Baseline scheme 1(SU-MIMO): UE would be always on to monitor the PDCCH for dynamic scheduling XR-specific traffic, in which there is no power saving scheme applied. It is the upper bound of XR capacity performance for existing dynamic scheduling scheme but worst UE power saving performance.

- Baseline scheme 2(SU-MIMO): UE is configured the C-DRX cycle with the parameters (DRX cycle, on-duration, inactive timer) = (16, 12, 4), in which the capacity of this C-DRX configuration is near to that of UE always on.

From Table 1 and Table 2, the XR-PMW based on MU-MIMO transmission without and with the different power saving schemes are considered for improve both capacity performance and power saving performance. The parameters (XR-PMW cycle, window length) = (16, 12) for XR-PMW are adopted.

- XR-PMW scheme 1(MU MIMO): the XR-PMW only that UE is configured with fixed PDCCH monitoring cycle and monitoring window disassociated with DRX. The fixed PDCCH monitoring cycle could be customized for the periodic traffic generation of XR service. The window size of PDCCH monitoring at each cycle could be dynamically adapted to the delay variation of packet arrival caused by network jitter. UE monitors only XR scheduled PDCCH during a preconfigured PDCCH monitoring window.

- XR-PMW scheme 2(MU MIMO): The XR-PMW with go-to-sleep that UE monitors the PDCCH for the dynamic scheduling in the XR-PMW. When traffic transmission is completed, UE would be indicated to go to sleep until the next XR-PMW.

- XR-PMW scheme 3(MU MIMO): The XR-PMW with PDCCH skipping and go-to-sleep that UE would be indicated to continuously skip the PDCCH monitoring at the beginning of the XR-PMW. UE skips monitoring the PDCCH at the Monitoring Occasion (MO) until the traffic packet arrives. When traffic transmission is completed, UE would be indicated to go to sleep until the next XR-PMW.

Furthermore, the capacity and power saving performance of the C-DRX with the different configurations are also provided for the comparison, which include the configurations (DRX cycle, on-duration, inactive timer) = (16, 12, 4), (16, 8, 4) and (6, 4, 2). The configurations (16, 12, 4) and (16, 8, 4) of C-DRX are considered, which are with the similar DRX cycle as the frame refresh rate, e.g. 60FPS, of video stream in XR traffic and the different on-duration timers for adapting the jitter delay. And the configuration (6, 4, 2) with shorter DRX cycle is also taken into account for the power saving purpose.

Table 1: The evaluation result comparison between the XR-PMW with MU-MIMO
and the Baseline scheme 1 (UE always on with SU-MIMO)

| Evaluation Schemes | Capacity |  |  | Power saving |
| --- | --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity enhancement Gain | Power Saving Gain (PSG) |
| DG scheduling and UE always on (SU-MIMO) (Baseline scheme 1) | 5.8 | 96.7% | 0.0% | - |
| XR-PMW scheme 1: XR-PMW (16,12) | 10.8 | 90.00% | 86.2% | 7.62% |
| XR-PMW scheme 2: XR-PMW (16,12) with go-to-sleep | 10.8 | 90.00% | 86.2% | 20.90% |
| XR-PMW scheme 3: XR-PMW (16,12) with PDCCH skipping and go-to-sleep | 10.7 | 89.17% | 84.5% | 26.68% |

From the evaluation results in Table 1, the capacity gain and power saving gain are both observed for the XR-PMW scheme based on the MU-MIMO compared with Baseline scheme 1: the DG scheduling based on SU-MIMO with UE always on. The XR-PMW scheme with MU-MIMO scheduling can obtain 84.5%~86.2% capacity gain compared with the Baseline scheme 1, the obvious capacity gain mainly comes from the higher resource utilization rate of the MU-MIMO scheduling due to the coordination of monitoring window among UEs in the XR-PMW scheme. Furthermore, when the power saving schemes, such as PDCCH skipping and go-to-sleep, are adopted in the XR-PMW, 20.9%~26.68% power saving gain can be obtained for reducing the unnecessary PDCCH monitoring during the monitoring window of the XR-PMW. UE is indicated to sleep state once it completes the reception of the PDSCH of XR packet and reports all ACKs for PDSCH. The detailed power saving schemes can be find in [5].

Thus, we have the following observation:

Observation 1: The XR-PMW based on MU-MIMO with coordinated monitoring window among co-scheduled UEs could obtain the 84.5%~86.2% capacity gain than that of the UE always on for DG scheduling based on SU-MIMO and obtain 20.9%~26.68% PSG compared to that of the UE always on for DG scheduling.

Furthermore, taken the C-DRX with configuration (16, 12, 4) based on SU-MIMO as the baseline scheme 2, the evaluation results including the capacity performance and power saving gain for the PMW wo/w the certain power saving schemes are provided in Table 2, in which the XR-PMW is evaluated based on the MU-MIMO transmission.

Table 2: The performance comparison between baseline scheme 2 (SU-MIMO) (C-DRX (16, 12, 4)) and 
XR-PMW with PDCCH skipping and go-to-sleep using MU-MIMO

| Evaluation Schemes | Capacity |  |  | Power saving |
| --- | --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity enhancement Gain | Power Saving Gain (PSG) |
| DG scheduling with C-DRX(16,12,4) (SU-MIMO) (Baseline scheme 2) | 3.7 | 92.5% | - | - |
| XR-PMW scheme 1: XR-PMW with (16,12) | 10.8 | 90.00% | 192% | 3.81% |
| XR-PMW scheme 2: XR-PMW with go-to-sleep | 10.8 | 90.00% | 192% | 17.64% |
| XR-PMW scheme 3: XR-PMW with PDCCH skipping and go-to-sleep | 10.7 | 89.17% | 189% | 23.66% |

We observe that the evaluation results of XR-PMW schemes in Table 2 are with the obvious capacity performance gain than the Baseline scheme 2: DG scheduling with C-DRX (16, 12, 4) based on the SU-MIMO transmission. The capacity gain of XR-PMW schemes can be achieved up to 189%~192%, while the 17.63%~23.66% power saving gain can be obtained. For the Baseline scheme 2, the UEs would spend more time to wait for the transmission the XR packets when the SU-MIMO scheduling is used, which mainly impact the capacity performance. When MU-MIMO is adopted during the monitoring window of XR-PMW, the efficient resource utilization rate would reduce the waiting delay to improve the XR capacity performance. Otherwise, in order to reduce the unnecessary PDCCH monitoring, UE could perform PDCCH skipping with certain duration until the packet arrival at the gNB in each cycle. UE is indicated to sleep state once it completes the reception of the PDSCH of XR packet and reports all ACKs for PDSCH. The details of power saving schemes, i.e. PDCCH skipping enhancement and go-to-sleep scheme, can be found in the [5].

Thus, we have the following observation:

Observation 2: The XR-PMW based on MU-MIMO with coordinate monitoring window of co-scheduled UEs with go-to-sleep and XR-PMW based on MU-MIMO with PDCCH skipping and go-to-sleep can obtain 189%~192% capacity gain and 17.64% and 23.66% PSG than the C-DRX with (16, 12, 4) using SU-MIMO, respectively.

Therefore, the XR-specific PDCCH monitoring window using MU-MIMO could obtain the obvious capacity gain and power saving gain than both the DG scheduling with UE always on and C-DRX (16, 12, 4) using SU-MIMO, but not to impact the power saving of other traffic with robust traffic arrival.

Proposal 2: The XR-dedicated PDCCH Monitoring Window should be considered as the dynamic scheduling enhancement scheme for improving the capacity and power saving performance of XR-specific traffic and not impact the other traffic.

In additional, gNB could collect the measurement results of the average transmission delay and jitter for the XR packets arriving at gNB of each UE. The measurement results could be used for adjusting the monitoring window size of the XR-PMW to achieve the UE power saving and to improve the performance of UE coordination in MU-MIMO scheduling. When the delay measurement result is not fit for the monitoring window size of XR-PMW, gNB would indicate UE to adapt the monitoring window or re-configure the parameters of XR-PMW. For instance, gNB measures the average transmission delay and jitter in a fixed duration e.g., every 200 ms and collects the measurement results via the sliding window method. When the measured delay is lower than the monitoring window size of XR-PMW, gNB could indicate UE to adjust the monitoring window or re-configure the parameters of XR-PMW in order to reduce unnecessary PDCCH monitoring. If the measured delay jitter is excessive over the monitoring window size, gNB needs to reconfigure the UE with longer PDCCH monitoring window size to prevent excessive delay in scheduling of late arrival packet and degrade the system capacity.  Thus, the measurement results of the average transmission delay and jitter could be used in XR-PMW scheme to improve the power saving performance.

Proposal 3: The measurement of the average transmission delay and jitter should be standardized to optimize the configure PDCCH monitoring window size of XR-PMW scheme and to improve system capacity and the power saving performance.

The XR-dedicated PDCCH monitoring window for UL

The XR-dedicated PDCCH monitoring window can also be used for XR packet transmission in UL, especially for UL scene and video with less jitter. The cycle of the XR-dedicated PDCCH monitoring window can be set to be the same as the periodicity of XR application generated from source codec, e.g. 16.67ms for 60 FPS. For the UL transmission based on dynamic grant, SR is sent at the configured SR resource at each XR packet arrival  to UE. The SR resource is configured periodically for UE to request the dynamic resource allocation of UL transmission based on robust traffic arrival. Since SR resources are the overhead of UL radio resource, the periodicity of SR resource would not be too short. The additional delay caused by SR transmitted at the configured SR resource would be introduced for UL dynamic scheduling, which can degrade the system capacity of UL XR transmission. During the XR-dedicated PDCCH monitoring window, UE is configured to monitor PDCCH within the window at each cycle with periodicity based on the XR packet generation. The BSR scheme could be enhanced to have XR-specific triggering mechanism of BSR report to minimize the scheduling delay. The XR-specific BSR report triggering mechanism is that the BSR report will be trigger at the 1st PUSCH of each XR packet at each cycle to inform gNB the remaining packet size to be transmitted after the 1st PUSCH transmission of the XR traffic. When gNB receive the BSR, gNB would schedule UE the UL PUSCH transmission until the completion of XR packet delivery in this cycle.

In this section, we also evaluate the capacity performance of the XR-PMW for UL and take the UL dynamic scheduling as the baseline scheme. The evaluation results are provided in Table 3. The SR delay is considered in the UL dynamic scheduling scheme based on 5ms periodicity of SR resource. For the UL XR-PMW scheme, the BSR enhancement that UE sends the BSR report at the 1st PUSCH of each XR packet periodicity is taken into account. According to the evaluation results in Table 3, we can find that the UL XR-PMW scheme with the BSR enhancement can obtain 33.3% capacity gain for reducing the SR/BSR delay.

Table 3: The evaluation result comparison between XR-PMW for UL 
and UL dynamic scheduling with SR/BSR delay (5ms)

|  | Capacity |  |
| --- | --- | --- |
|  | #satisfied UEs per cell / % of satisfied UEs | Capacity gain |
| DG scheduling with 5ms SR/BSR delay and UE always on (baseline) | 5.4 / 90.0% | - |
| XR-PMW (16.67, 8) for UL with the BSR enhancement | 7.2 / 90.0% | 33.3% |

Proposal 4: The BSR enhancement that UE sends the BSR report at the 1st PUSCH of each XR packet periodicity should be considered for the UL dynamic scheduling enhancement scheme for improving the capacity of XR-specific traffic.

The XR-specific playoutDelayForMediaStartup for the gNB scheduling awareness

The XR and CG services have a certain characteristics of the data streams (i.e., video) with data inter-arrival time varied while the services are transport through the NR network. Therefore, additional information on the XR services from higher layers, e.g. the QoS flow association, frame-level QoS, ADU-based QoS, XR specific QoS etc, would be beneficial to facilitate the selection of radio parameters for radio resource allocation. XR application awareness would help gNB to improve the user experience, the resource allocation to increase the NR system capacity in supporting XR services, and reduce the UE power consumption.

The specification of SA4 Rel-17 TS 26.247 [6] emphasizes that the playoutDelayForMediaStartup and appLayerBufferLevel are two important QoE metrics for operator to evaluate the quality of its service offer, where the playoutDelayForMediaStartup is the waiting time that the user experiences for media start-up. The other QoE metric appLayerBufferLevel is buffer level, which indicates the playout duration for which media data of all active media components is available starting from the current playout time [13]. A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement, i.e. when appLayerMeasConfig and SRB4 have been configured by the network. The QoE metrics can be included in the RAN visible application layer measurement report in TS38.331 [7], i.e. MeasurementReportAppLayer and appLayerBufferLevel, and reported to gNB.

TS 38.331, Section 5.7.16[7] Upon initiating the procedure, the UE shall: 1> for each measConfigAppLayerId: <omit the text> 2> if RAN visible application layer measurement report has been received from upper layers: 3> for each appLayerBufferLevel value in the received RAN visible application layer measurement report: 4> set the appLayerBufferLevel values in the appLayerBufferLevelList to the buffer level values received from the upper layer in the order with the first appLayerBufferLevel value set to the newest received buffer level value, the second appLayerBufferLevel value set to the second newest received buffer level value, and so on until all the buffer level values received from the upper layer have been assigned or the maximum number of values have been set according to appLayerBufferLevel, if configured; 3> set the playoutDelayForMediaStartup to the received value in the RAN visible application layer measurement report, if any; 3> for each PDU session ID value indicated in the received RAN visible application layer measurement report, if any: 4> set the PDU-SessionID field in the pdu-SessionIdList to the indicated PDU session ID value; <omit the text>

Based on the above specification, the XR-specific playoutDelayForMediaStartup and appLayerBufferLevel from the UE to the gNB for capacity enhancement is shown in Figure 2. The XR-specific playoutDelayForMediaStartup and appLayerBufferLevel in the application layer at UE are used to defend the delay jitter and out of sequence XR packet arrival. The XR-specific playoutDelayForMediaStartup and appLayerBufferLevel at UE would ensure the in-sequence and time interval alignment of XR video frames when it plays out to the user. The proposed scheme is for UE to feedback the XR-application type (XR-application awareness) including XR-specific playoutDelayForMediaStartup and appLayerBufferLevel at application layer to the gNB. The feedback of XR-specific playoutDelayForMediaStartup and appLayerBufferLevel from UE would give the gNB scheduler additional delay budget in scheduling the XR data transmission, which would increase the link adaptation gain.

Different UE might have different length of playout delay for XR starting up or buffer level based on the UE implementation. gNB could priority the UE scheduling based on the extended delay budge from the reported playout delay size and buffer level, in which a group UEs in the same priority queue are with the same length of playout delay. gNB could also collect the measurement results of the average transmission delay and jitter for the XR packets arriving at gNB of a UE. The measurement results along with UE extended delay budget of XR service provided by playout delay size would be incorporated in the proportional fair algorithm for XR packet scheduling. The average transmission delay and jitter would be smoothed out by the delay awareness in gNB scheduler to schedule XR packet within the extended delay budget. The gNB could also feed back the measurement results of the average transmission delay and jitter to the UE in helping UE setting the adapt the playout delay or buffer level in order to efficiently handle the XR packets, such as to warn the UE of the proper playout  buffer level.

Proposal 5: The gNB could also feed back the measurement results of the average transmission delay and jitter to UE to improve the XR capacity performance.

In the evaluation, the size of XR-specific playoutDelayForMediaStartup and appLayerBufferLevel is fed back from the UE to the gNB. The gNB takes the length of the XR-specific playoutDelayForMediaStartup and appLayerBufferLevel as the additional PDB, which could give gNB more time to schedule UE within the delay budget requirement of the XR service with link adaptation gain. gNB knowing the size of playout delay and buffer level can  schedule UE with packet within the extended delay budget to get better link adaptation gain.

Figure 2: The gNB scheduling awareness XR-specific playoutDelayForMediaStartup from UE

In this section, we evaluate the DL capacity performance and power saving gain of the XR-specific playoutDelayForMediaStartup scheme compared with the C-DRX with (16, 12, 4) for the VR/AR traffic based on MU-MIMO and SU-MIMO, which are provided in Table 4 and Table 5, respectively. For the XR-specific playoutDelayForMediaStartup scheme, the 2 frames to 4 frames playout delay are both reported to the gNB scheduler for relaxing the packet delay requirements and the C-DRX with (16, 8, 4) is configured for this scheme. In order to reduce the UE power consumption, some power saving schemes, such as PDCCH skipping and Go-to-sleep, are also considered in the XR-specific playoutDelayForMediaStartup scheme. The details of power saving schemes, i.e. PDCCH skipping enhancement and go-to-sleep scheme, can be found in the [5]. The evaluation results of the XR-specific playoutDelayForMediaStartup scheme with the power saving schemes are provided in Table 6 compared with the baseline scheme that DG scheduling with UE always on. The simple descriptions of the power saving schemes are listed as follows.

- XR-specific playoutDelayForMediaStartup scheme with go-to-sleep: UE is configured with C-DRX (16, 8, 4). UE transforms to sleep state once the all PDSCH of frame is received successfully and all ACKs for PDSCH are reported. The 3 frames playout delay is used for relaxing the packet delay requirements in the scheduler.

- XR-specific playoutDelayForMediaStartup scheme with PDCCH skipping and go-to-sleep: UE is configured with C-DRX (16, 8, 4). UE performs PDCCH skipping when On Duration starts until UE starts receiving PDSCH. UE transforms to sleep state once the all PDSCH of frame is received successfully and all ACKs for PDSCH are reported. The 3 frames playout delay is obtained by the gNB scheduler for relaxing the packet delay requirements.

Table 4: The evaluation results of the gNB scheduling awareness schemes 
with XR-specific playoutDelayForMediaStartup 
where the baseline is DG scheduling with C-DRX (16,12,4) for the MU-MIMO

| Evaluation Schemes | Capacity |  |  |
| --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity performance gain |
| DG scheduling with C-DRX(16,12,4) (Baseline) | 10.9 | 90.97% | - |
| XR-specific playoutDelayForMediaStartup scheme (2 frames playout delay) | 16 | 95% | 46.8% |
| XR-specific playoutDelayForMediaStartup scheme (3 frames playout delay) | 20 | 92% | 83.5% |
| XR-specific playoutDelayForMediaStartup scheme (4 frames playout delay) | 20 | 91% | 83.5% |

Table 5: The evaluation results of the gNB scheduling awareness schemes 
with XR-specific playoutDelayForMediaStartup 
where the baseline is DG scheduling with UE always on for the SU-MIMO

| Evaluation Schemes | Capacity |  |  |
| --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity performance gain |
| DG scheduling with UE always on (Baseline) | 5.8 | 96.7% | - |
| XR-specific playoutDelayForMediaStartup scheme (3 frames playout delay) | 7.3 | 91.3% | 25.86% |
| XR-specific playoutDelayForMediaStartup scheme (4 frames playout delay) | 11.8 | 98.3% | 103.45% |
| XR-specific playoutDelayForMediaStartup scheme (mixed playout delay {3, 4}) | 11.3 | 94.4% | 94.83% |

From the evaluation results in Table 4 and Table 5, the XR-specific playoutDelayForMediaStartup scheme could improve the link adaptation gain by gNB scheduler for the XR-specific traffic transmission based on the additional information of playoutDelayForMediaStartup, which can result in the capacity performance enhancement. The XR-specific playoutDelayForMediaStartup using MU-MIMO can obtain the obvious capacity performance gain from 46.8% to 83.5% for the 2 frames to 4 frames playout delay compared with the C-DRX with (16, 12, 4). The XR-specific playoutDelayForMediaStartup using SU-MIMO can obtain the obvious capacity performance gain from 25.86% to 103.45% for the 3 frames and 4 frames playout delay compared with the C-DRX with (16, 12, 4), respectively. When the UE feeds back the different playoutDelayForMediaStartup, e.g. 3 frames or 4 frames, it can obtain 94.83% capacity gain. Due to the XR-specific playoutDelayForMediaStartup reported from UE, the additional delay budget is added in the scheduling the XR packets, which can effectively increase the link adaptation gain and reduce the impact by delay jitter and transmission delay. For the MU-MIMO, when up to the 4 frames playout delay is reported, the maximum capacity gain is obtained. For the SU-MIMO, the longer playout delay is reported, the more capacity gain is obtained. Thus, the gNB scheduling with awareness of the UE feedback of QoE metrics, such as Rel-17 playoutDelayForMediaStartup, should be considered to improve the capacity performance of XR-specific traffic, such as the gNB scheduling awareness UE playout buffer.

Thus, we have the following observation:

Observation 3: The XR-specific playoutDelayForMediaStartup scheme could obtain the obvious capacity performance gain from 46.8% to 83.7% for the 2 frames to 4 frames playout delay based on MU-MIMO, while 25.86% to 103.45% capacity gain for 3 frames to 4 frames playout delay based on SU-MIMO, compared with DG scheduling with C-DRX with (16, 12, 4).

Table 6: The evaluation results of the gNB scheduling awareness schemes 
with XR-specific playoutDelayForMediaStartup
where the baseline is DG scheduling with UE always on (MU-MIMO)

| Evaluation Schemes | Capacity |  |  | Power saving |
| --- | --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity performance gain | PSG |
| DG scheduling with UE always on (Baseline) | 11.5 | 95.83% | - | - |
| XR-specific playoutDelayForMediaStartup scheme with go-to-sleep (3 frames playout delay) | 20 | 94.17% | 67% | 26.43%* |
| XR-specific playoutDelayForMediaStartup scheme with PDCCH skipping and go-to-sleep (3 frames playout delay) | 20 | 93.3% | 67% | 28.51%* |
| *Note: The power saving gain is based on the same capacity as that of the baseline scheme (DG scheduling with UE always on). |  |  |  |  |

We also observe from the evaluation results in Table 6 that the XR-specific playoutDelayForMediaStartup scheme with 3 frames playout delay report could obtain 67% capacity performance gain than the baseline scheme that DG scheduling with UE always on, while the 26.43%~ 28.51% power saving gain is obtained for the power saving schemes applied. When the power saving schemes such as go-to-sleep and PDCCH skipping are applied, the additional delay might be introduced. Since the XR-specific playoutDelayForMediaStartup can allow UE to easily receive packet with more relaxed packet delay requirements, which can provide additional packet delay budget, the UE has more time to sleep if the rest of delay budget is enough. Thus, the capacity would not be impacted by the power saving schemes and obtain the obvious power saving gain than the DG scheduling scheme with UE always on.

Thus, we have the following observation:

Observation 4: The XR-specific playoutDelayForMediaStartup scheme under the 3 frames playout delay can obtain 67% capacity performance gain and the 26.43%~ 28.51% power saving gain compared with UE always on.

Based on the description and the evaluation results, we have the following proposal.

Proposal 6: The XR-specific playoutDelayForMediaStartup scheme should be considered as the dynamic scheduling enhancement scheme for its significant capacity gain and obvious power saving gain.

The analysis of the other DG enhancement schemes

For the DG enhancement schemes, the single DCI scheduling multi-PDSCHs/PUSCHs in FR2-2 was proposed to be reused for XR capacity enhancement, which can reduce the DCI reception and the transmission latency [8][9][10]. For the NR in the frequency above 52.6GHz, the slot is too short to decode the PDCCH per slot based on the UE PDCCH monitoring capability. Thus, the PDCCH monitoring is applied in a slot group and the single DCI schedules multiple PDSCHs/PUSCHs in FR2-2. However, when UE uses the dynamic scheduling per slot in FR1/FR2 which is the most flexible scheduling manner, the PDCCH monitoring per slot would not be the challenge for UE capability. It is unnecessary for UEs to use the single DCI scheduling multi-PDSCHs/PUSCHs in FR1/FR2. Otherwise, the drawbacks of the single DCI scheduling multi-PDSCHs/PUSCHs in FR1/FR2 would not be ignored, such as the miss-detection of the scheduling DCI and the non-flexible resource assignment, which would increase the transmission delay and UE power consumption.

Therefore, the single DCI scheduling multi-PDSCHs/PUSCHs in FR1/FR2 would not be considered in XR capacity enhancement.

Proposal 7: The single DCI scheduling multi-PDSCHs/PUSCHs in FR1/FR2 would not be considered in XR capacity enhancement, since the PDCCH monitoring per slot is not be the challenge for UE capability and the drawbacks of this scheme is not ignored.

The SPS&CG enhancement

The dynamic PDSCH allocation introduced in a SPS period

As the XR-specific traffic has the characteristic of periodical transmission, the radio resource would be allocated to the XR-specific traffic. The Rel-15/16 C-DRX usually can be used for UE power saving for robust data arrival with delay insensitive data. However, the Rel-15/16 C-DRX would not be appropriate for the XR-specific packets’ arrival time with the delay jitter through the network transport. The SPS PDSCH is to have semi-persistent resource allocation and activated by the L1 signalling, which UE could receive the PDSCH without any scheduling grant at the SPS occasion regardless DRX ON or OFF as shown in Figure 3.

Thus, the SPS PDSCH and DRX could be configured together for UE with XR service and other application with robust traffic arrival.

Figure 3: SPS with DRX cycle configuration

However, the XR-specific traffic has variation in packet size from output of XR source codec, e.g. H.265/H.266, and inter-arrival time caused by network delay jitter. The SPS PDSCH with fixed resource allocation might not be sufficient to adapt to the arrival of XR traffic, which needs to be enhanced to adapt the resource allocation for the XR traffic. When XR traffic arrives at gNB, XR packet would be segmented to fit into the semi-static resource allocated resource, whose packet size is larger than the pre-allocated SPS resource. The enhancement of SPS is to introduce the PDCCH monitoring window to allocate additional resource over the reserved SPS resource for the transmission of remaining XR packets when XR packet is larger than the pre-allocated SPS resource, as shown in Figure 4.

For the SPS enhancement, there are two alternatives for introducing the additional dynamic grant for resource allocation in addition to the reserved SPS resource for PDSCH for the XR packet transmission as follows,

- Alt 1: The additional resource allocation is indicated by L1 signalling and/or MAC layer information for dynamically allocating the resource of remaining XR-specific traffic if any.

- Alt 2: The pre-configured monitoring window of dynamic scheduling is bundled with the SPS PDSCH occasion for improving the resource of remaining XR-specific traffic.

For Alt 1, the need of the additional resource allocation can be indicated by L1 signalling and/or MAC layer information. The additional XR resource indicated by L1 signalling in the Alt 1 enhancement is that the UE is required to monitor the PDCCH after SPS slot for dynamic resource allocation after SPS initial transmission. UE is pre-configured with the monitoring window following the SPS PDSCH occasion. If there is no XR packets in gNB buffer, UE can receive a scheduling DCI with a dummy grant as the indication of completion XR transmission and termination of the PDCCH monitoring in the monitoring window. The completion of XR data transmission and terminating the PDCCH monitoring can be carried in the MAC layer information at the last PDSCH at each SPS PDSCH occasion.

For the Alt 2, the pre-configured monitoring window of PDCCH monitoring is bundled with the SPS PDSCH occasion for the XR packet reception at every XR packet cycle. If the XR packet arrives at the SPS occasion, UE can receive the XR packet similar to that of Alt 1. gNB could indicate UE to terminate the PDCCH monitoring in the monitoring window after the XR packet transmission completed. If the XR packet does not arrive at the time of the reserved SPS resource for PDSCH, UE can continuously monitor the PDCCH with dynamic grant in the pre-configured monitoring window until the XR packet arrives. Alt 2 allows the gNB to reserve small size of radio resource for SPS and the subsequent dynamic grant for the remaining XR packet. This would minimize the overhead of SPS reserve resource when the XR packet arrives after the reserved resource.

The two alternatives of SPS enhancement can both support the variation of XR packets and network delay jitter based on the SPS configuration. When multiple SPS configurations are configured with the multiple occasions for the jitter, the Alt1 of SPS enhancement can further indicate the additional resource by the dynamic grant to adapt the packet size variation. When the reserved SPS resource is limited, the Alt2 of SPS enhancement could be chosen for the dynamically scheduling the XR packets with the varied arrival time and packet size.

Additionally, the PDCCH skipping indication or go-to-sleep indication could also be used for the power saving purpose, whose details can be found in [5]. These SPS enhancements would allow the configuration of the DRX not to adapt the XR traffic inter-arrival time. In order to support PDCCH skipping for XR periodic service with delay sensitive and other robust traffic arrival with delay insensitive data, the PDCCH skipping interval could be enhanced to be configured separately between XR and other data to optimize the PDCCH reduction for increasing UE power saving gain.

Thus, we have the following observation:

Observation 5: The two alternatives of SPS enhancement as following can both improve the capacity of XR traffic based on the SPS configuration:

- Alt 1: The additional resource allocation is indicated by L1 signalling and/or MAC layer information for improving the capacity of XR-specific traffic.

- Alt 2: The pre-configured monitoring window is bundled with the SPS PDSCH occasion for improving the capacity of XR-specific traffic.

However, from the flexibility perspective, the Alt 2 of SPS enhancement that the pre-configured PDCCH monitoring window bundled with the SPS reserved resource occasion has the advantage of minimizing the overhead and flexible in improving the resources for the variable XR-packet sizes and interarrival time caused by network delay jitter.

Figure 4: SPS with additional resource allocation

We evaluate the capacity performance and the power saving gain of the SPS enhancement that the PDCCH monitoring window introduced after the SPS occasion. Two baseline schemes including the single SPS configuration and DG scheduling with UE always on are both considered in the Table 7 and Table 8, respectively. Furthermore, the PDCCH skipping and Go-to-sleep are both considered in the SPS enhancement for the power saving purpose. For the baseline that the single SPS configuration, the 16ms periodicity is considered and the detailed evaluation assumptions are provided in Annex 2.

Table 7: The evaluation results of the SPS enhancement
where the baseline is DG scheduling with C-DRX(16,12,4)

| Evaluation Schemes | Capacity |  |  | Power saving |
| --- | --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity performance gain | PSG |
| DG scheduling with C-DRX(16,12,4) (Baseline) | 10.9 | 90.97% | - | - |
| Multiple SPS configurations | 0 | 0% | - | - |
| SPS enhancement | 10.8 | 90% | -0.9% | 9.9% |
| SPS enhancement with Go-To-Sleep | 10.6 | 88.3% | -2.7% | 38.1% |

We can see from the evaluation results, when the multiple SPS configurations are activated and with the distributed offsets, that it hardly provides UE the XR-specific service, since the fixed PDSCH reception occasions and pre-configured resource allocation does not adapt the varied XR-specific packets arrival time and packet size. The additional transmission delay is introduced for limited reserving SPS resource, when the XR packets are not completely transmitted at a SPS occasion and it needs more SPS PDSCH to be transmitted. Besides, the capacity loss is also the result of the misalignment delay between the XR packet arrival time and the next SPS occasion(s), when the XR packet does not arrive at the occasion of reserved SPS resource. From Table 7, the SPS enhancement that introducing dynamic PDSCH allocation introduced in a SPS PDSCH periodicity can obtain the capacity performance of 10.8 UEs per cell and 9.9%~38.1% power saving gain compared to that of DG scheduling with C-DRX(16,12,4). The SPS enhancement with dynamic scheduling of remaining XR packets in a XR packet cycle can resolve the problems of the SPS configuration especially for the fixed resource size per SPS periodicity. When the XR packet is transmitted at the SPS occasion and there are still remaining packets in the gNB buffer, gNB can indicate UE to monitor the dynamic scheduling DCI during the pre-configured monitoring window. The remaining packets can be transmitted in the SPS periodicity and don’t need to wait for the next SPS occasion(s). And the monitoring window can be pre-configured based on the jitter of packet arrival time.

Thus, we have the following observation:

Observation 6: The SPS enhancement scheme can obtain the capacity performance of 10.8 UEs per cell and 9.9%~38.1% power saving gain compared to that of DG scheduling with C-DRX(16,12,4), additionally the multiple SPS configurations hardly provide UE the XR-specific service.

Table 8: The evaluation results of the SPS enhancement
where the baseline is DG scheduling with UE always on

| Evaluation Schemes | Capacity |  |  | Power saving |
| --- | --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity performance gain | PSG |
| DG scheduling with UE always on | 11.5 | 95.83% | - | - |
| SPS enhancement | 10.8 | 90% | -6.1% | 12.6% |
| SPS enhancement with Go-To-Sleep | 10.6 | 88.3% | -7.8% | 39.9% |

As the evaluation results listed in Table 8, the capacity of the baseline scheme that DG scheduling with UE always on is 11.5 UEs per cell, which is the upper bound of capacity of the XR traffic. The capacity performance of the SPS enhancement in our contribution is 10.6~10.8 UEs per cell, which is near to that of the baseline, whose capacity performance gap is 6.1%~7.8%. Based on the power saving schemes applied in the monitoring window, the SPS enhancement schemes can obtain the obvious power saving gain, i.e. 12.6%~39.9%, than that of baseline scheme. If the XR packets are completely transmitted during the monitoring window, the UE can be indicated to go to sleep. If the XR packets arrive during the monitoring window, the UE can be indicated to skip the PDCCH monitoring until the packets arrive.

Thus, we have the following observation:

Observation 7: The capacity performance of the SPS enhancement in our contribution is near to that of DG scheduling with UE always on and with the 6.1%~7.8% capacity performance gap, while it can obtain the obvious power saving gain, i.e. 12.6%~39.9%, than that of baseline scheme.

Based on the description and the evaluation results, we give the following proposal.

Proposal 8: The Alt 2 of SPS enhancement that the pre-configured PDCCH monitoring window bundled with the reserved SPS resource for PDSCH would be provide the resource to meet the QoS requirement of XR-specific traffic.

The additional resource requirement in a CG period

Similar to DL XR-specific traffic, the UL XR-specific traffic has the characteristic of periodical data generation and variable packet size. UL configured grant is the candidate scheme for periodic UL XR traffic. The UL configured grant also has a fixed amount of resource allocation at each periodicity. The reserved resource allocation could be based on average packet size or minimum packet size of XR service with consideration of the system resource utilization. The larger the resource is allocated for CG, the less number of CG users is supported. It is clear that a fixed resource allocation could not be used for carrying XR data with variable packet size. It is challenge CG for NR without enhancement to support XR services with the high data rate, low latency and high reliability. The dedicated radio resource configured for CG-PUSCH could not be sufficiently large for high data rate transmission, which might cause excessive latency and performance penalty. If the XR packet size is larger than reserved resource allocation, additional resource allocation for remaining XR traffic needs to be included in the CG enhancement.

For the CG enhancement, the dynamic allocation of additional resource for XR data rate adaptation by dynamic grant could be included the at the CG occasion for the remaining packet data to be transmitted. The optimized configured grant is that the remaining packet data is scheduled by dynamic grant based on the UE reported by the BSR in the MAC subPDU at the CG MAC-PDU. If the BSR is reported by UE with the status “not empty”, UE would monitor the PDCCH in the subsequent slots for dynamic grant after XR packet transmission at the CG resource, in which the dynamic grant is used to schedule the remaining data of XR packet. In order for the same XR packet reception, the packets transmitted in CG and DG could be paired in the XR-specific CG configuration.

Figure 5: CG with additional resource allocation

We evaluate the capacity performance and the power saving gain of the CG enhancement that the additional resource requirement in a CG periodicity in Table 9. The capacity performance of the DG scheduling with UE always on is considered as the baseline. For the single CG configuration with the 16ms periodicity is also evaluated and the detailed evaluation assumptions are provided in Annex 2.

Table 9: The evaluation results of the CG enhancement
where the baseline is DG scheduling with UE always on

| Evaluation Schemes | Capacity |  |  |
| --- | --- | --- | --- |
|  | #satisfied UEs per cell | % of satisfied UEs | Capacity performance gain |
| DG scheduling with UE always on (Baseline) | 6 | 100.0% | - |
| Single CG configuration | 0 | 0.0% | - |
| CG enhancement | 5.9 | 98.3% | -1.7% |

The evaluation results listed in Table 9 show that the capacity of the scheme that DG scheduling with UE always on is 6 UEs per cell, which is as the baseline of UL capacity for XR traffic. When the single CG configuration is activated, it hardly provides UE the XR-specific service, since the pre-configured resource allocation does not adapt the varied XR-specific packet size. Since the UL XR packet arrival from the XR application to MAC/physical layer without the network delay jitter, the XR packet can arrives at the CG occasion periodically. The capacity degradation of CG transmission for XR is mainly due to the fixed reserved resource not adaptable for the XR packet size. In order to avoid the resource conflict for the UEs, the reserved resource would not be as large as possible, which is smaller than the XR packets. When the XR packet transmission is not completely, it would be transmitted at the next CG occasion, which results in the additional delay for the UL packets transmission. The capacity performance of the CG enhancement in our contribution is with the capacity performance gap is 1.7% compared with the baseline that the DG scheduling with UE always on. However, if the XR packets are completely transmitted during the monitoring window, the UE can be indicated to go to sleep. If the XR packets arrive during the monitoring window, the UE can be indicated to skip the PDCCH monitoring until the packets arrive.

Thus, we have the following observation:

Observation 8: The capacity performance of the CG enhancement that additional resource requirement in a CG period is near to that of DG scheduling with UE always on and with the 1.7% capacity performance gap.

Based on the description and the evaluation results, we have the following proposal.

Proposal 9: The enhancement of Configured Grant UL transmission that additional resource requirement in a CG period should be considered as the enhanced scheme to support low latency and large data rate transmission of XR traffic.

The analysis of the other SPS/CG enhancement schemes

For the SPS/CG enhancement schemes, the following schemes were mostly proposed for improving the capacity performance and power saving gain.

- Enhancement scheme 1: Single SPS/CG configuration with multiple occasions[8][12]

- Enhancement scheme 2: The SPS/CG configuration with adaptation parameters[8][11][12]

For the Enhancement scheme 1, i.e. the single SPS/CG configuration with multiple occasions, it was proposed that increasing the occasions in per SPS/CG periodicities to adapt to the variable number and sizes of packets per burst [8] and the supported number of SPS/CG configurations is limited[12]. The existing multiple SPS/CG configurations cannot actually resolve the random jitter problem and it would need to reserve more resource for the larger packet size; otherwise, it can result in the additional delay for less reserved resource. However, the problems of the Enhancement scheme 1 are similar to that of the multiple SPS/CG configurations. gNB cannot predict the XR-specific packet arrival time and the exact packet size at the instance of packet arrival at least for DL XR transmission. The inappropriate resource reservation would result in the resource wasted and resource confliction among the UEs. Furthermore, when the XR-specific packet size is large, the power saving gain of reduced PDCCH detection would be limited compared with that of packet transmission.

Proposal 10: According to the analysis, the Enhancement scheme 1, at least for the single SPS configuration with multiple occasions could not efficient to improve the capacity of XR-specific traffic for not predict the packet size and packet arrival time.

For the Enhancement scheme 2: The SPS/CG configuration with adaptation parameters, it was proposed that the parameters of the SPS/CG configuration were dynamic adapted to avoid the resource wasted or extra delay. However, the packet size and the packet arrival time with network delay jitter of XR packet transport are random and characterized by the truncated Gaussian distribution, which the random variable are difficult to be predicted by gNB. The adaptation adjustment for the parameters of the SPS/CG configuration would only depends on the statistical information but not precise. Although the parameter adaptation is needed, the SPS/CG parameter adjustment has been supported via the DCI indicating the SPS/Type2 CG activation as the existing specification in TS 38.321[4]. The detailed context of the specification is listed as following.

TS 38.321, v17.0.0, Section 5.3.1 DL Assignment reception[4] When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell: 1> if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI, or G-RNTI: <omit the text> 1> else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI or G-CS-RNTI: <omit the text> 2> if the NDI in the received HARQ information is 0: <omit the text> 3> else if PDCCH content indicates SPS activation: 4> store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment; 4> initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in clause 5.8.1; <omit the text> TS 38.321, v17.0.0, Section 5.4.1 UL Grant reception If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion: 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: <omit the text> 1> else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI: 2> if the NDI in the received HARQ information is 1: <omit the text> 2> else if the NDI in the received HARQ information is 0: 3> if PDCCH contents indicate configured grant Type 2 deactivation: 4> trigger configured uplink grant confirmation. 3> else if PDCCH contents indicate configured grant Type 2 activation: 4> trigger configured uplink grant confirmation; 4> store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant; 4> initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in clause 5.8.2; 4> stop the configuredGrantTimer for the corresponding HARQ process, if running; 4> stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

For the SPS PDSCH or configured grant type 2, if the PDCCH with the CRC scrambled by CS-RNTI and the PDCCH content indicates SPS activation or configured grant Type 2 activation, the configured downlink assignment or the configured uplink grant for this Serving Cell would be re-initialized. Thus, the Enhancement scheme 2, i.e. the SPS/CG configuration with adaptation parameters would not benefit for the XR-specific transmission.

Thus, we have the following observation:

Observation 9: The SPS/CG parameter adapted adjustment has been supported via the DCI indicating the /Type2 CG activation as the existing specification in TS 38.321[4].

Based on the above analysis, we give the following proposal.

Proposal 11: According to the analysis, the Enhancement scheme 2, i.e. the SPS/CG configuration with adaptation parameters, could not efficient to improve the capacity of XR-specific traffic for not predict the packet size and packet arrival time and existing specification has supported.

Conclusion

In this contribution, we discuss the capacity enhancement schemes and have the following observations and proposals.

Observation 1: The XR-PMW based on MU-MIMO with coordinated monitoring window among co-scheduled UEs could obtain the 84.5%~86.2% capacity gain than that of the UE always on for DG scheduling based on SU-MIMO and obtain 20.9%~26.68% PSG compared to that of the UE always on for DG scheduling.

Observation 2: The XR-PMW based on MU-MIMO with coordinate monitoring window of co-scheduled UEs with go-to-sleep and XR-PMW based on MU-MIMO with PDCCH skipping and go-to-sleep can obtain 189%~192% capacity gain and 17.64% and 23.66% PSG than the C-DRX with (16, 12, 4) using SU-MIMO, respectively.

Observation 3: The XR-specific playoutDelayForMediaStartup scheme could obtain the obvious capacity performance gain from 46.8% to 83.7% for the 2 frames to 4 frames playout delay based on MU-MIMO, while 25.86% to 103.45% capacity gain for 3 frames to 4 frames playout delay based on SU-MIMO, compared with DG scheduling with C-DRX with (16, 12, 4).

Observation 4: The XR-specific playoutDelayForMediaStartup scheme under the 3 frames playout delay can obtain 67% capacity performance gain and the 26.43%~ 28.51% power saving gain compared with UE always on.

Observation 5: The two alternatives of SPS enhancement as following can both improve the capacity of XR traffic based on the SPS configuration:

- Alt 1: The additional resource allocation is indicated by L1 signalling and/or MAC layer information for improving the capacity of XR-specific traffic.

- Alt 2: The pre-configured monitoring window is bundled with the SPS PDSCH occasion for improving the capacity of XR-specific traffic.

Observation 6: The SPS enhancement scheme can obtain the capacity performance of 10.8 UEs per cell and 9.9%~38.1% power saving gain compared to that of DG scheduling with C-DRX(16,12,4), additionally the multiple SPS configurations hardly provide UE the XR-specific service.

Observation 7: The capacity performance of the SPS enhancement in our contribution is near to that of DG scheduling with UE always on and with the 6.1%~7.8% capacity performance gap, while it can obtain the obvious power saving gain, i.e. 12.6%~39.9%, than that of baseline scheme.

Observation 8: The capacity performance of the CG enhancement that additional resource requirement in a CG period is near to that of DG scheduling with UE always on and with the 1.7% capacity performance gap.

Observation 9: The SPS/CG parameter adapted adjustment has been supported via the DCI indicating the /Type2 CG activation as the existing specification in TS 38.321[4].

Proposal 1：The dynamic scheduling enhancement for XR traffic should not impact the PDCCH monitoring and resource allocation of other traffic, such as robust traffic arrival of eMBB used by the UE in the same time in achieving UE power saving.

Proposal 2: The XR-dedicated PDCCH Monitoring Window should be considered as the dynamic scheduling enhancement scheme for improving the capacity and power saving performance of XR-specific traffic and not impact the other traffic.

Proposal 3: The measurement of the average transmission delay and jitter should be standardized to optimize the configure PDCCH monitoring window size of XR-PMW scheme and to improve system capacity and the power saving performance.

Proposal 4: The BSR enhancement that UE sends the BSR report at the 1st PUSCH of each XR packet periodicity should be considered for the UL dynamic scheduling enhancement scheme for improving the capacity of XR-specific traffic.

Proposal 5: The gNB could also feed back the measurement results of the average transmission delay and jitter to UE to improve the XR capacity performance.

Proposal 6: The XR-specific playoutDelayForMediaStartup scheme should be considered as the dynamic scheduling enhancement scheme for its significant capacity gain and obvious power saving gain.

Proposal 7: The single DCI scheduling multi-PDSCHs/PUSCHs in FR1/FR2 would not be considered in XR capacity enhancement, since the PDCCH monitoring per slot is not be the challenge for UE capability and the drawbacks of this scheme is not ignored.

Proposal 8: The Alt 2 of SPS enhancement that the pre-configured PDCCH monitoring window bundled with the reserved SPS resource for PDSCH would be provide the resource to meet the QoS requirement of XR-specific traffic.

Proposal 9: The enhancement of Configured Grant UL transmission that additional resource requirement in a CG period should be considered as the enhanced scheme to support low latency and large data rate transmission of XR traffic.

Proposal 10: According to the analysis, the Enhancement scheme 1, at least for the single SPS configuration with multiple occasions could not efficient to improve the capacity of XR-specific traffic for not predict the packet size and packet arrival time.

Proposal 11: According to the analysis, the Enhancement scheme 2, i.e. the SPS/CG configuration with adaptation parameters, could not efficient to improve the capacity of XR-specific traffic for not predict the packet size and packet arrival time and existing specification has supported.

References

  - RP-201145, Revised SI on XR Evaluations for XR, Qualcomm, RAN#88-e, June 29th - July 3rd, 2020.

  - TR 38.838, Study on XR (Extended Reality) Evaluations for NR, v17.0.0.

  - RP-220285, Revised SID : Study on XR Enhancements for NR, Nokia, RAN#95-e, March 17th-23rd, 2022.

  - TS 38.321, Medium Access Control (MAC) protocol specification, v17.0.0.

  - R1-2208952, UE Power saving techniques for XR, CATT, RAN1#110bis-e, October 10th – 19th, 2022.

  - TS 26.247, Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH), v17.0.0.

  - TS 38.331, Radio Resource Control (RRC) protocol specification, v17.1.0.

  - R1-2207254, Capacity enhancement techniques for XR, Qualcomm, RAN1#110, August 22nd – 26th, 2022.

  - R1-2206062, Discussion on XR specific capacity enhancements, vivo, RAN1#110, August 22nd – 26th, 2022.

  - R1-2206847, Considerations on XR Capacity Improvements, Samsung, RAN1#110, August 22nd – 26th, 2022.

  - R1-2207062, Discussion on XR specific capacity enhancements techniques, ZTE, RAN1#110, August 22nd – 26th, 2022.

  - R1-2207427, Discussion on XR specific capacity improvement enhancements, NTT DOCOMO, RAN1#110, August 22nd – 26th, 2022.

  - ISO/IEC 23009-1:2020/Amd. 1" Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats".

Annex

Annex 1. Traffic model

Traffic are generated from XR sever/device periodically and are transported to RAN with propagation delay and jitter. Each packet size is generated based on the Truncated Gaussian distribution. The jitter is considered as a variable followed Truncated Gaussian distribution. The detailed traffic parameters for XR evaluations are shown in Table 10. Note that these parameter values are those before the truncation. In SLS, the packet is obtained by capturing the sample from Gaussian distribution within a truncated range.

Table 10: Traffic parameters for XR evaluation

| Gaussian Parameters |  | DL AR | UL AR |
| --- | --- | --- | --- |
| Bit Rate | --- | 30Mbps | 10Mbps |
| FPS | --- | 60fps | 60fps |
| Packet size | Mean packet size | 62500bytes | 20800bytes |
|  | Std. packet size value | 6563bytes | 2184bytes |
|  | Min packet size | 31250bytes | 10400bytes |
|  | Max packet size | 93750bytes | 31200bytes |
| Inter-arrival time | Packet generated period | 16.7ms | 16.7ms |
|  | Mean jitter value | 4 | -- |
|  | STD. jitter value | 2 |  |
|  | Max jitter value | 8 |  |
|  | Min jitter value | 0 |  |

Annex 2. Simulation assumptions

The simulation assumptions of Indoor Hotspot are shown in Table 11.

Table 11: Simulation parameters of Indoor Hotspot

| Scenario | Indoor Hotspot |
| --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 |
| Carrier frequency | FR1:4GHz |
| Bandwidth | FR1:100MHz |
| Subcarrier spacing | FR1: 30 kHz |
| BS height | 3m |
| UE height | hUT=1.5 m |
| BS noise figure | FR1: 5 dB |
| UE noise figure | FR1: 9 dB |
| Receiver | MMSE-IRC |
| UE speed | 3 km/h |
| Channel estimation | Realistic |
| MCS | Up to 64QAM |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, |
| TX power | gNB: FR1: 24dBm/20MHz |
| gNB antenna configuration | gNB: - FR1:32TxRU, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| TDD configuration | DDDSU for DL DDDUU for UL |
| MIMO transmission | MU-MIMO for DL SU-MIMO for UL |
| Scheduling | PF, Dynamic scheduled |
| SPS Configuration | Periodicity= 16ms Mcs-Table ={qam64LowSE} Pdsch-Aggregation Factor =1 SLIV=223 BWP size=273PRBs Number of SPS configurations = 4 Note: 4 SPS configurations are considered with the different offset. |
| CG Configuration | Periodicity= 16ms 1 CG configuration is considered. |