3GPP TSG RAN WG1 #110bis-e                                         R1-2208969

e-Meeting, October 10th – 19th, 2022

Source: 	CATT

Title:	Evaluation on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion and Decision

- Introduction

In RAN1#110 meeting, the following agreements were made for evaluation on AI/ML for spatial-domain beam prediction [1]:

Agreement The Following updated based on the agreements in RAN 1 #109-e is adopted Parameters; Values. UE distribution; - FFS 10 UEs per sector/cell for system performance related KPI (if supported) [e.g,, throughput] for full buffer traffic (if supported) evaluation (model inference). - X UEs per sector/cell for system performance related KPI for FTP traffic (if supported) evaluation (model inference). - Other values are not precluded - Number of UEs per/sector per cell during data collection (training/testing) is reported by companies if relevant - More UEs per sector/cell for data generation is not precluded.. UE Antenna Configuration; - Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) - [Panel structure: (M,N,P) = (1,4,2)] - panels (left, right) with (Mg, Ng) = (1, 2) as baseline - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams. Agreement The Following updated based on the agreements in RAN 1 #109-e is adopted Parameters; Values. UE Speed; - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 3km/h(optional), 30km/h (baseline), 60km/h (optional), 90km/h (optional), 120km/h (optional) - Other values are not precluded. UE distribution; - For spatial domain beam prediction: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor. Agreement - If UE orientation is modeled, it can be independently modeled from UE moving trajectory model. - This is not precluded that UE orientation coupled with UE moving trajectory model. Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - FFS on the beams of Set B - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference) - FFS on fixed or variable number of beams (pairs) - FFS on the details - Other options are not precluded. - FFS on the number of beams (pairs) in Set B - Note: This does not preclude the alternative that Set B is different from Set A. Agreement - To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options. - FFS: number of UCI reports and UCI payload size

In this contribution, evaluation methodology and KPIs for AI/ML based beam management enhancement are discussed. Simulation results for beam pair and Tx beam prediction are also provided.

- Discussion

  - Sub use cases of beam management

During RAN1#109 e-meeting, it was reached a consensus on two types of sub use cases for beam management, including BM-Case1 and BM-Case2 [2].

Based on the last meeting agreements as following, for BM-Case1, there are two alternatives and for each alternative, DL Tx beam, DL Rx beam and beam pair prediction can be further studied [1]:

Agreement For the sub use case BM-Case1, support the following alternatives for further study: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The beam patterns of Set A and Set B can be clarified by the companies. Agreement For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact

In this contribution, we will focus on the evaluation of Alt.2 of BM-Case1, i.e., Set B is a subset of Set A. In the evaluation of Alt.2 of BM-Case1, we focus on the DL Tx beam prediction and beam pair prediction.

  - Simulation Assumptions

Based on the agreements in the RAN1#109 e-meeting and RAN #110 meeting [1][2], for dataset generation and performance evaluation for AI/ML in beam management, Dense Urban (macro-layer only, TR 38.913) is the basic scenario. According to the simulation assumptions for Dense Urban scenario for SLS, our simulation assumptions are shown in Table 12 in Annex.

In our simulation for beam pair prediction, the number of beam pairs in Set A is 128, which includes 32 gNB DL Tx beams and 4 UE DL Rx beams. The number of beam pairs in Set B is 32, which includes selected 8 gNB DL Tx beams and 4 UE DL Rx beams. To determine 8 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern and random pattern in our simulation.

For DL Tx beam prediction, the number of Tx beam in Set A is 32. The number of Tx beam pairs in Set B is 8. To determine 8 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern and random pattern in our simulation. The UE DL Rx beam is 1 selected from 4 UE DL Rx beam.

The fixed pattern selection is shown in Figure 1. For random pattern, the 8 gNB DL Tx beams are randomly selected from 32 gNB DL Tx beams.

Figure 1: Fixed pattern 1 selection of DL TX beam

  - Model description

In our simulation, we investigate ResNet based AI/ML model. This model is a beam prediction model based on ResNet as shown in Figure 2. The model is composed of a three-layer full connection structure and two Resblocks. In one Resblock, a three-layer convolution structure is used, and residual calculation between the first layer and the last convolution layer is conducted. The structure is repetitive between two Resblocks. The input data is the L1-RSRP (and beam ID) vector for 32 beam pairs. The output data is the index of the Top-N beam pair or Tx beam.

Figure 2: ResNet based Model

For ResNet based model, our simulation parameters are listed as Table 1:

Table 1: Simulation parameters for ResNet based model

| ## AI/ML model | ## ResNet based model |
| --- | --- |
| ## Training methodology | ## Offline training |
| ## Loss function | ## Cross entropy |
| ## Optimization function | ## Adam |
| ## Learning rate | ## First 100 epoch 0.001, then 0.0001 |
| ## Dataset size | ## 190k |
| ## Training data | ## 110k |
| ## Validation data | ## 40k |
| ## Testing data | ## 40k |
| ## Model Size | ## 104K |
| ## FLOPs | ## 2.84M |

  - KPI

To evaluate the performance of AI/ML in beam management, the KPI has been discussed during the previous meetings. Based on the previous meeting agreements, the KPI includes two types, which are beam prediction accuracy related KPIs and system level performance related KPIs.

In our simulation, we select the following KPIs for beam prediction accuracy:

- Beam prediction accuracy for Top-1 and Top-3 beams

- Average L1-RSRP difference of Top-1 predicted beam

Based on the agreements in RAN1#109 e-meeting, for the definition of beam prediction accuracy, we use Option2, which is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams” [2]. The L1-RSRP difference of Top-1 predicted beam is the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam.

Proposal 1: To evaluate the performance of AI/ML in beam management, the definition of beam prediction accuracy for Top-1 and/or Top-K beams is Option 2, i.e., the Top-1 genie-aided beam is one of the Top-K predicted beams.

    - Simulation results

We evaluate the beam pair prediction and DL Tx beam prediction accuracy related KPIs using fixed pattern and random pattern. The simulation results are shown for Top-1 accuracy, Top-3 accuracy and average Top-1 RSRP difference. Moreover, the generalization and system level performance are provided.

    - Beam pair prediction

In the evaluation of beam pair prediction, we use fixed pattern 1 in Figure 1 and random pattern to evaluate, respectively. The simulation assumption is described in Section 2.2. The simulation results of Top-1 accuracy, Top-3 accuracy and average Top-1 RSRP difference are shown in Table 2 and Table 3.

Table 2: Beam pair prediction accuracy with fixed pattern1

| ## Top-1 accuracy | ## Top-3 accuracy | ## Avg. Top-1 RSRP diff |
| --- | --- | --- |
| ## 0.9563 | ## 0.9866 | ## 0.0127 |

Table 3: Beam pair prediction accuracy with random pattern

| ## Top-1 accuracy | ## Top-3 accuracy | ## Avg. Top-1 RSRP diff |
| --- | --- | --- |
| ## 0.1289 | ## 0.3954 | ## 11.6323 |

Observation 1: For Beam pair prediction, beam prediction accuracy with fixed pattern has better performance than random pattern, since beam ID is implicit in the fixed pattern.

Besides the input of L1-RSRP, we also evaluate the random pattern with the additional input of beam ID. The simulation results are shown in Table 4. Compared with beam prediction accuracy with random pattern using L1-RSRP input only, additional Beam ID input have significant performance gain.

Table 4: Beam pair prediction accuracy with random pattern and beam ID

| ## Top-1 accuracy | ## Top-3 accuracy | ## Avg. Top-1 RSRP diff |
| --- | --- | --- |
| ## 0.7004 | ## 0.8700 | ## 1.4057 |

Observation 2: For Beam pair prediction, additional Beam ID input have significant performance gain compared with beam prediction accuracy with random pattern using L1-RSRP input only.

    - DL Tx beam prediction

The optimal DL Tx beam can be predicted by a selected DL Rx beam. In the evaluation of DL Tx beam prediction, we use fixed pattern 1 in Figure 1 and random pattern to evaluate, respectively. The simulation assumption is described in Section 2.2. The simulation results of Top-1 accuracy, Top-3 accuracy and average Top-1 RSRP difference are provided in Table 5 and Table 6.

Table 5: DL Tx beam prediction accuracy with fixed pattern1

| ## Top-1 accuracy | ## Top-3 accuracy | ## Avg. Top-1 RSRP diff |
| --- | --- | --- |
| ## 0.9842 | ## 0.9999 | ## 0.0202 |

Table 6: DL Tx beam prediction accuracy with random pattern

| ## Top-1 accuracy | ## Top-3 accuracy | ## Avg. Top-1 RSRP diff |
| --- | --- | --- |
| ## 0.1513 | ## 0.4252 | ## 12.7665 |

Observation 3: For DL Tx beam prediction, beam prediction accuracy with fixed pattern has better performance than random pattern, since beam ID is implicit in the fixed pattern.

Besides the input of L1-RSRP, we also evaluate the random pattern with the additional input of beam ID. The simulation results are shown in Table 7. Compared with beam prediction accuracy with random pattern using L1-RSRP input only, additional Beam ID input have significant performance gain.

Table 7: DL Tx beam prediction accuracy with random pattern and beam ID

| ## Top-1 accuracy | ## Top-3 accuracy | ## Avg. Top-1 RSRP diff |
| --- | --- | --- |
| ## 0.5833 | ## 0.7043 | ## 1.0647 |

Observation 4: For DL Tx beam prediction, additional Beam ID input have significant performance gain compared with beam prediction accuracy with random pattern using L1-RSRP input only.

Based on the above observations, additional Beam ID input is useful to improve the performance of beam predication accuracy. Thus, we have the following proposal.

Proposal 2: For AI/ML based beam management, DL Tx and/or Rx beam ID is supported as an additional input besides L1-RSRP measurement based on Set B.

    - Generalization

In the training phase, the size of input set B is 32 beam pairs. In the inference phase, we simulate different sizes of set B as inputs, which are 16, 32 and 64, respectively.

For 16 beam pairs in Set B, they include selected 4 gNB DL Tx beams and 4 UE DL Rx beams. To determine 4 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern 2 and random pattern in our simulation. The fixed pattern 2 selection is shown in Figure 3. For 32 beam pairs in Set B, they include selected 8 gNB DL Tx beams and 4 UE DL Rx beams. To determine 8 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern 1 and random pattern in our simulation. The fixed pattern 1 selection is shown in Figure 1. For 64 beam pairs in Set B, they include selected 16 gNB DL Tx beams and 4 UE DL Rx beams. To determine 16 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern 3 and random pattern in our simulation. The fixed pattern 3 selection is shown in Figure 4.

Figure 3: Fixed pattern 2 selection of DL TX beam

Figure 4: Fixed pattern 3 selection of DL TX beam

The simulation results of accuracy related KPIs using fixed pattern, i.e., training using fixed pattern 1 but inference using fixed pattern 1,2,3, respectively, are shown in Table 8. The simulation results of accuracy related KPIs using random pattern with beam ID are shown in Table 9.

Table 8: The number of beam pairs in Set B for training and inference are different with fixed pattern

| ## Test ## Train | ## Top-1 accuracy |  |  | ## Top-3 accuracy |  |  | ## Avg. Top-1 RSRP diff |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 |
| ## 32 | ## 0.2761 | ## 0.9563 | ## 0.8414 | ## 0.4550 | ## 0.9993 | ## 0.9710 | ## 12.4924 | ## 0.0323 | ## 4.3980 |

Table 9: The number of beam pairs in Set B for training and inference are different with random pattern and beam ID

| ## Test ## Train | ## Top-1 accuracy |  |  | ## Top-3 accuracy |  |  | ## Avg. Top-1 RSRP diff |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 |
| ## 32 | ## 0.4590 | ## 0.7004 | ## 0.7709 | ## 0.7695 | ## 0.8922 | ## 0.9359 | ## 4.2358 | ## 1.4057 | ## 0.1321 |

Observation 5: For AI/ML based beam management, there is significant performance degradation when the number of beams (pairs) of Set B for inference is smaller than that of Set B for training.

Scenario#2: In the training phase, the sizes of input set B are mixed 16, 32 and 64 beam pairs together. In the inference phase, we simulate different sizes of set B as inputs, which are 16, 32 and 64, respectively.  The simulation results of accuracy related KPIs using fixed pattern, i.e., training using fixed pattern 1,2 and 3 together, but inference using fixed pattern 1,2,3, respectively, are shown in Table 10. The simulation results of accuracy related KPIs using random pattern with beam ID are shown in Table 11.

Table 10: Training with mixed numbers of beam pairs in Set B with fixed pattern

| ## Test ## Train | ## Top-1 accuracy |  |  | ## Top-3 accuracy |  |  | ## Avg. Top-1 RSRP diff |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 |
| ## Mixed 16, 32 and 64 | ## 0.8560 | ## 0.9419 | ## 0.9469 | ## 0.9771 | ## 0.9977 | ## 0.9990 | ## 1.1749 | ## 0.0835 | ## 0.0108 |

Table 11: Training with mixed numbers of beam pairs in Set B with random pattern and beam ID

| ## Test ## Train | ## Top-1 accuracy |  |  | ## Top-3 accuracy |  |  | ## Avg. Top-1 RSRP diff |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 | ## 16 | ## 32 | ## 64 |
| ## Mixed 16, 32 and 64 | ## 0.7898 | ## 0.7902 | ## 0.7914 | ## 0.9025 | ## 0.9258 | ## 0.9301 | ## 1.0579 | ## 1.0683 | ## 1.0652 |

Observation 6: For AI/ML based beam management, the performance of AI/ML model, which is trained with mixed numbers of beams (pairs) in Set B, is similarity for different numbers of beams (pairs) in Set B.

Proposal 3: For AI/ML based beam management, the AI/ML model trained with mixed numbers of beams (pairs) in Set B has significant generalization performance for different numbers of beams (pairs) in Set B.

- Conclusions

In this contribution, the simulation results for Alt.2 of BM-Case1 are given and discussed. The observations and proposals are summarized as follows:

Observation 1: For Beam pair prediction, beam prediction accuracy with fixed pattern has better performance than random pattern, since beam ID is implicit in the fixed pattern.

Observation 2: For Beam pair prediction, additional Beam ID input have significant performance gain compared with beam prediction accuracy with random pattern using L1-RSRP input only.

Observation 3: For DL Tx beam prediction, beam prediction accuracy with fixed pattern has better performance than random pattern, since beam ID is implicit in the fixed pattern.

Observation 4: For DL Tx beam prediction, additional Beam ID input have significant performance gain compared with beam prediction accuracy with random pattern using L1-RSRP input only.

Observation 5: For AI/ML based beam management, there is significant performance degradation when the number of beams (pairs) of Set B for inference is smaller than that of Set B for training.

Observation 6: For AI/ML based beam management, the performance of AI/ML model, which is trained with mixed numbers of beams (pairs) in Set B, is similarity for different numbers of beams (pairs) in Set B.

Proposal 1: To evaluate the performance of AI/ML in beam management, the definition of beam prediction accuracy for Top-1 and/or Top-K beams is Option 2, i.e., the Top-1 genie-aided beam is one of the Top-K predicted beams.

Proposal 2: For AI/ML based beam management, DL Tx and/or Rx beam ID is supported as an additional input besides L1-RSRP measurement based on Set B.

Proposal 3: For AI/ML based beam management, the AI/ML model trained with mixed numbers of beams (pairs) in Set B has significant generalization performance for different numbers of beams (pairs) in Set B.

- References

- Chair's notes RAN1_110 v20, Chair, RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- R1-2205701, Report of RAN1#109-e meeting, Chair, RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- Annex

Table 12: Simulation assumptions for Dense Urban scenario for Alt.2 of BM-Case1

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (19 sites, 3 sectors/cells per site) |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | 3km/h |
| UE distribution | 10 UEs per sector/cell for full buffer traffic 80% indoor ,20% outdoor as in TR 38.901 |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| BF scheme | DFT codebook |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |