3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208980

e-Meeting, October 10th – 19th, 2022

Source:	CATT, GOHIGH

Title:	Evaluation methodology and performance evaluation for SL positioning

Agenda Item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

In RAN1#110 meeting, the following agreements on evaluation methodology for sidelink positioning were achieved [1]:

Agreement For SL positioning evaluation in IIOT use case, companies should report how to drop anchor UEs and how to select anchor UEs Agreement Adopt the tables in section 3 of R1-2207606 as templates to collect SL positioning simulation results from each company. Agreement In the evaluation, relative positioning or ranging is performed between two UEs within X m, where X value(s) are reported by companies, and companies should also report the minimum distance used in the evaluations for each use case. The assumption used for X will be included in the TR for each set of results. Agreement For SL positioning evaluation purpose, the following assumptions are further adopted - Companies should report whether SL-PRS and other SL signals are FDMed or not FDMed, and whether other SL signals are present - Adopting system level simulations (rather than the link level simulations) as the baseline tool - For SL positioning evaluation in highway scenario or urban grid scenario, the performance metrics can include absolute horizontal accuracy, relative horizontal accuracy, ranging with distance accuracy, and ranging with direction accuracy (optionally). - In highway and urban grid scenarios, companies can further consider other UE types, e.g. pedestrian UE or VRU devices.

In this contribution, we provide our sidelink positioning simulation results of both urban grid and highway scenarios for V2X use cases and IIoT use cases.

Performance evaluation for SL positioning in V2X use cases

In this section, we evaluate the performance of SL positioning in V2X use case. The common assumption for V2X use case is provided in Table 1. Parameters regarding the urban grid, highway scenarios are further given in Table 2.

Table 1. Common assumption for V2X use cases

| Parameter | V2X |
| --- | --- |
| Carrier frequency | 6GHz/3.5GHz |
| Subcarrier spacing | 30KHz |
| Reference Signal Transmission Bandwidth | 20M/40M/100M |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-2, Mode-2 |
| Reference signal including PRS, SRS and SL-PRS | SL-PRS(Gold), 1-port |
| Number of symbols used per occasion | 2 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | N/A |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm | MUSIC |
| Description of positioning technique / applied positioning algorithm | Gauss-Newton |
| Synchronization assumptions | Ideal Synchronization |
| UE/gNB RX and TX timing error assumption | No timing error |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |
| Additional notes, if any | N/A |

Table 2 Assumptions for urban and highway if they are different from or not specified in Agreements

| Parameters | urban | highway |
| --- | --- | --- |
| UE Antenna model | (1, 2, 2, 1, 1) |  |
| RSU Antenna model | (1, 2, 2, 1, 1) |  |
| RSU deployment for absolute positioning | - As shown in figure 1 | - As shown in figure 2 |
| Selected values of X | - 10/25m | - 20/25/100/150m |
| Positioning method | - TDOA (absolute positioning) - RTT+AoA (relative positioning/ranging) |  |

The topologies of urban and highway are shown in Figure 1 and Figure 2 separately. UE dropping option A defined in section 6.1.2 of TR 37.885 is used. The RSU dropping methods are as following:

- Urban: RSUs are located at the center of each intersection and uniformly located with 250m (lateral)/433m (longitudinal) spacing on both sides of each street unsymmetrically as shown in Figure 1.

- Highway: RSU are uniformly located with 200m spacing on both sides of highway unsymmetrically as shown in Figure 2.

Figure 1. RSU dropping in urban

Figure 2. RSU dropping in highway

Urban grid scenario

Absolute positioning

For absolute positioning of urban scenario, the simulation results of different bandwidths are provided in Table 3.

Table 3. Simulation results for urban grid for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #TDOA | 2.387 | 4.09 | 6.608 | 9.822 | No 38% of UEs satisfying the target positioning accuracy requirement | No 11% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method#TDOA, 6GHz | 0.9298 | 1.615 | 3.074 | 5.114 | No 65% of UEs satisfying the target positioning accuracy requirement | No 30% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 6GHz, positioning method#TDOA | 0.2903 | 0.5134 | 0.9177 | 1.559 | No 89.5% of UEs satisfying the target positioning accuracy requirement | No 66% of UEs satisfying the target positioning accuracy requirement |

The existing of NLOS will decrease the accuracy of the TDOA badly, the whole performance of urban absolute positioning is poor.

Observation 1: For urban grid scenario, the absolute positioning accuracy in horizontal can reach 1.559 m for 90% UEs under 100M bandwidth.

Observation 2: The positioning errors in urban grid almost satisfy the requirements of Set A under 100M bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Relative positioning

For relative positioning of urban scenario, the positioning method is RTT+AoA, and X=10/25m. The results are given in Table 4 and Table 5.

Table 4. Simulation results for urban grid for relative positioning (X=10m) - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#40M, 3.5GHz, positioning method #RTT+AoA | 0.599 | 0.894 | 1.497 | 1.974 | No 81.8% of UEs satisfying the target positioning accuracy requirement | No 40.4% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#100M, 3.5GHz, positioning method #RTT+AoA | 0.246 | 0.404 | 0.853 | 1.242 | No 93% of UEs satisfying the target positioning accuracy requirement | No 73% of UEs satisfying the target positioning accuracy requirement |

Table 5. Simulation results for urban grid for relative positioning (X=25m) - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #RTT+AoA | 1.619 | 2.377 | 3.344 | 4.55 | No 46.5% of UEs satisfying the target positioning accuracy requirement | No 11% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #RTT+AoA | 1.299 | 1.994 | 2.797 | 3.99 | No 55.8% of UEs satisfying the target positioning accuracy requirement | No 18.4% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 3.5GHz, positioning method #RTT+AoA | 1.146 | 1.804 | 2.653 | 3.821 | No 59% of UEs satisfying the target positioning accuracy requirement | No 23% of UEs satisfying the target positioning accuracy requirement |

As the relative positioning method is RTT+AoA, both the accuracy of RTT and AoA will affect the final positioning accuracy. As shown in section 2.1.3, we can see the main reason that affects the positioning performance is the measurement of AoA and the value of X.

Observation 3: For urban grid scenario, the relative positioning accuracy in horizontal can reach 1.242 m for 90% UEs under 100M bandwidth.

Observation 4: The positioning errors in urban grid can satisfy the requirements of Set A under 100M bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Ranging

For ranging, both the distance accuracy and angle accuracy are provided in Table 6~Table9.

- Distance accuracy:

Table 6. Simulation results for urban grid for ranging (X=10m)- distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#40M, 3.5GHz, positioning method #RTT | 0.115 | 0.2388 | 0.3944 | 0.6655 | Yes | No 86% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#100M, 3.5GHz, positioning method #RTT | 0.0889 | 0.1213 | 0.145 | 0.1715 | Yes | Yes |

Table 7. Simulation results for urban grid for ranging (X=25m)- distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #RTT | 0.3392 | 0.5669 | 0.8779 | 1.35 | Yes | No 63% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #RTT | 0.1342 | 0.2372 | 0.3917 | 0.6493 | Yes | No 85% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 3.5GHz, positioning method #RTT | 0.0374 | 0.0747 | 0.1271 | 0.2092 | Yes | Yes |

Observation 5: For urban grid scenario, the distance accuracy can reach 0.1715 m for 90% UEs under 100M bandwidth.

Observation 6: The distance accuracy in urban grid can satisfy the requirements of Set A under 100M bandwidth (1.5m @ 90% CDF UE), and also satisfy Set B (0.5m @ 90% CDF UE).

- Angle accuracy:

Table 8. Simulation results for urban grid for ranging (X=10m) - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#40M, 3.5GHz, positioning method #AoA | 2.28° | 4.55° | 9.48° | 12.6° | Yes | No 71% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#100M, 3.5GHz, positioning method #AoA | 3.456° | 5.903° | 8.72° | 12.5° | Yes | No 77.5% of UEs satisfying the target positioning accuracy requirement |

Table 9. Simulation results for urban grid for ranging (X=25m) - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #AoA | 6.051° | 8.873° | 12.01° | 16.47° | No 87.5% of UEs satisfying the target positioning accuracy requirement | No 62.3% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #AoA | 5.533° | 7.992° | 10.72° | 14.65° | Yes | No 67% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 3.5GHz, positioning method #AoA | 5.294° | 7.579° | 10.2° | 14° | Yes | No 69% of UEs satisfying the target positioning accuracy requirement |

From the above simulation results, it can be shown that the influence of bandwidth on the angle accuracy is limited in urban grid. The main reason is that most of the angles between the two UEs are close to 90 degrees, the measurement algorithm of MUSIC is badly conditioned in urban grid.

Observation 7: For urban grid scenario, the angle accuracy can reach 12.5°for 90% UEs under 100M bandwidth.

Observation 8: The angle accuracy in urban grid can satisfy the requirements of Set A under 100M bandwidth (±15°@ 90% CDF UE), but cannot satisfy Set B (±8°@ 90% CDF UE).

Highway scenario

Absolute positioning

For absolute positioning of highway scenario, the simulation results of different bandwidths are provided in Table 10.

Table 10. Simulation results for highway for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #TDOA | 0.695 | 1.05 | 1.455 | 2.126 | No 82% of UEs satisfying the target positioning accuracy requirement | No 36% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method#TDOA | 0.4425 | 0.6965 | 0.9968 | 1.420 | Yes | No 55% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 6GHz, positioning method#TDOA | 0.125 | 0.192 | 0.2874 | 0.4456 | Yes | Yes |

Observation 9: For highway scenario, the absolute positioning accuracy in horizontal can reach 0.4456 m for 90% UEs under 100M bandwidth.

Observation 10: The positioning errors in highway can satisfy the requirements of Set A under 100M bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Relative positioning

For relative positioning of urban scenario, the positioning method is RTT+AoA, and X=20/25m. The results are given in Table 11 and Table 12.

Table 11. Simulation results for highway for relative positioning (X=20m) - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #RTT+AoA | 1.325 | 2.154 | 2.685 | 3.231 | No 54% of UEs satisfying the target positioning accuracy requirement | No 23.5% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #RTT+AoA | 0.4605 | 0.80 | 1.191 | 1.493 | Yes | No 54% of UEs satisfying the target positioning accuracy requirement |

Table 12. Simulation results for highway for relative positioning (X=25m) - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #RTT+AoA | 1.975 | 2.97 | 3.7 | 4.363 | No 38.6% of UEs satisfying the target positioning accuracy requirement | No 12.7% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #RTT+AoA | 0.6280 | 0.815 | 1.417 | 1.974 | No 83% of UEs satisfying the target positioning accuracy requirement | No 39% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 3.5GHz, positioning method #RTT+AoA | 0.119 | 0.2450 | 0.5572 | 0.7594 | Yes | No 71% of UEs satisfying the target positioning accuracy requirement |

Table 13. Simulation results for highway for relative positioning (X=100m) - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #RTT+AoA | 1.739 | 2.932 | 4.504 | 7.27 | No 44% of UEs satisfying the target positioning accuracy requirement | No 16% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method #RTT+AoA | 1.325 | 2.154 | 3.385 | 5.231 | No 54% of UEs satisfying the target positioning accuracy requirement | No 23.5% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 6GHz, positioning method #RTT+AoA | 0.8705 | 1.5 | 2.891 | 4.467 | No 67% of UEs satisfying the target positioning accuracy requirement | No 35% of UEs satisfying the target positioning accuracy requirement |

Table 14. Simulation results for highway for relative positioning (X=150m) - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #RTT+AoA | 2.175 | 3.57 | 5.7 | 9.613 | No 38.6% of UEs satisfying the target positioning accuracy requirement | No 12.7% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method #RTT+AoA | 1.628 | 2.815 | 4.617 | 7.474 | No 48% of UEs satisfying the target positioning accuracy requirement | No 19% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 6GHz, positioning method #RTT+AoA | 1.119 | 2.25 | 3.572 | 6.197 | No 57% of UEs satisfying the target positioning accuracy requirement | No 29% of UEs satisfying the target positioning accuracy requirement |

As the relative positioning method is based on RTT+AoA, the value of X and angle result will mainly affect the performance. When the value of X is larger, the relative positioning accuracy is lower.

Observation 11: For highway scenario, the relative positioning accuracy in horizontal can reach 0.7594 m for 90% UEs under 100MHz bandwidth.

Observation 12: The positioning errors in highway scenario can satisfy the requirements of Set A under 100MHz bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Ranging

For ranging, both the distance accuracy and angle accuracy are provided in Table 15~ Table 18.

- Distance accuracy:

Table 15. Simulation results for highway for ranging (X=20m)- distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #RTT | 0.71 | 0.92 | 1.7814 | 2.309 | Yes | No If not, 36% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #RTT | 0. 47 | 0.5989 | 0.943 | 1.229 | Yes | No If not, 66% of UEs satisfying the target positioning accuracy requirement |

Table 16. Simulation results for highway for ranging (X=25m)- distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #RTT | 0. 453 | 0.69 | 1.533 | 2.01 | No, If not, 79% of UEs satisfying the target positioning accuracy requirement | No If not, 57% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #RTT | 0.2364 | 0.6455 | 0.8077 | 1.252 | Yes | No If not, 60% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 3.5GHz, positioning method #RTT | 0.0533 | 0.0983 | 0.1348 | 0.1922 | Yes | Yes |

Table 17. Simulation results for highway for ranging (X=100m)- distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #RTT | 0.31 | 0.52 | 0.7814 | 1.309 | Yes | No If not, 66% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method #RTT | 0.135 | 0.243 | 0.4 | 0.68 | Yes | No If not, 85% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 6GHz, positioning method #RTT | 0.047 | 0.0989 | 0.1655 | 0.2693 | Yes | Yes |

Table 18. Simulation results for highway for ranging (X=150m)- distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #RTT | 0.284 | 0.5 | 0.783 | 1.302 | Yes | No If not, 67% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method #RTT | 0.1364 | 0.2455 | 0.4091 | 0.683 | Yes | No If not, 85% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 6GHz, positioning method #RTT | 0.0533 | 0.0983 | 0.1748 | 0.2922 | Yes | Yes |

Observation 13: For highway scenario, the distance accuracy can reach 0.1922 m for 90% UEs under 100M bandwidth.

Observation 14: The distance accuracy in highway can satisfy the requirements of Set A under 100M bandwidth (1.5m @ 90% CDF UE), and also satisfy Set B (0.5m @ 90% CDF UE).

- Angle accuracy:

Table 19. Simulation results for highway for ranging (X=20m) - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #AoA | 2.143° | 3.571° | 5.357° | 8.52° | Yes | No 89% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #AoA | 1.03° | 1.5° | 1.93° | 3.1° | Yes | Yes |

Table 20. Simulation results for highway for ranging (X=25m) - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 3.5GHz, positioning method #AoA | 7.48° | 13.18° | 15° | 17.302° | No 80.0% of UEs satisfying the target positioning accuracy requirement | No 53.5% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 3.5GHz, positioning method #AoA | 1.589° | 5.325° | 9.733° | 12.678° | Yes | No 74.5% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW#100M, 3.5GHz, positioning method #AoA | 1.06° | 1.86° | 2.263° | 3.179° | Yes | Yes |

Table 21. Simulation results for highway for ranging (X=100m) - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #AoA | 2.143° | 3.571° | 5.357° | 8.52° | Yes | No 89% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method #AoA | 1.72° | 2.7° | 4.09° | 6.262° | Yes | Yes |
| Case #3, BW#100M, 6GHz, positioning method #AoA | 1.14° | 1.95° | 3.45° | 4.791° | Yes | Yes |

Table 22. Simulation results for highway for ranging (X=150m) - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#20M, 6GHz, positioning method #AoA | 2° | 3.18° | 5° | 8.302° | Yes | No 16.5% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#40M, 6GHz, positioning method #AoA | 1.589° | 2.515° | 3.733° | 5.878° | Yes | Yes |
| Case #3, BW#100M, 6GHz, positioning method #AoA | 1.06° | 1.86° | 3.263° | 4.645° | Yes | Yes |

Observation 15: For highway scenario, the angle accuracy can reach 3.1°for 90% UEs under 40M bandwidth.

Observation 16: The angle accuracy in highway can satisfy the requirements of Set A under 40M bandwidth (±15°@ 90% CDF UE), and also satisfy Set B (±8°@ 90% CDF UE).

Performance evaluation for SL positioning in IIoT use cases

In this section, we evaluate the performance of SL positioning in IIoT use cases. The common assumption for IIoT use cases is provided in Table 23. Other parameters are further given in Table 24.

Table 23. Common assumption for IIoT use cases

| Parameter | IIoT |
| --- | --- |
| Carrier frequency | 3.5GHz |
| Subcarrier spacing | 30KHz |
| Reference Signal Transmission Bandwidth | 20M/40M/100M |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-2, Mode-2 |
| Reference signal including PRS, SRS and SL-PRS | SL-PRS(Gold), 1-port |
| Number of symbols used per occasion | 2 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | N/A |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm | MUSIC |
| Description of positioning technique / applied positioning algorithm | Gauss-Newton |
| Synchronization assumptions | Ideal Synchronization |
| UE/gNB RX and TX timing error assumption | No timing error |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |
| Additional notes, if any | N/A |

Table 24 Assumptions for IIoT if they are different from or not specified in Agreements

| Parameters | IIoT |
| --- | --- |
| UE Antenna model | (1, 2, 2, 1, 1) |
| TRP antenna model | (1, 2, 2, 1, 1) |
| BS/RSU deployment for absolute positioning | - As shown in Figure 3 |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | - 10m |
| Positioning method | - RTT+AoA(relative positioning/ranging) |

For IIoT use cases, InF-SH scenario is simulated. 18 anchor UEs are located on a square lattice with spacing D, and D/2 from the walls. The deployment is shown in Figure 3, where L=300m, W=150m and D=50m. Each target terminal to be determined takes itself as the origin and takes 10m as the radius to determine a circular area range and find the anchor UE within the range.

Figure 3. RSU (anchor UEs) dropping in IIoT

IIoT InF-SH scenario

Relative positioning

For relative positioning of InF-SH scenario, the positioning method is RTT+AoA, and X=10m. The results are given in Table 25.

Table 25. Simulation results for IIoT for relative positioning (X=10m) - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#40M, 3.5GHz, positioning method #RTT+AoA | 0.379 | 0.414 | 0.5 | 0.79 | Yes | No 41% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#100M, 3.5GHz, positioning method #RTT+AoA | 0.1172 | 0.1468 | 0.1657 | 0.2471 | Yes | No 83% of UEs satisfying the target positioning accuracy requirement |

Observation 17: For IIoT scenario, the relative positioning accuracy in horizontal can reach 0.79 m for 90% UEs under 40MHz bandwidth and 0.2471 m for 90% UEs under 100MHz bandwidth.

Observation 18: The positioning errors in IIoT scenario can satisfy the requirements of Set A under 40M bandwidth (1m@ 90% CDF UE), but cannot satisfy Set B (0.2m@ 90% CDF UE).

Ranging

For ranging, both the distance accuracy and angle accuracy are provided in Table 26~ Table 27.

- Distance accuracy:

Table 26. Simulation results for IIoT for ranging (X=10m)- distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#40M, 3.5GHz, positioning method #RTT | 0.2364 | 0.3455 | 0.4091 | 0.683 | Yes | No If not, 85% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW#100M, 3.5GHz, positioning method #RTT | 0.0987 | 0.1194 | 0.14 | 0.1636 | Yes | Yes |

Observation 19: For IIoT scenario, the distance accuracy can reach 0.683 m for 90% UEs under 40MHz bandwidth and 0.1636 m for 90% UEs under 100MHz bandwidth.

Observation 20: The distance accuracy in IIoT can satisfy the requirements of Set A under 40M bandwidth (1 m @ 90% CDF UE), but cannot satisfy Set B (0.2m @ 90% CDF UE).

- Angle accuracy:

Table 27. Simulation results for IIoT for ranging (X=10m) - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW#40M, 3.5GHz, positioning method #AoA | 1.589° | 2.115° | 2.733° | 5.078° | Yes | Yes |
| Case #2, BW#100M, 3.5GHz, positioning method #AoA | 0.3981° | 0.6765° | 1.12° | 1.72° | Yes | Yes |

Observation 21: For IIoT scenario, the angle accuracy can reach 5.078°for 90% UEs under 40M bandwidth.

Observation 22: The angle accuracy in IIoT can satisfy the requirements of Set A under 40M bandwidth (±15°@ 90% CDF UE), and also satisfy Set B (±8°@ 90% CDF UE).

Conclusions

In this contribution, we provide simulation results for SL positioning. Both V2X use case and IIoT use case are evaluated. We have the following observations:

- Urban grid scenario

Observation 1: For urban grid scenario, the absolute positioning accuracy in horizontal can reach 1.559 m for 90% UEs under 100M bandwidth.

Observation 2: The positioning errors in urban grid almost satisfy the requirements of Set A under 100M bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Observation 3: For urban grid scenario, the relative positioning accuracy in horizontal can reach 1.242 m for 90% UEs under 100M bandwidth.

Observation 4: The positioning errors in urban grid can satisfy the requirements of Set A under 100M bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Observation 5: For urban grid scenario, the distance accuracy can reach 0.1715 m for 90% UEs under 100M bandwidth.

Observation 6: The distance accuracy in urban grid can satisfy the requirements of Set A under 100M bandwidth (1.5m @ 90% CDF UE), and also satisfy Set B (0.5m @ 90% CDF UE).

Observation 7: For urban grid scenario, the angle accuracy can reach 12.5°for 90% UEs under 100M bandwidth.

Observation 8: The angle accuracy in urban grid can satisfy the requirements of Set A under 100M bandwidth (±15°@ 90% CDF UE), but cannot satisfy Set B (±8°@ 90% CDF UE).

- Highway scenario

Observation 9: For highway scenario, the absolute positioning accuracy in horizontal can reach 0.4456 m for 90% UEs under 100M bandwidth.

Observation 10: The positioning errors in highway can satisfy the requirements of Set A under 100M bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Observation 11: For highway scenario, the relative positioning accuracy in horizontal can reach 0.7594 m for 90% UEs under 100MHz bandwidth.

Observation 12: The positioning errors in highway scenario can satisfy the requirements of Set A under 100MHz bandwidth (1.5m@ 90% CDF UE), but cannot satisfy Set B (0.5m@ 90% CDF UE).

Observation 13: For highway scenario, the distance accuracy can reach 0.1922 m for 90% UEs under 100M bandwidth.

Observation 14: The distance accuracy in highway can satisfy the requirements of Set A under 100M bandwidth (1.5m @ 90% CDF UE), and also satisfy Set B (0.5m @ 90% CDF UE).

Observation 15: For highway scenario, the angle accuracy can reach 3.1°for 90% UEs under 40M bandwidth.

Observation 16: The angle accuracy in highway can satisfy the requirements of Set A under 40M bandwidth (±15°@ 90% CDF UE), and also satisfy Set B (±8°@ 90% CDF UE).

- IIoT InF-SH scenario

Observation 17: For IIoT scenario, the relative positioning accuracy in horizontal can reach 0.79 m for 90% UEs under 40MHz bandwidth and 0.2471 m for 90% UEs under 100MHz bandwidth.

Observation 18: The positioning errors in IIoT scenario can satisfy the requirements of Set A under 40M bandwidth (1m@ 90% CDF UE), but cannot satisfy Set B (0.2m@ 90% CDF UE).
Observation 19: For IIoT scenario, the distance accuracy can reach 0.683 m for 90% UEs under 40MHz bandwidth and 0.1636 m for 90% UEs under 100MHz bandwidth.

Observation 20: The distance accuracy in IIoT can satisfy the requirements of Set A under 40M bandwidth (1 @ 90% CDF UE), but cannot satisfy Set B (0.2m @ 90% CDF UE).

Observation 21: For IIoT scenario, the angle accuracy can reach 5.078°for 90% UEs under 40M bandwidth.

Observation 22: The angle accuracy in IIoT can satisfy the requirements of Set A under 40M bandwidth (±15°@ 90% CDF UE), and also satisfy Set B (±8°@ 90% CDF UE).

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110.

Appendix 1: Evaluation Results for Urban scenario

Figure 4 Simulation results for urban grid for relative positioning (X=10m) - horizontal accuracy

Appendix 2: Evaluation Results for Highway scenario

Figure 5 Simulation results for Highway for relative positioning (X=20m, BW=40M) - horizontal accuracy

Appendix 3: Evaluation Results for IIoT scenario

Figure 6 Simulation results for IIoT for relative positioning (X=10m, BW=40M) - horizontal accuracy

Figure 7 Simulation results for IIoT for relative positioning (X=10m, BW=100M) - horizontal accuracy