3GPP TSG RAN WG1 #110bis-e	R1-2209005

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on minimum guard period Y between two SRS resources of an SRS resource set for antenna switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fujitsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It was agreed to support a minimum guard period Y between two SRS resources of an SRS resource set for antenna switching for 480 kHz and 960 kHz. However, the minimum guard period Y for 480 kHz and 960 kHz has not been defined in TS 38.214. 7 symbols for 480 kHz and 14 symbols for 960 kHz are the minimum number longer or equal to 15 us. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Table 6.2.1.2-1 in TS 38.214. The minimum guard period Y[symbol] is 7 for 480 kHz and 14 for 960 kHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum guard period Y between two SRS resources of an SRS resource set for antenna switching for 480 kHz and 960 kHz is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

*** Unchanged text omitted ***

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot.

For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols.

-	When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission.

The UE shall expect to be configured with the same number of SRS ports for all SRS resources in the SRS resource set(s) with higher layer parameter usage set as 'antennaSwitching'.

For 1T2R, 1T4R or 2T4R, or 1T6R or 1T8R, 2T6R, 2T8R, 4T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R or 4T=4R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

The value of Y is defined by Table 6.2.1.2-1.

Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |
| 5 | 480 | 7 |
| 6 | 960 | 14 |

*** Unchanged text omitted ***