3GPP TSG RAN WG1 #110bis-e	R1-2209017

e-Meeting, October 10th – 19th, 2022

Source:	Fujitsu

Title:	Discussion on channel access mechanism for SL-U

Agenda item:	9.4.1.1

Document for:	Discussion and Decision

Introduction

For Rel-18 sidelink evolution, sidelink on unlicensed band (SL-U) will be standardized. In the WID, the following objective for SL-U has been captured [1].

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

Based on the objective, the required changes to support SL-U are mainly on channel access and physical channel structure. In the RAN1#110 meeting, some agreements on those aspects have been achieved. In this contribution, we share some discussions on the part of channel access for SL-U.

Discussion

In the RAN1#110 meeting, the following agreement has been achieved for COT sharing [2].

Agreement - For UE-to-UE COT sharing, continue considering the following alternatives: - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS any additional conditions - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS how to determine a SL UE is a target receiverFFS: details of the channel type of the COT initiating UE’s transmission - FFS any additional conditions - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s). - FFS: details of the channel type of the responding UE’s transmission(s) - gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18 - FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA

The above agreement has established the fundamental framework for UE-to-UE COT sharing. Under the COT sharing framework, the major difference between Alt. 1 and Alt. 2 lies in how to determine a COT sharing UE. For Alt. 1, a COT sharing UE is a target receiver of a COT initiating UE’s PSSCH. For Alt. 2, a COT sharing UE is a target receiver of a COT initiating UE’s transmission, and the definition of the transmission is FFS. In our view, Alt. 1 follows the principle of NR-U COT sharingand thus is preferred. For NR-U, when a UE shares a COT initiated by a gNB, the UE is the target receiver of the gNB’s data transmission, and vice versa. Similarly, the same principle should apply to UE-to-UE COT sharing, which has been reflected by Alt. 1.

For COT sharing, it should be considered to support the following Alt. 1.

- Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT.

UE-A can share a COT initiated by UE-B when some conditions are met. According to the above agreement, one condition is related to the CAPC values of UE-A and UE-B. In our view, the RSRP or distance between UE-A and UE-B can be considered as an additional condition. The rationale is that UE-to-UE COT sharing is mainly based on the assumption that UE-A and UE-B have the same LBT results. If UE-A is relatively close to UE-B, it is expected that UE-A and UE-B will have the same LBT results. In this case, it would be safe to let UE-A share the COT initiated by UE-B. Therefore, the RSRP or distance between UE-A and UE-B can be leveraged to determine whether to share a COT. If the RSRP is larger than a threshold, or the distance is smaller than a threshold, UE-A can share the COT indicated by UE-B.

For COT sharing, the RSRP or distance between UE-A and UE-B should be considered to determine whether UE-A can share a COT initiated by UE-B.

Figure 1. An example of receiving two COT indicators transmitted by UE1 and UE2.

Generally, a COT sharing UE (UE-A) and a COT initiating UE (UE-B) should be the target receivers of each other according to the above agreement. However, for groupcast or broadcast, one UE-A can be the target receiver of multiple UE-Bs, and multiple UE-Bs can be the target receivers of one UE-A. In this case, UE-A may acquire more than one COT, and need to determine which COT to share. An example is shown in Figure 1. For UE1 and UE2, they have initiated COT#1 and COT#2, and thus transmit two COT indicators respectively. For UE-A, it receives two COT indicators from two UE-Bs (UE1 and UE2), and the COT indicators indicate COT#1 and COT#2 respectively. For these two COTs, the COT durations can be different, and the occupied slots within the COTs can be also different. Therefore, UE-A needs to determine whether to share COT#1 or COT#2. To this end, how to determine which COT to share should be further studied.

For COT sharing, it should be studied how to determine which COT to share if more than one COT is identified by a COT sharing UE.

In the RAN1#110 meeting, the following agreement has been achieved [2].

Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

Although it has been agreed to support multi-consecutive slots transmission (MCSt), it is not clear whether MCSt is transmitted by a single UE or multiple UEs. In our view, MCSt transmitted by a single UE should be supported. To efficiently share an unlicensed channel, a UE should be able to continuously occupy the channel once it has accessed the channel. By this way, the UE can avoid performing too much LBT and compete fairly with other devices from the coexisting systems. When consecutive slots are used for transmission, the gap (such as the guard symbol) between the slots can be omitted so that no additional LBT is needed. In other words, the guard symbol between the slots can be also used for transmission.

For multi-consecutive slots transmission (MCSt), it should be considered to support the follows.

- Multi-consecutive slots transmission is from a single UE.

- The guard symbol between the slots is used for transmission.

Figure 2: An example of triggering resource selection (Resource #n1) when a COT is obtained.

According to the agreement in RAN1#109, the existing mode 2 resource selection will used as the baseline, and it is FFS whether any enhancement is needed. In our view, mode 2 is not designed for sharing an unlicensed spectrum. Therefore, mode 2 may not be a strong competitor in terms of contending for the unlicensed spectrum or achieving an efficient channel access. Potentially, the enhancements on mode 2 can be considered. For example, it can be considered that resource selection is triggered once a COT is obtained. An example is given in Figure 2. If a UE has successfully initiated a COT or shared a COT initiated by another UE, it is triggered to perform resource selection (Resource#n1). The rationale is that the UE can quickly gain access to the channel once there is an opportunity.

For potential mode 2 enhancements, it should be further studied whether resource selection can be triggered when a COT is obtained.

Figure 3: An example of HARQ-ACK retransmission on PSFCH.

In sidelink, SL HARQ-ACK is conveyed on PSFCH. Also, PSFCH will not be transmitted in case of LBT failure. To cope with PSFCH dropping due to LBT failure, SL HARQ-ACK retransmission can be considered. In NR-U, HARQ-ACK retransmission has been supported. More specifically, a UE can be indicated to not transmit HARQ-ACK temporarily, but to retransmit HARQ-ACK together with other HARQ-ACK later. Following the design principle in NR-U, an example of SL HARQ-ACK retransmission is shown in Figure 3. Wherein, HARQ-ACK1 for PSSCH1 is not transmitted on PSFCH1. The reason could be that PSFCH1 is subject to LBT failure, or that PSFCH1 is outside of a COT. However, HARQ-ACK1 has a second chance to be transmitted on PSFCH2. Different from NR-U where HARQ-ACK resources and timing are dynamic, PSFCH resources and timing in sidelink are (pre-)configured in a semi-static manner. More specifically, PSFCH resources are periodic with only 1-bit HARQ-ACK carried per resource, and have a one-to-one mapping with PSSCH resources in timing. To support SL HARQ-ACK retransmission, the way of determining PSFCH resources would require some changes.

Considering PSFCH may endure LBT failure or be outside of a COT, SL HARQ-ACK retransmission should be supported for SL-U.

To support SL HARQ-ACK retransmission, the impact on PSFCH resource determination should be further studied.

Conclusion

In this contribution, we have expressed our views on unlicensed channel access for SL-U. The proposals are summarized as follows.

- For COT sharing, it should be considered to support the following Alt. 1.

- Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT.

- For COT sharing, the RSRP or distance between UE-A and UE-B should be considered to determine whether UE-A can share a COT initiated by UE-B.

For COT sharing, it should be studied how to determine which COT to share if more than one COT is identified by a COT sharing UE.

For multi-consecutive slots transmission (MCSt), it should be considered to support the follows.

- Multi-consecutive slots transmission is from a single UE.

- The guard symbol between the slots is used for transmission.

For potential mode 2 enhancements, it should be further studied whether resource selection can be triggered when a COT is obtained.

Considering PSFCH may endure LBT failure or be outside of a COT, SL HARQ-ACK retransmission should be supported for SL-U.

To support SL HARQ-ACK retransmission, the impact on PSFCH resource determination should be further studied.

Reference

- RP-221938, NR sidelink evolution, RAN#97, September 12-16, 2022.

- Chair’s notes, RAN1#110, August 22-26, 2022.