3GPP TSG RAN WG1 #110bis-e			R1-2209028

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.3.2

Source:	InterDigital, Inc.

Title:	Discussion on subband non-overlapping full duplex

Document for:	Discussion and Decision

# Introduction

For Rel-18, a study item on evolution of NR duplex operation has been approved [1], where the objectives identified for the study item are as follows:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In RAN1#110 meeting [3], the following agreements and conclusions were made:

Agreement: Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state. - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. UE capability discussion is held in work item phase. Agreement: For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline. Agreement For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair Agreement For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

In this contribution, we discuss subband non-overlapping full duplex (SBFD) operations, based on the analysis on effects of intra-/inter-subband CLI with simulation results, and discuss issues on power management, sensing-based CLI mitigation, UL/DL timing alignment, UE-to-UE CLI mitigation, resource allocations in SBFD, and benefits of SBFD operating through multiple BWPs.

# Discussions

## Subband non-overlapping FD operations

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-17, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-18, enhancements to support full duplex (FD) at least for gNB have been proposed and endorsed as the study item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on subband non-overlapping FD-gNB and HD-UEs in a cell

For a single UE perspective, a UE in a cell illustrated in Figure 2 can be informed of such a mixed D/U regions across RBs per symbol/slot, where the informed mixed D/U regions to the UE can be a new type of SFI, denoted by ‘MF’, which can be an enhanced version of ‘F’. The UE can perform a UL Tx when the scheduled UL Tx is matched with a UL region of the indicated ‘MF’ on a given symbol/slot.

For example, if the scheduled UL Tx for the UE across the 3 consecutive slots (as illustrated in Figure 2) is within a UL region of ‘MF’ per slot basis, the UE can perform the UL Tx across the 3 slots which results in a UL coverage/capacity enhancement, while in the cell perspective, parallel DL transmissions toward other UEs over other non-overlapped RBs can be performed by the FD-gNB.

To have a sufficient flexibility in terms of network implementation, multiple ‘MF’ types can be pre-configured to the UE with multiple variations on the D/U region separation and distribution, and one of the multiple can be dynamically selected per slot basis.

The granularity of a subband also needs to be discussed where the granularity should at least be a group of RBs, or BWP-level of the granularity may also be considered. For the case of BWP-level SBFD, a subband indication for the SBFD operations can be based on reusing an existing BWP indicator in a DCI. Or, alternatively, the BWP indication can be interpreted as muted RBs/BWP region in terms of DL reception or UL transmission to be rate-matched around the muted RBs, as a simplified SBFD operation. Both the RB-level and BWP-level granularity of subband has its own trade-off benefits and need to be considered in this study.

Observation 1. Mixed D/U regions informed to a UE per symbol/slot in a cell can be used for a subband-wise UL transmission or DL reception, which results in UL coverage/capacity enhancement while achieving parallel DL transmissions over non-overlapped RBs.

[Informing the time and freq. location of subbands for SBFD operation]

In RAN1#110, four operation alternatives were agreed to be studied on whether the time and frequency allocations of SBFD subbands should be informed to the UE and how respective indications should be performed. As we have discussed in this contribution, most benefits that come from the SBFD operations are based on the UE knowing the resource allocations for SBFD as subbands in terms of the time and frequency domains.

Since the symbol-level TDD allocations are already possible by using SFI, the time and frequency indication of SBFD subbands can be performed based on a combination of an enhanced SFI and one or more frequency resource allocations. As such, the UE can determine the time domain resources in slot-level or symbol-level for SBFD operation based on an enhanced SFI configuration.

The frequency resource allocations can be configured using below alternatives:

- Alt. 1. Indication of the first and the last PRBs corresponding to the SBFD subbands.

- Alt. 2. Indication of the starting PRB and the number of PRBs corresponding to the SBFD subbands.

- Alt. 3. Indication of the PRB resources corresponding to the SBFD subbands based on a bitmap indication.

In case of having multiple number of SBFD subbands/BWPs within the same carrier, Alt. 1 and 2 allow such flexible configuration by separately indicating respective PRBs, and Alt. 3 may need to incorporate multiple bitmaps or multi-bit based information delivery to the UE.

Proposal 1. Study mechanisms on how to inform UE of mixed D/U regions per symbol/slot as an enhancement of SFI to achieve subband non-overlapping FD (SBFD).

- Consider the cases where multiple BWPs/subbands are configured for SBFD operations within the same carrier.

[SBFD based on multiple DL/UL BWP pairs]

In RAN1#110, it was agreed to study semi-static configuration for SBFD subband time and frequency locations as baseline. However, the semi-static configuration may result in suboptimal NW operations. For example, the configuration cannot dynamically change based on the UL/DL traffic that can result in reduced scheduling flexibility, waste of resources, increased latency, and increased scheduling complexity due to the fixed resource allocations.

In RAN1#110, a working assumption was made that for SBFD operation within a TDD carrier, the SBFD will be studied within a single configured DL and UL BWP pair with aligned center frequencies as baseline. It was also agreed to further study the feasibility and potential benefits of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair.

Considering the semi-static configuration of SBFD subbands/BWPs, in case more than one SBFD BWP is configured, it may be possible to schedule the UE to operate in different BWPs, e.g., based on NW UL/DL traffic and scheduling requirements. For example, in case UL traffic is more than DL traffic, the NW may indicate a BWP pair in which the UE to operate in an associated SBFD configuration comprising a larger UL subband. Alternatively, in case the DL traffic is more than UL, the NW may select the UE to switch to operate in another SBFD configuration that is associated with a different BWP pair comprising larger DL subband(s) (i.e., smaller UL subband).

Observation 2. Using SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration could result in improving scheduling flexibility, optimal resource allocations, lower latency, and so forth.

Proposal 2. Study the aspects of BWP switching for SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration.

## Analysis on effects of intra-/inter-subband CLI

In this subsection, we discuss effects of intra-subband and inter-subband CLI based on initial LLS results, where we conducted the LLS to see how much negative impacts on DL reception on a subband (e.g., RBs) can be observed, when intra-subband CLI on the same overlapped subband or inter-subband CLI on adjacent or non-overlapped subband exist due to nearby other UE’s uplink transmissions.

Simulation assumptions for the LLS are summarized in Appendix (Table 3). We considered a duplexing scheme where the DL signal can be a couple of dBs less than, greater than or equal to the UL (CLI) signal. For a given UL (CLI) RB allocation, we evaluated the DL receiver performance for various RB allocations over the system bandwidth considering several degrees of intra and inter subband CLI overlaps as shown in Figure 3. For the initial results presented, no signal impairments due to PA nonlinearity or other imperfections, etc. are considered, where such impairments may further increase the adjacent inter-subband CLI level.

Additionally, the performance evaluations considered some timing advance (up to half an OFDM symbol) being applied between the DL and UL, which represents a realistic environment when UE-to-UE CLI exists, as an uplink (CLI) signal transmission timing is controlled by a timing advance mechanism from its serving cell/TRP.

Figure 3. Throughput vs SNR: DL signal and CLI having same power level

Figure 4. Throughput vs SNR: CLI power 18dB below DL signal

Figures 3 and 4 show throughput performance for the DL signal affected by intra- and inter-subband CLI (UL) with different power levels relative to the DL signal. Figure 3 is for the case when CLI and DL signal power are identical, while Figure 4 is for the case where CLI power level is 18dB below the DL signal. The throughput performance curves are generated based on applying the half symbol timing advance on the CLI signal in both figures.

It can be seen from both Figure 3 and Figure 4 that, for a given MCS index, i.e., MCS-27, the DL throughput performance suffers considerably from the effects of CLI and approaches almost zero when there is any amount of intra-subband CLI being overlapped with the DL signal. Moreover, the ICI as a result of the inter-subband CLI has a severe impact on the throughput when the inter-subband CLI is adjacent to the DL signal with 0-RB gap in between. Throughput performance improves significantly to 90% of the maximum throughput when the adjacent subband gap between the DL signal and the inter-subband CLI is at least 2RBs in this example scenario.

Figure 5. Normalized Throughput vs SNR, for various PRB allocations of DL signal (victim) 
and 25PRB allocation for CLI (UL)

Figure 5 shows the normalized throughput performance curves for the DL signal relative to the number of PRBs allocated to it. For the various number (2, 5, 10, 25, and 50) of PRB allocations to the DL (victim) signal, the CLI (UL) signal is allocated 25PRBs and it is assumed to be 18dB below the DL signal. The normalized throughput performance is shown for the case when there is no frequency gap (0PRB gap) and for a frequency gap of 1 PRB, between the DL signal and the CLI. It can be observed that as the frequency allocation (number of PRBs) of the DL signal relative to the frequency allocation of the CLI decreases, the performance of the DL signal degrades significantly. It can also be seen that the relative degradation with lower number of frequency (PRB) allocation is considerable when there is 0PRB frequency gap between DL signal and the CLI (UL) signal compared to a frequency gap of 1PRB.

Observation 3. DL throughput performance suffers considerably at high MCSs and approaches to almost zero when there is any degree of intra-subband CLI overlap with DL signal.

Observation 4. Inter-carrier interference, resulting from time advance on inter-subband CLI may impact DL throughput performance significantly, especially at high MCS indices, when there is no adjacent inter-subband distance between the DL signal and the CLI, i.e., 0-RB gap.

Observation 5. At high MCS indices, DL throughput performance recovers to 90% of the maximum throughput when the inter-subband distance between the DL signal and the CLI signal is at least 2-RB in this example scenario.

Observation 6. In addition to the frequency gap between the DL signal and the CLI (UL) signal, the amount of frequency resources (PRBs) allocated to the DL signal relative to that for the CLI signal also impacts the normalized throughput performance significantly.

Proposal 3. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands.

## Performance comparison between SBFD and legacy TDD

In this subsection we compare the performance of SBFD with legacy TDD via system level simulations. We consider the SBFD deployment case 1, where a single carrier is considered with all cells using the same SBFD subband configuration. We consider the FR1 - Urban macro deployment. We applied Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD) from the agreement made in RAN1#109-e [2], which includes the following:

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

Legacy TDD uses a shared Tx/Rx antenna array for downlink and uplink transmission/receptions, whereas SBFD utilizes separate Tx/Rx antenna array for simultaneous downlink and uplink transmission. The total number of TxRUs is the same for both legacy TDD and SBFD, however in the SBFD case, each of the downlink and uplink uses half of the total number of TxRUs. We consider two traffic ratios: (i) DL:UL ratio = 1:1, and (ii) DL:UL ratio = 2:1. Additionally, for both of these we consider varying DL:UL traffic arrival rates/loads. Table 1 presents results for the DL:UL ratio of 1:1, whereas Table 2 presents results for the 2:1 ratio. Additional details regarding simulation assumptions can be found in the Appendix (Table 4).

Table 1. legacy TDD vs. SBFD for 1:1 (DL:UL) ratio

| Reported parameters | DL/UL ratio 1/1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Legacy TDD | SBFD Opt.1 | SBFD Opt. 2 | Legacy TDD | SBFD Opt. 1 | SBFD Opt. 2 | Legacy TDD | SBFD Opt. 1 | SBFD Opt. 2 |
| Mean DL UPT [Mbps] | 356.863 | 284.333 | 303.109 | 318.477 | 250.690 | 271.109 | 282.937 | 219.118 | 241.258 |
| Mean UL UPT (Mbps) | 57.439 | 57.926 | 66.023 | 56.040 | 59.626 | 65.313 | 50.460 | 56.212 | 64.256 |
| RU | 0.317 | 0.457 | 0.441 | 0.398 | 0.552 | 0.530 | 0.483 | 0.649 | 0.618 |
| λd/λu | 0.95/0.95 |  |  | 1.43/1.43 |  |  | 1.90/1.90 |  |  |
|  | Additional comments: DL:UL ratio 1:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) |  |  |  |  |  |  |  |  |

Table 2. legacy TDD vs. SBFD for 2:1 (DL:UL) ratio

| Reported parameters | DL/UL ratio 2/1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Legacy TDD | SBFD Opt.1 | SBFD Opt. 2 | Legacy TDD | SBFD Opt. 1 | SBFD Opt. 2 | Legacy TDD | SBFD Opt. 1 | SBFD Opt. 2 |
| Mean DL UPT [Mbps] | 357.716 | 285.788 | 304.297 | 283.469 | 218.481 | 239.902 | 236.181 | 180.952 | 196.853 |
| Mean UL UPT (Mbps) | 59.497 | 57.800 | 66.795 | 57.158 | 57.171 | 66.132 | 56.865 | 59.036 | 64.881 |
| RU | 0.301 | 0.422 | 0.392 | 0.475 | 0.626 | 0.593 | 0.646 | 0.795 | 0.767 |
| λd/λu | 0.95/0.48 |  |  | 1.9/0.95 |  |  | 2.86/1.43 |  |  |
|  | Additional comments: DL:UL ratio 2:1 Legacy TDD Slot config: {DDDSU}, with S = [12D, 2G, 0U] SBFD slot config: {XXXXU}, where X denotes SBFD slot with [DUD] = [40 20 40] RB split and U is uplink only slot Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) |  |  |  |  |  |  |  |  |

We observe a drop in downlink UPT for SBFD compared with legacy TDD, in exchange for overall improvements in uplink UPT for SBFD. This trend is evident across all loads and traffic ratios. Legacy TDD offers four (full bandwidth) slots out of every five for downlink transmissions, while SBFD provides the same number slots, however only 80% of the bandwidth for each of these is available for downlink transmissions. Although these results show improvements in uplink UPT for SBFD which is a primary goal for this NR-Duplex study, it is evident that such a downlink performance drop is not satisfactory. In our view, one of main reasons on such a drop in downlink is because of the applied static/fixed subband partitioning, as [DUD] = [40 20 40] RB split all the time, which is not versatile in coping with varying traffic/channel conditions. Therefore, dynamic indications of flexible subband partitioning, e.g., [DUD] = [A B C] RB split for which values of A, B, and C can be flexibly selected by gNB and informed to UE dynamically, as discussed in Section 2.1, should be considered for further discussion.

Another observation point is that SBFD Option 2 provides better performance than SBFD Option 1. This directly results from the beamforming gain Option 2 offers (two elements per TxRU, compared to a 1-1 mapping between antenna element and TxRU for Option 1).

For uplink performance we observe that SBFD provides comparable (Option 1) or better performance (Option 2) when compared to legacy TDD. Legacy TDD offers one slot out of every five uplink transmissions, whereas SBFD offers one (full bandwidth) slot, as well as four (20% bandwidth) slots, providing additional opportunities for uplink transmissions. Also, similar to the downlink case, we observe that SBFD Option 2 provides better uplink performance than Option 1.

Observation 7. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance.

Observation 8. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink. Thus, flexible/versatile subband partitioning and its dynamic indication mechanisms should be considered to cope with varying traffic/channel conditions.

Observation 9. Utilizing SBFD option 2 (total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD) improves SBFD performance.

Proposal 4. Analysis on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study.

Proposal 5. To overcome the degraded downlink performance due to the static/fixed subband partitioning, flexible/versatile subband partitioning and its dynamic indication mechanisms should be further discussed to cope with varying traffic/channel conditions.

## On CLI Mitigation aspects

A granularity of a subband in the SBFD scenario can be a group of RBs. Since the gNB can flexibly schedule an UL transmission over a set of RBs on a subband for a UE, this may be adjacent to another subband being allocated for DL in SBFD. As observed by Figures 3, 4, 5, and 6 as the LLS results, the UL transmission may cause a severe UE-to-UE CLI leakage on the adjacent DL subband, which depends on the frequency gap between the UL RBs and the DL RBs.

In line with these aspects as well, in RAN1#110 it was agreed to study the potential solutions of UE-to-UE CLI measurement and reporting for SBFD scheme.

[Dynamic Power Management]

One way of handling this CLI issue is to consider dynamic UL power control depending on some factors such as the frequency gap, beam/spatial-domain parameter, a priority indication on the UL, and so on. Dynamic link adaptation mechanism can also be considered similarly. For example, MCS adjustment depending on the factors can be beneficial to mitigate the effects of the CLI dynamically.

Observation 10. In the SBFD scenario, an UL transmission over a subband causes significant UE-to-UE CLI leakage on the adjacent DL subband depending on a frequency gap between the UL RBs and DL RBs of each subband.

Proposal 6. Study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically.

On the other hand, gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation as a part of gNB implementation. This can have some impacts to UE behaviours unless the amount of power backoff is negligible on a given slot.

Since such a power management behaviour can be in a dynamic manner in the SBFD scenario, it is beneficial to consider informing UE of the dynamic power adjustment and related operations. For example, UE can apply the information to a CSI calculation by adjusting a channel power-level for compensation to improve CSI accuracy and depending on the information the UE may be better skipping some measurements on the SBFD slots/symbols. Details on when and how to trigger these behaviours are to be further studied to deal with the UE-to-UE CLI.

Observation 11. As a part of gNB implementation, the gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation, which can impact to UE behaviours depending on the amount of power backoff.

Proposal 7. Consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario.

[Subband-edge-specific CLI Measurement]

The UE-to-UE CLI leakage on the adjacent subbands in SBFD can be measured and reported as part of UE-to-UE CLI mitigation techniques. The inter-subband CLI in SBFD may impose higher interference on subband-edge RBs and resources, whereas the resources in the middle of the SBFD subband may not experience CLI as much. As such, measuring and averaging the CLI over the whole SBFD subband may result in down estimation of the CLI.

For example, the UE can measure a delta parameter that is based on measuring and calculating the difference between a first CLI-RSSI measured from the resources in the edge of the configured RBs and a second CLI-RSSI measured from the resources located in the middle of the configured RBs. As such, the difference between the CSI-RSSI measured in the edge and CLI-RSSI measured in the middle can be used as an indication of inter-subband CLI.

Observation 12. Inter-subband UE-to-UE CLI measurement in SBFD slots based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

Proposal 8. Study measurement resources and reporting configurations for subband-edge CLI measurement.

## Issues on sensing-based CLI mitigation

In general, DL/UL resource assignment for a UE can be based on CSI/beam measurement and reporting procedures supported in NR. Based on the CSI/beam measurements performed on a configured DL RS (e.g., CSI-RS) resource, the UE can report one or more preferred beam/RS indexes along with corresponding quality metrics such as CQI, L1-RSRP, or L1-SINR. The reported contents can be subband-wise, depending on gNB’s configuration, and those can be used for gNB’s scheduling on the DL/UL resource assignment for the UE. The beam information for the DL/UL resource assignment can be based on an indicated transmission configuration indicator (TCI) for a DL/UL channel/signal. When the UE receives a grant (DCI), the UE can apply a spatial filter determined by the indicated beam information to perform the DL reception or the UL transmission. These general procedures are not based on any dynamic information related to CLI (e.g., UE-to-UE CLI) as the UE’s measurement and reporting is based on a configured RS resource which is relevant only with the gNB.

A CLI due to a signal transmitted by other (aggressor) UE can be present and severely degrade a reception performance of a DL signal by the grant at the UE, where the presence of the interference may not be known prior to the grant, e.g., in case when the aggressor UE is associated with a different serving-cell/TRP. Due to the CLI which is not captured in the general CSI/beam reporting, the UE may fail to receive the DL signal, which degrades the DL performance. The failure in receiving the DL signal can continue to happen, in case when such unexpected UE-to-UE CLIs may exist for a duration of time.

Since such a UE-to-UE CLI can happen unexpectedly, it is beneficial to consider an event-based CLI sensing behaviour at the (victim) UE side, where the sensing behaviour can be conducted per subband which can be configured to the UE as performing subband-wise CLI measurement and reporting. Then, the victim UE can report the subband-wise CLI sensing results, which can be used at the gNB to determine whether the assigned DL/UL resource to the UE should be changed to a different subband to avoid the CLI on a subband. The event-based CLI sensing done at the victim UE may greatly simplify the network and UE implementation for CLI handling, as this procedure is transparent to the aggressor UE side. The event can at least include a case when the victim UE detects a PDSCH reception failure, e.g., along with NACK transmission in general.

Observation 13. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Proposal 9. Study an event-based CLI sensing behaviour at the victim UE side, where the event can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

## UL/DL timing alignment

In conventional TDD systems, the UL and DL slots are considered separately in time domain. Therefore, subsequent slots are split in DL-only, UL-only, and flexible slots. The symbols in flexible slots can be scheduled to be used as DL or UL based on received configurations. The flexible symbols can also be used as guard period for timing alignment requirements (e.g., DL/UL switching, UL timing advance (TA)).

In SBFD schemes with one or more subbands allocated for UL transmission in DL slots, a non-zero timing advance or switching time in SBFD could result in inter-slot interference. As an example, shown in Figure 6, where the UL signals in UL SBs and DL signals in DL SBs/slots are interfered by each other due to timing advance. Also, inter-slot interference is shown in Figure 7 that is affecting the SBFD slots due to required switching time for UE between DL and UL that is also a factor for the timing alignment requirements.

For example, especially for a semi-statically scheduled resource (e.g., configured grant PUSCH), every time when UE receives a TA command which affects all related UL transmission including the CG-PUSCH, the UE has to check whether the adjusted/accumulated TA and the switching time requirement result in overlap with DL symbol in a prior DL slot, if scheduled, for SBFD operation. This does not happen in legacy TDD system as there is the flexible slot for this reason at least, while in SBFD operation, it is always possible to face a back-to-back scheduling between DL and UL for a UE.

Figure 6. Inter-slot interference due to timing advance in SBFD UL SBs

Figure 7. Inter-slot interference due to switching time between DL and UL

While the gNB might address the required timing advance/switching time based on resource allocations and scheduling (e.g., as part of gNB implementation), this requires significantly increased scheduling complexity at the gNB side and also increases the signaling overhead, especially for taking such semi-statically scheduled resources for both DL and UL into account. In another aspect, in case a scheduled (dynamic) grant for UL or DL with high priority is received and if the timing alignment requirement including required Rx/Tx or Tx/Rx switching time is not met, e.g., resulting in some DL/UL overlapped symbols being faced at the UE, this could again result in a severe performance degradation or dropping corresponding slots.

Observation 14. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in inter-slot interference and dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

Therefore, there should be a means for UE to detect if the allocated UL/DL timing alignment is accurate and enough, especially for such back-to-back scheduling between DL and UL given to the UE. In case UE has detected timing discrepancies, the UE could report the issues along with requesting for adjustment or at least should be able to determine which overlapped portion of either DL symbol(s) or UL symbol(s) can be dropped or punctured. For example, the UE could also determine to include the time required for the timing alignment inside respective UL/DL SB in SBFD slot, where the scheduled UL transmission on the SBFD slot can have a rate-matched or punctured symbol(s) in the front symbol positions of the UL transmission, or alternatively DL symbol(s) can be punctured, which should depend on gNB’s flexible configurations for such UE behaviours.

Proposal 10. Study UL/DL timing alignment issues in subband non-overlapping full duplex systems.

## PUCCH/PUSCH Resource Allocation in SBFD Symbols

It is important to properly support UL resource allocations in SBFD symbols with UL subbands, which is one of main goals for this NR-Duplex feature. In a scenario, a HARQ-ACK/PUCCH transmission that is configured for a SPS PDSCH or by a dynamic-grant based PDSCH could be scheduled in an SBFD slot, where the respective PUCCH resource indicator (PRI) can point to a frequency location that is outside of the SBFD UL subband boundary. As such, the PRI that was originally configured to span the whole BWP in a legacy UL-only slot, may point to a frequency position outside of the SBFD UL subband. This could result in dropping the HARQ-ACK/PUCCH transmission that could lead to increase the latency, which is also one of the main objectives of employing SBFD in NR-Duplex. Alternatively, the UE could be configured with supplementary configurations so that the UE could reinterpret the original PRI to be mapped to different frequency resources inside the SBFD UL subband boundaries.

In another scenario, repetition-based PUSCH transmissions may be scheduled for a UE, where one or more of the repetitions can be occurred in SBFD slots. Since the SBFD slots include UL subbands, it needs to be addressed whether they could be counted as available slots or not. For example, the UE could decide whether to count the SBFD slots as well, or only count the legacy UL slots as available, based on configurations from gNB. In case the scheduled frequency resources are within the SBFD UL subbands, this should be straightforward for UE to be able to transmit PUSCH repetitions in respective SBFD slots. However, in case the scheduled frequency resources are outside of the SBFD UL subbands’ boundaries, the UE may need further configurations on how to reinterpret the scheduled frequency resources to be mapped within the SBFD boundaries.

Observation 15. In case the scheduled frequency resources for an UL transmission are located outside of the boundaries of the SBFD UL subbands, the UE may need further configurations to reinterpret the frequency resources to be mapped within the SBFD UL boundaries.

Proposal 11. Study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped to outside of the SBFD UL boundaries.

# Summary

In this contribution, we discussed subband non-overlapping full duplex (SBFD) operations, based on the analysis on effects of intra-/inter-subband CLI, and discussed issues on power management, sensing-based CLI mitigation, UL/DL timing alignment, UE-to-UE CLI mitigation, resource allocations in SBFD, and benefits of SBFD operating through multiple BWPs. From the discussions, we made following observations and proposals:

Observation 1. Mixed D/U regions informed to a UE per symbol/slot in a cell can be used for a subband-wise UL transmission or DL reception, which results in UL coverage/capacity enhancement while achieving parallel DL transmissions over non-overlapped RBs.

Observation 2. Using SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration could result in potential benefits in scheduling flexibility, optimal resource allocations, lower latency, and so forth.

Observation 3. DL throughput performance suffers considerably at high MCSs and approaches to almost zero when there is any degree of intra-subband CLI overlap with DL signal.

Observation 4. Inter-carrier interference, resulting from time advance on inter-subband CLI may impact DL throughput performance significantly, especially at high MCS indices, when there is no adjacent inter-subband distance between the DL signal and the CLI, i.e., 0-RB gap.

Observation 5. At high MCS indices, DL throughput performance recovers to 90% of the maximum throughput when the inter-subband distance between the DL signal and the CLI signal is at least 2-RB in this example scenario.

Observation 6. In addition to the frequency gap between the DL signal and the CLI (UL) signal, the amount of frequency resources (PRBs) allocated to the DL signal relative to that for the CLI signal also impacts the normalized throughput performance significantly.

Observation 7. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance.

Observation 8. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink. Thus, flexible/versatile subband partitioning and its dynamic indication mechanisms should be considered to cope with varying traffic/channel conditions.

Observation 9. Utilizing SBFD option 2 (total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD) improves SBFD performance.

Observation 10. In the SBFD scenario, a UL transmission over a subband causes significant UE-to-UE CLI leakage on the adjacent DL subband depending on a frequency gap between the UL RBs and DL RBs of each subband.

Observation 11. As a part of gNB implementation, the gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation, which can impact to UE behaviours depending on the amount of power backoff.

Observation 12. Inter-subband UE-to-UE CLI measurement in SBFD slots based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

Observation 13. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Observation 14. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in inter-slot interference and dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

Observation 15. In case the scheduled frequency resources for an UL transmission are located outside of the boundaries of the SBFD UL subbands, the UE may need further configurations to re-interpret the frequency resources to be mapped within the SBFD UL boundaries.

Proposal 1. Study mechanisms on how to inform UE of mixed D/U regions per symbol/slot as an enhancement of SFI to achieve subband non-overlapping FD (SBFD).

- Consider the cases where multiple BWPs/subbands are configured for SBFD within the same carrier.

Proposal 2. Study the aspects of BWP switching for SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration.

Proposal 3. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands.

Proposal 4. Analysis on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study.

Proposal 5. To overcome the degraded downlink performance due to the static/fixed subband partitioning, flexible/versatile subband partitioning and its dynamic indication mechanisms should be further discussed to cope with varying traffic/channel conditions.

Proposal 6. Study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically.

Proposal 7. Consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario.

Proposal 8. Study measurement resources and reporting configurations for subband-edge CLI measurement.

Proposal 9. Study an event-based CLI sensing behaviour at the victim UE side, where the event can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

Proposal 10. Study UL/DL timing alignment issues in subband non-overlapping full duplex systems.

Proposal 11. Study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped (partially or totally) to outside of the SBFD UL boundaries.

# References

- RP-220633, “Revised SID: Study on evolution of NR duplex operation”, CMCC.

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109-e, May 2022

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110, August 2022

# Appendix – Simulation assumptions

Table 3. LLS simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 |
| Antenna Configuration | 1Tx-2Rx |
| Carrier Frequency | 4GHz |
| SCS | 15kHz |
| MCS | 4, 10, 19 and 27 |
| System BW | 20MHz |
| Allocated no. of RBs for UL and DL signals | DL (Victim): 1RB, 2RBs, 5RBs, 10 RBs, 25RBs, 50RBs |
|  | UL (Aggressor): 25RBs |
| DL to UL power ratio (dB) | -6dB, 0dB, 6dB, 18dB |
| UL timing advance over DL | 0 , 1/2 of a symbol, 1/4 of a symbol |
| Propagation condition | TDL-A |
| Delay Spread | 30ns |
| UE Velocity | 3km/h |
| DMRS | DM-RS type-1, # of DMRS 1 + 1 |
| Channel estimation | Realistic |
| Receiver Type | MMSE |
| PA nonlinearity | None |

Table 4. SLS simulation assumptions

| Parameter | Deployment Scenario |
| --- | --- |
|  | Urban Macro (UMa) (from 38.913) |
| Layout | 21 cells with wraparound ISD: 500m |
| Channel Model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor |
| UE Mobility | 3 Km/hr |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25 m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |
| Open-loop power control | Default: P_0 = -92 dBm, alpha=1.0 |
| BS/UE TX power | BS: 49dBm, UE: 23dBm |
| BS antenna configuration | Baseline TDD: (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 1, 4, 4, 0.8, 0.5). Shared Tx/Rx antenna array with 32 TxRUs SBFD: Separate antenna array for Tx/Rx Option 1 (same # of antenna elements as TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 2, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. Option 2 (# of antenna elements for SBFD is two times that of TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. |
| Slot structure | Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD Subband configuration#1 with {DUD} pattern, one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. All cells use the same SBFD slot configuration |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports 4 Tx, 2 Rx |
| Channel estimation | Realistic |
| Transmission scheme | 16/32 Tx Type 1 Codebook |
| Scheduler | SU-MIMO (with PF) |
| Target BLER | 10% first transmission BLER |
| HARQ/repetition | 3 HARQ retransmission |
| Metric | DL/UL User Perceived Throughput |
| Traffic model | FTP3 (0.5MB as packet size) for DL/UL Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) DL:UL traffic ratio 1:1 and 2:1 are considered |