3GPP TSG RAN WG1 Meeting #110b-e					         R1-2209037

e-Meeting, October 10th – 19th, 2022

Agenda Item:	8.13

Source: 	Intel Corporation

Title:	Discussion on different SCSs between P(S)Cell and sSCell

Document for: 	Discussion/Decision

# Introduction

A UE can be configured with 4 BWPs with same or different SCS and the configuration of the BWPs can be different on different cells. It is possible that some combinations of BWPs on P(S)Cell and sSCell have μP≤μS, while some other combinations have μP>μS, where μP is the SCS of P(S)Cell and μS is the SCS of sSCell. It is not clear whether such configuration on BWPs on P(S)Cell and sSCell is allowed or not for a UE configured with sSCell scheduling P(S)Cell. In this contribution, we provide the analysis and propose to support such configuration. The corresponding change to the specification are also provided.

# Discussions

The NR DSS is only standardized for certain typical cases.

- μP≤μS, where μP, μS are respetively the SCS configuration for the active DL BWP on P(S)Cell and sSCell.

- r16monitoringcapability is not configured for PDCCH monitoring on P(S)Cell and not configured for PDCCH monitoring on sSCell; r16monitoringcapability can be configured for PDCCH monitoring on Scells other than sSCell

- Multiple CORESET pools are not configured for PDCCH monitoring on P(S)Cell and not configured for PDCCH monitoring on sSCell; Other Scells can be configured with multiple CORESET pools

In the current specification, DSS operation is specified when all the above three limitations are fulfilled. On the other hand, it is necessary to observe what is the specified behaviour if one or more above conditions are not satisfied. For the second condition on PDCCH monitoring capability, it is error case if r16monitoringcapability is configured for either P(S)Cell or sSCell. It would be fine to not explicitly capture the error case. Similarly, the third condition, i.e., multiple CORESET pools configured on either P(S)Cell or sSCell is error case and is not explicitly captured. However, Further clarifications are needed for the first condition.

If multiple BWPs with different SCSs are configured for P(S)Cell and/or sSCell, it is possible that a pair of BWPs on the P(S)Cell and sSCell have μP≤μS, while another pair of BWPs on the P(S)Cell and sSCell have μP>μS. The UE may switch between a combination μP≤μS and another combination with μP>μS, which can be tiggered by a DCI dynamically. Two options can be considered for such case. In a first option, dynamic switching to a combination with μP>μS is considered as valid configuration. A UE can do DSS operation when the active BWPs satisfies μP≤μS. On the other hand, if gNB switch the active BWPs which have μP>μS, the UE can do PDCCH monitoring as Rel-15/16. On the other hand, a second option is treated such BWP configuration/switching as error case. The first option is preferred for the scheduling flexibility. The corresponding behaviour should be clarified in the specification.

Proposal 1:

- A UE configured with sSCell scheduling P(S)Cell can switch between a combination of BWPs with μP≤μS and another combination of BWPs with μP>μS.

- Agree on the following CR for the fallback procedure in case that P(S)Cell and sSCell switch to BWPs with SCS μP>μS.

### 10.1.1 Self-carrier and cross-carrier scheduling on the primary cell A UE can be configured for scheduling on the primary cell from the primary cell and from a secondary cell [12, TS 38.331]. The UE is either not provided monitoringCapabilityConfig or the UE is provided only monitoringCapabilityConfig = r15monitoringcapability for the primary cell and for the secondary cell. The UE is not provided coresetPoolIndex on the primary cell or on the secondary cell. The SCS configuration μP for the active DL BWP on the primary cell is smaller than or equal to the SCS configuration μS for the active DL BWP on the secondary cell. Otherwise, the procedures are as described in clause 10.1 applies. If a UE indicates capability <FG 34-3> [18, TS 38.306] and the secondary cell is deactivated, or if the UE indicates capability <FG 34-4> [18, TS 38.306] and the active DL BWP of the secondary cell is a dormant DL BWP for the UE, α=1 applies for the procedures described in the remaining of this clause. If μP<μS, the UE determines MPDCCHtotal,slot,μP and CPDCCHtotal,slot,μP, and determines MPDCCHtotal,slot,μS and CPDCCHtotal,slot,μS, by including the primary cell only in the Ncells,0DL,μP downlink cells in j=03Ncells,0DL,j+γ∙Ncells,1DL,j, as described in clause 10.1. If μP=μS=μ, the UE determines MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ by including the primary cell once in the Ncells,0DL,μ downlink cells in j=03Ncells,0DL,j+γ∙Ncells,1DL,j, as described in clause 10.1. For scheduling on the primary cell from the primary cell, the UE is not required to monitor more than α∙minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP PDCCH candidates per slot or more than α∙minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP non-overlapping CCEs per slot on the active DL BWP of the primary cell, where α is provided by ccs-BlindDetectionSplit. For scheduling on the primary cell from the secondary cell, the UE is not required to monitor on the active DL BWP of the secondary cell more than - MPDCCHmax,slot,μS PDCCH candidates per slot or more than CPDCCHmax,slot,μS non-overlapping CCEs per slot of the active DL BWP of the secondary cell - minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP-α∙minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP PDCCH candidates per slot or more than minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP-α∙minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP non-overlapping CCEs per slot of the active DL BWP of the primary cell If μP<μS, the UE does not count PDCCH candidates and non-overlapping CCEs that the UE monitors for scheduling on the primary cell from the secondary cell towards MPDCCHtotal,slot,μS and CPDCCHtotal,slot,μS, respectively. If μP<μS, the UE counts PDCCH candidates and non-overlapping CCEs that the UE monitors for scheduling on the primary cell from the secondary cell towards MPDCCHtotal,slot,μP and CPDCCHtotal,slot,μP, respectively. For allocation of PDCCH candidates and non-overlapping CCEs to search space sets for scheduling on the primary cell from the primary cell, the UE applies the procedure in clause 10.1 using α∙minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ instead of minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ, and using α∙minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ instead of minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ for the primary cell. <Unchanged Text Omitted>

# Conclusion

In this contribution, we discuss the dynamic BWP switching on P(S)Cell and sSCell with SCS μP≤μS and μP>μS. It is preferred to support such combination for scheduling flexibility. We make the following proposal.

Proposal 1:

- A UE configured with sSCell scheduling P(S)Cell can switch between a combination of BWPs with μP≤μS and another combination of BWPs with μP>μS.

- Agree on the following CR for the fallback procedure in case that P(S)Cell and sSCell switch to BWPs with SCS μP>μS.

### 10.1.1 Self-carrier and cross-carrier scheduling on the primary cell A UE can be configured for scheduling on the primary cell from the primary cell and from a secondary cell [12, TS 38.331]. The UE is either not provided monitoringCapabilityConfig or the UE is provided only monitoringCapabilityConfig = r15monitoringcapability for the primary cell and for the secondary cell. The UE is not provided coresetPoolIndex on the primary cell or on the secondary cell. The SCS configuration μP for the active DL BWP on the primary cell is smaller than or equal to the SCS configuration μS for the active DL BWP on the secondary cell. Otherwise, the procedures are as described in clause 10.1 applies. If a UE indicates capability <FG 34-3> [18, TS 38.306] and the secondary cell is deactivated, or if the UE indicates capability <FG 34-4> [18, TS 38.306] and the active DL BWP of the secondary cell is a dormant DL BWP for the UE, α=1 applies for the procedures described in the remaining of this clause. If μP<μS, the UE determines MPDCCHtotal,slot,μP and CPDCCHtotal,slot,μP, and determines MPDCCHtotal,slot,μS and CPDCCHtotal,slot,μS, by including the primary cell only in the Ncells,0DL,μP downlink cells in j=03Ncells,0DL,j+γ∙Ncells,1DL,j, as described in clause 10.1. If μP=μS=μ, the UE determines MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ by including the primary cell once in the Ncells,0DL,μ downlink cells in j=03Ncells,0DL,j+γ∙Ncells,1DL,j, as described in clause 10.1. For scheduling on the primary cell from the primary cell, the UE is not required to monitor more than α∙minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP PDCCH candidates per slot or more than α∙minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP non-overlapping CCEs per slot on the active DL BWP of the primary cell, where α is provided by ccs-BlindDetectionSplit. For scheduling on the primary cell from the secondary cell, the UE is not required to monitor on the active DL BWP of the secondary cell more than - MPDCCHmax,slot,μS PDCCH candidates per slot or more than CPDCCHmax,slot,μS non-overlapping CCEs per slot of the active DL BWP of the secondary cell - minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP-α∙minMPDCCHmax,slot,μP,MPDCCHtotal,slot,μP PDCCH candidates per slot or more than minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP-α∙minCPDCCHmax,slot,μP,CPDCCHtotal,slot,μP non-overlapping CCEs per slot of the active DL BWP of the primary cell If μP<μS, the UE does not count PDCCH candidates and non-overlapping CCEs that the UE monitors for scheduling on the primary cell from the secondary cell towards MPDCCHtotal,slot,μS and CPDCCHtotal,slot,μS, respectively. If μP<μS, the UE counts PDCCH candidates and non-overlapping CCEs that the UE monitors for scheduling on the primary cell from the secondary cell towards MPDCCHtotal,slot,μP and CPDCCHtotal,slot,μP, respectively. For allocation of PDCCH candidates and non-overlapping CCEs to search space sets for scheduling on the primary cell from the primary cell, the UE applies the procedure in clause 10.1 using α∙minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ instead of minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ, and using α∙minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ instead of minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ for the primary cell. <Unchanged Text Omitted>