3GPP TSG RAN WG1 #110bis-e	                        			  R1-2209104

e-Meeting, October 10th – 19th, 2022

Agenda Item: 	9.5.1.1

Source: 	Sony

Title: 	Discussion on evaluation of SL positioning

Document for: 	Discussion & Decision

# Introduction

In RAN#110 meeting [1], some proposals were agreed upon SL positioning evaluation methodology, particularly on evaluation assumptions and potential methodology for V2X, IIoT and commercial use case. These agreements are shown below:

Agreement For SL positioning evaluation in IIOT use case, companies should report how to drop anchor UEs and how to select anchor UEs. Adopt the tables in section 3 of R1-2207606 as templates to collect SL positioning simulation results from each company. In the evaluation, relative positioning or ranging is performed between two UEs within X m, where X value(s) are reported by companies, and companies should also report the minimum distance used in the evaluations for each use case. The assumption used for X will be included in the TR for each set of results. For SL positioning evaluation purpose, the following assumptions are further adopted - Companies should report whether SL-PRS and other SL signals are FDMed or not FDMed, and whether other SL signals are present - Adopting system level simulations (rather than the link level simulations) as the baseline tool - For SL positioning evaluation in highway scenario or urban grid scenario, the performance metrics can include absolute horizontal accuracy, relative horizontal accuracy, ranging with distance accuracy, and ranging with direction accuracy (optionally). - In highway and urban grid scenarios, companies can further consider other UE types, e.g. pedestrian UE or VRU devices.

In this contribution, we present our system level simulation results for V2X use case, particularly on the accuracy of absolute/relative positioning and ranging accuracy. Furthermore, the observation and analysis are also provided.

# Discussion

## simulation assumptions

The simulation assumptions are divided into two parts:

- Common assumptions (Table A-1) and

- Scenario specific assumptions (Table A-2, A-7).

Table A-1 shown below is the common assumptions which apply in all the test cases. In the simulation, we used ZC-based (SRS-like) sequence as reference signal and adopted comb-2 and 2 symbols for each measurement occasion.

This contribution mainly focus on V2X positioning, which includes two scenarios: highway scenario and urban grid scenario. The geometry layout assumption, channel model, UE drop model follows description in [2] and [3]. All of the evaluation cases are for FR1, and also based on SL-only positioning.

Table A-2 and A-7 are the assumptions for highway and urban grid scenarios respectively. In this simulation, we defined 8 test cases. Case 1~4 correspond to highway scenario and case 5~8 are for urban grid. In each scenario, we further define 4 different test cases, aiming to investigate the impact from bandwidth size and LOS/NLOS condition.

## Simulation results for absolute positioning

The absolute positioning estimation is based on sidelink reference signal from RSUs to UE. The assumption is that vehicles can perform TDOA measurements based on multiple SL reference signals transmitted from the nearby RSUs. With the knowledge of the RSUs’ location, the vehicle can calculate its own coordinate. However, absolute positioning is only feasible for the case where vehicle can communicate with at least 3 RSUs. In urban grid scenario, there is only 1 RSU deployed in each intersection, which makes UE difficult to access to more than 1 RSU in its own communication range. Hence, we did not apply absolute positioning in urban grid simulation due to the lack of RSU.

Observation 1: Absolute positioning may not be feasible in urban grid scenario, due to the lack of RSUs.

Figure 1 below is the simulation results for absolute positioning in highway scenario.  The corresponding positioning error for 50%, 67%, 80% and 90% of UEs are summarized in Table A-3.

Figure 1. Simulation results for absolute positioning in highway scenario in FR1.

In Figure 1, we can observe that reducing bandwidth significantly degrades the positioning performance. The accuracy drops from 0.85m using 100MHz (Case 1) to 6m using 20MHz (Case 2). There is also a small gain in case of with or without perfect LOS condition (Case 3 and Case 4). Here, the impact from LOS condition is not as obvious as the bandwidth.

Observation 2: For V2X absolute positioning, reducing bandwidth significantly degrades the positioning performance.  Positioning accuracy drops from 0.85 m using 100MHz to 6 m when using 20MHz.

Observation 3: For V2X absolute positioning, requirement set A (<1.5m) can be achieved when using 100MHz bandwidth. Requirement set B (<0.5m) can almost be fulfilled considering 84% of the UEs satisfying the accuracy requirement.

## Simulation results for relative positioning

For relative positioning, the relative positioning estimation is based on RTT and AOA measurement from one PC5 link. Figure 2 shows the simulation results for relative positioning in highway (left) and urban grid scenario (right). To make it more intuitive, we also provide a table summarizing the V2X relative positioning measurement error captured in the figures. In the table, the error for {50% 67% 80% 90%} of the links can be found in the corresponding columns. The corresponding horizontal errors for X% UEs are summarized in Table A-4 and A-8 in the appendix.

Figure 2. Simulation results for relative positioning in highway scenario and urban grid scenario.

For the CDF error at 90%, relative positioning via V2X link has a poor accuracy (46.47m in urban grid scenario and 9.35 m in highway scenario), which is far off from the LOS case where accuracy is 3.74 m and 6.06 m for urban grid scenario and highway scenario, respectively. This indicates that NLOS component(s) is one of the key issues that degrades the performance.

Observation 4: For V2X relative positioning, the NLOS component degrades the positioning accuracy. In urban grid scenario, the accuracy in LOS-only case is 3.74m, while in LOS+NLOS case the accuracy decreases to 46.27m.

Comparing the simulation results with the positioning requirement, one can observe that there is still a huge performance gap between the simulation results and the requirement set A or B. This indicates that relative positioning only based on one RTT and one AOA measurement may not be sufficient.

Observation 5: For V2X relative positioning only using one PC5 link, the accuracy cannot be met either the requirement set A (<1.5m) or set B (<0.5m).

## Simulation results for V2X SL-RTT ranging

Figure 3 shows the simulation results for ranging distance measurement in highway (left) and urban grid (right). The performance is evaluated by the distance measurement accuracy (m). In both figures, not only the overall performance is provided (case 1,2,5,6), but also the positioning performance under perfect LOS conditions (corresponds to the case with LOS-only). The corresponding horizontal errors for X% UEs are summarized in Table A-5 and A-9 in the appendix.

Figure 3: Simulation results of SL V2X ranging in urban grid and highway scenario, using RTT method.

From this SL FR1 simulation results, we can observe that with 100MHz BW the horizontal error for 90% SL UE is 15.41 m in urban grid scenario and 0.54 m in highway scenario. Relative positioning in highway scenario meets the requirement set B and almost fulfilled requirement set A. For urban grid case, it is still tough to meet the requirement, considering the high NLOS probability and strong power distortion in NLOS channel. If there is a perfect LOS condition (LOS-only), the accuracy in urban grid case can be improved to 0.85m which meets the requirement set B.

Observation 6: For V2X ranging distance measurement, only highway scenario can fulfil the ranging requirement set B () and requirement set A. The urban grid scenario can meet the requirement set B only in LOS-only condition.

Observation 7: NLOS is one of the key issues for high accuracy SL ranging especially in urban grid scenario. The simulation results indicate a significant performance loss in NLOS case in both FR1 urban grid and highway scenarios.

Furthermore, comparing the two scenarios, it can be observed that performance in highway scenario is better than in urban grid scenario. This could be due to the higher LOS probability in highway scenario than in urban grid scenario. Besides that, urban grid and highway scenario have different NLOS setups. As defined in TR 37.885, two types of NLOS models are adopted for V2V links. One is V2V NLOSv model which is for the case where LOS path blocked by vehicles. The other one is V2V NLOS model for the case where LOS path blocked by buildings. In our simulation, urban grid case adopts both NLOS and NLOSv models, while highway case only uses NLOSv model because no building is modeled in the highway geometry layout.

Observation 8: The NLOS issue is particularly severe in urban grid scenario due to blockage by the building.

## 2.5 Simulation results for V2X SL-AoA ranging

For FR1 SL-AoA evaluation, we assumed that SL UEs have the perfect knowledge of the zenith of arrival information per measurement. That is because FR1 SL devices equip with a linear antenna array with only 2 horizontal elements, which is not sufficient for the zenith of arrival detection.  Figure 4 shows the relation among zenith of arrival θ, azimuth of arrival ϕ and the angle being measured from the Rx beam α in a cartesian coordinate system.

Figure 4: A Cartesian coordinate system with zenith of arrival θ, azimuth of arrival ϕ and the angle being measured from the Rx beam.

The Rx UE is located at the center of the coordinate system and the vector of the Rx beam is pointing toward the Tx UE. As shown in the figure, the angle measurement from Rx beam is not AoA, but an angle α which is an angle between Rx beam vector and x-axis. This angle α needs to be further calibrated to obtain the AoA ϕ by utilizing ZoA measurement θ. The calibration can be done by the equation:

ϕ=arccos⁡(sinθcos⁡(α)).

As we can observe from the equation, if θ=90°, the ϕ = α. It indicates that if two UEs have the same height (θ=90°), the angle measurement α is equivalent to the AoA ϕ. So the calibration is only needed for the case where two SL UEs have different height.

Observation 9: Azimuth of arrival (AoA) requires calibration by using zenith of arrival (ZoA) measurement, when two UEs have different height.

However, FR1 UE antenna cannot obtain ZoA θ due to the lack of vertical elements in the array. This may degrade the AoA measurement accuracy. This may also impact the measurement differently for UEs with different height. As a typical example, AoA measurement from V2V does not need to be calibrated due to the similar antenna height. On the contrary, the AoA from V2I (vehicle-to-infrastructure) suffer from the accuracy loss, because UE type infrastructure such as RSU (5 m) is much higher than the vehicle.

Observation 10: FR1 SL device cannot obtain zenith of arrival (ZoA) due to the lack of vertical elements in the antenna array. This may typically degrade the AoA measurement accuracy for V2I case.

Having the aforementioned assumptions (perfect ZoA knowledge), we performed simulation on SL-AoA for ranging direction measurement. The simulation results are shown in Figure 5. Likewise, the simulation is performed in both urban grid scenario (left) and highway scenario (right) using FR1 and 100 MHz bandwidth. The corresponding accuracy error is summarized in Table A-6 and A-10.

Figure 5: Simulation results of SL V2X ranging direction measurement in urban grid and highway scenario.

From the simulation results, we can observe that the azimuth of arrival measurement error for 90% SL UE is 17.43 degree in urban grid scenario and 4.4 degree in highway scenario. Almost with similar conclusion as in ranging distance measurement, the angle measurement requirement (both set A and B) can be only met in highway scenario, but not in urban grid. The urban gird case can fulfil requirement set A if there is perfect LOS condition (LOS-only).

Observation 11: In the FR1 SL-AoA simulation results, the ranging direction measurement error for 90% SL UE is 17.43 degree in urban grid scenario and 4.4 degree in highway scenario. Both angle requirement set A (< 15º) and requirement set B (< 8º) can be fulfilled in highway scenario. Urban grid can meet requirement set A only in perfect LOS condition.

We observe almost the same conclusion as in SL-RTT case, the NLOS links also degrades the AoA measurement accuracy both in urban grid and in highway scenario. We consider study on NLOS mitigation method targeting for SL-AoA measurement.

Observation 12: A significant AoA measurement performance loss due to the NLOS links can be observed from the simulation results, especially in urban grid scenario.

# Conclusion

In this contribution, we discuss a potential refinement for SL positioning evaluation and also present simulation results and the corresponding analysis. Our observations are given below:

Observation 1: Absolute positioning may not be feasible in urban grid scenario, due to the lack of RSUs.

Observation 2: For V2X absolute positioning, reducing bandwidth significantly degrades the positioning performance.  Positioning accuracy drops from 0.85 m using 100MHz to 6 m when using 20MHz.

Observation 3: For V2X absolute positioning, requirement set A (<1.5m) can be achieved when using 100MHz bandwidth. Requirement set B (<0.5m) can almost be fulfilled considering 84% of the UEs satisfying the accuracy requirement.

Observation 4: For V2X relative positioning, the NLOS component degrades the positioning accuracy. In urban grid scenario, the accuracy in LOS-only case is 3.74m, while in LOS+NLOS case the accuracy decreases to 46.27m.

Observation 5: For V2X relative positioning only using one UE-UE link, the accuracy always fails to meet either the requirement set A (<1.5m) or set B (<0.5m).

Observation 6: For V2X ranging distance measurement, only highway scenario can fulfil the ranging requirement set B () and requirement set A. The urban grid scenario can meet the requirement set B only in LOS-only condition.

Observation 7: NLOS is one of the key issues for high accuracy SL ranging especially in urban grid scenario. The simulation results indicate a significant performance loss in NLOS case in both FR1 urban grid and highway scenarios.

Observation 8: The NLOS issue is particularly severe in urban grid scenario due to blockage by the building.

Observation 9: Azimuth of arrival (AoA) requires calibration by using zenith of arrival (ZoA) measurement, when two UEs have different height.

Observation 10: FR1 SL device cannot obtain zenith of arrival (ZoA) due to the lack of vertical elements in the antenna array. This may typically degrade the AoA measurement accuracy for V2I case.

Observation 11: In the FR1 SL-AoA simulation results, the ranging direction measurement error for 90% SL UE is 17.43 degree in urban grid scenario and 4.4 degree in highway scenario. Both angle requirement set A (< 15º) and requirement set B (< 8º) can be fulfilled in highway scenario. Urban grid can meet requirement set A only in perfect LOS condition.

Observation 12: A significant AoA measurement performance loss due to the NLOS links can be observed from the simulation results, especially in urban grid scenario.

# References

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #110, Toulouse France, August 2022.

- 3GPP TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR

- 3GPP TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #109, e-Meeting, May 9th – 20th, 2022

# Appendix

##### Common assumption

Table A-1. Common assumption for all scenarios if they are different from or not specified in Agreements

| Parameter |  |
| --- | --- |
| Carrier frequency | 6GHz |
| Subcarrier spacing | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz or 20MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-2 |
| Reference signal including PRS, SRS and SL-PRS (type of sequence, number of ports, …) | ZC-based |
| Number of symbols used per occasion | 2 symbols |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 0dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution in channel estimation. Threshold based first path detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Maximum likelihood estimator for absolute coordinate calculation |
| Synchronization assumptions | Ideal synchronization |
| UE/gNB RX and TX timing error assumption | Ideal timing error calibration |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Codebook based beam sweeping |
| Additional notes, if any | Run 10 simulations per test case. Each simulation deploys with different channel and UE drops. |

##### 5.2 Highway scenario for V2X use case

Table A-2. Assumptions for highway if they are different from or not specified in Agreements

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| Simulation Bandwidth | 100 MHz | 20 MHz | 100 MHz | 20MHz |
| LOS condition | Channel model defined in 37.855 | Channel model defined in 37.855 | LOS only | LOS only |
| Vehicle antenna model, array configuration (M, N, P, Mg, Ng) | Rooftop antenna (1, 2, 2, 1, 1) | Rooftop antenna (1, 2, 2, 1, 1) | Rooftop antenna (1, 2, 2, 1, 1) | Rooftop antenna (1, 2, 2, 1, 1) |
| RSU antenna model, array configuration (M, N, P, Mg, Ng) | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) | (1, 2, 2, 1, 1) |
| RSU deployment for absolute positioning | Staggered RSU distribution with 200m RSU spacing like: |  |  |  |
| RSU deployment for relative positioning/ranging | Same as aboslute position |  |  |  |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | Vehicle: X = 50 RSU: X = 300 |  |  |  |
| Positioning method | Relative positioning based on one RTT and one AOA measurement Absolute positioning using TDOA. |  |  |  |

Table A-3. Simulation results for highway for absolute positioning - horizontal accuracy (m)

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1, BW100MHz, FR1, positioning method: TDOA, | 0.22 | 0.35 | 0.52 | 0.85 | Yes | No, 79% of UEs satisfying the target positioning accuracy requirement |
| Case 2, BW20M, FR1, positioning method: TDOA, | 2.03 | 2.82 | 4.41 | 6.04 | No, 34% of UEs satisfying the target positioning accuracy requirement | No, 7% of UEs satisfying the target positioning accuracy requirement |
| Case 3, BW100M, FR1, positioning method: TDOA, LOS only | 0.19 | 0.30 | 0.43 | 0.63 | Yes | No, 84% of UEs satisfying the target positioning accuracy requirement |
| Case 4, BW20M, FR1, positioning method: TDOA, LOS only | 1.92 | 2.29 | 3.63 | 5.42 | No, 40% of UEs satisfying the target positioning accuracy requirement | No, 8% of UEs satisfying the target positioning accuracy requirement |

Table A-4. Simulation results for highway for relative positioning - horizontal accuracy (m)

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1, BW100MHz, FR1, positioning method: RTT+AoA, | 0.48 | 1.38 | 3.46 | 9.35 | No, 68% of UEs satisfying the target positioning accuracy requirement | No, 50% of UEs satisfying the target positioning accuracy requirement |
| Case 2, BW20M, FR1, positioning method: RTT+AoA, | 1.67 | 2.90 | 5.73 | 11.70 | No, 46% of UEs satisfying the target positioning accuracy requirement | No, 12% of UEs satisfying the target positioning accuracy requirement |
| Case 3, BW100M, FR1, positioning method: RTT+AoA, LOS only | 0.38 | 0.92 | 2.14 | 6.06 | No, 74% of UEs satisfying the target positioning accuracy requirement | No, 56% of UEs satisfying the target positioning accuracy requirement |
| Case 4, BW20M, FR1, positioning method: RTT+AoA, LOS only | 1.46 | 2.33 | 4.07 | 8.16 | No, 50% of UEs satisfying the target positioning accuracy requirement | No, 13% of UEs satisfying the target positioning accuracy requirement |

Table A-5. Simulation results for highway for ranging - distance accuracy (m)

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1, BW100MHz, FR1, positioning method: RTT | 0.09 | 0.14 | 0.24 | 0.54 | Yes | ‘Yes’ (almost) |
| Case 2, BW20M, FR1, positioning method: TDOA, RTT | 0.88 | 1.34 | 2.05 | 3.98 | No, 71% of UEs satisfying the target positioning accuracy requirement | No, 29% of UEs satisfying the target positioning accuracy requirement |
| Case 3, BW100M, FR1, positioning method: RTT, LOS only | 0.09 | 0.13 | 0.21 | 0.44 | Yes | Yes |
| Case 4, BW20M, FR1, positioning method: RTT, LOS only | 0.84 | 1.26 | 1.88 | 3.50 | No, 73% of UEs satisfying the target positioning accuracy requirement | No, 29% of UEs satisfying the target positioning accuracy requirement |

Table A-6 Simulation results for highway for ranging positioning - angle accuracy (degree)

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1, BW100MHz, FR1, positioning method: AOA, | 0.55 | 1.05 | 1.97 | 4.40 | Yes | Yes |
| Case 2, BW20M, FR1, positioning method: AOA, | 1.20 | 2.10 | 3.52 | 6.77 | Yes | Yes |
| Case 3, BW100M, FR1, positioning method: AOA, LOS only | 0.49 | 0.89 | 1.62 | 3.52 | Yes | Yes |
| Case 4, BW20M, FR1, positioning method: AOA, LOS only | 1.09 | 1.85 | 3.04 | 5.73 | Yes | Yes |

##### 5.3 Urban grid scenario for V2X use case

Table A-7 Assumptions for urban grid if they are different from or not specified in the agreements

| Parameters | Case 5 | Case 6 | Case 7 | Case 8 |
| --- | --- | --- | --- | --- |
| Simulation Bandwidth | 100 MHz | 20 MHz | 100 MHz | 20MHz |
| LOS condition | Channel model defined in 37.855 | Channel model defined in 37.855 | LOS only | LOS only |
| Vehicle antenna model, array configuration (M, N, P, Mg, Ng) | Rooftop antenna - (1, 2, 2, 1, 1) | Rooftop antenna - (1, 2, 2, 1, 1) | Rooftop antenna - (1, 2, 2, 1, 1) | Rooftop antenna (1, 2, 2, 1, 1) |
| RSU antenna model, array configuration (M, N, P, Mg, Ng) | - (1, 2, 2, 1, 1) | - (1, 2, 2, 1, 1) | - (1, 2, 2, 1, 1) | - (1, 2, 2, 1, 1) |
| RSU deployment for absolute positioning | RSU deployment follows the description in TR 36.885 section A1.3. |  |  |  |
| RSU deployment for relative positioning/ranging | RSU deployment follows the description in TR 36.885 section A1.3. |  |  |  |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | 30 | - 30 | - 30 | - 30 |
| Positioning method | Relative positioning based on RTT and AOA measurement. |  |  |  |

Table A-8 Simulation results for urban grid for relative positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 5, BW100MHz, FR1, positioning method: RTT+AOA, | 0.51 | 2.64 | 12.73 | 46.47 | No, 62% of UEs satisfying the target positioning accuracy requirement | No, 50% of UEs satisfying the target positioning accuracy requirement |
| Case 6, BW20M, FR1, positioning method: RTT+AOA, | 2.82 | 6.09 | 15.47 | 42.85 | No, 31% of UEs satisfying the target positioning accuracy requirement | No, 7% of UEs satisfying the target positioning accuracy requirement |
| Case 7, BW100M, FR1, positioning method: RTT+AOA, LOS only | 0.23 | 0.43 | 0.92 | 3.74 | No, 84% of UEs satisfying the target positioning accuracy requirement | No, 31% of UEs satisfying the target positioning accuracy requirement |
| Case 8, BW20M, FR1, positioning method: RTT+AOA, LOS only | 1.71 | 2.70 | 4.18 | 8.72 | No, 45% of UEs satisfying the target positioning accuracy requirement | No, 11% of UEs satisfying the target positioning accuracy requirement |

Table A-9 Simulation results for urban grid for ranging - distance accuracy

|  | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 5, BW100MHz, FR1, positioning method: RTT, | 0.13 | 0.30 | 2.26 | 15.41 | No, 78% of UEs satisfying the target positioning accuracy requirement | No, 73% of UEs satisfying the target positioning accuracy requirement |
| Case 6, BW20M, FR1, positioning method: RTT, | 1.66 | 2.48 | 4.23 | 9.40 | No, 47% of UEs satisfying the target positioning accuracy requirement | No, 17% of UEs satisfying the target positioning accuracy requirement |
| Case 7, BW100M, FR1, positioning method: RTT, LOS only | 0.09 | 0.15 | 0.29 | 0.85 | Yes | No, 86% of UEs satisfying the target positioning accuracy requirement |
| Case 8, BW20M, FR1, positioning method: RTT, LOS only | 1.28 | 2.07 | 3.17 | 5.94 | No, 55% of UEs satisfying the target positioning accuracy requirement | No, 21% of UEs satisfying the target positioning accuracy requirement |

Table A-10 Simulation results for urban grid for ranging positioning - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 5, BW100MHz, FR1, positioning method: AOA, | 1.13 | 2.93 | 7.69 | 17.43 | No, 88% of UEs satisfying the target positioning accuracy requirement | No, 80% of UEs satisfying the target positioning accuracy requirement |
| Case 6, BW20M, FR1, positioning method: AOA, | 2.61 | 5.04 | 9.20 | 17.12 | No, 88% of UEs satisfying the target positioning accuracy requirement | No, 77% of UEs satisfying the target positioning accuracy requirement |
| Case 7, BW100M, FR1, positioning method: AOA, LOS only | 0.59 | 1.07 | 2.18 | 5.99 | Yes | Yes |
| Case 8, BW20M, FR1, positioning method: AOA, LOS only | 1.77 | 3.16 | 5.13 | 8.80 | Yes | No, 88% of UEs satisfying the target positioning accuracy requirement |