# 3GPP TSG RAN WG1 #110bis-e											 					     R1-2209133

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | Draft | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on monitoring occasion for individual PDCCH candidate overlapping with one of PDCCH repetitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Interpretation of counter/total DAI for any one of PDCCH repetitions and individual PDCCH is based on a union of two PDCCH monitoring occasions for two linked PDCCH candidates (i.e.Rel-17 rule). While for two linked PDCCH candidates, the union of two PDCCH monitoring occasions is applied (i.e. replacing legacy PDCCH monitoring occasion for each linked PDCCH candidate), but for the individual PDCCH candidate, the definition of PDCCH monitoring occasion is not changed (still legacy monitoring occasion), or in other words, the union of two PDCCH monitoring occasions is not applied for the individual PDCCH candidate which overlaps with one of the two linked PDCCH candidates. Which will cause the counter/total DAI definition incorrect, further details can be found in our companion contribution [R1-2209134]. Or in other words, counter/total DAI for two linked PDCCH candidates denotes accumulated/total occasions related to the union of two PDCCH monitoring occasions, but counter/total DAI for the individual PDCCH candidate denotes accumulated/total occasions related to the individual PDCCH monitoring occasion for the individual PDCCH candidate, how can a same rule of counter/total DAI for two linked PDCCH candidates and individual PDCCH candidate denote different monitoring occasions? Now that the counter/total DAI value for the individual PDCCH candidate follows same rule as the two PDCCH repetitions, the PDCCH monitoring occasion for the individual PDCCH candidate should also follow same rule as the two PDCCH repetitions (i.e. a union of the two PDCCH monitoring occasions), otherwise, the counter/total DAI value denoting the accumulated/total monitoring occasions up to current monitoring occasion is broken. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applying the union of two PDCCH monitoring occasions for two linked PDCCH candidates to the individual PDCCH candidate which overlaps with one of the two linked PDCCH candidate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of counter/total DAI for individual PDCCH candidate which overlaps with one of two linked PDCCH candidates is wrong. i.e. counter/total DAI value doesn’t denote the accumulated/total monitoring occasions up to the current monitoring occasion for the individual PDCCH candiate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9	UE procedure for reporting control information

<Unchanged part omitted>

In the remaining of this clause, when a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1, or when the PDCCH reception is for detection of a first DCI format, and there is a second PDCCH reception by the UE for detection of a second DCI format having a same size as the first DCI format includes a PDCCH candidate, and the PDCCH candidate and either one of the two PDCCH candidates included in the PDCCH reception have identical scrambling and use a same set of CCEs over same symbols in a slot in a CORESET,

-	a PDCCH monitoring occasion for the PDCCH reception or for the second PDCCH reception is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the start of the PDCCH reception or the start of the second PDCCH reception is the start of the earlier PDCCH candidate

-	the end of the PDCCH reception or the end of the second PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, and 11.1.1.

<Unchanged part omitted>

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

<Unchanged part omitted>

A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs in which PDSCH receptions, excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided, or HARQ-ACK information bits that are not in response for PDSCH receptions, associated with the DCI formats, excluding the SPS activation DCI, is present up to the current serving cell and current PDCCH monitoring occasion,

-	first, if the UE indicates by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion, in increasing order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair,

-	second in ascending order of serving cell index, and

-	third in ascending order of PDCCH monitoring occasion index m, where 0≤m<M.

If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the value of the counter DAI is in the order of the first CORESETs and then the second CORESETs for a same serving cell index and a same PDCCH monitoring occasion index.

The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s), excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided, or HARQ-ACK information that does not correspond to PDSCH receptions, associated with DCI formats, excluding the SPS activation DCI, is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the total DAI value counts the {serving cell, PDCCH monitoring occasion}-pair(s) for both the first CORESETs and the second CORESETs.

Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by VC-DAI,c,mDL the value of the counter DAI in a DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, on serving cell c in PDCCH monitoring occasion m according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by VT-DAI,mDL the value of the total DAI in a DCI format in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion m. A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field.

<Unchanged part omitted>