3GPP TSG RAN WG1 #110bis-e		R1-2209161

e-Meeting, October 10th –19th, 2022

Source:	Panasonic

Title: 	Discussion on AI/ML for CSI feedback enhancement

Agenda Item:		9.2.2.1

Document for:	Discussion

Introduction

A new study item “Study on Artificial Intelligence and Machine Learning for NR air-interface” has been approved for Rel.18 [1]. The objective is to study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. The initial set of use cases includes CSI feedback enhancement, beam management, and positioning accuracy enhancements. This document provides our view on AI/ML for CSI feedback enhancement.

Discussion

Selection of representative sub use case

In RAN1#110, the following conclusions were made on selection of representative sub use case.

Conclusion in RAN1#110

- CSI-RS configuration and overhead reduction is NOT selected as one representative sub use case for CSI feedback enhancement use case.

Conclusion in RAN1#110

- Resource allocation and scheduling is NOT selected as one representative sub use case for CSI feedback enhancement use case.

The remaining issue is additional sub use case(s) i.e., other than already agreed spatial-frequency domain CSI compression using two-sided AI model, is further studied or not. In our view, at least one sub use case with one-sided model could be studied since one-sided model is easier to implement with less specification impact and likely to have a much shorter time to market than two-sided model. As the selection of sub use case for one-sided model, we think CSI prediction in rapid channel variation scenarios, e.g., high mobility, is especially important but more challenging. In AI/ML approach, it would be possible to predict subsequent channels by exploiting the strong temporal correlation in channel history.

Proposal 1: CSI prediction using one-sided model could be considered as representative sub use cases.

Potential specification impact for CSI compression with two-sided model

In RAN1#109e, the following agreement was made.

Agreement:

- For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

  - At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

The above agreement can be basic principle of spatial-frequency domain CSI compression using two-sided AI model and it can be illustrated as Fig.1. At least AI/ML inference is located at both the UE and network. The input data would be raw channel matrix estimated by UE or eigenvector(s) of the raw channel matrix estimated by UE. UE’s inference (UE’s encoder) learns how to interpret the input and compress it to an “internal representation” defined by the bottleneck layer. UE feedback internal representations as CSI feedback. The network’s decoder takes the output of the encoder (the bottleneck layer) and attempts to recreate the input. Since AI/ML inference is employed at network and UE to compress CSI, network and UE need to synchronize the AI/ML model and/or parameters of the AI/ML model. How to align the model and other aspects such as scalability and life cycle management should be studied.

Fig.1 Spatial-frequency domain CSI compression using two-sided AI model

Training collaboration

CSI compression is realized by autoencoder. Autoencoder is typically trained end to end with a loss function to minimize the difference between input and reconstructed output. An auto-encoder has two main parts: an encoder that maps the original input into the “internal representation”, and a decoder that maps the “internal representation” to a reconstruction of the original input. Typically, the dimension of the “internal representation” can be smaller than the original input, and then, auto-encoder can realize the compression. An “encoder” of autoencoder corresponds to the UE’s processing in which the original input could be raw data (e.g., received CSI-RS value) or something after pre-computation (e.g., channel coefficient measured from CSI-RS, eigen vector, or coefficients before calculating Type II codebook). The output of UE encoder, which corresponds to “internal representation” in autoencoder, is transmitted from UE and received at gNB. An “decoder” of autoencoder corresponds to the gNB’s processing in which the original input is reconstructed from the “internal representation”.

Since autoencoder is employed at both gNB and UE to compress CSI reporting, gNB and UE need to synchronize the parameters of AI/ML module i.e., training collaboration is necessary. For training collaboration, following agreement was made in RAN1110.

Agreement in RAN1#110

- In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied.

  - Type 1: Joint training of the two-sided model at a single side / entity, e.g., UE-sided or network-sided.

  - Type 2: Joint training of the two-sided model at network and UE side, respectively.

  - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

  - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes).

  - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with network side training [, or parallel training: at UE and network.

  - Other collaboration types are not excluded.

In Type 1, the AI model is trained at network side or UE side. In case the AI model is trained at network side, CSI generation part (i.e., the CSI encoder) delivered to the UE (UE download AI model) via air interface. In case the AI model is trained at UE side, the AI model is trained at UE side and network-side CSI reconstruction part (i.e., CSI decoder) delivered to the network via air interface. Type 1 involves the exchange of AI model and then, these options require some common AI inference algorithm and common reference for model inference including bit length precision in order to obtain the same or similar output among different UEs with low complexity / low power consumption when the same input is provided. In Type 2 and Type 3, our view is that only offline training is feasible approach at least in Rel.18/19 timeline. For Type 2, joint training in offline engineering with multi-vendor agreements is possible. In this case, no model exchange is required after deployment (although model fine tuning would be necessary even after deployment). Type 3 also does not need model exchange. For the realization of Type 3, 3GPP may need to define some kind of requirement of CSI encoding by input and output relation, performance test or something else. In the defining the requirement, gNB feasibility based on the UE requirement is also checked. For the operation, UE declares whether it satisfy the requirement or not as UE capability and network enable AI/ML encoding if/when useful.

In our view, at least for Rel.18/19, Type 2 and 3 with offline training would be more feasible options. Large specification effort would be required for Type 1 since some common AI algorithm and common reference for model inference should be discussed and agreed. However, Type 1 with network-sided training might be interesting / potential in the long-term as more flexibility of the operation from the network is possible. Type 1 with UE-sided training might be risky approach since network vendor could not rely on UE manufacture’s AI model as it might not consider overall network efficiency.

Observation 1: Type 2 and 3 with offline training might be feasible options at least Re.18/19 timeline from standardization effort perspective. Type 1 with network sided training can be potential in the long-term.

Configuration and content for CSI report

The potential specification impacts related to the encoder input would be model input type / dimension / configuration. At least for Type 1 of training collaboration with network-sided training, since encoder / decoder is trained at the network, the network needs to know all required pre/post processing. If UE still choose the proper rank and corresponding model, CSI report can include the encoder output, plus RI and/or model ID. For Type 1 of training collaboration with UE-sided training, since encoder / decoder is trained at the UE side, output format and pre/post processing should be part of UE report so gNB is aware how to process it. For Type 2 of the training collaboration, since decoder is trained via offline, the network would know the characters of decoders. If UE still chooses the proper rank and corresponding model, CSI report can include the encoder output, plus RI and/or model ID. For Option 4 of the training collaboration, AI-based codebook may need to be specified.

Proposal 2: For each option of training collaboration, configuration and content for CSI report should be studied.

Handling of rank of AI/ML model

Once an AI/ML model has been trained, the input and output dimension of AI/ML model needs to be fixed. For CSI feedback, the dimension of input and output depend on the number of CSI-RS antenna ports, sub-bands, and the number of feedback bits, etc. However, if only one CSI compression model can be used for one fixed set of configurations, UE and gNB would need to store plenty of CSI compression models applicable for various configurations. Therefore, it is necessary to consider scalability of the AI/ML model for CSI compression so that it can adapt to various configurations. One of example of scalability consideration could be the handling of rank. For the handling of rank of AI/ML mode, following options could be considered.

- Option A: Separate AI/ML model is trained and applied for each rank to perform individual inference.

- Option B: A unified AI/ML model is trained and applied for each rank to perform individual inference.

- Option C: A unified AI/ML model is trained and applied for adaptive ranks to perform inference.

For Option A, data shaping effort might be larger. The amount of data might also be larger since the amount of data might depend on not only number of antenna ports and number of sub-bands but also on number of supported ranks. In addition, separate AI module for rank decision and each rank operation is necessary. On the other hand, since single AI/ML inference can be handled for each rank, model generation and/or parameter tuning could be less complicated than Option B and C. For Option C, the effort for data collection would be easier and single AI module may cover overall. The amount of the training data might depend on the number of antenna ports and number of sub-bands. Since single AI/ML inference needs to consider multiple rank, model generation and/or parameter tuning might be more complicated.

Proposal 3: For each option of training collaboration, handling of rank of AI/ML model should studied.

Life cycle management

In order to ensure availability of AI/ML model, the model performance needs to be monitored. UE and gNB should interact with some essential information related to the model, such as indicator related to model performance deterioration, information reflecting model performance and/or information related to both measurement results and inference results. For like cycle management, following solutions could be considered.

- Solution 1: gNB side performance monitoring

  - 1-1: UE transmits encoder input as CSI report periodically or occasionally.

  - 1-2: gNB may directly use system throughput or ratio of NACK.

- Solution 2: UE side performance monitoring

  - 2-1: UE calculates decoder output using virtual decoder in UE.

  - 2-2: UE may obtain the inference results indicated from gNB periodically or occasionally

  - 2-3: UE may use PDSCH decoding performance as KPI.

For Solution 1-1, encoder input means raw channel matrix or eigenvector(s). The network calculates loss function between the received CSI report (e.g., raw channel matrix or eigenvector(s) from UE) and decoder output. If the network detects the deterioration, network disable the AI/ML based CSI feedback and/or trigger the model update. Instead of the feedback of raw channel or eigenvector, SRS measurement at gNB side may be used for feedback overhead reduction. For Solution 1-2, the UE calculates loss function between the virtual decoder output and input to the encoder. If the UE detects the deterioration, UE triggers AI/ML model failure. For both Solution 1 and Solution 2, after evaluation, the gNB or UE may send the performance report to the other side to facilitate the potential determination of model deactivation or switching.

Proposal 4: The following options should be studied for life cycle management.

- Solution 1: gNB side performance monitoring

  - 1-1: UE transmit encoder input as CSI report periodically or occasionally.

  - 1-2: gNB may directly use system throughput or ratio of NACK.

- Solution 2: UE side performance monitoring

  - 2-1: UE calculate decoder output using virtual decoder in UE.

  - 2-2: UE may obtain the inference results indicated from gNB periodically or occasionally

  - 2-3: UE may use PDSCH decoding performance as KPI.

Conclusion

In this contribution, we provide our view on the specification impact of AI/ML for CSI feedback enhancement. We made following observation and proposals.

Proposal 1: CSI prediction using one-sided model could be considered as representative sub use cases.

Observation 1: Type 2 and 3 with offline training might be feasible options at least Re.18/19 timeline from standardization effort perspective. Type 1 with network sided training can be potential in the long-term.

Proposal 2: For each option of training collaboration, configuration and content for CSI report should be studied.

Proposal 3: For each option of training collaboration, handling of rank of AI/ML model should studied.

Proposal 4: The following options should be studied for life cycle management.

- Solution 1: gNB side performance monitoring

  - 1-1: UE transmit encoder input as CSI report periodically or occasionally.

  - 1-2: gNB may directly use system throughput or ratio of NACK.

- Solution 2: UE side performance monitoring

  - 2-1: UE calculate decoder output using virtual decoder in UE.

  - 2-2: UE may obtain the inference results indicated from gNB periodically or occasionally

  - 2-3: UE may use PDSCH decoding performance as KPI.

- Reference

[1]	RP-213599, “New SI: Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface,”	Qualcomm (Moderator), RAN#94e.