3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209176

# e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.3.3

Source:	Ericsson

Title:	Potential enhancements of dynamic TDD

Document for:	Discussion, Decision

# 1	Introduction

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

The Rel-18 study item on evolution of NR duplex operation contains the following list of objectives, and those RAN1 objectives related to dynamic/flexible TDD are highlighted in yellow.

The 1st and 2nd highlighted objectives direct RAN1 to study potential and identify potential enhancement schemes. In our view, the basic mechanics of dynamic TDD (dTDD) are already specified – albeit probably not tested – since most, if not all deployments of TDD to date have been based on static TDD (sTDD). Based on this, it is not clear that enhancements to the basic mechanics of dynamic/flexible TDD are warranted. One area, however, that has been discussed for potential enhancement, beyond basic mechanics, is potential solutions for handling inter-gNB and inter-UE cross link interference (CLI). The 3rd objective in the SID is to study such CLI handling schemes.

To this end, the below agreements were made in RAN1#110 related to inter-gNB and inter-UE CLI handling, respectively. We note that the CLI handling schemes listed in these agreements have been studied before (e.g., see  [1], [2]), and based on the Note at the bottom of the SID (see cyan highlight) repeated discussion should be avoided.

CLI Measurement

Agreement

Study the feasibility and potential benefits of gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes:

- Measurement resource configuration

- Measurement details

- Relevant information exchange

- Usage of measurement

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Co-ordinated Scheduling

Agreement

Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes:

- Details of coordinated scheduling for time/frequency resources

- Relevant information exchange

Agreement

Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes:

- Details of coordinated scheduling for time/frequency resources

- Relevant information exchange (if needed)

Co-ordianted Beamforming

Agreement

Study the feasibility and potential benefits of spatial domain coordination method for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes:

- Details for spatial domain coordination

- Relevant information exchange

Note1: Study can include method for FR1 and FR2

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI handling based on spatial domain coordination method which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic /flexible TDD, at least includes:

- Details for spatial domain coordination by gNB

- Relevant information exchange (if needed)

Note1: Study can include method for FR1 and FR2

# 2	Discussion

Figure 1 illustrates both gNB-gNB and UE-UE co-channel CLI for a dynamc TDD (dTDD) system consistent with the above agreements. CLI is present when two gNBs schedule transmissions in opposite directions at the same time; no CLI is present if both gNBs schedule transmission in the same direction at the same time (both DL or both UL), which is always the case in a sTDD system with coordinated TDD patterns. Note that gNB2 can be a gNB at another site, or a gNB in another sector in the same site. In this example, gNB2 transmits DL to UE2, while gNB1 receives UL from UE1. Hence gNB1 is the victim of gNB-gNB CLI and UE2 is the victim of UE-UE CLI. It is important to realize that whether or not CLI dominates over other sources of interference depends on a large number of factors, e.g., traffic load, DL/UL traffic split, inter-gNB distance, inter-UE distance, etc.

Figure 1: Illustration of CLI for dTDD in a single operator network. Co-channel CLI can occur when the two gNBs schedule transmissions in opposite directions at the same time.

As can be seen in the above agreements from RAN1#110, the proposed CLI mitigation mechanisms for study  are based on some form of coordination between nodes, e.g., coordinated scheduling and/or coordinated beam forming. These, along with other co-ordination approaches can be generally classified as CLI avoidance mechanisms. As has been discussed in previous studies, such coordination is often not practical, especially between nodes of different operators. For example, consider a two-operator scenario in which one operator wishes to deploy dTDD and legacy operator remains as sTDD. It is certainly not reasonable to require the legacy operator to avoid CLI for the benefit of the dTDD operator.

Even for nodes within the same operator, the time scale of any coordination needs to be fast, on the order of scheduling decisions which follow the traffic packet arrival statistics. Without such fast coordination, which may not be practical in the first place, the performance potential of such CLI handling mechanisms is likely to be modest at best. Moreover, since the mechanisms attempt to avoid CLI, there is an implicit cost in reduced scheduling opportunities, e.g., in the temporal and/or spatial domains. This cost needs to be balanced against potential benefits.

One simple approach for coordination that already exists without introducing any enhancements is coordination of TDD UL/DL patterns between nodes. In conventional sTDD systems, the pattern is fixed across all nodes of the same and different operators, in which case CLI does not occur at all. Typically, this level of coordination is required by regulation. On the other extreme is fully dynamic TDD (or in other words, “pure” dTDD) where there is no coordination between nodes at all. All slots are configured as flexible ('F'), and depending on the traffic packet arrivals in DL or UL, a slot can be used for either DL or UL transmission dynamically.

In our view, it is worthwhile to consider something in between these two extremes, and we refer to such a middle-ground solution as "protected" dynamic TDD (p-dTDD). With this CLI-mitigation scheme, one or more slots in the TDD UL/DL pattern are reserved as 'U' such that all nodes agree that only UL can be transmitted in these slot(s). These UL slot(s) are considered as "protected", since gNB-gNB CLI will not occur. Such protection is highly beneficial since it ensures that at least important UL control traffic can be received reliably (e.g., HARQ-ACK for DL). This is vital, since we have found that typically gNB-gNB CLI dominates over UE-UE CLI thus motivating provision of a protected UL slot. One can consider such a scheme as hybrid sTDD/dTDD operation. This level of coordination between nodes is very modest and could be practical both within and between operators.

Figure 2 illustrates the 3 different systems discussed above. From left-to right, sTDD is shown first in which case all nodes use a fixed D-D-D-D-U pattern. dTDD is shown next in which all slots are configured as flexible F-F-F-F-F, and any slot can be used by any node at any time for either DL or UL. Protected dTDD (p-dTDD) is shown last in which case the first four slots are configured as flexible and one slot is reserved for UL-only F-F-F-F-U.

Figure 2: Three different system operation modes with associated TDD UL/DL pattern.

In this contribution, we evaluate the system level performance of the 3 different operation modes shown in Figure 2 in a single-operator urban macro network with 3 sectors per site. The evaluation assumptions are listed in the Appendix. Figure 3 and Figure 4 show the mean and 5th percentile user throughput (UTP), respectively, at 3 different system load levels: low, medium, and high. These load levels are defined referring to baseline sTDD resource utilization, i.e. below 10% for low load, between 25% and 35% for medium load, and above 55% for high load.

(a)	(b)

Figure 3: Performance of dTDD and protected dTDD compared to sTDD in terms of mean user throughput for (a) UL, and (b) DL, at low, medium, and high loads.

(a)	(b)

Figure 4: Performance of dTDD and protected-dTDD compared to sTDD in terms of 5th percentile user throughput for (a) UL, and (b) DL at low, medium, and high loads.

From the above user-throughput results, we observe the following:

- Uplink (see Figure 3(a) and Figure 4(a))

  - Dynamic TDD (dTDD)

    - At low load, dTDD offers significant gain in UL UTP compared to sTDD. The gain is 200% / 150% for mean / 5th percentile UL UTP, respectively.

      - The large gain is due to the provision of more UL opportunities compared to sTDD for which UL is allowed in only one out of every 5 slots. The additional UL opportunities for dTDD are rarely impacted by gNB-gNB CLI since the load is low.

    - At medium and high load, the performance of dTDD collapses, i.e., the UL UTP is at or near zero, clearly much worse than sTDD

      - The reason for the collapsing performance is due to uncontrolled gNB-gNB CLI which occurs with higher probability as the load increases. Hence, one should be very careful about enabling dTDD since the load can change dynamically and cause the system to collapse.

  - Protected dynamic TDD (p-dTDD)

    - At medium and high load, p-dTDD avoids the performance collapse suffered by dTDD. In fact, the performance is on par or even better than sTDD where there is zero CLI inherently.

    - At low load, p-dTDD offers significant gain in UL UTP. The gain is 200% for mean UL UTP which is on par with dTDD. The gain is 100% for 5th percentile UL UTP. While the latter is somewhat less than dTDD, it is still significant. The reason for this reduction in 5th percentile UL UTP in comparison to dTDD is that 5th percentile UEs are obviously in challenging coverage/SINR conditions, so that the impact of CLI can easily have their service dropped. p-dTDD, however, is able to serve UL traffic over a protected UL slot, where CLI does not happen, and this allows to serve more traffic (98.36% of the offered traffic) than dTDD (96.83% of the offered traffic).  The trade-off is somewhat reduced 5th percentile UTP for p-dTDD in comparison to dTDD, since more challenged UEs are actually served.

- Downlink (see Figure 3(b) and Figure 4(b))

  - At all load levels, one can see that both the mean and 5th percentile DL UTP of p-dTDD is on par with sTDD (as expected for the equivalent amount of DL transmission opportunities), i.e., no degradation is observed due to UE-UE CLI. On the other hand, for low/medium load, dTDD offers modest gains in mean DL UTP compared to sTDD and p-dTDD (on the order of 20%), due to the availability of an additional slot for DL transmission opportunities. These gains, however, only exist until the UL collapses due to gNB-gNB CLI. For high load (and medium load in the case of 5th percentile UTP), when traffic cannot be served in the UL, traffic cannot be served in the DL either, since the DL relies on HARQ-ACK feedback in the UL. Again p-dTDD avoids this collapse in UL, which then also protects the DL allowing both links to operate at all load levels.

The above results illustrate the benefit of p-dTDD as a very simple coordination scheme where nodes coordinate on a semi-static basis to provide at least one protected UL slot. Fast exchange of coordination information to manage CLI is not required. The remaining slots can be used as with dTDD for DL or UL depending on the traffic arrival pattern. Provision of the protected UL slot enables "steering" of UL traffic to this CLI-free slot that otherwise can suffer from high CLI and thus low SINR. In this way, one can deploy p-dTDD without fear of load-dependent performance collapse due to gNB-gNB CLI as occurs in a dTDD system. This scheme can be considered under the umbrella of co-ordinated scheduling for time/frequency resources as agreed last meeting:

Agreement

Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes:

- Details of coordinated scheduling for time/frequency resources

- Relevant information exchange

Protected dTDD is a simple and robust scheme for mitigating the performance impact of CLI without requiring fast exchange of information between gNBs. The scheme is feasible for operation both within and between operators.

Capture the performance of protected dTDD in the TR as a beneficial CLI handling scheme under the umbrella of "co-ordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling."

## 2.1	Discussion on UE-UE CLI

In the previous section we showed both UL and DL performance in terms of mean and 5th percentile UTP for 3 different systems:

- Static TDD (sTDD)

  - No CLI in any slot

- Dynamic TDD (dTDD)

  - Potential CLI in all slots

- Protected dynamic TDD (p-dTDD)

  - Potential CLI in flexible slots; no CLI in protected UL slot

If gNB-gNB CLI is present, it will affect UL performance, and if UE-UE CLI is present, it will affect DL performance. In Figure 3 (mean UTP) and Figure 4 (5th percentile UTP), one can infer that it is the UL that it is most affected by CLI. For dTDD, the UL performance completely collapses at medium and high load due to gNB-gNB CLI. p-dTDD avoids this by steering UL traffic towards the UL-only slot that is CLI free.

In contrast, one can infer from the DL performance results that UE-UE CLI does not have an appreciable affect. Observing the DL results for p-dTDD, one can see that both the mean and 5th percentile DL UTP is on par with sTDD where there is no UE-UE CLI. dTDD even seems to offer a gain compared to sTDD at medium and high loads which indicates that UE-UE CLI does not create an issue. However, as we pointed out previously, these load points are not relevant since the UL performance completely collapses at medium and high load due to gNB-gNB CLI.

Generally, we have found this effect to be true across a broad range of evaluations, i.e, gNB-gNB CLI dominates over UE-UE CLI. One way to view this is that UE-UE CLI between two users only occurs only if all of the following are satisfied:

- The two UEs are very close to each other, i.e., path loss between them is small,

- There is a packet available in the UL and DL buffers of both UEs simultaneously,

- The two UEs are scheduled in opposite directions at the same time,

- The two UEs are at edge of their serving cells where the desired (DL) signal is weak.

Satisfying all of these conditions turns out to be relatively rare, and thus overall system performance in terms of mean and 5th percentile DL UTP is not strongly affected by UE-UE CLI.

As is well known, enhancements were made in Rel-16, to enable reporting of CLI-RSSI based on CLI measurement resources. If configured, this allows the gNB to learn if a particular UE is suffering from UE-UE CLI, even if it does not occur often. This knowledge can be taken into account in future scheduling decisions for such a UE. We note that the Rel-16 CLI reporting mechanism is based on Layer 3 reports, which have inherent latency. Hence, the gNB may not obtain very up-to-date information from the UE compared to the time scale of traffic arrivals and scheduling.

One approach that has been mentioned by several companies in this study item is to introduce L1 or L2 reporting of CLI-RSSI, similar to L1 reporting of RSRP introduced in Rel-15 to support beam management. While this can be faster than L3 reporting, we note that such reporting still has built-in latencies considering UE capabilities on CSI processing delay which can be quite long. Nonetheless, it could be worthwhile to study whether or not L1 CLI-RSSI reporting could bring some benefits to solve targeted CLI issues if there are UEs in bad CLI situations. In such a study, it would be important to model the CSI processing latency realistically in order to draw a proper benefit assessment. Such a scheme can be considered under the umbrella of UE-UE co-channel CLI measurement and reporting as agreed last meeting:

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Under the umbrella of "UE-to-UE co-channel CLI measurement and reporting," study whether or not L1 CLI-RSSI reporting can provide benefits for particular UEs that may suffer from UE-UE CLI. UE processing delays should be properly taken into account.

# Conclusion

In this contribution we make the following observations:

Observation 1	Protected dTDD is a simple and robust scheme for mitigating the performance impact of CLI without requiring fast exchange of information between gNBs. The scheme is feasible for operation both within and between operators.

In this contribution we make the following proposals:

Proposal 1	Capture the performance of protected dTDD in the TR as a beneficial CLI handling scheme under the umbrella of "co-ordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling."

Proposal 2	Under the umbrella of "UE-to-UE co-channel CLI measurement and reporting," study whether or not L1 CLI-RSSI reporting can provide benefits for particular UEs that may suffer from UE-UE CLI. UE processing delays should be properly taken into account.

# References

3GPP TR 38.802, "Study on New Radio Access Technology Physical Layer Aspects," v14.2.0, September 2017.

3GPP TR 38.828, " Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR," v16.1.0, September 2019.

# Appendix A: Parameters for system level evaluations

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 7 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | Carrier Frequency | 4 GHz |
|  | Duplex Type | Static TDD, Pure DTDD, Protected TDD |
|  | Base Static TDD pattern | 80:20 DL:UL |
|  | DTDD pattern | Pure DTDD: completely flexible UL:DL Protected DTDD: 80:20 DTDD slot:UL |
|  | Channel bandwidth | 100 MHz for STDD, Pure DTDD and Protected DTDD |
|  | Available resource blocks | 273 for STDD 273 for DTDD |
|  | Switching time | DL->UL: 2OS DTDD DL->UL: 2OS |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 7000 |
|  | Channel model | gNB-UE: UMa gNB-gNB: UMa with 100% LoS UE-UE: UMi |
|  | UE to BS min 2D distance | 25 m |
| BS | (Mg, Ng, M, N, P), where M, N indicate sub-array number | (1,1,4,8,2) (same antenna gain) (1,1,4,4,2) (same antenna area) |
|  | Sub-array configuration | 3x1 |
|  | Max gNB Tx Power (per polarization) | 50 dBm |
|  | (dv, dh) | (0.7λ, 0.5λ) |
|  | Antenna element gain | 6.4 dBi |
|  | Antenna element and sub-array model | R4-2109872, Table 1-2: Macro urban |
|  | Subarray electrical downtilt | 3 deg |
|  | Mechanical downtilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 |
|  | SNR target | 16 dB |
|  | Noise figure | 9 dB |
|  | Max modulation | 64 QAM |
|  | UE distribution outdoor:indoor | 30:0 Outside buildings 0:70 Inside buildings |
|  | Buildings | One 120x50 m building per cell placed randomly |
| Traffic | Traffic split | 87:13 DL:UL to achieve individual target RU in DL and UL |
|  | Target Resource utilization | <10%, 25-35%, 55% |
| Intra-network isolation | Front to back ratio (sector<->sector) | 45 dB |