3GPP TSG RAN WG1 #110bis-e                                            R1-2209212

e-Meeting, October 10th – 19th, 2022

Source:         ZTE, CMCC

Title:            Discussion on evaluation of SL positioning

Agenda item:    9.5.1.1

Document for:	Discussion and Decision

- Introduction

In RAN#94e meeting, the study item on Rel-18 NR positioning was approved [1], where one of the potential enhancements is for sidelink positioning. As shown in the SID, some bullets for sidelink positioning are to define evaluation methodology and evaluate performance.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In RAN1#109e and RAN1#110 meeting, evaluation methodology and scenarios have been agreed. In this contribution, we provide some SL positioning simulation results.

- Evaluation Results for TR 38.859 Section B.1

  - Description of evaluation scenarios

Based on the agreement in RAN1#109e and RAN1#110 meeting, sidelink positioning is evaluated in V2X use case with highway scenario and urban grid scenario, IIOT use case, public safety use case and commercial use case. For IIOT use case, indoor factory scenario defined in TR 38.857 [3] is used to evaluate. As a typical scenario of InF, InF-SH are evaluated. For public safety use case, RMa scenario can be adopted to be evaluated as a typical scenario of public safety. For commercial use case, indoor office scenario can be adopted to be evaluated as a typical scenario of commercial use case.

Common assumptions applicable to all evaluated scenarios are provided in Table 2.1-1.

Table 2.1-1 Common evaluation assumptions applicable to all use cases

| Parameter |  |
| --- | --- |
| Carrier frequency | Uu: 4GHz SL: 6GHz |
| Subcarrier spacing | 30KHz |
| UE or UE type RSU antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| Comb size | 4 |
| Interference modelling (ideal muting, or other) | Ideal muting |
| Description of measurement algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC algorithm |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Absolute positioning: TDOA with Gauss-Newton algorithm; Relative positioning/ranging: RTT, or AOA |
| Synchronization assumptions | Perfect synchronization |
| Precoding assumptions (codebook, etc) | No precoding |

Evaluation parameters for absolute positioning in highway use case are provided in Table 2.1-2. Evaluation parameters for relative positioning in highway use case are provided in Table 2.1-3.

Table 2.1-2 Evaluation parameters for absolute positioning in highway use case

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor nodes | Joint | Joint | Joint | SL only | SL only | SL only |
| Bandwidth | 100MHz | 40MHz | 20MHz | 100MHz | 40MHz | 20MHz |

Table 2.1-3 Evaluation parameters for relative positioning in highway use case

| Parameters | Case 7 | Case 8 | Case 9 | Case 10 | Case 11 | Case 12 | Case 13 | Case 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | 50 | 100 | 150 | 200 | 300 | 100 | 100 | - |
| Bandwidth | 40MHz | 40MHz | 40MHz | 40MHz | 40MHz | 20MHz | 100MHz | 40MHz |
| Rx antenna number | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

Evaluation parameters for absolute positioning in urban grid use case are provided in Table 2.1-4. Evaluation parameters for relative positioning in urban grid use case are provided in Table 2.1-5.

Table 2.1-4 Evaluation parameters for absolute positioning in urban grid use case

| Parameters | Case 15 | Case 16 | Case 17 | Case 18 | Case 19 | Case 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor nodes | Joint | Joint | Joint | SL only | SL only | SL only |
| Bandwidth | 100MHz | 40MHz | 20MHz | 100MHz | 40MHz | 20MHz |

Table 2.1-5 Evaluation parameters for relative positioning in urban grid use case

| Parameters | Case 21 | Case 22 | Case 23 | Case 24 | Case 25 | Case 26 | Case 27 | Case 28 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | 20 | 30 | 50 | 80 | 100 | 150 | 30 | 30 |
| Bandwidth | 40MHz | 40MHz | 40MHz | 40MHz | 40MHz | 40MHz | 20MHz | 100MHz |

Evaluation parameters for absolute positioning in InF-SH scenario are provided in Table 2.1-6. Evaluation parameters for relative positioning in InF-SH scenario are provided in Table 2.1-7.

Table 2.1-6 Evaluation parameters for absolute positioning in InF-SH scenario

| Parameters | Case 29 | Case 30 | Case 31 | Case 32 | Case 33 | Case 34 |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor nodes | Joint | Joint | Joint | SL only | SL only | SL only |
| Bandwidth | 100MHz | 40MHz | 20MHz | 100MHz | 40MHz | 20MHz |

Table 2.1-7 Evaluation parameters for relative positioning in InF-SH scenario

| Parameters | Case 35 | Case 36 | Case 37 | Case 38 | Case 39 | Case 40 |
| --- | --- | --- | --- | --- | --- | --- |
| X | 10 | 20 | 30 | 50 | 10 | 10 |
| Bandwidth | 40MHz | 40MHz | 40MHz | 40MHz | 20MHz | 100MHz |

Evaluation parameters for absolute positioning in public safety use case are provided in Table 2.1-8. Evaluation parameters for relative positioning in public safety use case are provided in Table 2.1-9.

Table 2.1-8 Evaluation parameters for absolute positioning in public safety use case

| Parameters | Case 41 | Case 42 | Case 43 | Case 44 | Case 45 | Case 46 |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor nodes | Joint | Joint | Joint | SL only | SL only | SL only |
| Bandwidth | 100MHz | 40MHz | 20MHz | 100MHz | 40MHz | 20MHz |
| BS location | 7 macro sites with 3 cells per site |  |  |  |  |  |
| UE drop | Antenna height 1.5m, speed 3km/h, uniformly and randomly distributed at the macro geographical area |  |  |  |  |  |
| Reference UE | 7 reference UEs per sector are dropped randomly |  |  |  |  |  |
| SL channel model | Modify RMa described in TR 38.901, i.e. replace BS with the reference UE in the channel model of BS2UE, where reference UE height, transmit power are used to replace gNB’s. |  |  |  |  |  |
| Uu channel model | RMa described in TR 38.901 |  |  |  |  |  |

Table 2.1-9 Evaluation parameters for absolute positioning in public safety use case

| Parameters | Case 47 | Case 48 | Case 49 | Case 50 |
| --- | --- | --- | --- | --- |
| X | 100 | 50 | 50 | 50 |
| Bandwidth | 40MHz | 40MHz | 100MHz | 20MHz |

Evaluation parameters for absolute positioning in commercial use case are provided in Table 2.1-10. Evaluation parameters for relative positioning in commercial use case are provided in Table 2.1-11.

Table 2.1-10 Evaluation parameters for absolute positioning in commercial use case

| Parameters | Case 51 | Case 52 | Case 53 | Case 54 | Case 55 | Case 56 |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor nodes | Joint | Joint | Joint | SL only | SL only | SL only |
| Bandwidth | 100MHz | 40MHz | 20MHz | 100MHz | 40MHz | 20MHz |
| BS location | 12 sites deployed as described in TR 38.901 |  |  |  |  |  |
| UE drop | Antenna height 1.5m, speed 3km/h, uniformly and randomly distributed at the macro geographical area |  |  |  |  |  |
| Reference UE | 12 reference UEs are dropped randomly |  |  |  |  |  |
| SL channel model | Modify indoor office channel model described in TR 38.901, i.e. replace BS with the reference UE in the channel model of BS2UE, where reference UE height, transmit power are used to replace gNB’s. |  |  |  |  |  |
| Uu channel model | Indoor office described in TR 38.901 |  |  |  |  |  |

Table 2.1-11 Evaluation parameters for absolute positioning in commercial use case

| Parameters | Case 57 | Case 58 | Case 59 | Case 60 | Case 61 |
| --- | --- | --- | --- | --- | --- |
| X | 10 | 20 | 50 | 10 | 10 |
| Bandwidth | 40MHz | 40MHz | 40MHz | 100MHz | 20MHz |

  - Positioning accuracy evaluation results for Sidelink Positioning

This subsection provides evaluation results for sidelink positioning, including absolute positioning and relative positioning, in V2X use case with highway scenario and urban grid scenario, IIOT use case, public safety use case and commercial use case.

### 2.2.1 Positioning accuracy evaluation results for Sidelink Positioning for Highway Scenarios for V2X

Figure 2.2.1-1, Table 2.2.1-1 and Table 2.2.1-2 show the CDF errors of simulation results for absolute positioning in highway scenario, involving horizontal positioning accuracy and vertical positioning accuracy. Figure 2.2.1-2, Table 2.2.1-3 and Table 2.2.1-4 show the CDF errors of simulation results for relative positioning in highway scenario, involving ranging accuracy and direction accuracy.

Figure 2.2.1-1 Curves of CDF errors for absolute positioning in highway scenario

Table 2.2.1-1 CDF errors for absolute positioning in highway scenario-horizontal accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-Joint positioning, BW=100MHz | 0.111 | 0.1505 | 0.1861 | 0.2169 | Yes | Yes |
| Case 2-Joint positioning, BW=40MHz | 0.3517 | 0.4583 | 0.6207 | 0.7354 | Yes | No 67% of UEs satisfying the requirement of Set B |
| Case 3-Joint positioning, BW=20MHz | 0.6651 | 0.8498 | 1.104 | 1.287 | Yes | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 4-SL positioning, BW=100MHz | 0.2397 | 0.3952 | 0.5869 | 0.8348 | Yes | No 67% of UEs satisfying the requirement of Set B |
| Case 5-SL positioning, BW=40MHz | 0.6787 | 1.02 | 1.412 | 2.343 | No 80% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 6-SL positioning, BW=20MHz | 1.013 | 1.542 | 2.405 | 3.08 | No 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |

Table 2.2.1-2 CDF errors for absolute positioning in highway scenario-vertical accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-Joint positioning, BW=100MHz | 0.784 | 1.193 | 1.445 | 1.578 | Yes | Yes |
| Case 2-Joint positioning, BW=40MHz | 1.423 | 1.57 | 2.965 | 4.481 | No 80% of UEs satisfying the requirement of Set A | No 67% of UEs satisfying the requirement of Set B |
| Case 3-Joint positioning, BW=20MHz | 2.725 | 7.534 | 9.946 | 12.76 | No 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 4-SL positioning, BW=100MHz | 7.236 | 8.714 | 9.927 | 11.81 | No Less than 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 5-SL positioning, BW=40MHz | 9.216 | 10.73 | 12.47 | 14.64 | No Less than 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 6-SL positioning, BW=20MHz | 12.12 | 14.2 | 17.62 | 23.06 | No Less than 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |

Figure 2.2.1-2 Curves of CDF errors for relative positioning in highway scenario

Table 2.2.1-3 CDF errors for relative positioning in highway scenario-ranging accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 7, X=50m, BW=40M | 0.1995 | 0.2837 | 0.3862 | 0.5556 | Yes | No 80% of UEs satisfying the requirement of Set B |
| Case 8, X=100m, BW=40M | 0.2075 | 0.2909 | 0.4125 | 0.6223 | Yes | No 80% of UEs satisfying the requirement of Set B |
| Case 9, X=150m, BW=40M | 0.2097 | 0.2945 | 0.4125 | 0.6555 | Yes | No 80% of UEs satisfying the requirement of Set B |
| Case10, X=200m, BW=40M | 0.2124 | 0.2984 | 0.4185 | 0.6714 | Yes | No 80% of UEs satisfying the requirement of Set B |
| Case 11, X=300m, BW=40M | 0.2284 | 0.3285 | 0.4564 | 0.7286 | Yes | No 80% of UEs satisfying the requirement of Set B |
| Case 12, X=100m, BW=20M | 0.4145 | 0.5466 | 0.8197 | 1.173 | Yes | No 50% of UEs satisfying the requirement of Set B |
| Case 13, X=100m, BW=100M | 0.1112 | 0.1611 | 0.2239 | 0.3187 | Yes | Yes |

Table 2.2.1-4 CDF errors for relative positioning in highway scenario-angle accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 14, BW=40M, 1Tx antenna, 4Rx antenna | 2.564 | 3.604 | 4.815 | 7.269 | Yes | Yes |

### 2.2.2 Positioning accuracy evaluation results for Sidelink Positioning for Urban Grid Scenarios for V2X

Figure 2.2.2-1 and Table 2.2.2-1 show the CDF errors of simulation results for absolute positioning in urban grid scenario. Figure 2.2.2-2 and Table 2.2.2-2 show the CDF errors of simulation results for relative positioning in urban grid scenario.

Figure 2.2.2-1 Curves of CDF errors for absolute positioning in urban grid scenario

Table 2.2.2-1 CDF errors for absolute positioning in urban grid scenario-horizontal accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 15-Joint positioning, BW=100MHz | 0.1718 | 0.226 | 0.2865 | 0.3423 | Yes | Yes |
| Case 16-Joint positioning, BW=40MHz | 0.6298 | 0.9333 | 1.325 | 1.918 | No 80% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 17-Joint positioning, BW=20MHz | 1.038 | 1.368 | 1.772 | 2.247 | No 67% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 18-SL positioning, BW=100MHz | 1.764 | 3.569 | 6.164 | 8.403 | No Less than 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 19-SL positioning, BW=40MHz | 6.644 | 8.542 | 10.95 | 13.28 | No Less than 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 20-SL positioning, BW=20MHz | 7.035 | 9.373 | 12.75 | 16.08 | No Less than 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |

Figure 2.2.2-2 Curves of CDF errors for relative positioning in urban grid scenario

Table 2.2.2-2 CDF errors for relative positioning in urban grid scenario-ranging accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 21, X=20m, BW=40M | 0.3272 | 0.5479 | 0.7734 | 1.014 | Yes | No 50% of UEs satisfying the requirement of Set B |
| Case 22, X=30m, BW=40M | 0.3559 | 0.5654 | 0.8703 | 1.413 | Yes | No 50% of UEs satisfying the requirement of Set B |
| Case 23, X=50m, BW=40M | 0.4467 | 0.752 | 1.11 | 2.59 | No 80% of UEs satisfying the requirement of Set A | No 50% of UEs satisfying the requirement of Set B |
| Case 24, X=80m, BW=40M | 0.4828 | 0.8532 | 1.587 | 6.793 | No 67% of UEs satisfying the requirement of Set A | No 50% of UEs satisfying the requirement of Set B |
| Case 25, X=100m, BW=40M | 0.4975 | 0.9022 | 1.771 | 7.139 | No 67% of UEs satisfying the requirement of Set A | No 50% of UEs satisfying the requirement of Set B |
| Case 26, X=150m, BW=40M | 0.5553 | 1.016 | 2.296 | 8.547 | No 67% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 27, X=30m, BW=20M | 0.7525 | 1.151 | 1.785 | 2.782 | No 67% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 28, X=30m, BW=100M | 0.1813 | 0.2942 | 0.444 | 0.752 | Yes | No 80% of UEs satisfying the requirement of Set B |

### 2.2.3 Positioning accuracy evaluation results for Sidelink Positioning for IIOT

Figure 2.2.3-1 and Table 2.2.3-1 show the CDF errors of simulation results for absolute positioning in InF-SH scenario. Figure 2.2.3-2 and Table 2.2.3-2 show the CDF errors of simulation results for relative positioning in InF-SH scenario.

Figure 2.2.3-1 Curves of CDF errors for absolute positioning in InF-SH scenario

Table 2.2.3-1 CDF errors for absolute positioning in InF-SH scenario-horizontal accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 29-Joint positioning, BW=100MHz | 0.1926 | 0.318 | 0.4258 | 0.5778 | Yes | No 50% of UEs satisfying the requirement of Set B |
| Case 30-Joint positioning, BW=40MHz | 0.5303 | 0.6574 | 0.8311 | 1.07 | No 80% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 31-Joint positioning, BW=20MHz | 0.9272 | 1.271 | 1.675 | 2.197 | No 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 32-SL positioning, BW=100MHz | 0.2535 | 0.3424 | 0.5115 | 0.7894 | Yes | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 33-SL positioning, BW=40MHz | 0.4399 | 0.5695 | 1.8338 | 1.257 | No 67% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 34-SL positioning, BW=20MHz | 1.003 | 1.398 | 1.961 | 3.265 | No Less than 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |

Figure 2.2.3-2 Curves of CDF errors for relative positioning in InF-SH scenario

Table 2.2.3-2 CDF errors for relative positioning in InF-SH scenario-ranging accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 35, X=10m, BW=40M | 0.3471 | 0.5429 | 0.8562 | 1.304 | No 80% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 36, X=20m, BW=40M | 0.3692 | 0.5666 | 0.8597 | 1.271 | No 80% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 37, X=30m, BW=40M | 0.3548 | 0.568 | 0.8629 | 1.295 | No 80% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 38, X=50m, BW=40M | 0.3572 | 0.5733 | 0.8861 | 1.481 | No 80% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 39, X=10m, BW=20M | 0.7925 | 1.2 | 1.81 | 2.536 | No 50% of UEs satisfying the requirement of Set A | No Less than 50% of UEs satisfying the requirement of Set B |
| Case 40, X=10m, BW=100M | 0.1741 | 0.2776 | 0.3925 | 0.714 | Yes | No 50% of UEs satisfying the requirement of Set B |

### 2.2.4 Positioning accuracy evaluation results for Sidelink Positioning for Public Safety

Figure 2.2.4-1 and Table 2.2.4-1 show the CDF errors of simulation results for absolute positioning in public safety use case. Figure 2.2.4-2 and Table 2.2.4-2 show the CDF errors of simulation results for relative positioning in public safety use case.

Figure 2.2.4-1 Curves of CDF errors for absolute positioning in public safety use case

Table 2.2.4-1 CDF errors for absolute positioning in public safety use case-horizontal accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 41-Joint positioning, BW=100MHz | 0.2579 | 0.336 | 0.4202 | 0.4901 | Yes |
| Case 42-Joint positioning, BW=40MHz | 0.6138 | 0.4337 | 1.515 | 2.421 | No 67% of UEs satisfying the requirement |
| Case 43-Joint positioning, BW=20MHz | 1.757 | 2.441 | 2.802 | 4.883 | No Less than 50% of UEs satisfying the requirement |
| Case 44-SL positioning, BW=100MHz | 0.3222 | 0.4337 | 0.5335 | 0.7002 | Yes |
| Case 45-SL positioning, BW=40MHz | 0.7694 | 1.177 | 1.545 | 2.571 | No 50% of UEs satisfying the requirement |
| Case 46-SL positioning, BW=20MHz | 1.958 | 2.622 | 3.89 | 5.891 | No Less than 50% of UEs satisfying the requirement |

Figure 2.2.4-2 Curves of CDF errors for relative positioning in public safety use case

Table 2.2.4-2 CDF errors for relative positioning in public safety use case-ranging accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 47, X=100m, BW=40M | 0.4947 | 0.8056 | 1.132 | 1.696 | No 67% of UEs satisfying the requirement |
| Case 48, X=50m, BW=40M | 0.4626 | 0.7163 | 1.013 | 1.358 | No 67% of UEs satisfying the requirement |
| Case 49, X=50m, BW=100M | 0.2806 | 0.3966 | 0.5738 | 0.9514 | Yes |
| Case 50, X=50m, BW=20M | 1.003 | 1.65 | 2.306 | 3.506 | No Less than 50% of UEs satisfying the requirement |

### 2.2.5 Positioning accuracy evaluation results for Sidelink Positioning for Commercial

Figure 2.2.5-1 and Table 2.2.5-1 show the CDF errors of simulation results for absolute positioning in commercial use case. Figure 2.2.5-2 and Table 2.2.5-2 show the CDF errors of simulation results for relative positioning in commercial use case.

Figure 2.2.5-1 Curves of CDF errors for absolute positioning in commercial use case

Table 2.2.5-1 CDF errors for absolute positioning in commercial use case-horizontal accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 51-Joint positioning, BW=100MHz | 0.1804 | 0.2366 | 0.3077 | 0.4194 | Yes |
| Case 52-Joint positioning, BW=40MHz | 0.542 | 0.7887 | 1.032 | 1.611 | No 67% of UEs satisfying the requirement |
| Case 53-Joint positioning, BW=20MHz | 0.8807 | 1.303 | 1.924 | 3.138 | No 50% of UEs satisfying the requirement |
| Case 54-SL positioning, BW=100MHz | 0.2097 | 0.2911 | 0.3851 | 0.6686 | Yes |
| Case 55-SL positioning, BW=40MHz | 0.5396 | 0.76 | 1.124 | 1.877 | No 67% of UEs satisfying the requirement |
| Case 56-SL positioning, BW=20MHz | 1.061 | 1.484 | 2.199 | 3.628 | No Less than 50% of UEs satisfying the requirement |

Figure 2.2.5-2 Curves of CDF errors for relative positioning in commercial use case

Table 2.2.5-2 CDF errors for relative positioning in commercial use case-ranging accuracy

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 57, X=10m, BW=40M | 0.3252 | 0.557 | 0.8612 | 1.324 | No 80% of UEs satisfying the requirement |
| Case 58, X=20m, BW=40M | 0.3983 | 0.6607 | 1.092 | 2.305 | No 67% of UEs satisfying the requirement |
| Case 59, X=50m, BW=40M | 0.5044 | 1.016 | 2.42 | 5.85 | No 50% of UEs satisfying the requirement |
| Case 60, X=10m, BW=100M | 0.1622 | 0.2491 | 0.3483 | 0.51 | Yes |
| Case 61, X=10m, BW=20M | 0.6704 | 1.039 | 1.489 | 2.358 | No 50% of UEs satisfying the requirement |

- Summary of Sidelink Positioning Evaluations

  - Evaluation of Bandwidth Requirements to meet Identified Accuracy Requirements

Observation 1: With increasing of bandwidth, the positioning accuracy increases for both absolute positioning and relative positioning/ranging.

- For absolute positioning in highway scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A without bandwidth limitation and satisfy the requirement of Set B with bandwidth 100MHz, the vertical accuracy of joint positioning technique can satisfy the requirement of Set A and Set B with bandwidth 100MHz. For SL only positioning, the horizontal accuracy satisfies the requirement of Set A with bandwidth 100MHz and cannot satisfy the requirement of Set B.

- For relative positioning in highway scenario, the ranging accuracy can satisfy the requirement of Set A without bandwidth limitation and distance X limitation. When X is no larger than 100m and bandwidth is 100MHz, the ranging accuracy can satisfy the requirement of Set B.

- For absolute positioning in urban grid scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A and Set B with bandwidth 100MHz. For SL only positioning, the horizontal accuracy cannot satisfy any of the requirement.

- For relative positioning in urban grid scenario, the ranging accuracy can satisfy the requirement of Set A with distance X<=30m and bandwidth>40MHz, cannot satisfy the requirement of Set B.

- For absolute positioning in InF-SH scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A with bandwidth 100MHz and cannot satisfy the requirement of Set B. For SL only positioning, the horizontal accuracy satisfies the requirement of Set A with bandwidth 100MHz and cannot satisfy the requirement of Set B.

- For relative positioning in InF-SH scenario, the ranging accuracy can satisfy the requirement of Set A when distance X<=10m and bandwidth =100MHz.

- For absolute positioning in public safety use case, the horizontal accuracy of joint positioning technique can satisfy the requirement with bandwidth 100MHz. For SL only positioning, the horizontal accuracy satisfies the requirement with bandwidth 100MHz.

- For relative positioning in public safety use case, the ranging accuracy can satisfy the requirement when distance X<=50m and bandwidth =100MHz.

- For absolute positioning in commercial use case, the horizontal accuracy of joint positioning technique can satisfy the requirement with bandwidth 100MHz. For SL only positioning, the horizontal accuracy satisfies the requirement with bandwidth 100MHz.

- For relative positioning in commercial use case, the ranging accuracy can satisfy the requirement when distance X<=10m and bandwidth =100MHz.

  - Evaluation of Absolute Positioning, Relative Positioning, and Ranging Methods

Observation 2: SL positioning is helpful to increase absolute positioning accuracy

Observation 3: Compared with SL only positioning technique, joint positioning technique combined with Uu measurements and SL measurements can obviously improve positioning accuracy.

- In highway scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A without bandwidth limitation, but SL only positioning can satisfy the requirement of Set A with bandwidth 100MHz.

- In urban grid scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A with bandwidth 100MHz, but SL only positioning cannot satisfy the requirement of Set A with bandwidth 100MHz.

- In InF-SH scenario, the horizontal accuracy of joint positioning technique with bandwidth 100MHz is 0.5778 for 90% of UEs, but that of SL only positioning with bandwidth 100MHz is 0.7894 for 90% of UEs.

- In public safety use case, the horizontal accuracy of joint positioning technique with bandwidth 100MHz is 0.4901 for 90% of UEs, but that of SL only positioning with bandwidth 100MHz is 0.7002 for 90% of UEs.

- In commercial use case, the horizontal accuracy of joint positioning technique with bandwidth 100MHz is 0.4194 for 90% of UEs, but that of SL only positioning with bandwidth 100MHz is 0.6686 for 90% of UEs.

Observation 4: SL positioning facilitates and/or enables relative positioning/ ranging between two UEs.

Observation 5: With increasing of the X value, the relative positioning accuracy decreases.

- In highway scenario, the ranging accuracy with bandwidth 40MHz is 0.5556 for 90% of UEs when X is 50m, but the ranging accuracy with bandwidth 40MHz decreases to 0.7286 for 90% of UEs when X is 300m.

- In urban grid scenario, the ranging accuracy with bandwidth 40MHz is 1.014 for 90% of UEs when X is 20m, but the ranging accuracy with bandwidth 40MHz decreases to 8.547 for 90% of UEs when X is 150m.

- In InF-SH scenario, the ranging accuracy with bandwidth 40MHz is 1.304 for 90% of UEs when X is 10m, but the ranging accuracy with bandwidth 40MHz decreases to 1.481 for 90% of UEs when X is 50m.

- In public safety use case, the ranging accuracy with bandwidth 40MHz is 1.358 for 90% of UEs when X is 50m, but the ranging accuracy with bandwidth 40MHz decreases to 1.696 for 90% of UEs when X is 100m.

- In commercial use case, the ranging accuracy with bandwidth 40MHz is 1.324 for 90% of UEs when X is 10m, but the ranging accuracy with bandwidth 40MHz decreases to 5.85 for 90% of UEs when X is 50m.

- Conclusion

In this contribution, we provide the evaluation results of SL positioning, including absolute positioning and relative positioning, in V2X use case with highway scenario and urban grid scenario, IIOT use case, public safety use case and commercial use case.

Observation 1: With increasing of bandwidth, the positioning accuracy increases for both absolute positioning and relative positioning/ranging.

- For absolute positioning in highway scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A without bandwidth limitation and satisfy the requirement of Set B with bandwidth 100MHz, the vertical accuracy of joint positioning technique can satisfy the requirement of Set A and Set B with bandwidth 100MHz. For SL only positioning, the horizontal accuracy satisfies the requirement of Set A with bandwidth 100MHz and cannot satisfy the requirement of Set B.

- For relative positioning in highway scenario, the ranging accuracy can satisfy the requirement of Set A without bandwidth limitation and distance X limitation. When X is no larger than 100m and bandwidth is 100MHz, the ranging accuracy can satisfy the requirement of Set B.

- For absolute positioning in urban grid scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A and Set B with bandwidth 100MHz. For SL only positioning, the horizontal accuracy cannot satisfy any of the requirement.

- For relative positioning in urban grid scenario, the ranging accuracy can satisfy the requirement of Set A with distance X<=30m and bandwidth>40MHz, cannot satisfy the requirement of Set B.

- For absolute positioning in InF-SH scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A with bandwidth 100MHz and cannot satisfy the requirement of Set B. For SL only positioning, the horizontal accuracy satisfies the requirement of Set A with bandwidth 100MHz and cannot satisfy the requirement of Set B.

- For relative positioning in InF-SH scenario, the ranging accuracy can satisfy the requirement of Set A when distance X<=10m and bandwidth =100MHz.

- For absolute positioning in public safety use case, the horizontal accuracy of joint positioning technique can satisfy the requirement with bandwidth 100MHz. For SL only positioning, the horizontal accuracy satisfies the requirement with bandwidth 100MHz.

- For relative positioning in public safety use case, the ranging accuracy can satisfy the requirement when distance X<=50m and bandwidth =100MHz.

- For absolute positioning in commercial use case, the horizontal accuracy of joint positioning technique can satisfy the requirement with bandwidth 100MHz. For SL only positioning, the horizontal accuracy satisfies the requirement with bandwidth 100MHz.

- For relative positioning in commercial use case, the ranging accuracy can satisfy the requirement when distance X<=10m and bandwidth =100MHz.

Observation 2: SL positioning is helpful to increase absolute positioning accuracy

Observation 3: Compared with SL only positioning technique, joint positioning technique combined with Uu measurements and SL measurements can obviously improve positioning accuracy.

- In highway scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A without bandwidth limitation, but SL only positioning can satisfy the requirement of Set A with bandwidth 100MHz.

- In urban grid scenario, the horizontal accuracy of joint positioning technique can satisfy the requirement of Set A with bandwidth 100MHz, but SL only positioning cannot satisfy the requirement of Set A with bandwidth 100MHz.

- In InF-SH scenario, the horizontal accuracy of joint positioning technique with bandwidth 100MHz is 0.5778 for 90% of UEs, but that of SL only positioning with bandwidth 100MHz is 0.7894 for 90% of UEs.

- In public safety use case, the horizontal accuracy of joint positioning technique with bandwidth 100MHz is 0.4901 for 90% of UEs, but that of SL only positioning with bandwidth 100MHz is 0.7002 for 90% of UEs.

- In commercial use case, the horizontal accuracy of joint positioning technique with bandwidth 100MHz is 0.4194 for 90% of UEs, but that of SL only positioning with bandwidth 100MHz is 0.6686 for 90% of UEs.

Observation 4: SL positioning facilitates and/or enables relative positioning/ ranging between two UEs.

Observation 5: With increasing of the X value, the relative positioning accuracy decreases.

- In highway scenario, the ranging accuracy with bandwidth 40MHz is 0.5556 for 90% of UEs when X is 50m, but the ranging accuracy with bandwidth 40MHz decreases to 0.7286 for 90% of UEs when X is 300m.

- In urban grid scenario, the ranging accuracy with bandwidth 40MHz is 1.014 for 90% of UEs when X is 20m, but the ranging accuracy with bandwidth 40MHz decreases to 8.547 for 90% of UEs when X is 150m.

- In InF-SH scenario, the ranging accuracy with bandwidth 40MHz is 1.304 for 90% of UEs when X is 10m, but the ranging accuracy with bandwidth 40MHz decreases to 1.481 for 90% of UEs when X is 50m.

- In public safety use case, the ranging accuracy with bandwidth 40MHz is 1.358 for 90% of UEs when X is 50m, but the ranging accuracy with bandwidth 40MHz decreases to 1.696 for 90% of UEs when X is 100m.

- In commercial use case, the ranging accuracy with bandwidth 40MHz is 1.324 for 90% of UEs when X is 10m, but the ranging accuracy with bandwidth 40MHz decreases to 5.85 for 90% of UEs when X is 50m.

- Reference

- RP-213588 Revised SID on Study on expanded and improved NR positioning, RAN#94e.

- 3GPP TR 36.885-e00. “Study on LTE-based V2X services (Release 14)”

- 3GPP TR 38.857. “Study on NR Positioning Enhancements”.

- 3GPP TR 36.843. “Study on LTE Device to Device Proximity Services”.

- 3GPP TR 38.901. “Study on channel model for frequencies from 0.5 to 100 GHz”..