3GPP TSG RAN WG1 #110bis-e                                            R1-2209217

e-Meeting, October 10th –19th, 2022

Source:         ZTE

Title:            Discussion on Positioning for RedCap UE

Agenda item:    9.5.3

Document for:	Discussion and Decision

Introduction

In RAN#94e meeting, the study item on positioning for RedCap UEs was approved [1]. As shown in the SID, the two key research directions for RedCap UE positioning is: performance evaluation under agreed scenarios and investigating enhancements for positioning of RedCap UEs if necessary.

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we mainly provide our evaluation assumptions and results including the assessment of frequency hopping on positioning for RedCap UE.

Potential Solutions for Positioning for RedCap UEs

2.1 Frequency hopping

The MIMO SRS frequency hopping has widely been applied to achieve power boosting and full frequency channel information (agreed in LTE, Rel-8 and NR Rel-15). But the frequency hopping of MIMO SRS is a hopping within single BWP. Similar to the idea above, a frequency hopping of a reference signal for positioning can extend the bandwidth of the reference signal for positioning as shown in the following Figure 2.1-1. By concatenating multiple hops of reference signal for positioning, the effective bandwidth of reference signal for positioning can be enlarged.

Figure 2.1-1 Frequency hopping of a reference signal for positioning

For each hop, because the RF chain (of a RedCap UE) should be tuned to another center frequency, a random phase between each hop may be introduced. Before concatenating these hops, this phase impact should be mitigated. In RAN1#110[2], the following agreement about resource frequency hopping has been made, companies are encouraged to investigate potential benefits and performance gains of frequency hopping. To verify the above analysis, some preliminary results for evaluating the influence of Doppler, phase offset and timing offset among hops are provided in section 3.

Agreement The potential benefits and performance gains of frequency hopping of the DL PRS and UL SRS can be investigated in release 18, which may take into account at least the following: - The impact of Doppler, phase offset, timing offset, power imbalance among hops - RedCap UE capability and complexity considerations - Impact of RF retuning during frequency hopping - Details of frequency hopping (including Tx hopping and/or Rx hopping, BWP switching) for the study are FFS

### 2.1.1 RS configuration for hopping

For positioning RS hopping for RedCap UE, we think there are several candidate solutions in term of specification enhancement as listed as follows:

- Frequency hopping within one resource

In this way, the frequency domain resource of one reference signal is divided into several parts, each part corresponds to a frequency hop, and several hops are received in different symbols with a combination as a whole (as shown in Figure 2.1-2). In other words, the three hopping parts refer to different symbols of one RS resource.

Figure 2.1-2. Example of frequency hopping within one resource

- Frequency hopping within one resource set

In this way, assuming that one reference signal resource set contains several reference signal resources, then different reference signal resources may have different frequency range and correspond to different hops (as shown in Figure 2.1-3). Similarly, the received signals from different resources are concatenated for positioning related processing.

Figure 2.1-3. Example of frequency hopping within one resource set

- Frequency hopping between different CCs/BWPs/PFLs/resource sets

- In this way, several reference signal resources belonging to different CCs/BWPs/PFLs/resource sets can be associated to compose frequency hopping. One example is, different reference signal resources with the same ID in different CCs/BWPs/PFLs correspond to different hops (as shown in Figure 2.1-4).

Figure 2.1-4. Example of frequency hopping between different CCs

Proposal 1: For RedCap UE, further study the following candidate solutions of RS resource frequency hopping:

- Frequency hopping within one resource

- Frequency hopping within one resource set

- Frequency hopping between different CCs/BWPs/PFLs/resource sets

### 2.1.2 Frequency overlapping between two frequency hops

Timing error or random phase rotation between two frequency hops may be caused by UE RF re-tuning. This kind of timing error or phase rotation is usually symbol basis. For instance, as shown in Figure 2.1-5, compared with symbol #0 and #1, there may be a phase rotation added in the channel response in symbol #2 and #3. The phase rotation is basically constant across all REs or subcarriers within single symbol.

One enhanced solution is to introduce predefined or configured overlapping frequency resources in adjacent frequency hops, then, phase between each hop can be correctly adjusted and the concatenated signal can be treated as a signal with extended bandwidth.

Proposal 2: For RedCap UE positioning RS, introduce partial overlapping in frequency for adjacent hops.

K PRBs

Figure 2.1-5 Frequency hopping with overlapping

### 2.1.3 Switching gap between two frequency hops

As Rel-17 RedCap UE only supports maximum 20/100 MHz bandwidth in FR1/FR2 respectively, UE has to operate RF re-tuning between two frequency hops. UE may need some switching gap between two frequency hops because of extra complexity from RF re-tuning.

Proposal 3: For RedCap UE positioning, further introduce switching gap between adjacent frequency hops. FFS the gap values.

Figure 2.1-6 Frequency hopping with time gap

Evaluation Results of RedCap UEs Positioning for TR 38.859 Section B.6

3.1 Description of evaluation scenarios

Based on the agreements in RAN1#109e and RAN1#110 meeting, RedCap UE positioning should be evaluated in IIOT use case and commercial use case. The corresponding evaluation methodology is agreed and captured in TR 38.859 [3]. IIOT use case with indoor factory scenario defined in TR 38.857 [4] is used to evaluate. As a typical scenario of InF, InF-SH is evaluated in this contribution.

Evaluation assumptions for system level analysis are provided in Table 3.1-1 to Table 3.1-5.

Table 3.1-1: NR RedCap UE positioning - evaluation scenarios and parameters for FR1 InF-SH (for comparing frequency hopping performance)

| Parameter | Case 1 (InF-SH, FR1, 20MHz Only) | Case 2 (InF-SH, FR1, 3x20 MHz, without Φ adjustment) | Case 3 (InF-SH, FR1, 3x20 MHz, with Φ adjustment) | Case 4 (InF-SH, FR1, Full 60MHz | Case 5 (InF-SH, FR1, 5x20 MHz, without Φ adjustment) | Case 6 (InF-SH, FR1, 5x20 MHz, with Φ adjustment) | Case 7 (InF-SH, FR1, Full 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | InF-SH |  |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |  |
| Subcarrier spacing | 30kHz |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 20MHz | 3x20MHz |  | 60MHz | 5x20MHz |  | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | CombSize=2 |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | DL PRS |  |  |  |  |  |  |
| Number of sites | 18 |  |  |  |  |  |  |
| Number of symbols used per occasion | 2 |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |
| Power-boosting level | Not applied |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g., super resolution, interference cancellation, ….) | MUSIC |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g., Least square, Taylor series, etc) | Least square |  |  |  |  |  |  |
| Network synchronization assumptions | Perfect synchronization |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |  |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) |  |  |  |  |  |  |
| Number of UE branches | 1Rx 1Tx |  |  |  |  |  |  |
| Description of enhancement solutions, if any | None | Frequency hopping, 3 hops without phase adjustment; Phase between hops satisfy uniform random, (-π, +π) | Frequency hopping, 3 hops with phase adjustment; Phase between hops satisfy uniform random, (-π, +π); 4PRB used for practical phase estimation | None | Frequency hopping, 5 hops without phase adjustment; Phase between hops satisfy uniform random, (-π, +π) | Frequency hopping, 5 hops with phase adjustment; Phase between hops satisfy uniform random, (-π, +π); 4PRB used for practical phase estimation | None |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |  |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |  |
| gNB antenna height | 8m |  |  |  |  |  |  |

Table 3.1-2: NR RedCap UE positioning - evaluation scenarios and parameters for FR1 InF-SH (Impact of Doppler)

| Parameter | Case 8 (InF-SH, FR1, 5x20 MHz, Doppler 0 Hz) | Case 9 (InF-SH, FR1, 5x20 MHz, Doppler 10 Hz) | Case 10 (InF-SH, FR1, 5x20 MHz, Doppler 100 Hz) | Case 11 (InF-SH, FR1, 5x20 MHz, Doppler 200 Hz) | Case 12 (InF-SH, FR1, 5x20 MHz, Doppler 500 Hz) | Case 13 (InF-SH, FR1, 5x20 MHz, Doppler 1000 Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | InF-SH |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |
| Subcarrier spacing | 15kHz |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5x20MHz |  |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | CombSize=2 |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | DL PRS |  |  |  |  |  |
| Number of sites | 18 |  |  |  |  |  |
| Number of symbols used per occasion | 2 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | Not applied |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g., super resolution, interference cancellation, ….) | MUSIC |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g., Least square, Taylor series, etc) | Least square |  |  |  |  |  |
| Network synchronization assumptions | Perfect synchronization |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) |  |  |  |  |  |
| Number of UE branches | 1Rx 1Tx |  |  |  |  |  |
| Description of enhancement solutions, if any | Frequency hopping |  |  |  |  |  |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |
| gNB antenna height | 8m |  |  |  |  |  |

Table 3.1-3: NR RedCap UE positioning - evaluation scenarios and parameters for FR1 InF-SH (Impact of phase offset)

| Parameter | Case 14 (InF-SH, FR1, 5x20 MHz, Phase offset 0.01*2π) | Case 15 (InF-SH, FR1, 5x20 MHz, Phase offset 0.05*2π) | Case 16 (InF-SH, FR1, 5x20 MHz, Phase offset 0.1*2π) | Case 17 (InF-SH, FR1, 5x20 MHz, Phase offset 0.2*2π) | Case 18 (InF-SH, FR1, 5x20 MHz, Phase offset 0.5*2π) | Case 19 (InF-SH, FR1, 5x20 MHz, Phase offset 1*2π) |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | InF-SH |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |
| Subcarrier spacing | 15kHz |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5x20MHz |  |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | CombSize=2 |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | DL PRS |  |  |  |  |  |
| Number of sites | 18 |  |  |  |  |  |
| Number of symbols used per occasion | 2 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | Not applied |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g., super resolution, interference cancellation, ….) | MUSIC |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g., Least square, Taylor series, etc) | Least square |  |  |  |  |  |
| Network synchronization assumptions | Perfect synchronization |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) |  |  |  |  |  |
| Number of UE branches | 1Rx 1Tx |  |  |  |  |  |
| Description of enhancement solutions, if any | Frequency hopping Random phase offset |  |  |  |  |  |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |
| gNB antenna height | 8m |  |  |  |  |  |

Table 3.1-4: NR RedCap UE positioning - evaluation scenarios and parameters for FR1 InF-SH (Impact of timing offset)

| Parameter | Case 20 (InF-SH, FR1, 5x20 MHz, STD=0) | Case 21 (InF-SH, FR1, 5x20 MHz, STD=0.5 ns) | Case 22 (InF-SH, FR1, 5x20 MHz, STD=1 ns) | Case 23 (InF-SH, FR1, 5x20 MHz, STD=2 ns) | Case 24 (InF-SH, FR1, 5x20 MHz, STD=3 ns) | Case 25 (InF-SH, FR1, 5x20 MHz, STD=4 ns) |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | InF-SH |  |  |  |  |  |
| Carrier frequency | 3.5GHz |  |  |  |  |  |
| Subcarrier spacing | 15kHz |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 5x20MHz |  |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | CombSize=2 |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | DL PRS |  |  |  |  |  |
| Number of sites | 18 |  |  |  |  |  |
| Number of symbols used per occasion | 2 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | Not applied |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g., super resolution, interference cancellation, ….) | MUSIC |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g., Least square, Taylor series, etc) | Least square |  |  |  |  |  |
| Network synchronization assumptions | Perfect synchronization |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) |  |  |  |  |  |
| Number of UE branches | 1Rx 1Tx |  |  |  |  |  |
| Description of enhancement solutions, if any | Frequency hopping Random timing offset |  |  |  |  |  |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |  |  |  |  |  |
| UE noise figure | 9dB |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |
| gNB antenna height | 8m |  |  |  |  |  |

Table 3.1-5: NR RedCap UE positioning - evaluation scenarios and parameters for FR2 InF-SH

| Parameter | Case 26 (InF-SH, FR2, 100MHz Only) | Case 27 (InF-SH, FR2, 100+100 MHz, with Φ adjustment |  | Case 28 (InF-SH, FR2, Full 200MHz) |  | Case 29 (InF-SH, FR2, 100+100+100 MHz, with Φ adjustment) |  | Case 30 (InF-SH, FR2, Full 300MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | InF-SH |  |  |  |  |  |  |  |
| Carrier frequency | 28 GHz |  |  |  |  |  |  |  |
| Subcarrier spacing | 120kHz |  |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 100MHz |  | 2 x 100 MHz |  | 200 MHz |  | 3 x 100 MHz | 300 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | CombSize=2 |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | DL PRS |  |  |  |  |  |  |  |
| Number of sites | 18 |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 2 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | Not applied |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g., super resolution, interference cancellation, ….) | MUSIC |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g., Least square, Taylor series, etc) | Least square |  |  |  |  |  |  |  |
| Network synchronization assumptions | Perfect synchronization |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |  |  |  |  |  |  |  |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |  |  |  |  |  |  |  |
| Number of UE branches | 2Rx 1Tx |  |  |  |  |  |  |  |
| Description of enhancement solutions, if any | None | Frequency hopping, 3 hops with phase adjustment; Phase between hops satisfy uniform random, (-π, +π); 4PRB used for practical phase estimation |  | None |  | Frequency hopping, 3 hops with phase adjustment; Phase between hops satisfy uniform random, (-π, +π); 4PRB used for practical phase estimation |  | None |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ One TXRU per polarization per panel is assumed |  |  |  |  |  |  |  |
| UE noise figure | 13dB |  |  |  |  |  |  |  |
| UE antenna height | 1.5m |  |  |  |  |  |  |  |
| gNB antenna height | 8m |  |  |  |  |  |  |  |

3.2 NR RedCap UE positioning accuracy evaluation results Evaluation results

The simulation results for FR1 in InF-SH scenario for comparing with and without frequency hopping are shown in the following Figure 3.2-1, Figure 3.2-1 and Table 3.2.2.

Figure 3.2-1 Simulation results for hopping for RedCap UE (3 hops, 60MHz)

Figure 3.2-2 Simulation results for hopping for RedCap UE (5 hops, 100MHz)

Table 3.2-1 Simulation results for hopping in FR 1, InF-SH scenario (UE Antenna = [1,1,1,1,1])

| Cases |  | 50% | 67% | 80% | 90% | Requirements met? (Yes/No) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1, InF-SH FR1, 20MHz Only | Convex UEs | 1.24 | 1.55 | 1.86 | 2.60 | No |
| Case 2, InF-SH FR1, 20+20+20 MHz, without Φ adjustment | Convex UEs | 1.56 | 2.01 | 2.45 | 3.05 | No |
| Case 3, InF-SH FR1, 20+20+20 MHz, with Φ adjustment | Convex UEs | 0.72 | 1.07 | 1.35 | 1.77 | No |
| Case 4, InF-SH FR1, Full 60MHz | Convex UEs | 0.61 | 0.83 | 1.17 | 1.79 | No |
| Case 5, InF-SH FR1, 5x20 MHz, without Φ adjustment | Convex UEs | 2.14 | 2.89 | 3.77 | 4.98 | No |
| Case 6, InF-SH FR1, 5x20 MHz, with Φ adjustment | Convex UEs | 0.27 | 0.37 | 0.45 | 0.57 | Yes |
| Case 7, InF-SH FR1, Full 100MHz | Convex UEs | 0.17 | 0.24 | 0.32 | 0.42 | Yes |

For the evaluation of Doppler/CFO impact:

The evaluation results for FR1 in InF-SH scenario for assessing Doppler impact are shown in the following Figure 3.2-3 and Table 3.2.2.

Figure 3.2-3 Simulation results for hopping for RedCap UE with Doppler impact (FR1, InF-SH scenario)

Table 3.2-2 Simulation results for hopping for RedCap UE with Doppler impact in FR 1, InF-SH scenario

| Cases |  | 50% | 67% | 80% | 90% | Requirements met? (Yes/No) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 8, InF-SH FR1, 5x20 MHz, Doppler 0 Hz | Convex UEs | 0.179 | 0.238 | 0.325 | 0.428 | Yes |
| Case 9, InF-SH FR1, 5x20 MHz, Doppler 10 Hz | Convex UEs | 0.195 | 0.257 | 0.347 | 0.429 | Yes |
| Case 10, InF-SH FR1, 5x20 MHz, Doppler 100 Hz | Convex UEs | 0.195 | 0.269 | 0.336 | 0.427 | Yes |
| Case 11, InF-SH FR1, 5x20 MHz, Doppler 200Hz | Convex UEs | 0.184 | 0.258 | 0.296 | 0.466 | Yes |
| Case 12, InF-SH FR1, 5x20 MHz, Doppler 500Hz | Convex UEs | 0.226 | 0.277 | 0.404 | 0.523 | Yes |
| Case 13, InF-SH FR1, 5x20 MHz, Doppler 1000Hz | Convex UEs | 0.407 | 0.526 | 0.644 | 0.814 | Yes |

For the evaluation of phase offset impact:

The evaluation results for FR1 in InF-SH scenario for assessing phase offset impact are shown in the following Figure 3.2-4 and Table 3.2.3.

Figure 3.2-4 Simulation results for hopping for RedCap UE with phase error (FR 1, InF-SH scenario)

Table 3.2-3 Simulation results for hopping for RedCap UE with phase error in FR 1, InF-SH scenario

| Cases |  | 50% | 67% | 80% | 90% | Requirements met? (Yes/No) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 14, InF-SH FR1, 5x20 MHz, Phase offset 0.01*2π | Convex UEs | 0.180 | 0.249 | 0.347 | 0.452 | Yes |
| Case 15, InF-SH FR1, 5x20 MHz, Phase offset 0.05*2π | Convex UEs | 0.198 | 0.273 | 0.361 | 0.441 | Yes |
| Case 16, InF-SH FR1, 5x20 MHz, Phase offset 0.1*2π | Convex UEs | 0.262 | 0.323 | 0.440 | 0.567 | Yes |
| Case 17, InF-SH FR1, 5x20 MHz, Phase offset 0.2*2π | Convex UEs | 0.422 | 0.563 | 0.677 | 0.968 | Yes |
| Case 18, InF-SH FR1, 5x20 MHz, Phase offset 0.5*2π | Convex UEs | 0.915 | 1.386 | 1.784 | 2.324 | No |
| Case 19, InF-SH FR1, 5x20 MHz, Phase offset 1*2π | Convex UEs | 2.145 | 2.874 | 3.769 | 4.981 | No |

For the evaluation of timing offset impact:

The evaluation results for FR1 in InF-SH scenario for assessing timing offset impact are shown in the following Figure 3.2-5 and Table 3.2.4.

Figure 3.2-5 Simulation results for hopping for RedCap UE with timing error

Table 3.2-4 Simulation results for hopping for RedCap UE with timing error in FR 1, InF-SH scenario

| Cases |  | 50% | 67% | 80% | 90% | Requirements met? (Yes/No) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 20, InF-SH FR1, 5x20 MHz, STD=0 | Convex UEs | 0.179 | 0.238 | 0.325 | 0.428 | Yes |
| Case 21, InF-SH FR1, 5x20 MHz, STD=0.5 ns | Convex UEs | 0.205 | 0.273 | 0.333 | 0.497 | Yes |
| Case 22, InF-SH FR1, 5x20 MHz, STD=1 ns | Convex UEs | 0.254 | 0.327 | 0.465 | 0.558 | Yes |
| Case 23, InF-SH FR1, 5x20 MHz, STD=2 ns | Convex UEs | 0.375 | 0.497 | 0.618 | 0.873 | Yes |
| Case 24, InF-SH FR1, 5x20 MHz, STD=3 ns | Convex UEs | 0.496 | 0.659 | 0.910 | 1.146 | No |
| Case 25, InF-SH FR1, 5x20 MHz, STD=4 ns | Convex UEs | 0.645 | 0.832 | 1.106 | 1.336 | No |

For the evaluation of FR2 InF-SH:

The simulation results for FR2 in InF-SH scenario are shown in the following Figure 3.2-6, Figure 3.2-7 and Table 3.2-5.

Figure 3.2-6 Simulation results for hopping for RedCap UE (2 hops, 200MHz)

Figure 3.2-7 Simulation results for hopping for RedCap UE (3 hops, 300MHz)

Table 3.2-5 Simulation results for hopping in FR 2, InF-SH scenario

| Cases |  | 50% | 67% | 80% | 90% | Requirements met? (Yes/No) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 26, InF-SH FR2,100MHz Only | Convex UEs | 0.26 | 0.35 | 0.41 | 0.50 | Yes |
| Case 27, InF-SH FR2, 100+100 MHz, with Φ adjustment | Convex UEs | 0.11 | 0.13 | 0.16 | 0.186 | Yes |
| Case 28, InF-SH FR2, Full 200MHz | Convex UEs | 0.092 | 0.12 | 0.15 | 0.185 | Yes |
| Case 29, InF-SH FR2,100+100+100 MHz, with Φ adjustment | Convex UEs | 0.065 | 0.088 | 0.10 | 0.12 | Yes |
| Case 30, InF-SH FR2,Full 300MHz | Convex UEs | 0.054 | 0.069 | 0.088 | 0.11 | Yes |

Summary of Evaluations for Positioning for RedCap UEs

For FR1 frequency hopping in InF-SH:

To check the positioning performance of a RedCap UE, some simulations are performed in InF-SH scenario for FR1. From these results, it can be observed that, the positioning accuracy in FR1 for a RedCap UE (with a bandwidth of 20MHz) is about 2.6 m for 90% UEs which can not satisfy the target positioning requirements.

Therefore, we have the following observation.

Observation 1: For a Rel-17 RedCap UE in FR1 and InF-SH scenario, the positioning performance is insufficient because of limited bandwidth.

From simulation results shown in Figure 3.2-1, Figure 3.2-1 and Table 3.2.2, it can be seen that, with multiple frequency hopping, the positioning accuracy can be significantly improved if the phase between each hop can be correctly adjusted. It can also be observed that, if the phase between each hop cannot be correctly adjusted, then the concatenated signal will be damaged by the random phase between each hop. It will even be worse than a single hop. Hence, we have the following observations.

Observation 2: For RedCap UE positioning in FR 1, InF-SH scenario

- The random phase between hops will damage the positioning performance if it was not adjusted.

- PRS frequency hopping can improve positioning performance if the random phase between hops can be adjusted.

For the evaluation of Doppler impact:

Base on our evaluation results, all cases for evaluating Doppler impact can meet the positioning requirement for RedCap UE. Therefore, we have the following observation.

Observation 3: The Doppler shift has little effect on positioning accuracy of RS hopping for RedCap UE under InF-SH scenario in FR1.

For the evaluation of phase offset impact:

When the random phase offset is set to 0.5*2π or larger than 0.5*2π (in STD of standard normal distribution), the positioning accuracy can not satisfy the agreed requirements, we have the following observation

Observation 4: For the case with 5 RS hops in FR1, if the phase offset between hops were not adjusted, it will affect positioning accuracy of hopping for RedCap UE.

- If the random phase offset is set to 0.5*2π or larger than 0.5*2π (in STD), the positioning accuracy requirement can not be satisfied. Otherwise, the requirement can still be met.

For the evaluation of timing offset impact:

Base on our evaluation results, time offset between hops will decrease positioning accuracy of hopping for RedCap UE. Especially when STD (of standard normal distribution) is equal to or larger than 3ns, the positioning accuracy requirement for RedCap UE can not be met.

Observation 5: For the case with 5 RS hops in FR1, the time offset between hops will decrease positioning accuracy of hopping for RedCap UE.

- When the random timing offset is equal to or larger than 3ns (in STD), the positioning accuracy requirement can not be met. Otherwise, the requirement can still be met.

For the evaluation of FR2 InF-SH:

Base on our evaluation results, all cases (case 26 to case 30) for evaluating FR2 can meet the positioning requirement for RedCap UE. Similar to that of FR1, we have the following observations in FR2.

Observation 6: For RedCap UE positioning in FR 2, InF-SH scenario, PRS frequency hopping can improve positioning performance if the random phase between hops can be adjusted.

Conclusion

In this contribution, we provide our views on positioning support for RedCap UE techniques as the following observations and proposals.

Observation 1: For a Rel-17 RedCap UE in FR1 and InF-SH scenario, the positioning performance is insufficient because of limited bandwidth.

Observation 2: For RedCap UE positioning in FR 1, InF-SH scenario

- The random phase between hops will damage the positioning performance if it was not adjusted.

- PRS frequency hopping can improve positioning performance if the random phase between hops can be adjusted.

Observation 3: The Doppler shift has little effect on positioning accuracy of RS hopping for RedCap UE under InF-SH scenario in FR1.

Observation 4: For the case with 5 RS hops in FR1, if the phase offset between hops were not adjusted, it will affect positioning accuracy of hopping for RedCap UE.

- When If the random phase offset is set to 0.5*2π or larger than 0.5*2π (in STD), the positioning accuracy requirement can not be satisfied. Otherwise, the requirement can still be met.

Observation 5: For the case with 5 RS hops in FR1, the time offset between hops will decrease positioning accuracy of hopping for RedCap UE.

- When the random timing offset is equal to or larger than 3ns (in STD), the positioning accuracy requirement can not be met. Otherwise, the requirement can still be met.

Observation 6: For RedCap UE positioning in FR 2, InF-SH scenario, PRS frequency hopping can improve positioning performance if the random phase between hops can be adjusted.

Proposal 1: For RedCap UE, further study the following candidate solutions of RS resource frequency hopping:

- Frequency hopping within one resource

- Frequency hopping within one resource set

- Frequency hopping between different CCs/BWPs/PFLs/resource sets

Proposal 2: For RedCap UE positioning RS, introduce partial overlapping in frequency for adjacent hops.

Proposal 3: For RedCap UE positioning, further introduce switching gap between adjacent frequency hops. FFS the gap values.

Reference

- RP-213588 Revised SID on Study on expanded and improved NR positioning, RAN#94e.

- Draft_Minutes_report_RAN1#110_v020, 3GPP RAN1#110

- 3GPP TR 38.859. “Study on Expanded and Improved NR Positioning; (Release 18)”.

- 3GPP TR 38.857. “Study on NR Positioning Enhancements”