3GPP TSG RAN WG1 Meeting #110b-e	R1-2209227

e-Meeting, October 10th – 19th, 2022

Agenda item:	8.8

Source: 	Lenovo

Title: 	Remaining issues on enhancement on PUSCH repetition type A

Document for:	Discussion

Introduction

According to the discussion in last meeting [1], there is still one remaining issue for enhancement on PUSCH repetition type A needed to be solved. We provide our views on this remaining issue in this contribution.

Discussion

## 2.1 Issue: Restriction on K2 offset for DG-PUSCH with K>1 and Out-of-order handling when AvailableSlotCounting is enabled

In RAN1#109-e, whether to have the restriction on K2 offset for DG-PUSCH with K>1 when AvailableSlotCounting is enabled was actively discussed with the following options [2].

For DG-PUSCH with K>1 when AvailableSlotCounting is enabled, - Alt 1: No restriction on the K2 offset. The reference of Out-of-order handling is PUSCH in the slot indicated by the K2 offset. - Alt 2: No restriction on the K2 offset. The reference of Out-of-order handling is PUSCH in the first available slot. - Alt 3: The K2 offset is restricted such that the slot indicated by the K2 offset is an available slot. - Note: In any case, Out-or-order check is done before PUSCH dropping.

During the discussion, some chipset vendors think the legacy UE implementation might be based on the assumption that the gNB never choose the K2 offset that indicates the invalid slot for PUSCH transmission, although that is not explicitly prohibited by Rel-15/16 RAN1 specifications. Rel-17 CovEnh UE implementation follows the same principle in terms of the use of the K2 offset. But some network vendors think Rel-15/16 RAN1 specifications do not have the restriction on the network’s choice of the K2 offset value. For Rel-17 CovEnh operation, any additional restriction on the K2 offset value is acceptable.

From our perspective, considering the process and complexity, we think we can use the first scheduled PUSCH as the reference of OfO handling, even if it is not in the available slot. Thus, we think no spec change is adopted.

Proposal: For DG-PUSCH with K>1 when AvailableSlotCounting is enabled, there is no consensus on whether the K2 offset is restricted to always indicate an available slot or not. No spec change is adopted in this regard.

Conclusion

In summary, we propose the followings for remaining issues on enhancement on PUSCH repetition type A

Proposal: For DG-PUSCH with K>1 when AvailableSlotCounting is enabled, there is no consensus on whether the K2 offset is restricted to always indicate an available slot or not. No spec change is adopted in this regard.

References

- 3GPP R1-2208188, “FL summary #2 for Rel-17 CovEnh maintenance: Enhancements on PUSCH repetition type A”, Moderator (China Telecom), RAN4#110, August 22nd – 26th, 2022.

- R1-2205421	“Summary on [109-e-R17_CovEnh-01] for maintenance on PUSCH enhancement (Enhancements on PUSCH repetition type A)”	Moderator (Sharp)