3GPP TSG RAN WG1 #110bis-e							R1-2209241

e-Meeting, October 10th – 19th, 2022

Source:         ZTE

Title:            Discussion on some remaining issues of Rel-17 UE features

Agenda item:    8.16.2

Document for:   Discussion and Decision

- Introduction

Based on the discussion in previous meetings, an updated UE feature list has been endorsed for NR [1]. In this contribution, we provide our views on some remaining issues for Rel-17 NR UE features.

- UE feature for NR above 52.6GHz

- New FGs: Multiple PDSCH/PUSCH scheduling by single DCI

In RAN1 #108-e meeting, the extending multiple PDSCH/PUSCH scheduling by single DCI to other SCSs has been captured in the note of the following agreement [2]. Wherein, multiple PDSCH/PUSCH scheduling by single DCI have been supported for 120/480/960 kHz in FR2-2 and 120 kHz in FR2-1.

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements [for both type 1 and type 2 HARQ codebook] for supporting multi-PDSCH scheduling with singe DCI | 24-1 | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A | FFS: to extend this FG to other frequency ranges | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1f | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements [for both type 1 and type 2 HARQ codebook] for supporting multi-PDSCH scheduling with singe DCI |  | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Continue discussion on extending 24-1f to other SCSs

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A | FFS: to extend this FG to other frequency ranges | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Continue discussion on extending 24-1g to other SCSs

In RAN1 #109-e meeting, extending multiple PDSCH/PUSCH scheduling by single DCI to 60 kHz in FR2-1 and 15/30/60 kHz in FR1 was discussed [3] and the possible proposal is as follows. However, it was unfortunate that no consensus was reached on this feature within the last limited time. In RAN1 #110-e meeting, this feature has not been discussed.

Possible Proposal: Introduce the following new FGs

| 24. NR_ext_to_71GHz | 24-1h | Multiple PDSCH scheduling by single DCI for 60kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 60 kHz SCSs in FR2-1 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | Yes | N/A | Multiple PDSCH scheduling by single DCI for 15/30/60kHz is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1i | Multiple PDSCH scheduling by single DCI for for 15/30/60kHz in FR1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 15/30/60 kHz SCSs in FR1 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | Yes | N/A | Multiple PDSCH scheduling by single DCI for 15/30/60kHz is not supported in FR1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1j | Multiple PUSCH scheduling by single DCI for 60kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 60 kHz SCSs with non-contiguous allocation in FR2-1 | Yes | N/A | Multiple PUSCH scheduling by single DCI for 15/30/60kHz is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1k | Multiple PUSCH scheduling by single DCI for 15/30/60kHz in FR1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 15/30/60 kHz SCSs with non-contiguous allocation in FR1 | Yes | N/A | Multiple PUSCH scheduling by single DCI for 15/30/60kHz is not supported in FR1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

In this meeting, it is necessary to further discuss applicability of this feature and agree extending it to other SCSs (e.g., 60 kHz in FR2-1 and 15/30/60 kHz in FR1) considering that it is band-agnostic and beneficial to degrade the overhead of DCI signalling. Given that, we recommend extending the applicability of this feature to 60 kHz in FR2-1 and 15/30/60 kHz in FR1 and no differentiation licensed and unlicensed spectrum.

Proposal 1: It is recommended to extend the applicability of multiple PDSCH/PUSCH scheduling by single DCI to 60 kHz in FR2-1 and 15/30/60 kHz in FR1.

Proposal 2: Adopt the following new FGs on multiple PDSCH/PUSCH scheduling by single DCI:

| 24. NR_ext_to_71GHz | 24-1h | Multiple PDSCH scheduling by single DCI for 60kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 60 kHz SCSs in FR2-1 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | Yes | N/A | Multiple PDSCH scheduling by single DCI for 15/30/60kHz is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1i | Multiple PDSCH scheduling by single DCI for for 15/30/60kHz in FR1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 15/30/60 kHz SCSs in FR1 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | Yes | N/A | Multiple PDSCH scheduling by single DCI for 15/30/60kHz is not supported in FR1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1j | Multiple PUSCH scheduling by single DCI for 60kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 60 kHz SCSs with non-contiguous allocation in FR2-1 | Yes | N/A | Multiple PUSCH scheduling by single DCI for 15/30/60kHz is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1k | Multiple PUSCH scheduling by single DCI for 15/30/60kHz in FR1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 15/30/60 kHz SCSs with non-contiguous allocation in FR1 | Yes | N/A | Multiple PUSCH scheduling by single DCI for 15/30/60kHz is not supported in FR1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Conclusions

In this contribution, we discuss some remaining issues for Rel-17 NR UE features, the following proposals are suggested.

For NR above 52.6GHz

Proposal 1: It is recommended to extend the applicability of multiple PDSCH/PUSCH scheduling by single DCI to 60 kHz in FR2-1 and 15/30/60 kHz in FR1.

Proposal 2: Adopt the following new FGs on multiple PDSCH/PUSCH scheduling by single DCI:

| 24. NR_ext_to_71GHz | 24-1h | Multiple PDSCH scheduling by single DCI for 60kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 60 kHz SCSs in FR2-1 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | Yes | N/A | Multiple PDSCH scheduling by single DCI for 15/30/60kHz is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1i | Multiple PDSCH scheduling by single DCI for for 15/30/60kHz in FR1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 15/30/60 kHz SCSs in FR1 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | Yes | N/A | Multiple PDSCH scheduling by single DCI for 15/30/60kHz is not supported in FR1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1j | Multiple PUSCH scheduling by single DCI for 60kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 60 kHz SCSs with non-contiguous allocation in FR2-1 | Yes | N/A | Multiple PUSCH scheduling by single DCI for 15/30/60kHz is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1k | Multiple PUSCH scheduling by single DCI for 15/30/60kHz in FR1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 15/30/60 kHz SCSs with non-contiguous allocation in FR1 | Yes | N/A | Multiple PUSCH scheduling by single DCI for 15/30/60kHz is not supported in FR1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- References

- R1-2207923 - Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2202851 Summary of UE features for supporting NR from 52.6 GHz to 71 GHz, RAN1#108-e, Moderator (AT&T)

- R1-2204850 Summary of UE features for supporting NR from 52.6 GHz to 71 GHz , RAN1#109-e, Moderator (AT&T)